| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0353-v3 |
| --- | --- |

| Title: | CE1-related: Reference picture resampling filters |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Name(s) Jiancong Luo Yan Ye Mohammed Golam Sarwer | Tel: Email: | +1 862 596 2086 jiancong.luo@alibaba-inc.com |
| Source: | Alibaba Group Inc. |  |  |

_____________________________

# Abstract

The CE1 test 3 in JVET-O2021 tests the application of 12-tap SHM filter for reference picture down-sampling. In order to avoid increase of complexity and memory access bandwidth, this contribution proposes to use cosine windowed sinc filters with the filter length, gain and number of phases matching those of the existing motion compensation interpolation filters (MCIF) in VVC. The simulation results reportedly show that comparing to the CE1 anchors which use existing MCIF for reference down-sampling, average BD rate savings of PSNR1 are { -5.96%, -2.40%} for ratio 2:1 and 1.5:1, and average BD rate savings of PSNR2 are { X.XX%, X.XX%} for ratio 2:1 and 1.5:1.

# Introduction

The contribution JVET-O0641 [1] proposed to use 12-tap down-sampling filters for reference picture down-sampling. The proposed 12-tap filters, which are the same as the source down-sampling filters in SHM 12.4, are tested in CE1-3. In CE1-3, the proposed 12-tap filters for the following MC interpolation filtering: regular MC, 4x4 sub-block MC and chroma MC. Comparing to the existing MCIF in VVC 6, which uses 8-tap filters for regular MC, 6-tap filters for 4x4 sub-block MC and 4-tap filters for chroma MC, both computation complexity and memory bandwidth are increase with the 12-tap filter. In addition, the gain of 12-tap filters is 128, comparing to 64 in the existing VVC MCIF, the phase for 12-tap filtering is 16, comparing to 32 in the existing chroma MCIF. In order to unify the interpolation filter designs for cases with and without reference picture resampling, this contribution proposes to use cosine windowed sinc filter with the filter length, gain and number of phases matching the same length, gain and number of phases as the existing MCIF in VVC.

# Proposal

This contribution proposes to use cosine windowed sinc filters for MC interpolation filtering when the resolution of a reference picture is larger than that of the current picture. The equation used to derive the filter coefficients as shown in (1) is the same as that used to derive 12-tap SHM filter, except that different lengths and gains are used.

fn=hnwn,  n=-L-12, …, L-12                          (1)

The filter coefficient fn is a product of the impulse response of the ideal low-pass filter hn and a window function wn.

hn=fcrsincfcrn),   n=-∞, …, +∞                 (2)

The sinc function is defined below:

Sincx=1,           x=0sin⁡(πx)πx,   x≠0					(3)

a window function is applied to truncate the filter length to L. To obtain a smooth tapered curve, a cosine-windowed function is used, which is given by:

wn=cosπn)L-1, n=-L-12, …, L-12				(4)

where fc is the cut-off frequency, r is the down-sampling ratio.

In this contribution, fc is 0.9, the same value used to derive the 12-tap SHM filter coefficients. The real value coefficients obtained in (1) are multiplied by 64 and rounded to integer. The filter coefficients are further refined so that the gain of the filter is equal to 64.

The filter coefficients for down-sample ratio 2:1 and 1.5:1 are shown in the following tables:

Table 1 8-tap filter for MC interpolation with reference down-sampling at ratio 2:1

| Fractional sample position p | Filter coefficients |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | p0 | p1 | p2 | p3 | p4 | p5 | p6 | p7 |
| 0 | -4 | 2 | 20 | 28 | 20 | 2 | -4 | 0 |
| 1 | -4 | 0 | 19 | 29 | 21 | 5 | -4 | -2 |
| 2 | -4 | -1 | 18 | 29 | 22 | 6 | -4 | -2 |
| 3 | -4 | -1 | 16 | 29 | 23 | 7 | -4 | -2 |
| 4 | -4 | -1 | 16 | 28 | 24 | 7 | -4 | -2 |
| 5 | -4 | -1 | 14 | 28 | 25 | 8 | -4 | -2 |
| 6 | -3 | -3 | 14 | 27 | 26 | 9 | -3 | -3 |
| 7 | -3 | -1 | 12 | 28 | 25 | 10 | -4 | -3 |
| 8 | -3 | -3 | 11 | 27 | 27 | 11 | -3 | -3 |
| 9 | -3 | -4 | 10 | 25 | 28 | 12 | -1 | -3 |
| 10 | -3 | -3 | 9 | 26 | 27 | 14 | -3 | -3 |
| 11 | -2 | -4 | 8 | 25 | 28 | 14 | -1 | -4 |
| 12 | -2 | -4 | 7 | 24 | 28 | 16 | -1 | -4 |
| 13 | -2 | -4 | 7 | 23 | 29 | 16 | -1 | -4 |
| 14 | -2 | -4 | 6 | 22 | 29 | 18 | -1 | -4 |
| 15 | -2 | -4 | 5 | 21 | 29 | 19 | 0 | -4 |

Table 2 8-tap filter for MC interpolation with reference down-sampling at ratio 1.5:1

| Fractional sample position p | Filter coefficients |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | p0 | p1 | p2 | p3 | p4 | p5 | p6 | p7 |
| 0 | -1 | -6 | 20 | 38 | 20 | -6 | -1 | 0 |
| 1 | 1 | -7 | 18 | 37 | 22 | -5 | -4 | 2 |
| 2 | 1 | -7 | 16 | 37 | 23 | -4 | -4 | 2 |
| 3 | 1 | -7 | 14 | 36 | 26 | -3 | -5 | 2 |
| 4 | 1 | -7 | 12 | 36 | 27 | -2 | -5 | 2 |
| 5 | 2 | -7 | 11 | 35 | 28 | -1 | -6 | 2 |
| 6 | 2 | -7 | 9 | 34 | 30 | 0 | -6 | 2 |
| 7 | 2 | -7 | 7 | 33 | 31 | 2 | -6 | 2 |
| 8 | 2 | -7 | 5 | 32 | 32 | 5 | -7 | 2 |
| 9 | 2 | -6 | 2 | 31 | 33 | 7 | -7 | 2 |
| 10 | 2 | -6 | 0 | 30 | 34 | 9 | -7 | 2 |
| 11 | 2 | -6 | -1 | 28 | 35 | 11 | -7 | 2 |
| 12 | 2 | -5 | -2 | 27 | 36 | 12 | -7 | 1 |
| 13 | 2 | -5 | -3 | 26 | 36 | 14 | -7 | 1 |
| 14 | 2 | -4 | -4 | 23 | 37 | 16 | -7 | 1 |
| 15 | 2 | -4 | -5 | 22 | 37 | 18 | -7 | 1 |

Table 3 6-tap filter for MC interpolation with reference down-sampling at ratio 2:1 with 16 phases

| Fractional sample position p | Filter coefficients |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | p0 | p1 | p2 | p3 | p4 | p5 |
| 0 | 0 | 19 | 26 | 19 | 0 | 0 |
| 1 | -1 | 18 | 27 | 20 | 2 | -2 |
| 2 | -1 | 16 | 27 | 21 | 3 | -2 |
| 3 | -2 | 15 | 27 | 22 | 5 | -3 |
| 4 | -2 | 14 | 26 | 23 | 6 | -3 |
| 5 | -2 | 13 | 26 | 24 | 6 | -3 |
| 6 | -3 | 12 | 25 | 25 | 8 | -3 |
| 7 | -3 | 11 | 25 | 25 | 9 | -3 |
| 8 | -3 | 10 | 25 | 25 | 10 | -3 |
| 9 | -3 | 9 | 25 | 25 | 11 | -3 |
| 10 | -3 | 8 | 25 | 25 | 12 | -3 |
| 11 | -3 | 6 | 24 | 26 | 13 | -2 |
| 12 | -3 | 6 | 23 | 26 | 14 | -2 |
| 13 | -3 | 5 | 22 | 27 | 15 | -2 |
| 14 | -2 | 3 | 21 | 27 | 16 | -1 |
| 15 | -2 | 2 | 20 | 27 | 18 | -1 |

Table 4 6-tap filter for MC interpolation with reference down-sampling at ratio 1.5:1 with 16 phases

| Fractional sample position p | Filter coefficients |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | p0 | p1 | p2 | p3 | p4 | p5 |
| 0 | -5 | 18 | 38 | 18 | -5 | 0 |
| 1 | -5 | 16 | 39 | 20 | -4 | -2 |
| 2 | -5 | 14 | 39 | 22 | -4 | -2 |
| 3 | -5 | 13 | 38 | 24 | -3 | -3 |
| 4 | -5 | 11 | 37 | 26 | -2 | -3 |
| 5 | -5 | 9 | 36 | 28 | -1 | -3 |
| 6 | -5 | 8 | 35 | 30 | -1 | -3 |
| 7 | -4 | 6 | 34 | 31 | 1 | -4 |
| 8 | -4 | 3 | 33 | 33 | 3 | -4 |
| 9 | -4 | 1 | 31 | 34 | 6 | -4 |
| 10 | -3 | -1 | 30 | 35 | 8 | -5 |
| 11 | -3 | -1 | 28 | 36 | 9 | -5 |
| 12 | -3 | -2 | 26 | 37 | 11 | -5 |
| 13 | -3 | -3 | 24 | 38 | 13 | -5 |
| 14 | -2 | -4 | 22 | 39 | 14 | -5 |
| 15 | -2 | -4 | 20 | 39 | 16 | -5 |

Table 5 4-tap filter for MC interpolation with reference down-sampling at ratio 2:1 with 32-phases

| Fractional sample position p | Filter coefficients |  |  |  |
| --- | --- | --- | --- | --- |
|  | p0 | p1 | p2 | p3 |
| 0 | 17 | 30 | 17 | 0 |
| 1 | 17 | 30 | 18 | -1 |
| 2 | 16 | 30 | 18 | 0 |
| 3 | 16 | 30 | 18 | 0 |
| 4 | 15 | 30 | 18 | 1 |
| 5 | 14 | 30 | 18 | 2 |
| 6 | 13 | 29 | 19 | 3 |
| 7 | 13 | 29 | 19 | 3 |
| 8 | 12 | 29 | 20 | 3 |
| 9 | 11 | 28 | 21 | 4 |
| 10 | 10 | 28 | 22 | 4 |
| 11 | 10 | 27 | 22 | 5 |
| 12 | 9 | 27 | 23 | 5 |
| 13 | 9 | 26 | 24 | 5 |
| 14 | 8 | 26 | 24 | 6 |
| 15 | 7 | 26 | 25 | 6 |
| 16 | 7 | 25 | 25 | 7 |
| 17 | 6 | 25 | 26 | 7 |
| 18 | 6 | 24 | 26 | 8 |
| 19 | 5 | 24 | 26 | 9 |
| 20 | 5 | 23 | 27 | 9 |
| 21 | 5 | 22 | 27 | 10 |
| 22 | 4 | 22 | 28 | 10 |
| 23 | 4 | 21 | 28 | 11 |
| 24 | 3 | 20 | 29 | 12 |
| 25 | 3 | 19 | 29 | 13 |
| 26 | 3 | 19 | 29 | 13 |
| 27 | 2 | 18 | 30 | 14 |
| 28 | 1 | 18 | 30 | 15 |
| 29 | 0 | 18 | 30 | 16 |
| 30 | 0 | 18 | 30 | 16 |
| 31 | -1 | 18 | 30 | 17 |

Table 6 4-tap filter for MC interpolation with reference down-sampling at ratio 1.5:1 with 32-phases

| Fractional sample position p | Filter coefficients |  |  |  |
| --- | --- | --- | --- | --- |
|  | p0 | p1 | p2 | p3 |
| 0 | 13 | 38 | 13 | 0 |
| 1 | 13 | 38 | 14 | -1 |
| 2 | 12 | 38 | 15 | -1 |
| 3 | 11 | 37 | 17 | -1 |
| 4 | 10 | 38 | 18 | -2 |
| 5 | 9 | 38 | 19 | -2 |
| 6 | 8 | 38 | 20 | -2 |
| 7 | 7 | 38 | 21 | -2 |
| 8 | 6 | 37 | 22 | -1 |
| 9 | 6 | 36 | 24 | -2 |
| 10 | 5 | 36 | 25 | -2 |
| 11 | 4 | 35 | 26 | -1 |
| 12 | 4 | 34 | 27 | -1 |
| 13 | 3 | 34 | 28 | -1 |
| 14 | 3 | 33 | 29 | -1 |
| 15 | 2 | 32 | 30 | 0 |
| 16 | 1 | 31 | 31 | 1 |
| 17 | 0 | 30 | 32 | 2 |
| 18 | -1 | 29 | 33 | 3 |
| 19 | -1 | 28 | 34 | 3 |
| 20 | -1 | 27 | 34 | 4 |
| 21 | -1 | 26 | 35 | 4 |
| 22 | -2 | 25 | 36 | 5 |
| 23 | -2 | 24 | 36 | 6 |
| 24 | -1 | 22 | 37 | 6 |
| 25 | -2 | 21 | 38 | 7 |
| 26 | -2 | 20 | 38 | 8 |
| 27 | -2 | 19 | 38 | 9 |
| 28 | -2 | 18 | 38 | 10 |
| 29 | -1 | 17 | 37 | 11 |
| 30 | -1 | 15 | 38 | 12 |
| 31 | -1 | 14 | 38 | 13 |

# Simulation results

The proposed filters are implemented on top of VTM-6.1 with the patch for CE1 applied. Test condition described in JVET-O2021 is used in the simulation. The anchor of the comparison is CE1 anchor, which uses the existing MCIF for reference picture down-sampling. Both results of PSNR1 (codec PSNR measured between the input of encoder and output of decoder at coding resolution by calculating MSE for each picture, accumulating MSEs for all pictures and calculating average PSNR from the accumulated MSE) and PSNR2 (end-to-end PSNR measured between the original video source and the up-sampled (if needed) decoded pictures using the existing VTM interpolation filters) are reported. The simulation results are shown in Tables 7 - 10.

# Conclusion

In order to avoid increase of complexity and memory bandwidth, this contribution proposes to use cosine windowed sinc filters with filter lengths, filter gain and number of phases matching the existing MCIF in VVC when the reference picture resolution is of larger resolution than the current picture. In other cases, the existing MCIF in VVC are used. Comparing using existing MCIF in VVC for reference down-sampling, the proposed filters achieve average { -5.96%, -2.40%} bit rate saving for ratio 2:1 and 1.5:1 in terms of PSNR1, and average { X.XX%, X.XX%} bit rate savings for ratio 2:1 and 1.5:1 in terms of PSNR1.

Table 7: Simulation results of PSNR1 for down-sample ratio 2:1

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over CE1 anchor |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | - | - | - | - | - |
| Class A2 | - | - | - | - | - |
| Class B | -3.16% | -1.73% | -1.67% | 99% | 101% |
| Class C | -6.65% | -4.37% | -4.10% | 99% | 101% |
| Class E | -9.71% | -4.54% | -3.71% | 99% | 100% |
| Overall | -5.96% | -3.31% | -2.99% | 99% | 101% |
| Class D | - | - | - | - | - |
| Class F | - | - | - | - | - |

Table 8: Simulation results of PSNR1 for down-sample ratio 1.5:1

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over CE1 anchor |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | - | - | - | - | - |
| Class A2 | - | - | - | - | - |
| Class B | - | - | - | - | - |
| Class C | -3.65% | -2.30% | -1.94% | 98.15% | 96.33% |
| Class E | -3.04% | -1.06% | -0.63% | 99% | 97% |
| Overall | - | - | - | - | - |
| Class D | - | - | - | - | - |
| Class F | - | - | - | - | - |

Table 9: Simulation results of PSNR2 for down-sample ratio 2:1

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over CE1 anchor |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | - | - | - | - | - |
| Class A2 | - | - | - | - | - |
| Class B | -2.16% | -1.24% | -1.22% | 99% | 101% |
| Class C | -4.07% | -2.53% | -2.46% | 99% | 101% |
| Class E | -6.19% | -3.80% | -3.45% | 99% | 100% |
| Overall | -3.80% | -2.31% | -2.19% | 99% | 101% |
| Class D | - | - | - | - | - |
| Class F | - | - | - | - | - |

Table 10: Simulation results of PSNR2 for down-sample ratio 1.5:1

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over CE1 anchor |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | - | - | - | - | - |
| Class A2 | - | - | - | - | - |
| Class B | - | - | - | - | - |
| Class C | -2.68% | -2.24% | -1.95% | 98.15% | 96.33% |
| Class E | -2.23% | -3.26% | -2.08% | 99% | 97% |
| Overall | - | - | - | - | - |
| Class D | - | - | - | - | - |
| Class F | - | - | - | - | - |

# References

[1] M. G. Sarwer, R. -L Liao, J. Chen, J. Luo, Y. Ye, “AHG 8: JVET Common test conditions for adaptive resolution change (ARC)”, JVET-O0641, JVET 15th Meeting: Gothenburg, SE, 3–12 July 2019

[2] VVC reference software (VTM) version 6.1, available at: https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

# Patent rights declaration(s)

Alibaba Group Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).