| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0354-r1 |
| --- | --- |

| Title: | Non-CE6: LFNST Up to 64x64 CU’s |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Karam NASER Tangi POIRIER Fabrice LE LEANNEC Franck GALPIN | Tel: Email: | karam.naser@InterDigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

During the last JVET meeting, LFNST was restricted to CU’s not larger than the maximum TU size in order to solve the VPDU issue. However, due to the adoption of JVET-O0545, the maximum transform size changes between 64x64 and 32x32 depending on the SPS flag sps_max_luma_transform_size_64_flag. Despite no VPDU issue, LFNST is restricted to 32x32 when sps_max_luma_transform_size_64_flag is set to zero according to the current specification. This contribution proposes to remove this restriction and allow LFNST up to 64x64 regardless sps_max_luma_transform_size_64_flag, where TU tiling is performed when CU size is larger than TU. It is reported that 0.04% AI luma gain is achieved.

# Introduction

In VVC draft 6 specification, lfnst_idx is coded when both CU width and height are less than or equal to the maximum transform size. The maximum transform size (MaxTbSizeY) depends on the SPS flag sps_max_luma_transform_size_64_flag as followes:

MaxTbLog2SizeY = sps_max_luma_transform_size_64_flag  ?  6  :  5	(7-28)

MaxTbSizeY = 1  <<  MaxTbLog2SizeY		(7-30)

The initial motivation of restricting LFNST size to the maximum transform size is to avoid the VPDU issue when CU’s of size larger than 64x64 decodes LFNST index, in which multiple TU’s need to be decoded before decoding the LFNST index. Therefore, LFNST was restricted to the maximum TU size that was 64x64. Meanwhile, ‎[1] was adopted and cause the maximum transform size to be either 64x64 or 32x32. However, when maximum TU size is 32x32, there is no VDPU issue and no need for restricting LFNST to 32x32.

This contribution proposes enabling LFNST up to 64x64 even if the maximum transform size is 32x32, where TU tiling is used if CU size is larger than TU size. This is to allow more flexibility to the encoder and to improve the coding efficiency when the maximum transform size is set to 32x32.

# Proposal

The proposal allows LFNST for CU sizes up to 64x64. When the maximum TU size is 32x32, TU tiling is performed. To be inline with the adoption of JVET-O0466, LFNST is only allowed if all TU’s uses DCT2. That is, no LFNST if any TU has MTS index larger than 0.

The corresponding specification change is:

| coding_unit( x0, y0, cbWidth, cbHeight, cqtDepth, treeType, modeType ) { | Descriptor |
| --- | --- |
| if( cu_cbf ) { |  |
| if( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTER && sps_sbt_enabled_flag && !ciip_flag[ x0 ][ y0 ] && !MergeTriangleFlag[ x0 ][ y0 ] ) { |  |
| … |  |
| MTS_notDCT2 = 0 |  |
| LfnstDcOnly = 1 |  |
| LfnstZeroOutSigCoeffFlag = 1 |  |
| transform_tree( x0, y0, cbWidth, cbHeight, treeType ) |  |
| lfnstWidth = ( treeType = = DUAL_TREE_CHROMA ) ? cbWidth / SubWidthC : cbWidth |  |
| lfnstHeight = ( treeType = = DUAL_TREE_CHROMA ) ? cbHeight / SubHeightC : cbHeight |  |
| if( Min( lfnstWidth, lfnstHeight ) >= 4 && sps_lfnst_enabled_flag = = 1 && CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && IntraSubPartitionsSplitType = = ISP_NO_SPLIT && ( !intra_mip_flag[ x0 ][ y0 ] ¦ ¦ Min( lfnstWidth, lfnstHeight ) >= 16 ) && tu_mts_idx[ x0 ][ y0 ] = = 0 && Max( cbWidth, cbHeight ) <= MaxTbSizeY 64) { |  |
| if( LfnstDcOnly = = 0 && MTS_notDCT2 == 0 && LfnstZeroOutSigCoeffFlag = = 1 ) |  |
| lfnst_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |

| transform_unit( x0, y0, tbWidth, tbHeight, treeType, subTuIndex, chType ) { | Descriptor |
| --- | --- |
| if( sps_joint_cbcr_enabled_flag && ( ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && ( tu_cbf_cb[ x0 ][ y0 ] ¦ ¦ tu_cbf_cr[ x0 ][ y0 ] ) ) ¦ ¦ ( tu_cbf_cb[ x0 ][ y0 ] && tu_cbf_cr[ x0 ][ y0 ] ) ) ) |  |
| tu_joint_cbcr_residual_flag[ x0 ][ y0 ] | ae(v) |
| if( tu_cbf_luma[ x0 ][ y0 ] && treeType != DUAL_TREE_CHROMA && ( tbWidth <= 32 ) && ( tbHeight <= 32 ) && ( IntraSubPartitionsSplit[ x0 ][ y0 ] = = ISP_NO_SPLIT ) && ( !cu_sbt_flag ) ) { |  |
| if( sps_transform_skip_enabled_flag && !BdpcmFlag[ x0 ][ y0 ] && tbWidth <= MaxTsSize && tbHeight <= MaxTsSize ) |  |
| transform_skip_flag[ x0 ][ y0 ] | ae(v) |
| if( ( ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTER && sps_explicit_mts_inter_enabled_flag ) ¦ ¦ ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && sps_explicit_mts_intra_enabled_flag ) ) && ( !transform_skip_flag[ x0 ][ y0 ] ) ) |  |
| tu_mts_idx[ x0 ][ y0 ] | ae(v) |
| if ( tu_mts_idx[x0][y] > 0) |  |
| MTS_notDCT2 = 1 |  |
| } |  |
| } |  |

# Experimental Results

The proposed method was implemented on top of VTM6.0. The tests were conducted under the CTC. Both anchor and test use maximum transform size of 32x32 (--Log2MaxTbSize=5). The results are shown in the table below:

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 --Log2MaxTrSize=5 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.15% | -0.07% | -0.11% | 101% | 100% |
| Class A2 | -0.03% | 0.00% | -0.03% | 102% | 101% |
| Class B | -0.03% | 0.03% | 0.08% | 100% | 98% |
| Class C | 0.00% | 0.00% | -0.03% | 100% | 103% |
| Class E | -0.04% | 0.14% | -0.03% | 104% | 104% |
| Overall | -0.04% | 0.02% | -0.01% | 101% | 101% |
| Class D | -0.01% | -0.04% | 0.02% | 103% | 106% |
| Class F | -0.03% | 0.04% | 0.02% | 100% | 103% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-6.0 --Log2MaxTrSize=5 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.31% | -0.51% | -0.54% | 102% | 100% |
| Class A2 | -0.05% | 0.13% | -0.13% | 101% | 101% |
| Class B | -0.04% | -0.04% | -0.11% | 101% | 100% |
| Class C | 0.00% | 0.07% | 0.03% | 101% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.09% | -0.07% | -0.17% | 101% | 100% |
| Class D | 0.01% | 0.12% | 0.08% | 100% | 100% |
| Class F | -0.05% | 0.03% | 0.04% | 100% | 99% |

# Conclusion

This contribution proposes enabling LFSNT up to 64x64 regardless of the maximum transform size, where TU tiling is performed for larger CU’s. It is asserted that this approach does not cause any VPDU issue and improves the coding performance.

# References

- X. Zhao, X. Li, S. Liu, “Non-CE6: Configurable maximum transform size in VVC”, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Doc. JVET- O0545, 15th Meeting: Gothenburg, SE, 3–12 July 2019.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).