| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET- P0355 |
| --- | --- |

| Title: | Non CE3: Simplified coding of chroma intra modes |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Saverio Blasi, Andre Seixas Dias, Gosala Kulupana 201 Wood Lane The Lighthouse W12 7TQ, London, UK | Email: | saverio.blasi@bbc.co.uk andre.seixasdias@bbc.co.uk gosala.kulupana@bbc.co.uk |
| Source: | British Broadcasting Corporation |  |  |

_____________________________

# Abstract

In the latest VVC draft specification, the process of decoding a chroma intra-prediction mode that is not CCLM allows the decoder to either select the same mode as in the co-located luma block, or to select one of four explicitly signalled modes in a chroma candidate list. The pre-defined modes in this list are Planar, DC, horizontal and vertical. If the co-located luma mode is equal to one of these modes, the corresponding entry in the list is replaced by the vertical diagonal mode. This contribution proposes to reduce the number of modes that are explicitly signalled from four to two. The horizontal and vertical modes are removed from the list. This simplifies the decoding process by reducing the number of bits to parse and reducing the number of comparisons needed to determine whether a mode in the list should be replaced. It also reduces the complexity at the encoder side as less modes need to be tested. Average BD-rates of 0.00%, 0.67% and 0.73% are reported for the Y, Cb and Cr components, respectively, for the AI configuration, with 96% encoding time and 99% decoding time. Average BD-rates of 0.00%, 0.40% and 0.36% are reported for the Y, Cb and Cr components, respectively, for the RA configuration, with 98% encoding time and 99% decoding time.

# Proposed method

The process of decoding a chroma intra-prediction mode in the VVC WD [1] consists in determining first if the mode is CCLM. If the mode is not CCLM, then a flag is decoded to indicate whether the chroma intra prediction mode should assume the value of lumaIntraPredMode, namely the corresponding intra-prediction mode used by the co-located luma block. If the chroma intra-prediction mode is not lumaIntraPredMode, then a list of four possible explicitly-signalled chroma intra mode candidates is constructed. An index between 0 and 3 is decoded to determine which element in the list is to be used, and such element is then extracted as the current chroma intra mode for the PU. The process of constructing the list of chroma intra candidate modes depends on the value of lumaIntraPredMode. The four elements in the list are first initialized to Planar, Horizontal, Vertical and DC. Then, each element in the list is compared with the value of lumaIntraPredMode. If the two are identical, then such element in the list is replaced by the Vertical Diagonal mode (VDIA).

This contribution proposes to reduce the number of intra prediction modes that can be explicitly signaled from four to two. In particular, a list of two chroma intra mode candidates is considered instead of a list of four. The two elements in the list are initialized to Planar and DC. This means that the Horizontal and Vertical modes are not considered in the list, and cannot be explicitly signaled anymore. They can, of course, still be used to predict a block in case the co-located luma block is predicted using the Horizontal or Vertical mode. Each element in the list is then compared with the value of lumaIntraPredMode. If the two are identical, then such element in the list is replaced by the Vertical Diagonal mode (VDIA), as in the current VVC draft specifications. The rest of the decoding process is unchanged.

The proposed modification simplifies the decoding process by reducing the number of bits that are needed to be parsed to determine the index of the cand mode in the list. It also halves the maximum number of comparisons necessary to check whether a mode in the list needs to be replaced with the vertical diagonal mode or not.

Moreover, the proposed changes reduce the complexity of the encoder, as less modes need to be tested while performing chroma intra-prediction.

The following changes to Table 8-2 are proposed:

Table 8-2 – Specification of IntraPredModeC[ xCb ][ yCb ] depending on cclm_mode_flag, cclm_mode_idx, intra_chroma_pred_mode and lumaIntraPredMode

| cclm_mode_flag | cclm_mode_idx | intra_chroma_pred_mode | lumaIntraPredMode |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | 50 | 18 | 1 | X ( 0 <= X <= 66 ) |
| 0 | - | 0 | 66 | 0 | 0 | 0 | 0 |
| 0 | - | 1 | 50 | 66 | 50 | 50 | 50 |
| 0 | - | 2 | 18 | 18 | 66 | 18 | 18 |
| 0 | - | 3 1 | 1 | 1 | 1 | 66 | 1 |
| 0 | - | 4 2 | 0 | 50 | 18 | 1 | X |
| 1 | 0 | - | 81 | 81 | 81 | 81 | 81 |
| 1 | 1 | - | 82 | 82 | 82 | 82 | 82 |
| 1 | 2 | - | 83 | 83 | 83 | 83 | 83 |

The following changes to Table 9-80 are proposed:

Table 9-80 – Binarization for intra_chroma_pred_mode

| Value of intra_chroma_pred_mode | Bin string |
| --- | --- |
| 0 | 100 10 |
| 1 | 101 11 |
| 2 | 110 0 |
| 3 | 111 |
| 4 | 0 |

# Performance results

Tests were performed using the JVET common test conditions [2] with parallel encoding/decoding used for RA. The software modifications were made on top of VTM 6.0 [3], compiled with gcc 4.8.2. All experiments were run using AVX.

Table 1: Performance of the proposed luma intra mode coding with no MPM derivation

|  |  |  | AI |  |  |  |  | RA |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | 0.73% | 0.74% | 96% | 99% | 0.00% | 0.17% | 0.29% | 97% | 98% |
| Class A2 | -0.04% | 0.83% | 0.92% | 95% | 99% | 0.02% | 0.44% | 0.43% | 97% | 99% |
| Class B | 0.02% | 0.65% | 0.66% | 98% | 100% | 0.00% | 0.54% | 0.47% | 98% | 99% |
| Class C | 0.02% | 0.35% | 0.50% | 96% | 100% | -0.01% | 0.37% | 0.24% | 98% | 99% |
| Class E | 0.04% | 0.92% | 0.94% | 96% | 100% |  |  |  |  |  |
| Overall | 0.00% | 0.67% | 0.73% | 96% | 99% | 0.00% | 0.40% | 0.36% | 98% | 99% |
| Class D | 0.02% | 0.46% | 0.43% | 96% | 99% | -0.06% | 0.08% | 0.38% | 98% | 99% |
| Class F | 0.07% | 0.46% | 0.60% | 96% | 96% | 0.05% | 0.27% | 0.37% | 97% | 97% |

# Conclusions

This contribution proposes to reduce the number of chroma intra prediction modes that can be explicitly signalled from 4 to 2. The Horizontal and Vertical modes are removed from the possible expliclty-signalled modes. Average BD-rates of 0.00%, 0.67% and 0.73% are reported for the Y, Cb and Cr components, respectively, for the AI configuration, with 96% encoding time and 99% decoding time. Average BD-rates of 0.00%, 0.40% and 0.36% are reported for the Y, Cb and Cr components, respectively, for the RA configuration, with 98% encoding time and 99% decoding time.

# References

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 6),” JVET-O2001-vE, Gothenburg, July 2019.

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, Geneva, March 2019.

- VTM-6.0 software, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM.git, February 2019

# Patent rights declaration(s)

The British Broadcasting Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).