| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0357 |
| --- | --- |

| Title: | AHG12: Signal the CTU size in both SPS and PPS |  |  |  |
| --- | --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |  |
| Purpose: | Proposal |  |  |  |
| Author(s) or Contact(s): | Rickard Sjöberg Martin Pettersson Mitra Damghanian | Email: | rickard.sjoberg@ericsson.com martin.m.pettersson@ericsson.com mitra.damghanian@ericsson.com |
| Source: | Ericsson |  |  |  |

_____________________________

# Abstract

This contribution claims that the current VVC draft specification requires information from both the SPS and PPS before the layout of the tile and brick partitioning structure can be derived by the decoder and that it would be cleaner to enable derivation of the partitioning structure from the PPS alone.

The contribution proposes that the size of the CTU is signaled in the PPS in addition to signaling it in the SPS and claims that this enables derivation of the tile and brick partitioning structure as well as many other variables by parsing only the PPS.

The proposed changes to the draft specification are as follows:

- To rename the SPS u(2) syntax element log2_ctu_size_minus5 as sps_log2_ctu_size_minus5

- To add a PPS u(2) syntax element pps_log2_ctu_size_minus5

- Require that pps_log2_ctu_size_minus5 is equal to sps_log2_ctu_size_minus5

- To remove the PPS ue(v) syntax element num_tiles_in_pic_minus1 and instead derive and use NumTilesInPic

The proponents claim that removal of the num_tiles_in_pic_minus1 syntax element without adding the pps_log2_ctu_size_minus5 syntax element would introduce parsing dependencies between the SPS and PPS.

Proposed specification text on top of JVET-O2001-vE.docx using change marks is provided with the contribution.

# Introduction and summary

In the current VVC draft specification, the PPS is parsing independent from the SPS. This means that it is possible to parse the PPS without using any data from the SPS. However, it is not possible to derive all variable values related to the PPS without using data from the SPS. In particular, it is not possible to derive the layout of the tile and brick partitioning structure from the PPS alone.

In this contribution, we propose to enable the tile and brick partitioning structure to be derived from the PPS alone.

# Proposal

We propose to signal the size of the CTU in the PPS in addition to signaling it in the SPS, with the requirement that the CTU size signaled in the PPS is identical to the CTU size signaled in the SPS. This simple change makes it possible to derive the following values from the PPS alone:

- CtbSizeY = 1 << (pps_log2_ctu_size_minus5 + 5)

- PicWidthInCtbsY = Ceil( pic_width_in_luma_samples ÷ CtbSizeY )

- PicHeightInCtbsY = Ceil( pic_height_in_luma_samples ÷ CtbSizeY )

- PicSizeInCtbsY = PicWidthInCtbsY * PicHeightInCtbsY

In turn, this enables derivation of a large number of variables when parsing only the PPS, such as:

- NumTilesInPic

- NumBricksInPic

- colWidth[ i ] and colHeight[ i ]

- num_tiles_columns_minus1 and num_tile_rows_minus1 (in case of uniform tiles)

- tileColBd[ i ] and tileRowBd[ i ]

- BrickColBd[ i ], BrickRowBd[ i ], BrickWidth[ i ], BrickHeight[ i ]

- rowHeight2[ j ], rowBd2[ j ]

- num_brick_rows_minus2 (in case of uniform tiles)

- CtbAddrRsToBs[ ctbAddrRs ]

- CtbAddrBsToRs[ ctbAddrBs ]

- FirstCtbAddrBs[ brickIdx ]

The solution introduces a 2-bit pps_log2_ctu_size_minus5 code word in the PPS when there are multiple tiles in the PPS, but the syntax element num_tiles_in_pic_minus1 with a ue(v) code word can be removed from the specification.

It can be noted that at the previous meeting in Gothenburg, we proposed this solution during the discussion of JVET-O0236. The notes in JVET-O1076-v5.docx says: “It was commented that another option would be to indicate the CTB size in PPS (with the constraint that it shall be the same as the signalled CTB size in SPS). This would also enable deriving the tile-and-brick partitioning (in units of samples) by parsing the PPS only (without a need to resolve the referencing to SPS).”

We propose the following VVC draft specification changes:

- To rename the SPS u(2) syntax element log2_ctu_size_minus5 as sps_log2_ctu_size_minus5

- To add a PPS u(2) syntax element pps_log2_ctu_size_minus5

- Require that pps_log2_ctu_size_minus5 is equal to sps_log2_ctu_size_minus5

- To remove the PPS ue(v) syntax element num_tiles_in_pic_minus1 and instead derive and use NumTilesInPic

Proposed specification text on top of JVET-O2001-vE.docx using change marks is provided with the contribution.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).