| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0362 |
| --- | --- |

| Title: | AHG17: NAL unit header extension to extend the number of layers |  |  |  |
| --- | --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |  |
| Purpose: | Proposal |  |  |  |
| Author(s) or Contact(s): | Rickard Sjöberg Martin Pettersson Mitra Damghanian | Email: | rickard.sjoberg@ericsson.com martin.m.pettersson@ericsson.com mitra.damghanian@ericsson.com |
| Source: | Ericsson |  |  |  |

_____________________________

# Abstract

This contribution proposes a variable length NAL unit header design to provide a larger range of layer id values. It is pointed out in the contribution that the current VVC design has a 2-byte NAL unit header with 6 bits for nuh_layer_id that provides the use of 64 layer id values. It is claimed that the proposed design provides the use of 56 layer id values with a 2-byte NAL unit header and 2104 layer id values with a 3-byte NAL unit header. The proposed syntax is as follows where nuh_layer_id_extension_value is a proposed new syntax element:

| nal_unit_header( ) { | Descriptor |
| --- | --- |
| forbidden_zero_bit | f(1) |
| nuh_reserved_zero_bit | u(1) |
| nuh_layer_id | u(6) |
| nal_unit_type | u(5) |
| nuh_temporal_id_plus1 | u(3) |
| if( nuh_layer_id >= 56 ) |  |
| nuh_layer_id_extension_value | u(8) |
| } |  |

The proposed derivation of the layer id value to use, LayerID, is as follows:

if( nuh_layer_id < 56 )
  LayerID = nuh_layer_id
else
  LayerID = 56 + ( ( nuh_layer_id − 56 ) * 256 ) + nuh_layer_id_extension_value

Proposed specification text relative to JVET-O2001-vE.docx is provided with the contribution.

# Introduction and Problem Statement

The current VVC draft specification uses the following NAL unit header syntax:

| nal_unit_header( ) { | Descriptor |
| --- | --- |
| forbidden_zero_bit | f(1) |
| nuh_reserved_zero_bit | u(1) |
| nuh_layer_id | u(6) |
| nal_unit_type | u(5) |
| nuh_temporal_id_plus1 | u(3) |
| } |  |

The 6 bits used for nuh_layer_id provide a maximum of 64 layer ids. This is sufficient for many applications, but more layer id values are desirable for some applications such as multilens array (lenslet) use-cases.

# Proposal

We propose a NAL unit header design that supports 56 layers using a 2-byte NAL unit header and 2104 layers using a 3-byte NAL unit header. The proposed syntax and semantics look as follows, where nuh_layer_id_extension_value is a proposed new syntax element:

| nal_unit_header( ) { | Descriptor |
| --- | --- |
| forbidden_zero_bit | f(1) |
| nuh_reserved_zero_bit | u(1) |
| nuh_layer_id | u(6) |
| nal_unit_type | u(5) |
| nuh_temporal_id_plus1 | u(3) |
| if( nuh_layer_id >= 56 ) |  |
| nuh_layer_id_extension_value | u(8) |
| } |  |

nuh_layer_id is used to derive the variable LayerID.

nuh_layer_id_extension_value is used to derive the variable LayerID. When nuh_layer_id_extension_value is not present, it is inferred to be equal to 0. The variable LayerID, which specifies the identifier of the layer to which a VCL NAL unit belongs or the identifier of a layer to which a non-VCL NAL unit applies, is derived as follows:

–	If nuh_layer_id is smaller than 56, LayerID is set equal to nuh_layer_id

–	Otherwise, LayerID is derived as follows:

LayerID = 56 + ( ( nuh_layer_id − 56 ) * 256 ) + nuh_layer_id_extension_value	(X-XX)

The value of LayerID shall be the same for all VCL NAL units of a coded picture. The value of LayerID of a coded picture or a layer access unit is the value of the LayerID of the VCL NAL units of the coded picture or the layer access unit.

NOTE X – LayerID values 0 to 55 are signaled using a 2 byte NAL unit header. LayerID values 56 to 2103 are signaled using a 3 byte NAL unit header.

[Ed. note: Replace occurrencies of nuh_layer_id with LayerID throughout the specification.]

It can be noted that the number of LayerID values N achievable by using a 3-byte NAL unit header as a function of the number of LayerID values V provided using a 2-byte NAL unit header by the design is:

N = 16384 – 255 * V

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).