| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0365-v1 |
| --- | --- |

| Title: | AHG15: Scaling matrices for LFNST-coded blocks |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Adarsh K. Ramasubramonian Geert Van der Auwera Vadim Seregin Hilmi Enes Egilmez Marta Karczewicz 5775 Morehouse Dr San Diego, CA 92121 USA | Email: | aramasub@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This document proposes a modified method of applying scaling matrices for LFNST-coded blocks. It is asserted that the current VVC design of applying the scaling matrices has some shortcomings. For all LFNST-coded blocks, it is observed that the LFNST coefficients are restricted to be contained in a 4x4 region of the transform block. It is proposed that when scaling matrices are applied to an LFNST-coded block, the scaling matrix corresponding to the 4x4 transform block size be applied to the LFNST coefficients. For applications that use quantization matrices for rate control, this method is asserted to be sufficient. For use of scaling matrices for perceptual quantization, it is not evident that the scaling matrices that are specified for the primary matrices may be useful for LFNST coefficients. Until such evidence is available, it is proposed that flexibility be provided to disable the scaling matrices for LFNST coefficients.

# Introduction

Quantization matrices or scaling matrices (QM/SM) are used for frequency-dependent quantization of the various transform blocks. In VVC, scaling lists are signalled in the APS, and scaling matrices are derived therefrom. QM/SM may be used to apply perceptual quantization to the transform coefficients, where quantization of the coefficients may be optimized for the human visual system. More recently, QM/SM are also used for rate control where the QM/SM coefficients provide an encoder flexibility to apply different quantization to different coefficients in the transform block. The use of the QP parameter results in a uniform scaling factor/quantization step size applied to all the coefficients, which may not be appropriate or desired in all the situations (e.g., finer quantization of low-frequency coefficients and coarser quantization of high-frequency coefficients).

Before LFNST (secondary transform) was introduced, scaling matrices were directly applied on the quantized coefficients of the primary transform. In current VVC, when scaling matrices are enabled, they are applied to all the transform blocks excluding transform skip blocks. In VVC, for LFNST-coded blocks, the scaling matrices (that are typically designed on the primary transform coefficients) are directly applied on the quantized LFNST coefficients.

When a block is coded with LFNST, only 8 or 16 LFNST coefficients may be non-zero. These coefficients are stored in the top-left 4x4 region of the transform block. When a scaling matrix associated with an MxN block is applied to the 4x4 region of transform blocks, the top-left 4x4 matrix coefficients are only applied to the LFNST coefficients. For example, if the block is an 8x8 block, the top-left 4x4 region of the primary transform block may correspond to low-frequency coefficients. The scaling matrix may be specified such that the low frequency coefficients are not quantized as heavily as the high frequency coefficients, resulting in a relatively flat profile of the scaling coefficients in the 4x4 region. When the block is coded with LFNST, the scaling matrix may not result in any relative quantization of coefficients due to the flat nature of the coefficients in the 4x4 region of the scaling matrix.

In another example, for a 32x32 TB, scaling matrix coefficients are specified on an 8x8 grid (or rather 64 coefficients) and upsampled with replication to obtain the 32x32 scaling matrix (a DC coefficient may also be applied to all the coefficients). So effectively, the top-left 4x4 region of the matrix is filled with one value that corresponds to the top-left coefficient of the 8x8 grid. When scaling matrix is applied to a 32x32 block where LFNST is applied, the LFNST coefficients (restricted to the top-left 4x4 region) are scaled by a constant value (i.e., no relative scaling of the coefficients). This effectively results in a “flat” scaling matrix which would be contrary to the desired behaviour of scaling matrices.

# Proposal

It is proposed to signal a flag in the APS to indicate whether scaling matrices are to be applied for LFNST coefficients. When scaling matrices are applied to LFNST coefficients, it is proposed to use the scaling matrix corresponding to the 4x4 transform block. These two modes of operation provide the flexibility to handle LFNST-coded blocks when scaling matrices are applied for the various use cases.

| scaling_list_data( ) { | Descriptor |
| --- | --- |
| scaling_matrix_for_lfnst_disable_flag | u(1) |
| for( sizeId = 1; sizeId < 7; sizeId++ ) |  |
| for( matrixId = 0; matrixId < 6; matrixId ++ ) { |  |
| if( ! ( ( ( sizeId = = 1 ) && ( matrixId % 3 = = 0 ) ) ¦ ¦ ( ( sizeId = = 6 ) && ( matrixId % 3 != 0 ) ) ) ) { |  |

scaling_matrix_for_lfnst_disable_flag equal to 1 specifies that scaling matrices are not applied to blocks coded with LFNST. Scaling_matrix_for_lfnst_disable_flag equal to 0 specifies that the scaling matrices may apply to the blocks coded with LFNST.

The following changes are made to the Section 8.7.3 Scaling process for transform coefficients.

For the derivation of the scaled transform coefficients d[ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1, the following applies:

…

- The intermediate scaling factor m[ x ][ y ] is  derived as follows:

–	If one or more of the following conditions are true, m[ x ][ y ] is set equal to 16:

- sps_scaling_list_enabled_flag is equal to 0.

- transform_skip_flag[ xTbY ][ yTbY ] is equal to 1.

- scaling_matrix_for_lfnst_disable_flag is equal to 1 and lfnst_idx[ xTbY ][ yTbY ] is not equal to 0

- All of the following conditions are true:

  - scaling_matrix_for_lfnst_disable_flag is equal to 0

  - lfnst_idx[ xTbY ][ yTbY ] is not equal to 0

  - x is greater than or equal to 4

  - y is greater than or equal to 4

–	Otherwise, the following applies:

scW = ( lfnst_idx[ xTbY ][ yTbY ] != 0 ) ? 4 : nTbW
scH = ( lfnst_idx[ xTbY ][ yTbY ]  != 0 ) ? 4 : nTbH
m[ x ][ y ] = ScalingFactor[ Log2( nTbscW ) ][ Log2( nTbscH ) ][ matrixId ][ x ][ y ], 
			with matrixId as specified in Table 7-5		(8-958)

- The scaling factor ls[ x ][ y ] is derived as follows:

…

# Patent rights declaration

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).