| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0367 |
| --- | --- |

| Title: | AHG17: On AUD signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Martin Pettersson Rickard Sjöberg Mitra Damghanian | Email: | martin.m.pettersson@ericsson.com rickard.sjoberg@ericsson.com mitra.damghanian@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution proposes to signal access unit delimiters (AUDs) in VVC only when it is assessed to be required in order to separate the access units. The contribution proposes to signal an aud_in_next_access_unit_present_flag codeword in the AUD NAL unit. When aud_in_next_access_unit_present_flag is equal to 1, the access unit following the current access unit in decoding order shall start with an AUD NAL unit. When aud_in_next_access_unit_present_flag is equal to 0, an AUD NAL unit may or may not be present for the access unit following the current access unit.

It is further proposed to add the following restrictions:

- Access units in which the coded picture is an IRAP or GDR picture shall start with an access unit delimiter NAL unit.

- An access unit comprising more than one NAL unit shall start with an access unit delimiter.

- aud_in_next_access_unit_present_flag shall be equal to 1 for access units with more than one NAL unit, excluding the access unit delimiter NAL unit.

# Introduction

At the 15th JVET meeting in Gothenburg it was decided to mandate the use of AUDs for all access units in the bitstream. This was done to simplify the division of NAL units into access units in the bitstream, which can be a complex task in HEVC.

However, the AUD is not needed when there is only one NAL unit in an access unit in the bitstream. We are proposing a method to signal AUDs only when it is needed in VVC.

# Proposal

It is proposed to signal an aud_in_next_access_unit_present_flag codeword in the AUD NAL unit. When aud_in_next_access_unit_present_flag is equal to 1, the access unit following the current access unit in decoding order shall start with an AUD NAL unit. When aud_in_next_access_unit_present_flag is equal to 0, an AUD NAL unit may or may not be present for the access unit following the current access unit in decoding order.

It is further proposed to have the following restrictions:

- Access units in which the coded picture is an IRAP or GDR picture shall start with an access unit delimiter NAL unit.

- An access unit comprising more than one NAL unit shall start with an access unit delimiter.

- aud_in_next_access_unit_present_flag shall be equal to 1 for access units with more than one NAL unit, excluding the access unit delimiter NAL unit

This will ensure that an access unit that contains more than one NAL unit, excluding any AUD NAL unit in the count, will contain an AUD NAL unit, and the following access unit in decoding order will also contain an AUD NAL unit.

It is proposed to add the following changes to the VVC specification.

## Proposed changes to the VVC specification

The proposed changes on top of the current VVC draft (JVET-O2001vE) are marked in yellow.

| access_unit_delimiter_rbsp( ) { | Descriptor |
| --- | --- |
| pic_type | u(3) |
| aud_in_next_access_unit_present_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

aud_in_next_access_unit_present_flag equal to 1 specifies that the access unit following the current access unit in decoding order shall start with an access unit delimiter NAL unit. aud_in_next_access_unit_present_flag equal to 0 specifies that an access unit delimiter NAL unit may or may not be present for the access unit following the current access unit in decoding order. aud_in_next_access_unit_present_flag shall be 1 for access units with more than one NAL unit, excluding the access unit delimiter NAL unit.

It is a requirement of bitstream conformance that:

- access units in which the coded picture is an IRAP or GDR picture shall start with an access unit delimiter NAL unit.

- an access unit comprising more than one NAL unit shall start with an access unit delimiter.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).