| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0368-v2 |
| --- | --- |

| Title: | AHG17: On selectively signal slice header parameters in PPS |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Martin Pettersson Rickard Sjöberg Mitra Damghanian | Email: | martin.m.pettersson@ericsson.com rickard.sjoberg@ericsson.com mitra.damghanian@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

At the 15th JVET meeting in Gothenburg JVET-O0238 was adopted which added support to signal some of the slice header parameters either in the PPS or in the slice headers. An encoder may signal parameters that it knows will stay constant in the PPS and not repeat them in every slice. The functionality is enabled by the constant_slice_header_params_enabled_flag.

This contribution proposes to add the following additional syntax elements to PPS and use the same mechanism provided by the constant_slice_header_params_enabled_flag to either signal a parameter value in the PPS or in the slice headers:

- pps_num_ref_idx_active_override_idc

- pps_disable_bdof_dmvr_idc

- pps_lmcs_aps_id_idc

- pps_chroma_residual_scale_idc

- pps_collocated_ref_idx_plus1

- pps_log2_diff_min_qt_min_cb_luma_plus1

- pps_max_mtt_hierarchy_depth_luma_plus1

- pps_log2_diff_max_bt_min_qt_luma_plus1

- pps_log2_diff_max_tt_min_qt_luma_plus1

The luma BD-rate numbers for the CTC, using VTM-master (1a3e0974) from 20190913 as the anchor, are reported to be 0.00%/-0.04%/-0.13%/-0.14% for AI/RA/LDB/LDP respectively. The luma BD-rate numbers for the CTC using VTM-master (1a3e0974) with constant_slice_header_params_enabled_flag set equal to 0 as the anchor are reported to be 0.00%/-0.05%/-0.21%/-0.17% for AI/RA/LDB/LDP respectively.

Proposed specification text relative to JVET-O2001-vE.docx is provided with the contribution.

Version 2 of the contribution contains the full simulation results.

# Introduction

At the 15th JVET meeting in Gothenburg JVET-O0238 was adopted which added support to signal some of the slice header parameters either in the PPS or in the slice headers. An encoder may signal parameters that it knows will stay constant in the PPS and not repeat them in every slice.

It has been observed by the proponents when studying the new version of VTM that there are additional slice parameters that stay constant in the bitstream. In the table below the additional parameters are listed with information of in which CTC configuration they stay constant, which constant value they have and the estimated number of potential bits saved per slice for each parameter.

Table 1	- Slice parameters that stay constant for each CTC configuration using VTM

| Parameter | Code word | In CTC always equal to |  |  | Estimated number of bits saved in a slice |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RA | LDB | LDP | RA | LDB | LDP |
| num_ref_idx_active_override_flag | u(1) | 0 | 0 | 0 | 1 | 1 | 1 |
| slice_disable_bdof_dmvr_flag | u(1) | 0 |  |  | 1 |  |  |
| slice_lmcs_aps_id | u(2) | 0 | 0 | 0 | 2 | 2 | 2 |
| slice_chroma_residual_scale_flag | u(1) |  | 1 | 1 |  | 1 | 1 |
| collocated_ref_idx | ue(v) | 0 | 0 | 0 | 1 | 1 | 1 |
| slice_log2_diff_min_qt_min_cb_luma | ue(v) | 1 | 1 | 1 | 3 | 3 | 3 |
| slice_max_mtt_hierarchy_depth_luma | ue(v) | 3 | 3 | 3 | 5 | 5 | 5 |
| slice_log2_diff_max_bt_min_qt_luma | ue(v) |  |  |  |  |  |  |
| slice_log2_diff_max_tt_min_qt_luma | ue(v) | 3 | 3 | 3 | 5 | 5 | 5 |
| Summary |  |  |  |  | 18 | 18 | 18 |

# Proposal

This contribution proposes to add the following syntax elements to PPS and use the same mechanism provided by the constant_slice_header_params_enabled_flag to either signal a parameter value in PPS or in slice headers:

- pps_num_ref_idx_active_override_idc

- pps_disable_bdof_dmvr_idc

- pps_lmcs_aps_id_idc

- pps_chroma_residual_scale_idc

- pps_collocated_ref_idx_plus1

- pps_log2_diff_min_qt_min_cb_luma_plus1

- pps_max_mtt_hierarchy_depth_luma_plus1

- pps_log2_diff_max_bt_min_qt_luma_plus1

- pps_log2_diff_max_tt_min_qt_luma_plus1

It is proposed to add the additional parameters to the VVC specification and use them as shown in Table 1 for the CTC.

## Proposed syntax and semantics for the VVC specification

The proposed syntax and semantics on top of the current VVC draft (JVET-O2001-vE) are marked in yellow with removed text in red strike-through.

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| constant_slice_header_params_enabled_flag | u(1) |
| if( constant_slice_header_params_enabled_flag ) { |  |
| pps_dep_quant_enabled_idc | u(2) |
| pps_num_ref_idx_active_override_idc | u(2) |
| for( i = 0; i < 2; i++ ) |  |
| pps_ref_pic_list_sps_idc[ i ] | u(2) |
| pps_temporal_mvp_enabled_idc | u(2) |
| pps_disable_bdof_dmvr_idc | u(2) |
| pps_lmcs_aps_id_idc | u(3) |
| pps_chroma_residual_scale_idc | u(2) |
| pps_mvd_l1_zero_idc | u(2) |
| pps_collocated_from_l0_idc | u(2) |
| pps_collocated_ref_idx_plus1 | ue(v) |
| pps_six_minus_max_num_merge_cand_plus1 | ue(v) |
| pps_five_minus_max_num_subblock_merge_cand_plus1 | ue(v) |
| pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1 | ue(v) |
| pps_log2_diff_min_qt_min_cb_luma_plus1 | ue(v) |
| pps_max_mtt_hierarchy_depth_luma_plus1 | ue(v) |
| pps_log2_diff_max_bt_min_qt_luma_plus1 | ue(v) |
| pps_log2_diff_max_tt_min_qt_luma_plus1 | ue(v) |
| } |  |
| … |  |
| } |  |

constant_slice_header_params_enabled_flag equal to 0 specifies that pps_dep_quant_enabled_idc, pps_num_ref_idx_active_override_idc, pps_ref_pic_list_sps_idc[ i ], pps_temporal_mvp_enabled_idc, pps_disable_bdof_dmvr_idc, pps_lmcs_aps_id_idc, pps_chroma_residual_scale_idc, pps_mvd_l1_zero_idc, pps_collocated_from_l0_idc, pps_collocated_ref_idx_plus1, pps_six_minus_max_num_merge_cand_plus1, pps_five_minus_max_num_subblock_merge_cand_plus1, pps_max_num_merge_cand_minus_max_num_triangle_cand_minus1, pps_log2_diff_min_qt_min_cb_luma_plus1, pps_max_mtt_hierarchy_depth_luma_plus1, pps_log2_diff_max_bt_min_qt_luma_plus1 and pps_log2_diff_max_tt_min_qt_luma_plus1 are inferred to be equal to 0. constant_slice_header_params_enabled_flag equal to 1 specifies that these syntax elements are present in the PPS.

pps_num_ref_idx_active_override_idc equal to 0 specifies that the syntax element slice_num_ref_idx_active_override_flag is present in slice header of slices referring to the PPS. pps_num_ref_idx_active_override_idc equal to 1 or 2 specifies that the syntax element slice_num_ref_idx_active_override_flag is not present in slice header of slices referring to the PPS.  pps_num_ref_idx_active_override_idc equal to 3 is reserved for future use by ITU-T | ISO/IEC.

pps_disable_bdof_dmvr_idc equal to 0 specifies that the syntax element slice_disable_bdof_dmvr_flag is present in slice header of slices referring to the PPS. pps_disable_bdof_dmvr_idc equal to 1 or 2 specifies that the syntax element slice_disable_bdof_dmvr_flag is not present in slice header of slices referring to the PPS.  pps_disable_bdof_dmvr_idc equal to 3 is reserved for future use by ITU-T | ISO/IEC.

pps_lmcs_aps_id_idc equal to 0 specifies that the syntax element slice_lmcs_aps_id is present in slice header of slices referring to the PPS. pps_lmcs_aps_id_idc in the range 1 to 4 specifies that the syntax element slice_lmcs_aps_id is not present in slice header of slices referring to the PPS.  pps_lmcs_aps_id_idc in the range 5 to 7 are reserved for future use by ITU-T | ISO/IEC.

pps_chroma_residual_scale_idc equal to 0 specifies that the syntax element slice_chroma_residual_scale_flag is present in slice header of slices referring to the PPS. pps_chroma_residual_scale_idc equal to 1 or 2 specifies that the syntax element slice_chroma_residual_scale_flag is not present in slice header of slices referring to the PPS.  pps_chroma_residual_scale_idc equal to 3 is reserved for future use by ITU-T | ISO/IEC.

pps_collocated_ref_idx_plus1 equal to 0 specifies that the syntax element slice_collocated_ref_idx is present in slice header of slices referring to the PPS. pps_collocated_ref_idx_plus1 greater than 0 specifies that the syntax element slice_collocated_ref_idx is not present in slice header of slices referring to the PPS.

pps_log2_diff_min_qt_min_cb_luma_plus1 equal to 0 specifies that the syntax element slice_log2_diff_min_qt_min_cb_luma is present in slice header of slices referring to the PPS. pps_log2_diff_min_qt_min_cb_luma_plus1 greater than 0 specifies that the syntax element slice_log2_diff_min_qt_min_cb_luma is not present in slice header of slices referring to the PPS. The value of the syntax element pps_log2_diff_min_qt_min_cb_luma_plus1 shall be in the range of 0 to CtbLog2SizeY − MinCbLog2SizeY + 1, inclusive.

pps_max_mtt_hierarchy_depth_luma_plus1 equal to 0 specifies that the syntax element slice_max_mtt_hierarchy_depth_luma is present in slice header of slices referring to the PPS. pps_max_mtt_hierarchy_depth_luma_plus1 greater than 0 specifies that the syntax element slice_max_mtt_hierarchy_depth_luma is not present in slice header of slices referring to the PPS. The value of the syntax element pps_max_mtt_hierarchy_depth_luma_plus1 shall be in the range of 0 to CtbLog2SizeY − MinCbLog2SizeY + 1, inclusive.

pps_log2_diff_max_bt_min_qt_luma_plus1 equal to 0 specifies that the syntax element slice_log2_diff_max_bt_min_qt_luma is present in slice header of slices referring to the PPS. pps_log2_diff_max_bt_min_qt_luma_plus1 greater than 0 specifies that the syntax element slice_log2_diff_max_bt_min_qt_luma is not present in slice header of slices referring to the PPS. The value of the syntax element pps_log2_diff_max_bt_min_qt_luma_plus1 shall be in the range of 0 to CtbLog2SizeY − MinQtLog2SizeY + 1, inclusive.

pps_log2_diff_max_tt_min_qt_luma_plus1 equal to 0 specifies that the syntax element slice_log2_diff_max_tt_min_qt_luma is present in slice header of slices referring to the PPS. pps_log2_diff_max_tt_min_qt_luma_plus1 greater than 0 specifies that the syntax element slice_log2_diff_max_tt_min_qt_luma is not present in slice header of slices referring to the PPS. The value of the syntax element pps_log2_diff_max_tt_min_qt_luma_plus1 shall be in the range of 0 to CtbLog2SizeY − MinQtLog2SizeY + 1, inclusive.

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( ( nal_unit_type != IDR_W_RADL && nal_unit_type != IDR_N_LP ) ¦ ¦ sps_idr_rpl_present_flag ) { |  |
| … |  |
| if( ( slice_type != I && num_ref_entries[ 0 ][ RplsIdx[ 0 ] ] > 1 ) ¦ ¦ ( slice_type = = B && num_ref_entries[ 1 ][ RplsIdx[ 1 ] ] > 1 ) ) { |  |
| if( !pps_num_ref_idx_active_override_idc ) |  |
| slice_num_ref_idx_active_override_flag | u(1) |
| if( slice_num_ref_idx_active_override_flag ) |  |
| for( i = 0; i < ( slice_type = = B ? 2: 1 ); i++ ) |  |
| if( num_ref_entries[ i ][ RplsIdx[ i ] ] > 1 ) |  |
| num_ref_idx_active_minus1[ i ] | ue(v) |
| } |  |
| } |  |
| if( partition_constraints_override_enabled_flag ) { |  |
| partition_constraints_override_flag | ue(v) |
| if( partition_constraints_override_flag ) { |  |
| if ( !pps_log2_diff_min_qt_min_cb_luma_plus1 ) |  |
| slice_log2_diff_min_qt_min_cb_luma | ue(v) |
| if( !pps_max_mtt_hierarchy_depth_luma_plus1 ) |  |
| slice_max_mtt_hierarchy_depth_luma | ue(v) |
| if( slice_max_mtt_hierarchy_depth_luma != 0 ) { |  |
| if( !pps_log2_diff_max_bt_min_qt_luma_plus1 ) |  |
| slice_log2_diff_max_bt_min_qt_luma | ue(v) |
| if( !pps_log2_diff_max_tt_min_qt_luma_plus1 ) |  |
| slice_log2_diff_max_tt_min_qt_luma | ue(v) |
| } |  |
| … |  |
| } |  |
| } |  |
| if ( slice_type != I ) { |  |
| … |  |
| if( slice_temporal_mvp_enabled_flag ) { |  |
| if( slice_type = = B && !pps_collocated_from_l0_idc ) |  |
| collocated_from_l0_flag | u(1) |
| if( ( ( collocated_from_l0_flag && NumRefIdxActive[ 0 ] > 1 ) ¦ ¦ ( !collocated_from_l0_flag && NumRefIdxActive[ 1 ] > 1 ) ) && !pps_collocated_ref_idx_plus1 ) |  |
| slice_collocated_ref_idx | ue(v) |
| } |  |
| … |  |
| if( sps_bdof_dmvr_slice_present_flag && !pps_disable_bdof_dmvr_idc ) |  |
| slice_disable_bdof_dmvr_flag | u(1) |
| … |  |
| } |  |
| … |  |
| if( sps_lmcs_enabled_flag ) { |  |
| slice_lmcs_enabled_flag | u(1) |
| if( slice_lmcs_enabled_flag ) { |  |
| if( !pps_lmcs_aps_id_idc ) |  |
| slice_lmcs_aps_id | u(2) |
| if( ChromaArrayType != 0 && !pps_chroma_residual_scale_idc ) |  |
| slice_chroma_residual_scale_flag | u(1) |
| } |  |
| } |  |
| … |  |
| } |  |

slice_num_ref_idx_active_override_flag equal to 1 specifies that the syntax element num_ref_idx_active_minus1[ 0 ] is present for P and B slices and that the syntax element num_ref_idx_active_minus1[ 1 ] is present for B slices. slice_num_ref_idx_active_override_flag equal to 0 specifies that the syntax elements num_ref_idx_active_minus1[ 0 ] and num_ref_idx_active_minus1[ 1 ] are not present. When not present, the value of slice_num_ref_idx_active_override_flag is inferred to be equal to 1pps_num_ref_idx_active_override_idc − 1.

slice_log2_diff_min_qt_min_cb_luma specifies the difference between the base 2 logarithm of the minimum size in luma samples of a luma leaf block resulting from quadtree splitting of a CTU and the base 2 logarithm of the minimum coding block size in luma samples for luma CUs in the current slice. The value of slice_log2_diff_min_qt_min_cb_luma shall be in the range of 0 to CtbLog2SizeY − MinCbLog2SizeY, inclusive. When not present, the value of slice_log2_diff_min_qt_min_cb_luma is inferred as follows:

- If partition_constraints_override_flag is equal to 1, the value of slice_log2_diff_min_qt_min_cb_luma is inferred to be equal to pps_log2_diff_min_qt_min_cb_luma_plus1 – 1.

- Else if slice_type equal to 2 (I), the value of slice_log2_diff_min_qt_min_cb_luma is inferred to be equal to sps_log2_diff_min_qt_min_cb_intra_slice_luma

- Otherwise (slice_type equal to 0 (B) or 1 (P)), the value of slice_log2_diff_min_qt_min_cb_luma is inferred to be equal to sps_log2_diff_min_qt_min_cb_inter_slice.

slice_max_mtt_hierarchy_depth_luma specifies the maximum hierarchy depth for coding units resulting from multi-type tree splitting of a quadtree leaf in the current slice. The value of slice_max_mtt_hierarchy_depth_luma shall be in the range of 0 to CtbLog2SizeY − MinCbLog2SizeY, inclusive. When not present, the value of slice_max_mtt_hierarchy_depth_luma is inferred as follows:

- If partition_constraints_override_flag is equal to 1, the value of slice_ max_mtt_hierarchy_depth_luma is inferred to be equal to pps_max_mtt_hierarchy_depth_luma_plus1 – 1.

- Else if slice_type equal to 2 (I), the value of slice_max_mtt_hierarchy_depth_luma is inferred to be equal to sps_max_mtt_hierarchy_depth_intra_slice_luma

- Otherwise (slice_type equal to 0 (B) or 1 (P)), the value of slice_max_mtt_hierarchy_depth_luma is inferred to be equal to sps_max_mtt_hierarchy_depth_inter_slice.

slice_log2_diff_max_bt_min_qt_luma specifies the difference between the base 2 logarithm of the maximum size (width or height) in luma samples of a luma coding block that can be split using a binary split and the minimum size (width or height) in luma samples of a luma leaf block resulting from quadtree splitting of a CTU in the current slice. The value of slice_log2_diff_max_bt_min_qt_luma shall be in the range of 0 to CtbLog2SizeY − MinQtLog2SizeY, inclusive. When not present, the value of slice_log2_diff_max_bt_min_qt_luma is inferred as follows:

- If partition_constraints_override_flag is equal to 1, the value of slice_log2_diff_max_bt_min_qt_luma is inferred to be equal to pps_log2_diff_max_bt_min_qt_luma_plus1 – 1.

- Else if slice_type equal to 2 (I), the value of slice_log2_diff_max_bt_min_qt_luma is inferred to be equal to sps_log2_diff_max_bt_min_qt_intra_slice_luma

- Otherwise (slice_type equal to 0 (B) or 1 (P)), the value of slice_log2_diff_max_bt_min_qt_luma is inferred to be equal to sps_log2_diff_max_bt_min_qt_inter_slice.

slice_log2_diff_max_tt_min_qt_luma specifies the difference between the base 2 logarithm of the maximum size (width or height) in luma samples of a luma coding block that can be split using a ternary split and the minimum size (width or height) in luma samples of a luma leaf block resulting from quadtree splitting of a CTU in in the current slice. The value of slice_log2_diff_max_tt_min_qt_luma shall be in the range of 0 to CtbLog2SizeY − MinQtLog2SizeY, inclusive. When not present, the value of slice_log2_diff_max_tt_min_qt_luma is inferred as follows:

- If partition_constraints_override_flag is equal to 1, the value of slice_log2_diff_max_tt_min_qt_luma is inferred to be equal to pps_log2_diff_max_tt_min_qt_luma_plus1 – 1.

- Else if slice_type equal to 2 (I), the value of slice_log2_diff_max_tt_min_qt_luma is inferred to be equal to sps_log2_diff_max_tt_min_qt_intra_slice_luma

- Otherwise (slice_type equal to 0 (B) or 1 (P)), the value of slice_log2_diff_max_tt_min_qt_luma is inferred to be equal to sps_log2_diff_max_tt_min_qt_inter_slice.

slice_collocated_ref_idx specifies the reference index of the collocated picture used for temporal motion vector prediction.

When slice_type is equal to P or when slice_type is equal to B and collocated_from_l0_flag is equal to 1, slice_collocated_ref_idx refers to a picture in list 0, and the value of slice_collocated_ref_idx shall be in the range of 0 to NumRefIdxActive[ 0 ] − 1, inclusive.

When slice_type is equal to B and collocated_from_l0_flag is equal to 0, slice_collocated_ref_idx refers to a picture in list 1, and the value of slice_collocated_ref_idx shall be in the range of 0 to NumRefIdxActive[ 1 ] − 1, inclusive.

When slice_collocated_ref_idx is not present, and ( ( collocated_from_l0_flag  &&  NumRefIdxActive[ 0 ] > 1 )  | |
( !collocated_from_l0_flag  &&  NumRefIdxActive[ 1 ] > 1 ) is not true, the value of slice_collocated_ref_idx is inferred to be equal to 0.

When slice_collocated_ref_idx is not present, and ( ( collocated_from_l0_flag  &&  NumRefIdxActive[ 0 ] > 1 )  | |
( !collocated_from_l0_flag  &&  NumRefIdxActive[ 1 ] > 1 ) is true, the value of slice_collocated_ref_idx is inferred to be equal to pps_collocated_ref_idx_plus1 – 1.

It is a requirement of bitstream conformance that the picture referred to by slice_collocated_ref_idx shall be the same for all slices of a coded picture.

It is a requirement of bitstream conformance that the resolutions of the reference picture referred to by slice_collocated_ref_idx and the current picture shall be the same.

slice_disable_bdof_dmvr_flag equal to 1 specifies that neither of bi-directional optical flow inter prediction and decoder motion vector refinement based inter bi-prediction is enabled in the current slice. slice_disable_bdof_dmvr_flag equal to 0 specifies that bi-directional optical flow inter prediction or decoder motion vector refinement based inter bi-prediction may or may not be enabled in the current slice. When slice_disable_bdof_dmvr_flag is not present, the value of slice_disable_bdof_dmvr_flag is inferred to be 0pps_disable_bdof_dmvr_idc - 1.

slice_lmcs_aps_id specifies the adaptation_parameter_set_id of the LMCS APS that the slice refers to. The TemporalId of the APS NAL unit having aps_params_type equal to LMCS_APS and adaptation_parameter_set_id equal to slice_lmcs_aps_id shall be less than or equal to the TemporalId of the coded slice NAL unit.

When present, the value of slice_lmcs_aps_id shall be the same for all slices of a picture. When not present, the value of slice_lmcs_aps_id is inferred to be equal to pps_lmcs_aps_id_idc – 1.

slice_chroma_residual_scale_flag equal to 1 specifies that chroma residual  scaling is enabled for the current slice. slice_chroma_residual_scale_flag equal to 0 specifies that chroma residual scaling is not enabled for the current slice. When slice_chroma_residual_scale_flag is not present and ChromaArrayType is equal to 0, it is inferred to be equal to 0. When slice_chroma_residual_scale_flag is not present and ChromaArrayType is not equal to 0, it is inferred to be equal to pps_chroma_residual_scale_idc – 1.

# Results

The BD-rate numbers for the CTC comparing the proposed solution over VTM-6.1+ (called VTM-6.1+ for convenience, the full reference is VTM-master 1a3e0974 from 20190913 at 10:37 CET) is shown below. For both the anchor and test configurations constant_slice_header_params_enabled_flag is set to 1 for RA, LDB and LDP and to 0 for AI, since for AI a PPS is sent for each picture. Note that the gains do not come from change in quality but purely from saving bits by signaling constant parameters in the PPS instead of in the slice headers.

Table 2	- BD-rate numbers for the proposed solution compared to VTM 6.1+ with constant_slice_header_params_enabled_flag equal to 1 for RA, LDB and LDP and 0 for AI.

| All intra. Y; U; V. Class A1; 0.00%; 0.00%; 0.00%. Class A2; 0.00%; 0.00%; 0.00%. Class B; 0.00%; 0.00%; 0.00%. Class C; 0.00%; 0.00%; 0.00%. Class E; 0.00%; 0.00%; 0.00%. Overall; 0.00%; 0.00%; 0.00%. Class D; 0.00%; 0.00%; 0.00%. Class F; 0.00%; 0.00%; 0.00%. | Random Access. Y; U; V. Class A1; -0.02%; -0.02%; -0.02%. Class A2; -0.02%; -0.02%; -0.01%. Class B; -0.03%; -0.03%; -0.03%. Class C; -0.07%; -0.07%; -0.07%. Class E. Overall; -0.04%; -0.04%; -0.04%. Class D; -0.21%; -0.21%; -0.21%. Class F; -0.07%; -0.07%; -0.07%. |
| --- | --- |
| Low delay B. Y; U; V. Class A1. Class A2. Class B; -0.03%; -0.03%; -0.03%. Class C; -0.08%; -0.07%; -0.07%. Class E; -0.36%; -0.34%; -0.35%. Overall; -0.13%; -0.12%; -0.12%. Class D; -0.22%; -0.21%; -0.21%. Class F; -0.21%; -0.18%; -0.19%. | Low delay P. Y; U; V. Class A1. Class A2. Class B; -0.03%; -0.03%; -0.03%. Class C; -0.08%; -0.07%; -0.07%. Class E; -0.41%; -0.38%; -0.38%. Overall; -0.14%; -0.13%; -0.13%. Class D; -0.21%; -0.20%; -0.21%. Class F; -0.20%; -0.19%; -0.19%. |

For information, the BD-rate results are also shown below for the proposed solution compared to setting the constant_slice_header_params_enabled_flag to 0 for the VTM 6.1+ anchor for all of AI, RA, LDB and LDP.

Table 3	- BD-rate numbers for the proposed solution compared to VTM 6.1+ with constant_slice_header_params_enabled_flag equal to 0 for AI, RA, LDB and LDP.

| All intra. Y; U; V. Class A1; 0.00%; 0.00%; 0.00%. Class A2; 0.00%; 0.00%; 0.00%. Class B; 0.00%; 0.00%; 0.00%. Class C; 0.00%; 0.00%; 0.00%. Class E; 0.00%; 0.00%; 0.00%. Overall; 0.00%; 0.00%; 0.00%. Class D; 0.00%; 0.00%; 0.00%. Class F; 0.00%; 0.00%; 0.00%. | Random Access. Y; U; V. Class A1; -0.02%; -0.02%; -0.02%. Class A2; -0.02%; -0.02%; -0.02%. Class B; -0.04%; -0.05%; -0.05%. Class C; -0.09%; -0.09%; -0.09%. Class E. Overall; -0.05%; -0.05%; -0.05%. Class D; -0.27%; -0.27%; -0.27%. Class F; -0.08%; -0.08%; -0.08%. |
| --- | --- |
| Low delay B. Y; U; V. Class A1. Class A2. Class B; -0.05%; -0.05%; -0.05%. Class C; -0.12%; -0.11%; -0.11%. Class E; -0.61%; -0.57%; -0.58%. Overall; -0.21%; -0.20%; -0.20%. Class D; -0.35%; -0.32%; -0.33%. Class F; -0.33%; -0.30%; -0.31%. | Low delay P. Y; U; V. Class A1. Class A2. Class B; -0.04%; -0.04%; -0.04%. Class C; -0.10%; -0.09%; -0.09%. Class E; -0.49%; -0.45%; -0.46%. Overall; -0.17%; -0.16%; -0.16%. Class D; -0.27%; -0.26%; -0.26%. Class F; -0.24%; -0.23%; -0.23%. |

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).