| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0374-v1 |
| --- | --- |

| Title: | Non-CE3: One directional Intra planar mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hua Yang, Yuwen He 4 Research Way 3rd Floor, Princeton, NJ 08540 USA | Tel: Email: | +1-609-608-0479 Hua.Yang@InterDigital.com Yuwen.He@InterDigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

The current Intra planar mode is essentially an equally weighted combination of both the horizontal and the vertical prediction components. We propose that for coding blocks with large aspect ratio (i.e. ≥ 4), only one direction of either the horizontal or the vertical prediction component is used in Intra planar mode. Simulation result shows +0.04% Y BD-rate change for RA, with similar coding time.

# Introduction

Intra planar mode is an important and commonly used Intra mode in practice. The current Intra planar prediction is essentially an equal weight average of the horizontal and vertical prediction component, as illustrated in Figure 3.

Vertical

prediction

Horizontal

prediction

Bottom left reference sample repeated padding to generate the bottom row

Top right corner reference sample repeated padding to generate the right column

Figure 1. The existing Intra planar prediction mode

The vertical prediction component is a weighted averge of the top row and bottom row reference samples, where the weight used is actually the inverse of the respective prediction distance. Similarly, the horizontal prediction component is also an inverse prediction distance weighted average over the left column and the right column of reference samples. Herein, the bottom row and the right column reference samples are actually padded by the left bottom and the top right corner reference sample, respectively.

However, in practice, if a coding block is of a large aspect ratio, i.e. its width (or height) is much larger than its height (or weight), one direction of prediction component may have much more dominant impact than the other. In this case, equal weights used in the current planar mode may not be appropriate or efficient.

# One directional Intra planar mode

We propose that in case of coding blocks with large aspect ratio, i.e. CB_aspect_ratio = max( CB_width / CB_height, CB_height / CB_width ) ≥ 4, either one of the horizontal or vertical prediction component, but not the equally weighted combination of both, is used as the Intra planar prediction.

# Simulation results

CTC test result with VTM-6.0 is provided [1]. So far, only AI result is provided. Note that In the current test, in addition to the aspect ratio constraint, we also restrict the CU size be larger than or equal to 32 samples. More and complete results will be provided later on.

Table 1. Performance of the proposed one directional Intra planar mode for large aspect ratio coding blocks

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | 0.01% | -0.01% | 101% | 99% |
| Class A2 | 0.00% | 0.01% | -0.02% | 101% | 99% |
| Class B | 0.05% | 0.10% | 0.07% | 101% | 98% |
| Class C | 0.01% | -0.02% | 0.10% | 101% | 98% |
| Class E | 0.06% | 0.12% | 0.19% | 100% | 100% |
| Overall | 0.04% | 0.05% | 0.07% | 101% | 99% |
| Class D | 0.04% | 0.16% | 0.05% | 100% | 97% |
| Class F | 0.02% | 0.03% | -0.04% | 100% | 100% |
| Class TGM | -0.06% | -0.06% | -0.05% | 101% | 97% |

# Conclusion

We propose to use either the horizontal or the vertical prediction component as calculated in the current Intra planar mode to be the Intra planar prediction for large aspect ratio coding blocks. Simulation result shows that the proposed method yields +0.04% Y BD-rate change on RA with similar coding time.

# Reference

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring,"JVET common test conditions and software reference configurations for SDR video", JVET-O2010, Gothenburg, Sweden, July 2019.

# Patent rights declaration(s)

InterDigital Communications, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).