| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0376 |
| --- | --- |

| Title: | Non-CE6: Refined LFNST restriction with MIP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hua Yang, Yuwen He 4 Research Way 3rd Floor, Princeton, NJ 08540 USA | Tel: Email: | +1-609-608-0479 Hua.Yang@InterDigital.com Yuwen.He@InterDigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

In VTM-6.0, LFNST (low-frequency non-separable transform) is allowed to be used together with MIP (Matrix weighted intra prediction) for large coding blocks (i.e. min(width, height) ≥ 16). In this proposal, we propose to always disallow LFNST with MIP for inter slices. Simulation result shows +0.02% Y BD-rate change of RA, with similar encoding and decoding time.

# Introduction

Currently in VVC VTM-6.0, LFNST is allowed to be used with MIP for large coding blocks (i.e. min(width, height) ≥ 16). This restriction may be further improved for better complexity vs coding efficiency trade-off.

Herein, we propose to disallow LFNST with MIP for inter slices, no matter a coding block is large or not. For I-slices, LFNST is still allowed for large coding blocks.

# Simulation results

CTC test result with VTM-6.0 is provided. Herein, LDB test is not needed, as LFNST is disabled for LDB [1]. Since our proposed change is only for inter slices, AI result is the same as the anchor, and thus, omitted.

Table 1. Performance of disallowing LFNST with MIP for inter slices only

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.09% | 0.16% | 0.23% | 99% | 101% |
| Class A2 | 0.01% | -0.02% | -0.07% | 99% | 101% |
| Class B | 0.00% | 0.13% | 0.04% | 99% | 100% |
| Class C | 0.00% | 0.12% | 0.02% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.10% | 0.05% | 99% | 100% |
| Class D | -0.02% | 0.03% | 0.07% | 99% | 99% |
| Class F | -0.02% | 0.00% | -0.04% | 100% | 100% |
| Class TGM | 0.00% | -0.01% | 0.00% | 99% | 100% |

# Conclusion

We propose to disallow LFNST with MIP for inter slices, while for I-slices, we still keep the same as the current VVC standard, which allows LFNST with MIP for larger blocks. Simulation result shows the proposed change yields +0.02% Y BD-rate loss on RA with no coding time difference.

# Reference

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring,"JVET common test conditions and software reference configurations for SDR video", JVET-O2010, Gothenburg, Sweden, July 2019.

# Patent rights declaration(s)

InterDigital Communications, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).