| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0378-v1 |
| --- | --- |

| Title: | AHG12: Removal of dependency between sub-pictures |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kai Zhang, Li Zhang, Hongbin Liu, Zhipin Deng, Jizheng Xu, Yue Wang 8910 University Center Lane San Diego, CA, U.S.A. 92122 | Tel: Email: | +1-858-380-8522 zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

In this contribution, two aspects are proposed to remove the dependency between different sub-pictures.

Aspect #1: The location in the collocated picture, where the TMVP is fetched by an affine constructed candidate, is constrained to be in the same sub-picture.

Aspect #2: Extended integer reference samples required by BDOF/PROF are clipped to be in the same sub-picture.

No performance change is reported under the common test conditions.

# Introduction

The concept of sub-pictures requires that sub-pictures should be independent with each other when a sub-picture is treated as a picture. However, there are two issues resulting in dependency between sub-pictures in VVC.

Issue #1: The location in the collocated picture, where the TMVP is fetched by an affine constructed candidate, may be out of the current sub-picture.

Issue #2: Extended integer reference samples required by BDOF/PROF may be out of the current sub-picture.

# Proposed methods

To remove the dependency between sub-pictures, two aspects are proposed as below.

Aspect #1: The location in the collocated picture, where the TMVP is fetched by an affine constructed candidate, is constrained in the current sub-picture.

Aspect #2: Extended integer reference samples required by BDOF/PROF are clipped into the current sub-picture.

# Changes on the Working Draft

The working draft specified in JVET-O2001-v14 are changed as below. Newly added changes are highlighted in yellow and deleted texts are marked with strikethrough.

## Aspect #1

#### 8.5.5.6 Derivation process for constructed affine control point motion vector merging candidates

Inputs to this process are:

- …

Output of this process are:

- …

…

The fourth (collocated bottom-right) control point motion vector cpMvLXCorner[ 3 ], reference index refIdxLXCorner[ 3 ], prediction list utilization flag predFlagLXCorner[ 3 ] and the availability flag availableFlagCorner[ 3 ] with X being 0 and 1 are derived as follows:

- The reference indices for the temporal merging candidate, refIdxLXCorner[ 3 ], with X being 0 or 1, are set equal to 0.

- The variables mvLXCol and availableFlagLXCol, with X being 0 or 1, are derived as follows:

- If slice_temporal_mvp_enabled_flag is equal to 0, both components of mvLXCol are set equal to 0 and availableFlagLXCol is set equal to 0.

- Otherwise (slice_temporal_mvp_enabled_flag is equal to 1), the following applies:

xColBr = xCb + cbWidth	(8-601)

yColBr = yCb + cbHeight	(8-602)

rightBoundaryPos = subpic_treated_as_pic_flag[ SubPicIdx ] ?
SubPicRightBoundaryPos : pic_width_in_luma_samples − 1	(8-602)

botBoundaryPos = subpic_treated_as_pic_flag[ SubPicIdx ] ?
SubPicBotBoundaryPos : pic_height_in_luma_samples – 1	(8-602)

- If yCb  >>  CtbLog2SizeY is equal to yColBr  >>  CtbLog2SizeY, yColBr is less than or equal to botBoundaryPos and xColBr is less than or equal to rightBoundaryPos, yColBr is less than pic_height_in_luma_samples and xColBr is less than pic_width_in_luma_samples, the following applies:

- The variable colCb specifies the luma coding block covering the modified location given by ( ( xColBr  >>  3 )  <<  3, ( yColBr  >>  3 )  <<  3 ) inside the collocated picture specified by ColPic.

- The luma location ( xColCb, yColCb ) is set equal to the top-left sample of the collocated luma coding block specified by colCb relative to the top-left luma sample of the collocated picture specified by ColPic.

- The derivation process for collocated motion vectors as specified in clause 8.5.2.12 is invoked with currCb, colCb, ( xColCb, yColCb ), refIdxLXCorner[ 3 ] and sbFlag set equal to 0 as inputs, and the output is assigned to mvLXCol and availableFlagLXCol.

- Otherwise, both components of mvLXCol are set equal to 0 and availableFlagLXCol is set equal to 0.

…

## Aspect #2

##### 8.5.6.3.3 Luma integer sample fetching process

Inputs to this process are:

- a luma location in full-sample units ( xIntL, yIntL ),

- the luma reference sample array refPicLXL,

Output of this process is a predicted luma sample value predSampleLXL

The variable shift is set equal to Max( 2, 14 − BitDepthY ).

The variable picW is set equal to pic_width_in_luma_samples and the variable picH is set equal to pic_height_in_luma_samples.

The luma locations in full-sample units ( xInt, yInt ) are derived as follows:

–	If subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 1, the following applies:

xInt = Clip3( SubPicLeftBoundaryPos, SubPicRightBoundaryPos, xInt)	 (8-782)
yInt = Clip3( SubPicTopBoundaryPos, SubPicBotBoundaryPos, yInt)	(8-782)

–	Otherwise:

xInt = Clip3( 0, picW − 1, sps_ref_wraparound_enabled_flag ?	(8-782)
		ClipH( ( sps_ref_wraparound_offset_minus1 + 1 ) * MinCbSizeY, picW, xIntL ) : xIntL )

yInt = Clip3( 0, picH − 1, yIntL )		(8-783)

The predicted luma sample value predSampleLXL is derived as follows:

predSampleLXL = refPicLXL[ xInt ][ yInt ] << shift3	(8-784)

# Experimental Results

There is no coding performance impact under the common test conditions [1].

# Conclusions

Considering the constraints are necessary to guarantee the independency between sub-pictures, it is recommended that the proposed methods be adopted into VVC.

# References

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, Mar. 2019.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).