| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 Oct. 2019 | Document: JVET-P383-v1 |
| --- | --- |

| Title: | Non-CE3: JCCR and LMCS interaction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kui Fan, Li Zhang, Kai Zhang, Yue Wang 8910 University Center Lane San Diego, CA, U.S.A. 92122 | Tel: Email: | +1-858-265-9942 kui.fan@bytedance.com lizhang.idm@bytedance.com zhangkai.video@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

In VVC6, Joint Coding of Chroma Residuals (JCCR) and Luma Mapping with Chroma Scaling (LMCS) could be applied sequentially for the chroma residual coding. When JCCR is enabled, the two chroma residual blocks are firstly derived, followed by LMCS applied twice on each of the two chroma components separately. In this contribution, LMCS is firstly applied once to the joint chroma residual, followed by JCCR to derive the residuals of the two chroma components. With the proposed method, it is asserted that the computational complexity of the per-sample chroma scaling and residual clipping of two chroma blocks could be saved by 50% when JCCR is enabled. Experimental results reportedly show that the BD-rate changes are negligible, i.e., 0.00%/0.02%/0.01%/, 0.00%/0.07%/-0.06%, -0.04%/0.17%/0.21% for Y/Cb/Cr under AI, RA, and LB configurations, respectively.

# Introduction

LMCS consists of two major parts. For the luma component, the in-loop mapping is applied based on adaptive piecewise linear models. As to the chroma components, the per-sample scaling based on the corresponding luma reconstruction samples is applied to the residual blocks.

In JCCR, the residual blocks for chroma components are combined into one joint residual block and only the joint residual block needs to be coded. At the decoder side, the joint chroma residual block is decoded and the residual blocks for chroma components are derived from it.

In VVC6, after the inverse transform, the inverse JCCR is performed before the inverse chrome scaling in LMCS. More specifically, after the joint residual block is generated, the residual blocks of chroma components are derived. After that, the inverse scaling in LMCS is applied to each of chroma residual blocks separately. In other words, the chroma scaling is operated twice for chroma residual blocks, even for JCCR mode.

Figure 1. Decoding flow of chroma blocks with JCCR and LMCS enabled (VVC)

# Proposed methods

In this contribution, the order of JCCR and the chroma scaling in LMCS is exchanged, as shown in Figure 2. At the encoder, JCCR is used to generate the joint chroma residual block, followed by the chroma scaling operation defined in LMCS. At the decoder, the inverse chroma scaling takes place before the inverse JCCR.

With the proposed modification, the chroma scaling operation is only performed once for the JCCR blocks at both encoder and decoder. As a result, the per-sample chroma scaling computation could be saved by 50% when JCCR is enabled. In addition, since the clipping operation for chroma residual values to the range of BitDepth is applied before inverse chroma scaling in LMCS, the number of clipping operations is reduced by 50% with the proposed method.

In addition, in the JCCR modes 1 and 3, the residual of the second component is obtained by right-shifting the residual of the coded component by 1. To improve the computation precision, 1 is added before the shifting operation in the proposed method.

Figure 2. Decoding flow of chroma blocks with JCCR and LMCS enabled (proposed solution)

# Experimental results

The experiments were carried out under “All Intra”, “Random Access”, and “Low Delay B” configurations as defined in the JVET Common Test Condition (CTC) [1] on top of VTM-6.0.

Table I shows the CTC results of the proposed method.

Table 1. CTC test results of the proposed method on top of VTM-6.0

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.08% | 0.06% | 100% | 98% |
| Class A2 | 0.02% | 0.07% | 0.01% | 99% | 99% |
| Class B | 0.00% | 0.02% | 0.05% | 100% | 99% |
| Class C | 0.01% | -0.07% | -0.07% | 99% | 97% |
| Class E | 0.00% | 0.05% | -0.02% | 99% | 98% |
| Overall | 0.00% | 0.02% | 0.01% | 99% | 98% |
| Class D | -0.02% | 0.03% | -0.06% | 100% | 104% |
| Class F | 0.00% | -0.05% | 0.01% | 100% | 104% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.05% | -0.01% | 99% | 100% |
| Class A2 | 0.01% | 0.07% | 0.04% | 100% | 100% |
| Class B | -0.01% | 0.08% | -0.11% | 99% | 100% |
| Class C | -0.01% | 0.08% | -0.11% | 100% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.07% | -0.06% | 100% | 99% |
| Class D | 0.01% | -0.29% | -0.18% | 100% | 105% |
| Class F | -0.01% | -0.12% | 0.01% | 100% | 101% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.31% | 0.18% | 100% | 100% |
| Class C | 0.01% | 0.00% | 0.02% | 99% | 98% |
| Class E | -0.19% | 0.16% | 0.51% | 99% | 100% |
| Overall | -0.04% | 0.17% | 0.21% | 100% | 99% |
| Class D | 0.00% | 0.10% | 0.11% | 100% | 102% |
| Class F | 0.00% | 0.30% | 0.30% | 101% | 98% |

# Conclusion

In this contribution, we propose to exchange the order of LMCS and JCCR. With the proposed method, the chroma scaling operation in LMCS needs to be invoked only once for JCCR mode and the computation is saved accordingly. The performance change is marginal. It is recommended to adopt this method in the next VVC WD and VTM software.

# Reference

[1]	F. Bossen, J. Boyce, X. Li, K. Sühring, V. Seregin, V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 14th Meeting: Geneva, CH, 19–27 Mar. 2019.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).