| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0384-v2 |
| --- | --- |

| Title: | Flexible MMVD candidates |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Franck Galpin, Antoine Robert, Fabrice Le Léannec 975 Avenue des champs blancs 35576 Cesson-Sévigné, FRANCE | Tel: Email: | +33.2.56.85.62.76 franck.galpin@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

It is proposed to unify the MMVD candidate’s derivation with other merge modes (regular merge, CIIP merge).

# Introduction

In VTM-6.0 [1], only the first 2 MMVD candidates are used to derive a MMVD candidates. While it offers a good trade-off complexity/gains at the encoder, having such a heuristic hard coded in the codec may penalize encoder with other heuristics or high complexity encoder (typically used in streaming application).

Typically, an encoder choosing merge candidates based on a predefined motion field (and not a full search on a list of candidates as in VTM) will have a reduced choice of candidates in case of MMVD mode.

# Proposal

We propose to let the freedom to encoder maker to design such heuristic, without impacting current VTM heuristics choice. Impact on VTM-6.0 is minimal:

- the codec design is unchanged (no new mode is introduced),

- the worst case is unchanged

- default heuristics used by the VTM give similar performance and complexity,

- a hard-coded heuristic is transformed into a soft coded heuristic.

The only change is to allow having a MMVD merge index similar to other merge modes, taking into account the current VTM heuristic in the CABAC context derivation.

## Default VTM mode results

|  |  | Random access Main10 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.06% | -0.01% | 102% | 105% |
| Class A2 | 0.02% | 0.10% | 0.04% | 104% | 110% |
| Class B | 0.00% | 0.10% | 0.02% | 97% | 99% |
| Class C | -0.01% | 0.01% | -0.04% | 100% | 96% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.04% | 0.00% | 100% | 102% |
| Class D | -0.05% | -0.05% | -0.14% | 100% | 88% |
| Class F | 0.01% | 0.00% | -0.02% | 101% | 95% |

|  |  | Low delay B Main10 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.09% | -0.14% | 102% | 104% |
| Class C | -0.01% | -0.02% | 0.06% | 104% | 109% |
| Class E | 0.04% | 0.67% | 0.92% | 97% | 94% |
| Overall | 0.01% | 0.20% | 0.19% | 102% | 103% |
| Class D | 0.04% | 0.06% | 0.37% | 100% | 98% |
| Class F | 0.02% | 0.07% | 0.29% | 103% | 104% |

## Results using an extended MMVD search mode

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.05% | -0.01% | 109% | 107% |
| Class A2 | -0.11% | -0.09% | -0.19% | 110% | 109% |
| Class B | -0.10% | -0.07% | -0.16% | 105% | 99% |
| Class C | -0.07% | 0.08% | 0.04% | 106% | 94% |
| Class E |  |  |  |  |  |
| Overall | -0.08% | -0.03% | -0.08% | 107% | 101% |
| Class D | -0.14% | -0.15% | -0.19% | 107% | 88% |
| Class F | -0.04% | -0.11% | -0.08% | 105% | 94% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.03% | -0.02% | -0.43% | 106% | 104% |
| Class C | -0.06% | -0.14% | -0.14% | 109% | 109% |
| Class E | 0.05% | 0.64% | 0.61% | 102% | 94% |
| Overall | -0.02% | 0.10% | -0.07% | 106% | 103% |
| Class D | -0.07% | 0.41% | 0.36% | 104% | 98% |
| Class F | 0.04% | 0.19% | 1.06% | 103% | 105% |

# Specifications changes

# The merge_data syntax 7.3.8.7 is updated as follow:

| merge_data( x0, y0, cbWidth, cbHeight, chType ) { | Descriptor |
| --- | --- |
| if ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_IBC ) { |  |
| if( MaxNumIbcMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( MaxNumSubblockMergeCand > 0 && cbWidth >= 8 && cbHeight >= 8 ) |  |
| merge_subblock_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_subblock_flag[ x0 ][ y0 ] = = 1 ) { |  |
| if( MaxNumSubblockMergeCand > 1 ) |  |
| merge_subblock_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( ( cbWidth * cbHeight ) >= 64 && ( (sps_ciip_enabled_flag && cu_skip_flag[ x0 ][ y0 ] = = 0 && cbWidth < 128 && cbHeight < 128) ¦ ¦ ( sps_triangle_enabled_flag && MaxNumTriangleMergeCand > 1 && slice_type = = B ) ) ) |  |
| regular_merge_flag[ x0 ][ y0 ] | ae(v) |
| if ( regular_merge_flag[ x0 ][ y0 ] = = 1 ){ |  |
| if( sps_mmvd_enabled_flag ) |  |
| mmvd_merge_flag[ x0 ][ y0 ] | ae(v) |
| if( mmvd_merge_flag[ x0 ][ y0 ] = = 1 ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| mmvd_cand_flag[ x0 ][ y0 ] | ae(v) |
| if (mmvd_cand_flag[ x0 ][ y0 ]) { |  |
| mmvd_merge_idx_minus1[ x0 ][ y0] | ae(v) |
| } |  |
| mmvd_distance_idx[ x0 ][ y0 ] | ae(v) |
| mmvd_direction_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| } else { |  |
| if( sps_ciip_enabled_flag && sps_triangle_enabled_flag && MaxNumTriangleMergeCand > 1 && slice_type = = B && cu_skip_flag[ x0 ][ y0 ] = = 0 && ( cbWidth * cbHeight ) >= 64 && cbWidth < 128 && cbHeight < 128 ) { |  |
| ciip_flag[ x0 ][ y0 ] | ae(v) |
| if( ciip_flag[ x0 ][ y0 ] && MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| if( !ciip_flag[ x0 ][ y0 ] && MaxNumTriangleMergeCand > 1 ) { |  |
| merge_triangle_split_dir[ x0 ][ y0 ] | ae(v) |
| merge_triangle_idx0[ x0 ][ y0 ] | ae(v) |
| merge_triangle_idx1[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

- If mmvd_merge_flag[ x0 ][ y0 ] is equal to 1, merge_idx[ x0 ][ y0 ] is inferred to be equal to mmvd_cand_flag[ x0 ][ y0 ]+ mmvd_merge_idx_minus1[ x0 ][ y0].

# Conclusion

The results show that current VTM performance and complexity are unchanged, while allowing other encoding methods or more complex encoder to improve performances. We propose to have this change included in the next VTM version.

# References

- Text of Versatile Video Coding (Draft 6), document JVET-O2001, 15th Meeting: Gothenburg, SE, 3–12 July 2019.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).