| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0385-v1 |
| --- | --- |

| Title: | Non-CE4: MV rounding in sbTMVP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kai Zhang, Li Zhang, Hongbin Liu, Zhipin Deng, Na Zhang, Yue Wang 8910 University Center Lane San Diego, CA, U.S.A. 92122 | Tel: Email: | +1-858-380-8522 zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

In the current design of VVC, MV rounding required by sbTMVP and AMVR are performed in different ways. In this contribution, it is proposed to round a MV in sbTMVP in the same way as that in AMVR. Experimental results reportedly show that the BD-rate changes are 0.00% and -0.03% for RA and LB, respectively.

# Introduction

In the current design of VVC, a MV are rounded to the integer precision required by sbTMVP and AMVR, but in different ways. In sbTMVP, MV is rounded to MV’ toward negative as: MV’ = MV >> 4.In AMVR, MV is rounded to MV’ toward zero as: MV’ = (MV + (MV < 0 ? 8 : 7 ))>>4.

# Proposed methods

Since MVs are rounded to zero in all cases except sbTMVP, it is proposed to round a MV in sbTMVP in the same way as that in AMVR, i.e. toward zero.

# Experimental Results

The proposed method has been implemented on top of VTM-6.0. The simulations were performed following the JVET CTC [1]. Results are tabulated in Table 1. More detail results can be found in the attached spreadsheets.

Table 1: Results of the proposed method

|  | RA |  |  |  |  | PSNR2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.02% | -0.02% | 98% | 99% |  |  |  |  |  |
| Class A2 | 0.03% | 0.06% | 0.04% | 99% | 99% |  |  |  |  |  |
| Class B | 0.00% | 0.00% | -0.02% | 100% | 99% | 0.01% | -0.07% | 0.05% | 98% | 100% |
| Class C | -0.02% | 0.01% | 0.02% | 99% | 100% | -0.04% | 0.02% | -0.19% | 99% | 94% |
| Class E |  |  |  |  |  | -0.07% | 0.18% | 0.12% | 99% | 93% |
| Overall | 0.00% | 0.02% | 0.00% | 99% | 99% | -0.03% | 0.02% | -0.01% | 99% | 96% |
| Class D | -0.03% | -0.06% | 0.03% | 99% | 99% | -0.07% | 0.18% | 0.09% | 101% | 99% |
| Class F | -0.01% | -0.03% | 0.00% | 100% | 101% | -0.02% | -0.10% | 0.57% | 99% | 97% |

# Changes on the Working Draft

The working draft specified in JVET-O2001-v14 are changed as below. The newly added parts are highlighted in yellow.

#### 8.5.5.3 Derivation process for subblock-based temporal merging candidates

Inputs to this process are:

- …

Outputs of this process are:

- ….

…

- For xSbIdx = 0..numSbX − 1 and ySbIdx = 0 .. numSbY − 1, the motion vectors mvLXSbCol[ xSbIdx ][ ySbIdx ] and prediction list utilization flags predFlagLXSbCol[ xSbIdx ][ ySbIdx ] are derived as follows:

- The luma location ( xSb, ySb ) specifying the top-left sample of the current coding subblock relative to the top-left luma sample of the current picture is derived as follows:

xSb  =  xCb + xSbIdx * sbWidth + sbWidth / 2	(8-551)

ySb  =  yCb + ySbIdx * sbHeight + sbHeight / 2	(8-552)

- The location ( xColSb, yColSb ) of the collocated subblock inside ColPic is derived as follows.

    - The following applies:

yColSb = Clip3( yCtb,
	Min( CurPicHeightInSamplesY − 1, yCtb + ( 1  <<  CtbLog2SizeY ) − 1 ), 	ySb + ( (tempMv[1] + (tempMv[1] >= 0 ? 7 : 8 ))   >>  4 ) ) 	(8-553)

  - If subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 1, the following applies:

xColSb = Clip3( xCtb, 
	Min( SubPicRightBoundaryPos, xCtb + ( 1  <<  CtbLog2SizeY ) + 3 ), 	xSb + ( (tempMv[0] + (tempMv[0] >= 0 ? 7 : 8 ))  >>  4 ) )	 (8-554)

  - Otherwise ( subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 0), the following applies:

xColSb = Clip3( xCtb,
	Min( CurPicWidthInSamplesY − 1, xCtb + ( 1  <<  CtbLog2SizeY ) + 3 ), xSb + ( (tempMv[0] + (tempMv[0] >= 0 ? 7 : 8 ))  >>  4 ) )	(8-555)

…

#### 8.5.5.4 Derivation process for subblock-based temporal merging base motion data

Inputs to this process are:

- ...

Outputs of this process are:

- ...

The variable tempMv is set as follows:

tempMv[ 0 ] = 0		(8-558)

tempMv[ 1 ] = 0		(8-559)

The variable currPic specifies the current picture.

When availableFlagA1 is equal to TRUE, the following applies:

- If all of the following conditions are true, tempMv is set equal to mvL0A1:

  - predFlagL0A1 is equal to 1,

  - DiffPicOrderCnt(ColPic, RefPicList[ 0 ][refIdxL0A1]) is equal to 0,

- Otherwise, if all of the following conditions are true, tempMv is set equal to mvL1A1:

  - slice_type is equal to B,

  - predFlagL1A1 is equal to 1,

  - DiffPicOrderCnt(ColPic, RefPicList[ 1 ][refIdxL1A1]) is equal to 0.

The location ( xColCb, yColCb ) of the collocated block inside ColPic is derived as follows.

  - The following applies:

yColCb = Clip3( yCtb,
	Min( CurPicHeightInSamplesY − 1, yCtb + ( 1  <<  CtbLog2SizeY ) − 1 ), yColCtrCb + ( (tempMv[1] + (tempMv[1] >= 0 ? 7 : 8 ))  >>  4 ) ) 	(8-560)

  - If subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 1, the following applies:

xColCb = Clip3( xCtb,
	Min( SubPicRightBoundaryPos, xCtb + ( 1  <<  CtbLog2SizeY ) + 3 ), xColCtrCb + ( (tempMv[0] + (tempMv[0] >= 0 ? 7 : 8 ))  >>  4 ) )	 (8-561)

  - Otherwise ( subpic_treated_as_pic_flag[ SubPicIdx ] is equal to o, the following applies:

xColCb = Clip3( xCtb,
	Min( CurPicWidthInSamplesY − 1, xCtb + ( 1  <<  CtbLog2SizeY ) + 3 ), xColCtrCb + ( (tempMv[0] + (tempMv[0] >= 0 ? 7 : 8 ))  >>  4 ) ) 	(8-562)

# Conclusions

It is recommended MV rounding be unified in VVC.

# References

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, Mar. 2019.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).