| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 Oct. 2019 | Document: JVET-P0392-v3 |
| --- | --- |

| Title: | Non-CE6: Combination of ISP and LFNST |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Santiago De-Luxán-Hernández, Valeri George, Gayathri Venugopal, Jens Brandenburg, Benjamin Bross, Heiko Schwarz, Detlev Marpe, Thomas Wiegand Einsteinufer 37 10587 Berlin, Germany Moonmo Koo, Mehdi Salehifar, Jaehyun Lim, Seung Hwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Email: | santiago.de.luxan@hhi.fraunhofer.de firstname.lastname@hhi.fraunhofer.de moonmo.koo@lge.com mehdi.salehifar@lge.com jaehyun.lim@lge.com seunghwan3.kim@lge.com |
| Source: | Fraunhofer HHI, LG Electronics Inc. |  |  |

_____________________________

# Abstract

In VTM-6.0 LFNST and ISP are mutually exclusive. This contribution proposes to remove this restriction. In case of ISP, the LFNST index is also signalled at the CU-Level and applied to all ISP TUs. Test 1 obtains a gain of 0.19% and 111% and 100% encoder and decoder run-times respectively for AI. In the case of RA, the gain is 0.12% and 102% and 101% encoder and decoder run-times respectively.

# Introduction

In VTM-6.0 the LFNST cannot be utilized in CUs using ISP and accordingly the decoder does not parse the LFNST flags in this case. The present contribution proposes to eliminate this restriction, so that the LFNST can be used in combination with ISP. To do so, a CU-Level signalling approach is suggested. This implies that the same LFNST will be applied to all TUs that have a non-zero CBF and therefore it will be signalled only once for the whole CU. The details of the implementation are listed below:

- ISP is used with LFNST at the CU-Level, provided that the the resulting ISP TUs have at least 4×4 dimensions. In case of subpartitions with one side less than 4, LFNST will not be signalled.

- No new kernels are introduced.

- The already existing kernels have not been retrained.

- All TUs must respect the maximum last position value constraint. Therefore, if one or more TUs violate the condition, then the LFNST will not be used and its flags will not be parsed.

- The condition that establishes that LFNST can only be used if at least there is a significant coefficient other than DC is ignored for the ISP cases.

- If LFNST is used, the primary transform for the ISP TUs is the DCT-II

# Experimental results

According to the common test conditions, the proposed method is evaluated for intra-only (AI) and random-access (RA) configuration with the VTM-6.0 software. The corresponding simulations were conducted on an Intel Xeon cluster (E5-2697A v4, AVX2 on, turbo boost off) with Linux OS and a GCC 7.2.1 compiler. SCC TGM sequences have also been simulated.

Two encoder configurations are provided:

- Test 1: the estimation algorithms used for ISP in VTM-6.0 are extended to the cases of ISP-LFNST1 and ISP-LFNST2.

- Test 2: same normative changes but additional fast encoder strategies are introduced to obtain a faster total encoder run-time.

## Test 1

Table I: Results for the AI configuration

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | 0.04% | 0.25% | 110% | 99% |
| Class A2 | -0.09% | 0.01% | -0.03% | 110% | 101% |
| Class B | -0.15% | -0.12% | -0.16% | 111% | 102% |
| Class C | -0.38% | -0.18% | -0.02% | 112% | 99% |
| Class E | -0.24% | -0.23% | -0.21% | 110% | 101% |
| Overall | -0.19% | -0.10% | -0.05% | 111% | 100% |
| Class D | -0.25% | -0.07% | -0.28% | 111% | 99% |
| Class F | -0.31% | -0.14% | -0.15% | 106% | 101% |
| Class TGM | -0.04% | -0.02% | -0.04% | 102% | 102% |

Table II: Results for the RA configuration

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.07% | -0.02% | 0.03% | 103% | 101% |
| Class A2 | -0.07% | 0.02% | 0.03% | 102% | 102% |
| Class B | -0.11% | -0.06% | -0.15% | 102% | 101% |
| Class C | -0.22% | 0.00% | -0.11% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.12% | -0.02% | -0.07% | 102% | 101% |
| Class D | -0.16% | -0.35% | -0.19% | 101% | 99% |
| Class F | -0.18% | -0.09% | 0.15% | 101% | 98% |
| Class TGM | -0.07% | -0.07% | -0.07% | 100% | 97% |

## Test 2

Table I: Results for the AI configuration

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Table II: Results for the RA configuration

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

# Draft changes

Changes are highlighted in yellow

## LFNST CU-Level signalling

LfnstDcOnly = 1

LfnstZeroOutSigCoeffFlag = 1

transform_tree( x0, y0, cbWidth, cbHeight, treeType )

lfnstWidth = ( treeType = = DUAL_TREE_CHROMA ) ? cbWidth / SubWidthC : cbWidth : ( IntraSubPartitionsSplitType = = ISP_VER_SPLIT) ? cbWidth / NumIntraSubPartitions : cbWidth

lfnstHeight = ( treeType = = DUAL_TREE_CHROMA ) ? cbHeight / SubHeightC : cbHeight : ( IntraSubPartitionsSplitType = = ISP_HOR_SPLIT) ? cbHeight / NumIntraSubPartitions : cbHeight

if( Min( lfnstWidth, lfnstHeight ) >= 4 && sps_lfnst_enabled_flag = = 1 && CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && IntraSubPartitionsSplitType = = ISP_NO_SPLIT && ( !intra_mip_flag[ x0 ][ y0 ] ¦ ¦ Min( lfnstWidth, lfnstHeight ) >= 16 ) && tu_mts_idx[ x0 ][ y0 ] = = 0 && Max( cbWidth, cbHeight ) <= MaxTbSizeY) {

if( ( IntraSubPartitionsSplitType ! = ISP_NO_SPLIT ¦ ¦ LfnstDcOnly ) = = 0 && LfnstZeroOutSigCoeffFlag = = 1 )

lfnst_idx[ x0 ][ y0 ]

}

## Primary transform

The variable trTypeHor specifying the horizontal transform kernel and the variable trTypeVer specifying the vertical transform kernel are derived as follows:

- If cIdx is greater than 0, trTypeHor and trTypeVer are set equal to 0.

- When any of the following conditions is true, trTypeHor and TrTypeVer are set equal to 0:

  - cIdx is greater than 0

  - IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT and lfnst_idx[ xTbY ][ yTbY ] is not equal to 0

- Otherwise, if implicitMtsEnabled is equal to 1, the following applies:

- If IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT or sps_explicit_mts_intra_enabled_flag is equal to 0 and CuPredMode[ 0 ][ xTbY ][ yTbY ] is equal to MODE_INTRA, trTypeHor and trTypeVer are derived as follows:

trTypeHor = ( nTbW >= 4  &&  nTbW <= 16 ) ? 1 : 0	(8-975)

trTypeVer = ( nTbH >= 4  &&  nTbH <= 16 ) ? 1 : 0	(8-976)

- Otherwise (cu_sbt_flag is equal to 1), trTypeHor and trTypeVer are specified in Table 8-15 depending on cu_sbt_horizontal_flag and cu_sbt_pos_flag.

- Otherwise, trTypeHor and trTypeVer are specified in Table 8-14 depending on tu_mts_idx[ xTbY ][ yTbY ].

# Conclusion

TBD

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).