| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0393 |
| --- | --- |

| Title: | On BD rate computation for tools affecting quantization |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | For Information |  |  |
| Author(s) or Contact(s): | Steve Keating Karl Sharman Adrian Browne | Email: | steve.keating@sony.com karl.sharman@sony.com adrian.browne@sony.com |
| Source: | Sony Europe |  |  |

_____________________________

# Abstract

In JVET-N0031 it was proposed to increase the number of points used in rate-distortion curves to improve the accuracy of BD rate computation.  This paper studies LMCS, which modifies the effective quantization, as an example of a tool where the BD rate computation is affected.  The use of a combined YUV metric, as proposed in JVET-N0341, is also studied for the same example.

# Introduction

It has previously been proposed in [1] that the number of QP values currently used in Common Test Conditions [3] can cause inaccurate BD rate numbers to be reported.  This is especially true for tools which modify the effective quantisation either directly, such as Dependent Quantisation, or by modifying the tone curve, such as LMCS.

It has also been proposed in [2] that combined YUV BD rate numbers should be reported and this is important for tools which give significant divergence between luma and chroma PSNRs such as LM_Chroma.

LMCS was chosen as an example as it has both the effect of modifying the effective quantisation points as well as modifying the relative quantisation of luma vs. chroma.

# Results and Analysis

## Number of points in BD rate calculation

The BD-rate figures for the LMCS tool-off test under CTC are shown below.

Using 4 points:

|  | Over VTM6.1 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A1 | 1.88% | 0.42% | 2.72% |
| Class A2 | 3.02% | -0.34% | -0.75% |
| Class B | 0.71% | -0.05% | 0.14% |
| Class C | 0.29% | -2.35% | -2.39% |
| Class E | 0.06% | -3.03% | -3.04% |
| Overall | 1.09% | -1.03% | -0.67% |
| Class D | 0.18% | -2.17% | -2.37% |
| Class F | 0.67% | -1.21% | -1.27% |
| Class TGM | 1.20% | -0.06% | 0.23% |

When the test is run with 16 points instead of 4 some of the figures are notably different:

|  | Over VTM6.1 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Y | U | V | YUV |
| Class A1 | 1.11% | 1.33% | 3.56% | -0.09% |
| Class A2 | 0.92% | 1.44% | 1.03% | -0.51% |
| Class B | 0.52% | -0.08% | 0.21% | -0.33% |
| Class C | 0.29% | -2.27% | -2.38% | -0.21% |
| Class E | 0.06% | -3.14% | -3.01% | -0.35% |
| Overall | 0.56% | -0.59% | -0.30% | -0.30% |
| Class D | 0.17% | -2.16% | -2.19% | -0.27% |
| Class F | 0.39% | -1.23% | -1.22% | 0.04% |
| Class TGM | 0.67% | 0.10% | 0.28% | 0.50% |

On examining the figures for the class A sequences it can be seem that some are quite similar whereas others, such as Tango, CatRobot and DaylightRoad show a larger disparity.

Using 4 points:

|  |  | BD-rate (piecewise cubic) |  |  | BD-rate (cubic) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V |
| Class A1 | Tango2 | 1.48% | 1.26% | 1.71% | 1.29% | 1.56% | 2.09% |
|  | FoodMarket4 | 2.49% | 4.89% | 5.80% | 2.55% | 4.95% | 5.84% |
|  | CampfireParty2 | 1.68% | -4.89% | 0.64% | 2.26% | -5.06% | 0.53% |
| Class A2 | CatRobot1 | 1.05% | 2.01% | 1.45% | 0.80% | 2.20% | 1.68% |
|  | DaylightRoad2 | 3.16% | 2.40% | 1.91% | 1.87% | 2.78% | 2.47% |
|  | ParkRunning3 | 4.84% | -5.43% | -5.59% | 4.84% | -5.43% | -5.59% |

Using 16 points:

|  |  | BD-rate (piecewise cubic) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | YUV |
| Class A1 | Tango2 | -0.51% | 3.77% | 4.05% | 0.02% |
|  | FoodMarket4 | 2.44% | 4.97% | 5.96% | -0.60% |
|  | CampfireParty2 | 1.39% | -4.75% | 0.67% | 0.30% |
| Class A2 | CatRobot1 | -0.75% | 3.99% | 3.40% | 0.10% |
|  | DaylightRoad2 | -1.30% | 5.78% | 5.30% | -0.58% |
|  | ParkRunning3 | 4.82% | -5.45% | -5.62% | -1.04% |

It can also be seen in the 4-point table above that there are disparities in the figures calculated using cubic interpolation and piecewise cubic interpolation; this can provide some indication of the reliability of the 4-point BD rate calculation.  It is suggested that if the disparity is 10% then more points should be tested.

The plots for DaylightRoad help illustrate the difference in the calculation using 4 points vs. 16 points.

Using 4 points (Y):

Using 16 points (Y):

It can be seen that LMCS has the effect of broadening the range of bit-rates for the QP range, which causes the points in the plot to be less aligned; indeed at the higher QPs there is almost a shift of one QP (e.g. QP37  QP36).

Inaccuracy in interpolation is evident in the chroma plots.

Using 4 points (U):

Using 16 points (U):

This has the effect of reporting a chroma loss instead of a chroma gain.  It can also be seen that the plot is non-monotonic around QP34 to QP33 and in some cases it was necessary to modify (swap over) these values for the BD-rate calculation to work (highlighted in yellow in the accompanying spreadsheet), this illustrates one risk in using so many points.  At lower QPs the curve becomes more linear, as expected.

## Use of Combined YUV PSNR

It has been proposed in [2] that combined YUV BD rate numbers should be used and these are shown again below for the 16-point tool-off test:

|  | Over VTM6.1 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Y | U | V | YUV |
| Class A1 | 1.11% | 1.33% | 3.56% | -0.09% |
| Class A2 | 0.92% | 1.44% | 1.03% | -0.51% |
| Class B | 0.52% | -0.08% | 0.21% | -0.33% |
| Class C | 0.29% | -2.27% | -2.38% | -0.21% |
| Class E | 0.06% | -3.14% | -3.01% | -0.35% |
| Overall | 0.56% | -0.59% | -0.30% | -0.30% |
| Class D | 0.17% | -2.16% | -2.19% | -0.27% |
| Class F | 0.39% | -1.23% | -1.22% | 0.04% |
| Class TGM | 0.67% | 0.10% | 0.28% | 0.50% |

In many cases this would seem to be a useful measure but for class A this shows a gain in YUV when the separate Y, U and V calculations each show a loss.  Looking in more detail:

|  |  | BD-rate (piecewise cubic) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | YUV |
| Class A1 | Tango2 | -0.51% | 3.77% | 4.05% | 0.02% |
|  | FoodMarket4 | 2.44% | 4.97% | 5.96% | -0.60% |
|  | CampfireParty2 | 1.39% | -4.75% | 0.67% | 0.30% |
| Class A2 | CatRobot1 | -0.75% | 3.99% | 3.40% | 0.10% |
|  | DaylightRoad2 | -1.30% | 5.78% | 5.30% | -0.58% |
|  | ParkRunning3 | 4.82% | -5.45% | -5.62% | -1.04% |

Taking the example of FoodMarket this may be due to the fact that the final figure is calculated by averaging the PSNR for each POC rather than calculating the average difference for the whole sequence.  For this sequence the PSNR varies as can be seen below:

Averaging these per-POC PSNR values (which is what VTM currently does) gives 40.81dB whereas averaging the (back calculated) per-POC MSE values and then calculating the PSNR gives 41.74dB. This discrepancy is also found in each of the individual Y, U, and V PSNR values.

Looking at the BD rate for each POC it can be seen that the tool has a chroma loss for the first half of the sequence but a chroma gain for the second half of the sequence, whereas the opposite is the case for luma; the combined YUV BD rate does not have such a variation:

For objective measurement of such sequences it might be appropriate for them to be split into sub-parts.

Currently low QP tests only use the first 100 frames of a sequence so for some sequences it would not be valid to make comparisons with CTC tests which use frames from the full length of the sequence.  Consideration should be given to changing low QP so that, for example, every 32nd frame is used across the whole sequence in AI instead of every 8th frame for the first 100 frames; RA could test a group of 17 frames every 64 across the whole sequence (although this would increase the effect of I-frames).

# Conclusion

Analysis of this example tool shows that the current method of BD rate calculation can give unreliable results where the effective quantization points are modified and that using more points in the calculation should improve the reliability; it is likely that this only affects a few tools.  Figures are already reported for two different interpolation methods and if there is, say, a 10% disparity between these that would indicate that more points should be tested.

The use of a combined YUV metric would be useful for evaluating tools which have a significant performance difference between luma and chroma, although the separate Y, U and V metrics are still important for subjective understanding.

Only All Intra tests have been analysed as this keeps the QP unchanged throughout the sequence.

To conclude, it is proposed that:

- If the differences between the reported piecewise cubic and cubic interpolation methods for BD-rate differ by 10%, the tool should be tested at more QP points.

  - It is likely that only a few tools will be affected – AHG13 can confirm.

- YUV BD-rate is added to the spreadsheets.

- If YUV metrics differ from Y, U and V metrics, then they should be justified

  - E.g. monochrome testing

  - E.g. luma/chroma QP/lambda testing

  - E.g. per-POC BD-rates should be examined

- YUV metrics are added to the per-POC lines in the software.

- The impact of sequences that have a change in PSNR, bit-rate or BD-rate from one end to the other should be studied further.

  - A plot of VTM6.1 PSNRs for each POC of each sequence at each QP for AI, RA and LB is included in this contribution.

- To increase diversity in the source data, low QP CTCs should be changed, especially for intra, to cover more than just the first 100 frames:

  - AI: code every 32nd frame of the source data, over all frames.

  - RA: for every 64 frames (=intra period), code the first 17 frames.

# Patent rights declaration(s)

Sony Corporation or Sony Group companies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- “On interpolation for BD rate computation”, JVET-N0331, F. Bossen

- “On reporting combined YUV BD rates”, JVET-N0341, F. Bossen

- “JVET common test conditions and software reference configurations for SDR video”, JVET-M1010, F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring,