| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0394 |
| --- | --- |

| Title: | AHG13: Luma Clipping instead of LMCS |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | For Information |  |  |
| Author(s) or Contact(s): | Steve Keating Karl Sharman Adrian Browne | Email: | steve.keating@sony.com karl.sharman@sony.com adrian.browne@sony.com |
| Source: | Sony Europe |  |  |

_____________________________

# Abstract

This contribution compares the effect of clipping the reconstructed luma with that obtained with LMCS, as commented during the 15th JVET meeting. An overall luma BD-rate gain of 0.08% for the CTC and in particular a gain of 1.58% on the TGM image class was found over the LMCS tool-off anchor at AI. However, losses are reported against VTM6.1 (with LMCS enabled) of 1.01% and 1.64% for luma in AI and RA respectively (but similar gains for chroma).

# Introduction

At the 15th JVET meeting it was commented that LMCS should be tested against clipping, as LMCS includes a form of clipping as part of its function.

# Implementation

Clipping was applied to the reconstructed luma image before loop filtering.  The clip range was taken from the min and max values of the source image; these values were signalled in the bit-stream using APS.  This differs from the adaptive clipping in JEM [1] in the signalling method (in the APS) and that only luma is clipped and only at one point, i.e. the reconstructed luma image.  The VTM6.1 code base was used.

# Results

The results below are for the LMCS tool-off anchor.

All Intra:

|  |  |  | Over LMCS off |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.14% | -0.08% | -0.22% | 101% | 101% |
| Class A2 | -0.01% | 0.01% | 0.01% | 100% | 100% |
| Class B | -0.16% | -0.01% | -0.07% | 100% | 102% |
| Class C | -0.08% | 0.02% | 0.04% | 100% | 103% |
| Class E | 0.02% | -0.03% | 0.06% | 100% | 102% |
| Overall | -0.08% | -0.01% | -0.04% | 100% | 102% |
| Class D | -0.14% | 0.11% | 0.23% | 99% | 104% |
| Class F | -0.43% | -0.10% | -0.13% | 100% | 103% |
| Class TGM | -1.58% | -0.25% | -0.28% | 99% | 101% |

Random Access:

|  |  |  | Over LMCS off |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | -0.14% | 0.29% | 0.28% | 100% | 99% |
| Class C | 0.22% | 0.59% | 0.70% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | 0.45% | 0.99% | 1.33% | 100% | 100% |
| Class F | -0.05% | 0.33% | 0.27% | 100% | 100% |
| Class TGM | -1.01% | -0.24% | -0.26% | 100% | 98% |

The results below are for the VTM6.1 anchor.

All Intra:

|  |  |  | Over VTM6.1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 1.75% | 0.34% | 2.50% | 105% | 99% |
| Class A2 | 3.00% | -0.32% | -0.73% | 104% | 98% |
| Class B | 0.55% | -0.06% | 0.06% | 98% | 99% |
| Class C | 0.21% | -2.33% | -2.35% | 96% | 97% |
| Class E | 0.09% | -3.06% | -2.98% | 94% | 97% |
| Overall | 1.01% | -1.04% | -0.71% | 99% | 98% |
| Class D | 0.03% | -2.06% | -2.15% | 97% | 97% |
| Class F | 0.24% | -1.31% | -1.40% | 98% | 98% |
| Class TGM | -0.41% | -0.32% | -0.05% | 100% | 99% |

Random Access:

|  |  |  | Over VTM6.1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.98% | -2.62% | -2.01% | 93% | 97% |
| Class A2 | 2.35% | -3.91% | -3.86% | 93% | 96% |
| Class B | 1.63% | 1.09% | 0.98% | 97% | 98% |
| Class C | 1.61% | -0.38% | -0.98% | 98% | 99% |
| Class E |  |  |  |  |  |
| Overall | 1.64% | -1.04% | -1.11% | 96% | 98% |
| Class D | 1.36% | 0.30% | 0.12% | 98% | 99% |
| Class F | 1.23% | 0.52% | 0.19% | 100% | 100% |
| Class TGM | -0.46% | -0.03% | -0.01% | 97% | 96% |

Please see JVET-P0393 for a discussion on the BD-rate calculation for tools such as LMCS.

# Conclusion

Simple clipping of the reconstructed luma image gives only a small proportion of the gain of LMCS (even allowing for the limitations of a 4-point BD-rate calculation) but it does give significant gain for screen content.

# Patent rights declaration(s)

Sony Corporation or Sony Group companies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- “Adaptive Clipping in JEM2.0”, JVET-C0040, Galpin, F., Bordes, P., Le Léannec, F.