| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0396 |
| --- | --- |

| Title: | Switching between CABAC context coded bins and bypass coded bins |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Adrian Browne Steve Keating Karl Sharman | Email: | adrian.browne@sony.com steve.keating@sony.com karl.sharman@sony.com |
| Source: | Sony Europe BV |  |  |

_____________________________

# Abstract

This contribution describes a modified coding method for entropy coding, proposing to remove the need to use cabac_zero_word padding and to simplify coefficient coding.  The modification works by switching between CABAC and bypass coding at discrete points.  An implementation is described which uses 5 of these points.  Results are given which show gains in low QP conditions (-0.33% overall for RA), without harming performance in CTC.  In addition results are given for class TGM which show gains of -0.43% for AI in CTC, and -2.49% for RA in low QP.

# Introduction

JVET-O0519[1] introduced the concept of switching between CABAC and bypass coding to avoid the need for padding with cabac_zero_word.  This means that the padding is implicitly coded into the bit stream, and thus splicing of tiles or pictures at the higher (NALU) syntax layer is potentially simpler.

The modification was implemented by introducing symmetric counters in both the encoder and the decoder.  These counters determined if the next context coded bin should be coded using CABAC as requested, or as a bypass/EP bin.  By dynamically controlling the bin to bit ratio, the threshold for adding cabac_zero_word (see section 7.4.3.11 of the specification[2]) would never be exceeded.  In addition, JVET-O0519 removed the limits on context coded bins introduced in JVET-N0280[3] and, JVET-L0274[4], by controlling the bin to bit ratio in the entropy coder rather than during coefficient coding. Therefore, the multiple counters of that latter have been replaced with one counter.

When presented during the 15th JVET meeting in Gothenburg[5] it was noted that the decision path required for switching might increase the length of the critical path in the CABAC engine. In addition it was felt that the proposal allowed switching between CABAC and bypass coding in too many places.

This proposal attempts to resolve the issues identified in Gothenburg whilst maintaining the gains that were achieved.  In addition, following the adoption of JVET-O052[6], the new limits on context coded bins introduced have been removed to allow for bin to bit management in the entropy layer.

# Proposed Implementation

As with the previous proposal this proposal uses symmetric counters in the encoder and decoder.  The role of these counters is to determine if CABAC or bypass encoding is used whenever a context coded bin is encoded. As the cabac_zero_word threshold defined in the threshold allows 323 bins per byte in the stream, the counter counts in thirds of a bin.  Whenever a bin is encoded (CABAC, bypass or TRM) 3 is subtracted from the count.  Whenever a bit is appended to the output stream 4 is added to the count.

If the threshold formula was changed, these numbers could be easily modified.  Though beyond the scope of this proposal, it would be possible to define these values at the profile level.  This would allow different profiles to define different encoder/decoder performance including the possibilities of unlimited or zero CABAC usage.

The proposed implementation uses 5 switch points at which it is possible to switch between CABAC and bypass coding.

- At the beginning of each CTU

- Immediately after encoding a merge skip CU

- Immediately before encoding a sub-TU residual block

- Immediately before encoding a sub-TU transform skip residual block

- Immediately after encoding a TU if CBF=0 (when no sub-TU residual has been encoded).

Previously, at each switch point, the counter is compared to a threshold. This was indicated to have potential ramifications due to CABAC pipelining. To circumvent this, now at each switch point the following is done:

- The decision from the last switch point is applied

- The value of the counter is compared with zero.

  - If the value is greater than zero, the decision is to use CABAC encoding from the next switch point

  - Otherwise, the decision is to use bypass encoding the next switch point

Thus, the decision at each switch point is determined using the state of the counter at the previous switch point.  This decision can be made in parallel to the rest of the entropy encoder, removing the potential for an increase in the critical path identified in the previous proposal.

Using empirical techniques across CTC and low QP, it has been determined that the number of context coded bins encoded between switch points does not exceed (or even approach) 256 even if context coded coefficients are disabled.  The precise number at this point is in a state of flux as the standard is still in draft; the number could be kept at the conservative value of 256, or it could be set lower at a later point.

Thus to implement the counter / switch point mechanism the counter is initialized at the beginning of each slice with enough to allow for two successive switch points as follows:

counter = 3 * (( RawMinCuBits * PicSizeInMinCbsY )  32 – 2 * 256)

The calculation is based upon a component of the cabac_zero_word threshold.

# Results

For CTC:

|  |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM6.1 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.02% | -0.01% | 100% | 101% |
| Class A2 | 0.00% | -0.01% | 0.03% | 100% | 100% |
| Class B | 0.01% | 0.03% | -0.02% | 100% | 100% |
| Class C | -0.05% | -0.20% | -0.09% | 100% | 101% |
| Class E | 0.01% | 0.05% | 0.04% | 100% | 100% |
| Overall | -0.01% | -0.03% | -0.02% | 100% | 100% |
| Class D | -0.04% | -0.08% | -0.24% | 100% | 99% |
| Class F | -0.18% | -0.34% | -0.23% | 100% | 101% |
| Class TGM | -0.43% | -0.41% | -0.38% | 100% | 100% |

|  |  |  | RA |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM6.1 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.06% | 0.07% | 101% | 100% |
| Class A2 | 0.03% | 0.11% | -0.02% | 100% | 100% |
| Class B | -0.01% | 0.19% | 0.09% | 100% | 100% |
| Class C | -0.02% | 0.03% | -0.04% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.10% | 0.03% | 100% | 100% |
| Class D | -0.05% | -0.41% | -0.14% | 100% | 101% |
| Class F | -0.21% | -0.35% | -0.14% | 100% | 100% |
| Class TGM | -0.38% | -0.33% | -0.35% | 101% | 100% |

|  |  |  | LDB |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM6.1 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.27% | -0.22% | 101% | 99% |
| Class C | -0.04% | -0.19% | 0.12% | 100% | 100% |
| Class E | -0.23% | 0.27% | 0.32% | 101% | 101% |
| Overall | -0.07% | 0.12% | 0.03% | 101% | 100% |
| Class D | 0.01% | -0.09% | 0.61% | 100% | 99% |
| Class F | -0.04% | -0.36% | 0.08% | 101% | 101% |
| Class TGM | -0.38% | -0.27% | -0.23% | 100% | 100% |

For low QP:

|  |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM6.1 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.40% | -0.47% | -0.49% | 101% | 104% |
| Class A2 | -0.34% | -0.53% | -0.54% | 102% | 107% |
| Class B | -0.06% | -0.07% | -0.11% | 103% | 107% |
| Class C | -0.03% | -0.10% | -0.09% | 104% | 108% |
| Class E | -0.10% | -0.26% | -0.21% | 100% | 102% |
| Overall | -0.16% | -0.25% | -0.26% | 102% | 106% |
| Class D | -0.04% | -0.13% | -0.11% | 102% | 105% |
| Class F | -1.69% | -0.83% | -0.85% | 100% | 101% |
| Class TGM | -2.24% | -1.83% | -1.81% | 101% | 101% |

|  |  |  | RA |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM6.1 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.36% | -0.40% | -0.46% | 100% | 102% |
| Class A2 | -0.56% | -0.85% | -0.85% | 102% | 104% |
| Class B | -0.29% | -0.34% | -0.40% | 101% | 103% |
| Class C | -0.19% | -0.35% | -0.40% | 102% | 104% |
| Class E |  |  |  |  |  |
| Overall | -0.33% | -0.46% | -0.50% | 101% | 103% |
| Class D | -0.23% | -0.42% | -0.39% | 102% | 103% |
| Class F | -2.46% | -0.86% | -0.74% | 100% | 101% |
| Class TGM | -2.49% | -1.65% | -1.66% | 103% | 101% |

|  |  |  | LDB |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM6.1 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.34% | -0.40% | -0.45% | 101% | 103% |
| Class C | -0.26% | -0.44% | -0.49% | 102% | 105% |
| Class E | -0.22% | -0.08% | -0.14% | 100% | 101% |
| Overall | -0.28% | -0.34% | -0.39% | 101% | 103% |
| Class D | -0.26% | -0.39% | -0.42% | 102% | 105% |
| Class F | -0.78% | -0.62% | -0.71% | 101% | 101% |
| Class TGM | -1.55% | -1.28% | -1.28% | 104% | 101% |

Note that in the software, for readability, no optimisation is made between switch points.

# Specification changes

The following changes are relative to JVET-O2001-vE. They do not include the removal of the coefficient limits, as these changes should be obvious to the editor.

7.2	Specification of syntax functions and descriptors

…

–	ae(v): context-adaptive arithmetic entropy-coded syntax element. The parsing process for this descriptor is specified in clause 9.3.

–	aes(): a switch point in the arithmetic entropy-coded stream. The parsing process for this is specified in clause 9.4.

–	b(8): byte having any pattern of bit string (8 bits). The parsing process for this descriptor is specified by the return value of the function read_bits( 8 ).

…

7.3.2.11	RBSP slice trailing bits syntax

| rbsp_slice_trailing_bits( ) { | Descriptor |
| --- | --- |
| rbsp_trailing_bits( ) |  |
| while( more_rbsp_trailing_data( ) ) |  |
| cabac_zero_word /* equal to 0x0000 */ | f(16) |
| } |  |

7.3.8.2	Coding tree unit syntax

| coding_tree_unit( ) { | Descriptor |
| --- | --- |
| xCtb = ( CtbAddrInRs % PicWidthInCtbsY ) << CtbLog2SizeY |  |
| yCtb = ( CtbAddrInRs / PicWidthInCtbsY ) << CtbLog2SizeY |  |
| switch_point | aes() |
| if( slice_sao_luma_flag ¦ ¦ slice_sao_chroma_flag ) |  |
| sao( xCtb >> CtbLog2SizeY, yCtb >> CtbLog2SizeY ) |  |
| … |  |

7.3.8.5	Coding unit syntax

| … |  |
| --- | --- |
| lfnst_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| else if ( cu_skip_flag[ x0 ][ y0 ] ) |  |
| switch_point | aes() |

7.3.8.10	Transform unit syntax

| … |  |
| --- | --- |
| } |  |
| if ( ! tu_cbf_luma [ x0 ][ y0 ] && ! tu_cbf_cb[ x0 ][ y0 ] && ! tu_cbf_cr[ x0 ][ y0 ]) |  |
| switch_point | aes() |
| } |  |

7.3.8.11	Residual coding syntax

| … |  |
| --- | --- |
| LfnstZeroOutSigCoeffFlag = 0 |  |
| QState = 0 |  |
| for( i = lastSubBlock; i >= 0; i− − ) { |  |
| switch_point | aes() |
| startQStateSb = QState |  |
| xS = DiagScanOrder[ log2TbWidth − log2SbW ][ log2TbHeight − log2SbH ] [ i ][ 0 ] |  |
| … |  |

| … |  |
| --- | --- |
| MaxCcbs = 2 * ( 1 << log2TbWidth ) * ( 1<< log2TbHeight ) |  |
| for( i =0; i <= lastSubBlock; i++ ) { |  |
| switch_point | aes() |
| xS = DiagScanOrder[ log2TbWidth − log2SbSize ][ log2TbHeight − log2SbSize ][ i ][ 0 ] |  |
| yS = DiagScanOrder[ log2TbWidth − log2SbSize ][ log2TbHeight − log2SbSize ][ i ][ 1 ] |  |
| … |  |

7.4.2.1	General NAL unit semantics

3)	One or more cabac_zero_word 16-bit syntax elements equal to 0x0000 may be present in some RBSPs after the rbsp_trailing_bits( ) at the end of the RBSP.

7.4.2.3	Encapsulation of an SODB within an RBSP (informative)

To enable identification of the end of the SODB within the NAL unit by searching the RBSP for the rbsp_stop_one_bit starting at the end of the RBSP,

–	To enable a NAL unit to have a size greater than that of the SODB under some circumstances (using one or more cabac_zero_word syntax elements).

The encoder can produce a NAL unit from an RBSP by the following procedure:

- The RBSP data are searched for byte-aligned bits of the following binary patterns:

'00000000 00000000 000000xx' (where 'xx' represents any two-bit pattern: '00', '01', '10', or '11'),

and a byte equal to 0x03 is inserted to replace the bit pattern with the pattern:

'00000000 00000000 00000011 000000xx',

and finally, when the last byte of the RBSP data is equal to 0x00 (which can only occur when the RBSP ends in a cabac_zero_word), a final byte equal to 0x03 is appended to the end of the data. The last zero byte of a byte-aligned three-byte sequence 0x000000 in the RBSP (which is replaced by the four-byte sequence 0x00000300) is taken into account when searching the RBSP data for the next occurrence of byte-aligned bits with the binary patterns specified above.

7.4.3.11	RBSP slice trailing bits semantics

cabac_zero_word is a byte-aligned sequence of two bytes equal to 0x0000.

Let NumBytesInVclNalUnits be the sum of the values of NumBytesInNalUnit for all VCL NAL units of a coded picture.

Let BinCountsInNalUnits be the number of times that the parsing process function DecodeBin( ), specified in clause TBD, is invoked to decode the contents of all VCL NAL units of a coded picture.

Let the variable RawMinCuBits be derived as follows:

RawMinCuBits = MinCbSizeY * MinCbSizeY *
				( BitDepthY + 2 * BitDepthC / ( SubWidthC * SubHeightC ) )	(7-46)

The value of BinCountsInNalUnits shall be less than or equal to ( 32 ÷ 3 ) * NumBytesInVclNalUnits + ( RawMinCuBits * PicSizeInMinCbsY )  32.

NOTE – The constraint on the maximum number of bins resulting from decoding the contents of the coded slice NAL units can be met by inserting a number of cabac_zero_word syntax elements to increase the value of NumBytesInVclNalUnits. Each cabac_zero_word is represented in a NAL unit by the three-byte sequence 0x000003 (as a result of the constraints on NAL unit contents that result in requiring inclusion of an emulation_prevention_three_byte for each cabac_zero_word).

9.3.2	Initialization process

9.3.2.1	General

Outputs of this process are initialized CABAC internal variables.

If the CTU is the first CTU of a brick, the 32 bit decoding engine register availableCount must be initialized as follows:

Let the variable RawMinCuBits be derived as follows:

RawMinCuBits = MinCbSizeY * MinCbSizeY *
				( BitDepthY + 2 * BitDepthC / ( SubWidthC * SubHeightC ) )

Let the variable BrickSizeInMinCbsY be defined as

BrickWidthInMinCbsY = brick_width_in_luma_samples / MinCbSizeX

BrickHeightInMinCbsY = brick_height_in_luma_samples / MinCbSizeY

BrickSizeInMinCbsY = BrickWidthInMinCbsY * BrickHeightInMinCbsY

The initial value of availableBinFractionalCount is initialized as:

3 * (  ( RawMinCuBits * BrickSizeInMinCbsY )   32 – 2*256).

The initial values of nextForceBypassFlag and forceBypassFlag are both seq equal to 0.

9.3.4.3	Arithmetic decoding process

9.3.4.3.1	General

If bypassFlag is equal to 1 or forceBypassFlag is equal to 1, DecodeBypass( ) as specified in subclause 9.3.4.3.4 is invoked.

–	Otherwise, if bypassFlag is equal to 0 and forceBypassFlag is equal to 0, ctxTable is equal to 0, and ctxIdx is equal to 0, DecodeTerminate( ) as specified in subclause 9.3.4.3.5 is invoked.

–	Otherwise (bypassFlag is equal to 0, forceBypassFlag is equal to 0 and ctxTable is not equal to 0), DecodeDecision( ctxTable, ctxIdx ) as specified in subclause 9.3.4.3.2 is invoked.

9.3.4.3.3	Renormalization process in the arithmetic decoding engine

Inputs to this process are bits from slice data and the variables ivlCurrRange and ivlOffset.

Outputs of this process are the updated variables ivlCurrRange and ivlOffset.

A flowchart of the renormalization is shown in Figure 9-6. The current value of ivlCurrRange is first compared to 256 and further steps are specified as follows:

–	If ivlCurrRange is greater than or equal to 256, decrement availableBinFractionalCount by 3. No renormalization is needed and the RenormD process is finished;

–	Otherwise (ivlCurrRange is less than 256), the renormalization loop is entered. Within this loop, the value of ivlCurrRange is doubled, i.e. left-shifted by 1, and a single bit is shifted into ivlOffset by using read_bits( 1 ) and availableBinFractionalCount is incremented by 4.

The bitstream shall not contain data that result in a value of ivlOffset being greater than or equal to ivlCurrRange upon completion of this process.

9.3.4.3.4	Bypass decoding process for binary decisions

…

First, the value of ivlOffset is doubled, i.e. left-shifted by 1 and a single bit is shifted into ivlOffset by using read_bits( 1 ). Then, the value of ivlOffset is compared to the value of ivlCurrRange and further steps are specified as follows:

–	If ivlOffset is greater than or equal to ivlCurrRange, the variable binVal is set equal to 1 and ivlOffset is decremented by ivlCurrRange.

–	Otherwise (ivlOffset is less than ivlCurrRange), the variable binVal is set equal to 0.

Then availableBinFractionalCount is incremented by 1.

…

9.4	CABAC switch point

9.4.1	General

This process is invoked when the descriptor of a syntax element in the syntax tables is equal to aes().

There are no specific inputs or outputs to this syntax element.

When invoked, first forceBypassFlag is set equal to nextForceBypassFlag.

Then nextForceBypassFlag is set equal to 0 if availableBinFractionalCount is less than 0, or 1 otherwise.

# Conclusion

A modified coding technique is described for entropy coding. The technique essentially rations the use of CABAC bins to ensure that the constraint given in section 7.4.3.11 of the specification is never broken.

An implementation is described which exhibits no net loss in CTC and gains at low QP with gains across screen content for all tested QP.

# Patent rights declaration(s)

Sony Corporation or Sony Group companies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- “Slice/tile level CABAC zero-word constraints”, JVET-O0517, A. Browne, S. Keating and K. Sharman.

- “Versatile Video Coding (Draft 5)”, JVET-N1001-v8, B. Bross, J. Chen, and S. Liu.

- “CE8: Residual Coding for Transform Skip Mode (CE8-4.3a, CE8-4.3b, CE8-4.4a, and CE8-4.4b)”, JVET-N0280, B. Bross, T. Nguyen, H. Schwarz, D. Marpe, T. Wiegand.

- “CE7: Transform coefficient coding with reduced number of regular coded bins (tests 7.1.3a, 7.1.3b)”, JVET-L0274, H.Schwarz, T.Nguyen, D.Marpe, T.Wiegand, M.Karczewicz, M.Coban, J.Dong

- “Meeting Report of the 15th JVET Meeting (Gothenburg, SE, 3–12 July 2019)”, JVET-O2000, G. J. Sullivan and J.-R. Ohm.

- “CE7-1: TB-level constraints on context-coded bins for coefficient coding”, JVET-O0052, T.-D. Chuang, C.-Y. Chen, S.-T. Hsiang, Y.-W. Huang, S.-M. Lei, M. Coban, M. Karczewicz.

- “JVET common test conditions and software reference configurations”, JVET-M1010, F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring