| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0397-v3 |
| --- | --- |

| Title: | CE7-related: Simplified two-pass transform-skip residual coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mohammed Golam Sarwer, Ruling Liao, Jiancong Luo, Yan Ye 525 Almanor Ave, Sunnyvale, CA | Tel: Email: | +1 (408) 858-6780 m.sarwer@alibaba-inc.com |
| Source: | Alibaba Group |  |  |

_____________________________

# Abstract

This contribution proposes a two-pass transform skip (TS) residual coding method. The proposed method reduces the worst case number of checking of whether to perform bypass or context coding from 8 per coefficient position to 1 per coefficient position. Similar to VVC6 transform residual coding, after reaching to the maximum limit of the context coded bin, the proposed method switches to the Rice-Golomb coding. Following results are reported.

- CTC QP:

  - Overall: 0.00% (AI), 0.01% (RA), -0.05% (LB) luma BD-rate

  - Class SCC: 0.25% (AI), 0.17% (RA), -0.03% (LB) luma BD-rate

- Low QP:

  - Class SCC: -0.69% (AI), -1.46% (RA), -0.46% (LB) luma BD-rate

# Problem statement

In VVC 6, the transform skip (TS) residual levels of a coefficient group (CG) are coded in three passes over the scan positions as follows:

- Pass 1: Following flags are signalled

  - sig_coeff_flag

  - coeff_sign_flag

  - greater 1 flag (abs_level_gtx_flag[0])

  - parity (par_level_flag) flag

- Pass 2:

  - greater than 3 flag (abs_level_gtx_flag[1])

  - greater than 5 flag (abs_level_gtx_flag[2])

  - greater than 7 flag (abs_level_gtx_flag[3])

  - greater than 9 flag (abs_level_gtx_flag[4])

- Pass 3: Rice-Golomb coding of remaining absolute level (abs_remainder)

This contribution solves the following issues of VVC 6 TS residual coding.

- In TS residual coding, before coding each flag of each coefficient, it is required to check if the number of context coded bins are available or not. If the context coded bins are available, the flag is context coded. Otherwise, the flag is by-pass coded. Since there are total 8 flags, in worst case scenario, the number of checking is 8 per coefficient. This worst case checking is significantly higher than that of the regular transform coefficient coding, which performs only 1 such checking 1 per coefficient.

- The syntax structure of the by-pass coded coefficient of the transform and TS residual coding is different. In transform residual coding, once the context coded bins exceed the limit, the rest of the coefficients that have not been coded yet are coded using the syntax dec_abs_level using only Rice-Golomb coding. However, in case of TS residual coding, when the context coding bins have been used up, the coefficients that have not been coded yet are coding using the remaining flags followed by Rice-Golomb Coding of the remainder.  For instance, assume the number of context coded bins reached to maximum limit before coding of sig_coeff_flag of a coefficient. For that coefficient (and all subsequent coefficients), following compares the syntax of the transform residual and TS residual coding of VVC 6.

- TS residual coding: sig sign  gt1 parity gt3 gt5 gt7  gt9 abs_remainder

- Transform residual coding: dec_abs_level sign

# Proposed method

In the proposed method, a CG is coded in two passes. In the first pass, all of the flags are context coded and in the second pass remainder values are Rice-Golomb coded. In the first pass, instead of the checking the availability of the context coded bins at each flag, the proposed method checks the availability of the context coded bins once at each coefficient position. At the beginning of each coefficient coding, a condition is checked if the remaining number of context coded bins greater than or equal to 8 or not. If the remaining number of context coded bins is greater than or equal to 8, all of the flags are context coded; otherwise, pass 1 is skipped and remainder value is coded using Rice-Golomb code.

The pseudo-code of the proposed method is given below.

Pass 1: for(n = 0; n <= numSbCoeff  - 1 && remainingCtxBin >= 8; n++ )

decode sig_coeff_flag

decode coeff_sign_flag

decode abs_level_gtx_flag[0] // gt1 flag

decode par_level_flag

decode abs_level_gtx_flag[1] // gt3

decode abs_level_gtx_flag[2] // gt5

decode abs_level_gtx_flag[3] // gt7

decode abs_level_gtx_flag[4] // gt9

iFirstPassBypassPos = n;

Pass 2 (a): for(n = 0; n < iFirstPassBypassPos; n++ )

decode abs_remainder_using_RG_Coding

Pass 2 (b): for(n = iFirstPassBypassPos; n <= numSbCoeff  - 1; n++ )

decode abs_remainder_using_RG_Coding

decode coeff_sign_flag

In the proposed method, a coefficient can be classified into one of the following two categories:

- The decoded coefficients before reaching to the maximum limit of the context coded bins. In the above pseudo-code, the coefficients which satisfy n < iFirstPassBypassPos condition are classified into this category. For these coefficients, all of the flags are context coded and the minimum level value (baseLevel) is 10.

- The decoded coefficients after reaching to the maximum limit of context coded bins. In the above pseudo-code, the coefficients which do not satisfy n < iFirstPassBypassPos condition are classified into this category. For these coefficients, none of the flags are signalled and the minimum level value (baseLevel) is 0.

Similar to VVC draft 6, the Rice parameter is derived based on sum of top and left coefficients, however, sum is adjusted based on the minimum level ( baseLevel). In the above pseudo-code, if n < iFirstPassBypassPos , baseLevel = 10; otherwise baseLevel = 0. Assume, locSumAbs is the sum of top and left coefficients. The highlighted portion is the additional adjustment ( as compared to VVC draft 6) of  locSumAbs  value based on the base level.

locSumAbs  -= (2 * baseLevel)

locSumAbs = clip3 (0  , 31, locSumAbs)

Similar to VVC6, the proposed method also selects Rice parameter value from the predefined look-up table.  However, the proposed method uses same look-up table for both TS and transform residual coding whereas VVC6 uses two separate look-up tables (one for transform residual coding and another for TS residual coding). In the proposed method, both transform and TS residual coding method share same look-up table and that look-up table is same as VVC 6 transform residual coding look-up table. The entries of the look-up table is given below.

const uint32_t auiGoRicePars[32] =

{

0, 0, 0, 0,

0, 0, 0, 1, 1, 1,

1, 1, 1, 1, 2, 2, 2, 2, 2, 2, 2, 2,

2, 2, 2, 2, 2, 2, 3, 3, 3, 3

};

Similar to VVC6 transform residual coding, after reaching to the maximum context coded bin limit, the proposed method switches to the Rice-Golomb coding without signaling any flag. For instance, assume the number of context coded bins reached to maximum limit before coding of sig_coeff_flag of a coefficient. For that coefficient (and all subsequent coefficients), following compares the syntax of the transform residual and TS residual coding of VVC 6 and proposed method.

- TS residual coding: sig sign  gt1 parity gt3 gt5 gt7  gt9 abs_remainder

- Transform residual coding: dec_abs_level sign

- Proposed TS residual coding: abs_remainder  sign

# Simulation results

The proposed methods are tested under both CTC and low QP (2, 7, 12, 17) settings. Table 1 and Table 2 shows the results of the proposed method under CTC and low QP setting.

Table 1:  results of proposed method under CTC QP

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.01% | -0.01% | 100% | 101% |
| Class A2 | 0.00% | 0.00% | 0.02% | 100% | 101% |
| Class B | 0.00% | -0.02% | -0.04% | 101% | 102% |
| Class C | 0.01% | -0.02% | -0.01% | 100% | 104% |
| Class E | 0.00% | 0.08% | 0.00% | 101% | 104% |
| Overall | 0.00% | 0.01% | -0.01% | 100% | 102% |
| Class D | 0.01% | 0.01% | -0.07% | 99% | 106% |
| Class F | 0.10% | 0.07% | 0.03% | 101% | 99% |
| Class SCC | 0.25% | 0.30% | 0.29% | 100% | 99% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.01% | 0.01% | 100% | 100% |
| Class A2 | 0.02% | 0.11% | 0.07% | 101% | 100% |
| Class B | -0.02% | 0.04% | 0.09% | 101% | 102% |
| Class C | 0.01% | -0.01% | 0.04% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.03% | 0.06% | 101% | 101% |
| Class D | -0.04% | -0.17% | -0.16% | 100% | 101% |
| Class F | 0.10% | 0.05% | 0.19% | 100% | 100% |
| Class SCC | 0.17% | 0.14% | 0.20% | 99% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.04% | 0.11% | -0.32% | 101% | 101% |
| Class C | 0.01% | -0.01% | -0.03% | 101% | 104% |
| Class E | -0.13% | 0.07% | -0.14% | 102% | 102% |
| Overall | -0.05% | 0.06% | -0.17% | 101% | 102% |
| Class D | -0.04% | 0.10% | -0.18% | 101% | 107% |
| Class F | 0.16% | 0.11% | 0.54% | 99% | 99% |
| Class SCC | -0.03% | 0.01% | 0.09% | 100% | 104% |

Table 2: results of proposed method under low QP setting (results of RA camera capture contents will be updated)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.02% | 0.04% | 100% | 97% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 98% |
| Class B | -0.01% | 0.00% | 0.01% | 100% | 98% |
| Class C | -0.04% | -0.03% | -0.02% | 98% | 99% |
| Class E | -0.06% | -0.04% | -0.05% | 100% | 98% |
| Overall | -0.02% | -0.01% | -0.01% | 99% | 98% |
| Class D | -0.04% | -0.04% | -0.03% | 99% | 101% |
| Class F | -1.00% | -0.28% | -0.28% | 99% | 98% |
| Class SCC | -0.69% | -0.40% | -0.38% | 98% | 97% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.01% | 100% | 101% |
| Class C | -0.02% | -0.02% | -0.02% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.06% | -0.04% | 0.00% | 99% | 101% |
| Class F | -1.33% | -0.44% | -0.35% | 100% | 109% |
| Class SCC | -1.46% | -0.39% | -0.41% | 100% | 113% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.01% | 0.00% | 100% | 98% |
| Class C | -0.01% | 0.01% | -0.03% | 100% | 100% |
| Class E | -0.07% | 0.00% | 0.01% | 100% | 99% |
| Overall | -0.03% | 0.00% | -0.01% | 100% | 99% |
| Class D | -0.04% | 0.05% | -0.08% | 99% | 100% |
| Class F | -0.34% | -0.18% | -0.19% | 100% | 100% |
| Class SCC | -0.46% | -0.19% | -0.13% | 100% | 98% |

# Additional results

The proposed method is also tested with maximum number of context coded bins 1.75 per coefficient. Table 3 shows the simulation results under CTC QP condition. In Table 3, the maximum number of context coded bins in anchor is 2.0 per coeff whereas in the test it is 1.75 per coeff.

Table 3: proposed method with number of max context coded bins 1.75 per coefficient CTC QP ( Anchor is VTM6.0 where number of context coded bins is 2.0 per coefficient)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | 0.25% | 0.16% | 0.23% | 101% | 102% |
| Class SCC | 0.53% | 0.55% | 0.52% | 101% | 100% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | 0.21% | 0.38% | 0.30% | 101% | 103% |
| Class SCC | 0.34% | 0.39% | 0.35% | 101% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | 0.02% | -0.08% | 0.66% | 101% | 103% |
| Class SCC | 0.11% | 0.25% | 0.23% | 101% | 106% |

# Conclusion

The proposed method reduces the worst case number of checking of whether to perform bypass or context coding from 8 per coefficient position to 1 per coefficient position. Experimental results show that 0.25% (AI), 0.17% (RA), -0.03% (LB) luma BD-rate of SCC sequences under CTC test conditions. In low QP test settings, the proposed method can achieve -0.69% (AI), -1.46% (RA), -0.46% (LB) luma BD-rate of SCC sequences.

# Patent rights declaration(s)

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).