| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0402-v1 |
| --- | --- |

| Title: | CE7-related: Unification of CCB count method between transform residual and transform skip residual coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Y. Chen, F. Le Léannec, T. Poirier, K.Naser | Tel: Email: | +33(0)256856325 ya.chen@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

In order to unify the context-coded bins (CCB) count method between transform residual and transform skip (TS) residual coding, this contribution proposes: 1) align the number of CCB per coefficient for transform residual coding and TS residual coding by assigning 1.75 bin/coefficient at TB-level, and exclude coeff_sign_flag from CCB count in TS residual coding; 2) on top of Test 1, assign another CCB_SIGN (0.5 bin/coefficient at TB-level) to restrict the maximum number of context coded bins for coeff_sign_flag. The proposed Test 1 and Test2 reportedly provides the following luma BD-rates results, respectively, versus the VTM6.0:

- Test 1: Align CCB to 1.75 + exclude coeff_sign_flag from CCB count in TS residual coding

  - CLF: -0.06% AI, -0.10% RA, -0.05% LDB

  - TGM: -0.11% AI, -0.09% RA, -0.02% LDB

- Test 2: Test 1 + CCB_SIGN to 0.5 for coeff_sign_flag

  - CLF: -0.05% AI, -0.06% RA, 0.05% LDB

  - TGM: -0.09% AI, -0.07% RA, -0.09% LDB

# Introduction

In VTM6.0[1], the constraint on the maximum number of context-coded bins (CCB) is applied for the transform residual and transform skip (TS) residual coding.

According to VVC Draft, it limits the maximum number of CCB per coefficient for luma and chroma coefficients to be 1.75 bin/coefficient at TB-level, coefficient flags such as sig_coeff_flag, abs_level_gtx_flag[0] (gt1_flag), par_level_flag, and abs_level_gtx_flag[1] (gt3_flag) are included in CCB count in transform residual coding.

On the other hand, TS residual coding sets this maximum number of CCB limit to 2 bin/coefficient at TB-level, coefficient flags such as sig_coeff_flag, coeff_sign_flag, abs_level_gtx_flag[0], par_level_flag, and abs_level_gtx_flag[j] (1≤j≤4, gt3, gt5, gt7, and gt9 flags) are included in CCB count in TS residual coding. There are some differences: 1) the maximum number of CCB per coefficient; 2) whether coeff_sign_flag is included in CCB count.

Table 1 shows two residual coding structures of the two residual coding modes in VTM6.0.

Table 1. Two residual coding structures of transform residual and TS residual coding in VTM6.0

| Transform residual coding | TS residual coding |
| --- | --- |
| residual_coding( ) { | residual_ts_coding( ) { |
| MaxCcbs = (TbWidth_Zosize * TbHeight_Zosize *28) >> 4 | MaxCcbs = 2 * TbWidth * TbHeight |
| for( SubBlock loop ) { | for( SubBlock loop ) { |
| coded_sub_block_flag | coded_sub_block_flag |
| for( Pass 1 && MaxCcbs ≥ 4 ) { | for( Pass 1) { |
| sig_coeff_flag | sig_coeff_flag |
| abs_level_gtx_flag[0] | MaxCcbs –= 1 |
| par_level_flag Count CCB | coeff_sign_flag Count CCB |
| abs_level_gtx_flag[1] | MaxCcbs –= 1 |
| MaxCcbs –= 4 | abs_level_gtx_flag[0] |
| } | MaxCcbs –= 1 |
| for( Pass 2-1 ) { | par_level_flag |
| abs_remainder | MaxCcbs –= 1 |
| } | } |
| for( Pass 2-2 ) { | for( Pass 2 ) { |
| dec_abs_level | for( j=1; j<5; j++ ) { |
| } | abs_level_gtx_flag[j] |
| for( Pass 3 ) { | MaxCcbs –= 1 |
| coeff_sign_flag | } |
| } | } |
| } | for( Pass 3 ) { |
| } | abs_remainder |
|  | } |
|  | } |
|  | } |

# CE description

In this contribution, Test 1 proposed to align the number of CCB per coefficient at TB level for transform residual coding and TS residual coding schemes by setting the limit to 1.75 bin/coefficient, and exclude coeff_sign_flag from CCB count in TS residual coding as shown in Table 2. As a result, it aligns the CCB count method of TS residual coding to the transform residual coding.

On the top of Test 1, Test 2 assign another maximum CCB_SIGN for coeff_sign_flag to control the CABAC throughput of coeff_sign_flag independently. In this proposal, the maximum number of CCB_SIGN is set the limit to 0.5 bin/coefficient at TB level as shown in Table 3.

Table 2. Two residual coding structures of transform residual and TS residual coding in Test 1 of this proposal

| Transform residual coding | TS residual coding |
| --- | --- |
| residual_coding( ) { | residual_ts_coding( ) { |
| MaxCcbs = (TbWidth_Zosize * TbHeight_Zosize *28) >> 4 | MaxCcbs = 2 * TbWidth * TbHeight MaxCcbs = (TbWidth * TbHeight *28) >> 4) |
| for( SubBlock loop ) { | for( SubBlock loop ) { |
| coded_sub_block_flag | coded_sub_block_flag |
| for( Pass 1 && MaxCcbs ≥ 4 ) { | for( Pass 1) { |
| sig_coeff_flag | sig_coeff_flag Count CCB |
| abs_level_gtx_flag[0] | MaxCcbs –= 1 |
| par_level_flag Count CCB | coeff_sign_flag |
| abs_level_gtx_flag[1] | MaxCcbs –= 1 |
| MaxCcbs –= 4 | abs_level_gtx_flag[0] |
| } | MaxCcbs –= 1 |
| for( Pass 2-1 ) { | par_level_flag |
| abs_remainder | MaxCcbs –= 1 |
| } | } Count CCB |
| for( Pass 2-2 ) { | for( Pass 2 ) { |
| dec_abs_level | for( j=1; j<5; j++ ) { |
| } | abs_level_gtx_flag[j] |
| for( Pass 3 ) { | MaxCcbs –= 1 |
| coeff_sign_flag | } |
| } | } |
| } | for( Pass 3 ) { |
| } | abs_remainder |
|  | } |
|  | } |
|  | } |

Table 3. Two residual coding structures of transform residual and TS residual coding in Test 2 of this proposal

| Transform residual coding | TS residual coding |
| --- | --- |
| residual_coding( ) { | residual_ts_coding( ) { |
| MaxCcbs = (TbWidth_Zosize * TbHeight_Zosize *28) >> 4 | MaxCcbs = 2 * TbWidth * TbHeight MaxCcbs = (TbWidth * TbHeight *28) >> 4) |
|  | MaxCcbs_sign = (TbWidth * TbHeight) >> 1 |
| for( SubBlock loop ) { | for( SubBlock loop ) { |
| coded_sub_block_flag | coded_sub_block_flag |
| for( Pass 1 && MaxCcbs ≥ 4 ) { | for( Pass 1) { |
| sig_coeff_flag | sig_coeff_flag Count CCB |
| abs_level_gtx_flag[0] | MaxCcbs –= 1 |
| par_level_flag Count CCB | coeff_sign_flag Count CCB_SIGN |
| abs_level_gtx_flag[1] | MaxCcbs_sign –= 1 |
| MaxCcbs –= 4 | abs_level_gtx_flag[0] |
| } | MaxCcbs –= 1 |
| for( Pass 2-1 ) { | par_level_flag |
| abs_remainder | MaxCcbs –= 1 |
| } | } Count CCB |
| for( Pass 2-2 ) { | for( Pass 2 ) { |
| dec_abs_level | for( j=1; j<5; j++ ) { |
| } | abs_level_gtx_flag[j] |
| for( Pass 3 ) { | MaxCcbs –= 1 |
| coeff_sign_flag | } |
| } | } |
| } | for( Pass 3 ) { |
| } | abs_remainder |
|  | } |
|  | } |
|  | } |

# Experimental results

The proposed method was integrated into the VTM6.0 reference software [2]. The coding results generated by the proposed method were evaluated against the anchor results generated by the unmodified VTM6.0 reference software under the CE7 test conditions [3]. Table 4 and Table 5 summarize the experimental results for Test 1 and Test 2 of this proposal over standard QPs 22, 27, 32, and 37, respectively. Table 6 and Table 7 summarize the experimental results for Test 1 and Test 2 of this proposal over low QPs 2, 7, 12, and 17, respectively. Please note the run time results may not be accurate in the tables. The detailed simulation results can be found in the attached spreadsheets.

Table 4. Summary of the average BD bitrate results of Test 1 over standard QPs 22, 27, 32, and 37

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.01% | 0.00% | 105% | 104% |
| Class A2 | 0.00% | 0.00% | 0.01% | 104% | 104% |
| Class B | 0.00% | 0.00% | -0.05% | 104% | 104% |
| Class C | -0.01% | -0.05% | 0.07% | 106% | 106% |
| Class E | 0.00% | 0.04% | -0.04% | 107% | 106% |
| Overall | 0.00% | -0.01% | 0.00% | 105% | 105% |
| Class D | -0.02% | -0.02% | -0.05% | 104% | 107% |
| Class F | -0.06% | -0.11% | 0.09% | 106% | 126% |
| TGM | -0.11% | -0.07% | -0.09% | 104% | 119% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.02% | 0.05% | 104% | 105% |
| Class A2 | 0.00% | 0.12% | 0.07% | 104% | 106% |
| Class B | -0.01% | 0.11% | 0.11% | 98% | 97% |
| Class C | -0.02% | -0.13% | -0.16% | 101% | 94% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.03% | 0.02% | 101% | 100% |
| Class D | -0.05% | -0.21% | 0.00% | 101% | 90% |
| Class F | -0.10% | -0.05% | 0.04% | 101% | 102% |
| TGM | -0.09% | -0.08% | -0.03% | 101% | 106% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.17% | -0.21% | 105% | 104% |
| Class C | -0.03% | -0.02% | -0.12% | 105% | 108% |
| Class E | -0.06% | 0.05% | -0.40% | 100% | 94% |
| Overall | -0.03% | 0.08% | -0.23% | 104% | 103% |
| Class D | 0.02% | -0.11% | 0.07% | 103% | 101% |
| Class F | -0.05% | -0.28% | 0.23% | 112% | 138% |
| TGM | -0.02% | 0.04% | 0.08% | 99% | 96% |

Table 5. Summary of the average BD bitrate results of Test 2 over standard QPs 22, 27, 32, and 37

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | -0.01% | 104% | 103% |
| Class A2 | 0.00% | 0.00% | 0.00% | 104% | 105% |
| Class B | 0.00% | 0.01% | -0.03% | 106% | 105% |
| Class C | 0.01% | -0.06% | -0.02% | 105% | 106% |
| Class E | 0.00% | 0.04% | -0.03% | 106% | 105% |
| Overall | 0.00% | -0.01% | -0.02% | 105% | 105% |
| Class D | -0.01% | -0.02% | 0.05% | 105% | 108% |
| Class F | -0.05% | -0.06% | 0.04% | 88% | 94% |
| TGM | -0.09% | -0.06% | -0.06% | 88% | 91% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.01% | 0.03% | 103% | 105% |
| Class A2 | 0.00% | 0.08% | 0.09% | 103% | 106% |
| Class B | -0.01% | 0.11% | 0.06% | 98% | 99% |
| Class C | 0.00% | 0.08% | -0.09% | 101% | 93% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.07% | 0.02% | 101% | 100% |
| Class D | -0.04% | -0.22% | 0.09% | 101% | 86% |
| Class F | -0.06% | -0.04% | 0.09% | 104% | 96% |
| TGM | -0.07% | -0.06% | -0.10% | 98% | 99% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.10% | 0.12% | 103% | 102% |
| Class C | -0.05% | 0.02% | -0.15% | 105% | 108% |
| Class E | -0.07% | -0.28% | -0.28% | 98% | 93% |
| Overall | -0.03% | -0.02% | -0.07% | 102% | 101% |
| Class D | 0.01% | -0.17% | 0.45% | 102% | 98% |
| Class F | 0.05% | 0.03% | 0.60% | 88% | 97% |
| TGM | -0.09% | -0.07% | 0.00% | 89% | 93% |

Table 6. Summary of the average BD bitrate results of Test 1 over Low QPs 2, 7, 12, and 17

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.03% | -0.03% | 118% | 93% |
| Class A2 | 0.00% | 0.00% | 0.01% | 112% | 90% |
| Class B | -0.01% | -0.01% | 0.03% | 114% | 90% |
| Class C | 0.00% | 0.00% | 0.00% | 111% | 92% |
| Class E | -0.04% | -0.05% | -0.05% | 115% | 91% |
| Overall | -0.01% | -0.02% | -0.01% | 114% | 91% |
| Class D | -0.01% | -0.03% | 0.03% | 112% | 93% |
| Class F | -0.33% | -0.13% | -0.15% | 110% | 90% |
| TGM | -0.18% | -0.12% | -0.13% | 112% | 93% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | 0.02% | 0.03% | 0.00% | 113% | 97% |
| Class C | 0.00% | 0.00% | -0.02% | 113% | 97% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.01% | -0.01% | 0.01% | 112% | 96% |
| Class F | -0.58% | -0.21% | -0.09% | 107% | 87% |
| TGM | -0.17% | -0.15% | -0.15% | 101% | 83% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.02% | -0.01% | 98% | 88% |
| Class C | 0.00% | 0.01% | 0.01% | 102% | 91% |
| Class E | -0.01% | -0.01% | 0.00% | 103% | 90% |
| Overall | 0.00% | 0.01% | 0.00% | 101% | 90% |
| Class D | 0.00% | 0.00% | 0.00% | 89% | 85% |
| Class F | -0.21% | -0.22% | -0.28% | 101% | 79% |
| TGM | -0.15% | -0.23% | -0.18% | 102% | 73% |

Table 7. Summary of the average BD bitrate results of Test 2 over Low QPs 2, 7, 12, and 17

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.03% | -0.04% | 91% | 84% |
| Class A2 | -0.01% | 0.00% | 0.01% | 86% | 83% |
| Class B | -0.01% | -0.01% | 0.02% | 87% | 81% |
| Class C | 0.00% | 0.00% | 0.01% | 82% | 80% |
| Class E | -0.03% | -0.03% | -0.03% | 86% | 80% |
| Overall | -0.01% | -0.01% | 0.00% | 86% | 81% |
| Class D | 0.00% | -0.02% | -0.02% | 88% | 85% |
| Class F | -0.18% | -0.12% | -0.13% | 82% | 78% |
| TGM | -0.11% | -0.09% | -0.08% | 93% | 83% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | 0.02% | 0.03% | 0.00% | 95% | 88% |
| Class C | 0.01% | 0.02% | 0.00% | 93% | 87% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.01% | -0.07% | 0.04% | 93% | 87% |
| Class F | -0.35% | -0.15% | -0.10% | 87% | 77% |
| TGM | -0.14% | -0.12% | -0.10% | 89% | 77% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.00% | -0.02% | 87% | 80% |
| Class C | -0.01% | -0.01% | 0.02% | 88% | 83% |
| Class E | -0.02% | 0.00% | 0.03% | 99% | 89% |
| Overall | -0.01% | 0.00% | 0.00% | 90% | 83% |
| Class D | 0.01% | 0.01% | 0.00% | 88% | 85% |
| Class F | -0.17% | -0.25% | -0.19% | 85% | 72% |
| TGM | -0.11% | -0.19% | -0.16% | 85% | 65% |

Bin to bit ratio analysis is conducted according to CE7 testing conditions. For CTC conditions, the bin to bit ratio analysis is provided in Table 8 and Table 10. For low QP conditions, the bin to bit ratio analysis is provided in Table 9 and Table 11.

Table 8. Summary of CABAC bin to bit results for Test1 under CTCs.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  | Max - Bin2Bit Ratio |  |  |  |  |  | TRF of Max - Bin2Bit Ratio |  |  | Avg TRF - Bin2Bit Ratio |  |  | Avg - Bin2Bit Ratio |  |  |  |  |
|  | Y |  | U |  | V | Ref |  |  | Test |  | Diff (%) | Ref | Test | Diff (%) | Ref | Test | Diff (%) | Ref | Test | Diff (%) | EncT | DecT |
| Class A1 | 0.00% |  | -0.01% |  | 0.00% | 1.19 |  |  | 1.19 |  | 0.20% | 1.15 | 1.16 | 0.26% | 1.16 | 1.16 | 0.00% | 1.19 | 1.19 | 0.00% | 105% | 104% |
| Class A2 | 0.00% |  | 0.00% |  | 0.01% | 1.18 |  |  | 1.18 |  | -0.01% | 1.16 | 1.16 | -0.01% | 1.16 | 1.16 | 0.00% | 1.18 | 1.18 | 0.00% | 104% | 104% |
| Class B | 0.00% |  | 0.00% |  | -0.05% | 1.15 |  |  | 1.15 |  | 0.03% | 1.13 | 1.13 | 0.02% | 1.13 | 1.13 | 0.01% | 1.15 | 1.15 | 0.00% | 104% | 104% |
| Class C | -0.01% |  | -0.05% |  | 0.07% | 1.08 |  |  | 1.08 |  | -0.29% | 1.09 | 1.09 | -0.28% | 1.09 | 1.09 | 0.07% | 1.07 | 1.07 | 0.05% | 106% | 106% |
| Class E | 0.00% |  | 0.04% |  | -0.04% | 1.11 |  |  | 1.11 |  | 0.05% | 1.10 | 1.10 | -0.04% | 1.10 | 1.10 | 0.01% | 1.10 | 1.10 | 0.01% | 107% | 106% |
| Overall | 0.00% |  | -0.01% |  | 0.00% | 1.14 |  |  | 1.14 |  | -0.02% | 1.13 | 1.13 | -0.02% | 1.13 | 1.13 | 0.02% | 1.14 | 1.14 | 0.01% | 105% | 105% |
| Class D | -0.02% |  | -0.02% |  | -0.05% | 1.06 |  |  | 1.05 |  | -0.06% | 1.07 | 1.07 | 0.07% | 1.08 | 1.08 | 0.16% | 1.04 | 1.04 | 0.09% | 104% | 107% |
| Class F | -0.06% |  | -0.11% |  | 0.09% | 1.16 |  |  | 1.16 |  | 0.30% | 1.17 | 1.17 | 0.43% | 1.18 | 1.18 | 0.30% | 1.16 | 1.17 | 0.21% | 106% | 126% |
| Class TGM | -0.11% |  | -0.07% |  | -0.09% | 1.20 |  |  | 1.21 |  | 0.47% | 1.33 | 1.34 | 0.84% | 1.33 | 1.34 | 0.51% | 1.20 | 1.20 | 0.38% | 104% | 119% |
|  | Random access Main10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  | Max - Bin2Bit Ratio |  |  |  |  |  | TRF of Max - Bin2Bit Ratio |  |  | Avg TRF - Bin2Bit Ratio |  |  | Avg - Bin2Bit Ratio |  |  |  |  |
|  | Y | U |  | V |  | Ref | Test |  |  | Diff (%) |  | Ref | Test | Diff (%) | Ref | Test | Diff (%) | Ref | Test | Diff (%) | EncT | DecT |
| Class A1 | 0.01% |  | 0.02% |  | 0.05% | 1.18 |  | 1.18 |  | -0.03% |  | 1.14 | 1.14 | 0.00% | 1.18 | 1.18 | -0.01% | 1.22 | 1.22 | -0.01% | 104% | 105% |
| Class A2 | 0.00% |  | 0.12% |  | 0.07% | 1.17 |  | 1.17 |  | 0.04% |  | 1.14 | 1.14 | 0.00% | 1.21 | 1.21 | 0.00% | 1.22 | 1.22 | -0.01% | 104% | 106% |
| Class B | -0.01% |  | 0.11% |  | 0.11% | 1.14 |  | 1.14 |  | -0.04% |  | 1.10 | 1.10 | -0.04% | 1.17 | 1.17 | 0.01% | 1.19 | 1.19 | 0.01% | 98% | 97% |
| Class C | -0.02% |  | -0.13% |  | -0.16% | 1.05 |  | 1.05 |  | 0.18% |  | 1.03 | 1.03 | 0.30% | 1.13 | 1.13 | 0.06% | 1.09 | 1.09 | 0.02% | 101% | 94% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.00% |  | 0.03% |  | 0.02% | 1.13 |  | 1.13 |  | 0.03% |  | 1.10 | 1.10 | 0.06% | 1.17 | 1.17 | 0.02% | 1.17 | 1.17 | 0.00% | 101% | 100% |
| Class D | -0.05% |  | -0.21% |  | 0.00% | 1.02 |  | 1.02 |  | 0.07% |  | 1.01 | 1.01 | 0.23% | 1.14 | 1.14 | 0.13% | 1.02 | 1.02 | 0.06% | 101% | 90% |
| Class F | -0.10% |  | -0.05% |  | 0.04% | 1.11 |  | 1.12 |  | 0.43% |  | 1.08 | 1.09 | 0.60% | 1.16 | 1.17 | 0.31% | 1.15 | 1.15 | 0.17% | 101% | 102% |
| Class TGM | -0.09% |  | -0.08% |  | -0.03% | 1.15 |  | 1.15 |  | 0.48% |  | 1.22 | 1.23 | 0.71% | 1.36 | 1.37 | 0.37% | 1.22 | 1.23 | 0.26% | 101% | 106% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  | Max - Bin2Bit Ratio |  |  |  |  |  | TRF of Max - Bin2Bit Ratio |  |  | Avg TRF - Bin2Bit Ratio |  |  | Avg - Bin2Bit Ratio |  |  |  |  |
|  | Y | U |  | V |  | Ref | Test |  |  | Diff (%) |  | Ref | Test | Diff (%) | Ref | Test | Diff (%) | Ref | Test | Diff (%) | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.00% |  | 0.17% |  | -0.21% | 1.18 | 1.18 |  |  | -0.05% |  | 1.16 | 1.16 | -0.15% | 1.29 | 1.29 | 0.01% | 1.24 | 1.24 | 0.00% | 105% | 104% |
| Class C | -0.03% |  | -0.02% |  | -0.12% | 1.09 | 1.09 |  |  | 0.15% |  | 1.11 | 1.11 | 0.13% | 1.27 | 1.27 | 0.01% | 1.15 | 1.15 | 0.01% | 105% | 108% |
| Class E | -0.06% |  | 0.05% |  | -0.40% | 1.11 | 1.11 |  |  | -0.07% |  | 1.11 | 1.11 | -0.08% | 1.36 | 1.36 | 0.01% | 1.18 | 1.18 | 0.00% | 100% | 94% |
| Overall | -0.03% |  | 0.08% |  | -0.23% | 1.13 | 1.13 |  |  | 0.01% |  | 1.13 | 1.13 | -0.05% | 1.30 | 1.30 | 0.02% | 1.19 | 1.19 | 0.00% | 104% | 103% |
| Class D | 0.02% |  | -0.11% |  | 0.07% | 1.05 | 1.05 |  |  | 0.11% |  | 1.08 | 1.08 | 0.28% | 1.31 | 1.31 | 0.09% | 1.10 | 1.10 | 0.03% | 103% | 101% |
| Class F | -0.05% |  | -0.28% |  | 0.23% | 1.19 | 1.19 |  |  | 0.09% |  | 1.21 | 1.21 | 0.11% | 1.30 | 1.30 | 0.11% | 1.21 | 1.21 | 0.09% | 112% | 138% |
| Class TGM | -0.02% |  | 0.04% |  | 0.08% | 1.19 | 1.19 |  |  | 0.57% |  | 1.29 | 1.29 | 0.58% | 1.49 | 1.49 | 0.13% | 1.30 | 1.30 | 0.07% | 99% | 96% |

Table 9. Summary of CABAC bin to bit results for Test1 under Low QP conditions..

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  | Max - Bin2Bit Ratio |  |  |  |  |  | TRF of Max - Bin2Bit Ratio |  |  | Avg TRF - Bin2Bit Ratio |  |  | Avg - Bin2Bit Ratio |  |  |  |  |
|  | Y |  | U |  | V | Ref |  |  | Test |  | Diff (%) | Ref | Test | Diff (%) | Ref | Test | Diff (%) | Ref | Test | Diff (%) | EncT | DecT |
| Class A1 | -0.02% |  | -0.03% |  | -0.03% | 1.02 |  |  | 1.02 |  | 0.08% | 0.95 | 0.95 | 0.09% | 0.96 | 0.96 | 0.14% | 1.02 | 1.02 | 0.11% | 118% | 93% |
| Class A2 | 0.00% |  | 0.00% |  | 0.01% | 0.90 |  |  | 0.90 |  | 0.01% | 0.84 | 0.84 | 0.00% | 0.84 | 0.84 | 0.01% | 0.91 | 0.91 | 0.01% | 112% | 90% |
| Class B | -0.01% |  | -0.01% |  | 0.03% | 0.93 |  |  | 0.93 |  | 0.13% | 0.86 | 0.86 | 0.13% | 0.87 | 0.87 | 0.12% | 0.94 | 0.94 | 0.10% | 114% | 90% |
| Class C | 0.00% |  | 0.00% |  | 0.00% | 0.83 |  |  | 0.83 |  | 0.13% | 0.74 | 0.74 | 0.17% | 0.75 | 0.75 | 0.20% | 0.83 | 0.84 | 0.15% | 111% | 92% |
| Class E | -0.04% |  | -0.05% |  | -0.05% | 1.01 |  |  | 1.01 |  | 0.54% | 0.94 | 0.95 | 0.62% | 0.94 | 0.95 | 0.59% | 1.01 | 1.01 | 0.49% | 115% | 91% |
| Overall | -0.01% |  | -0.02% |  | -0.01% | 0.93 |  |  | 0.93 |  | 0.15% | 0.86 | 0.86 | 0.17% | 0.87 | 0.87 | 0.19% | 0.94 | 0.94 | 0.15% | 114% | 91% |
| Class D | -0.01% |  | -0.03% |  | 0.03% | 0.80 |  |  | 0.80 |  | 0.11% | 0.71 | 0.71 | 0.13% | 0.73 | 0.73 | 0.13% | 0.81 | 0.81 | 0.08% | 112% | 93% |
| Class F | -0.33% |  | -0.13% |  | -0.15% | 0.96 |  |  | 0.97 |  | 1.36% | 0.87 | 0.89 | 1.85% | 0.85 | 0.87 | 1.55% | 0.94 | 0.95 | 1.18% | 110% | 90% |
| Class TGM | -0.18% |  | -0.12% |  | -0.13% | 0.87 |  |  | 0.87 |  | 0.58% | 0.80 | 0.81 | 0.69% | 0.80 | 0.81 | 0.83% | 0.87 | 0.87 | 0.61% | 112% | 93% |
|  | Random access Main10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  | Max - Bin2Bit Ratio |  |  |  |  |  | TRF of Max - Bin2Bit Ratio |  |  | Avg TRF - Bin2Bit Ratio |  |  | Avg - Bin2Bit Ratio |  |  |  |  |
|  | Y | U |  | V |  | Ref | Test |  |  | Diff (%) |  | Ref | Test | Diff (%) | Ref | Test | Diff (%) | Ref | Test | Diff (%) | EncT | DecT |
| Class A1 | #VALUE! |  | #VALUE! |  | #VALUE! | 0.96 |  | #DIV/0! |  | #DIV/0! |  | 0.89 | #DIV/0! | #DIV/0! | 1.09 | #DIV/0! | #DIV/0! | 1.15 | #DIV/0! | #DIV/0! | #NUM! | #NUM! |
| Class A2 | #VALUE! |  | #VALUE! |  | #VALUE! | 0.84 |  | #DIV/0! |  | #DIV/0! |  | 0.77 | #DIV/0! | #DIV/0! | 1.03 | #DIV/0! | #DIV/0! | 1.09 | #DIV/0! | #DIV/0! | #NUM! | #NUM! |
| Class B | 0.02% |  | 0.03% |  | 0.00% | 0.87 |  | 0.88 |  | 0.18% |  | 0.80 | 0.80 | 0.25% | 1.03 | 1.04 | 0.95% | 1.11 | 1.12 | 0.79% | 113% | 97% |
| Class C | 0.00% |  | 0.00% |  | -0.02% | 0.77 |  | 0.77 |  | 0.14% |  | 0.68 | 0.68 | 0.11% | 0.96 | 0.97 | 1.91% | 1.03 | 1.04 | 1.41% | 113% | 97% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall | #VALUE! |  | #VALUE! |  | #VALUE! | 0.86 |  | #DIV/0! |  | #DIV/0! |  | 0.78 | #DIV/0! | #DIV/0! | 1.02 | #DIV/0! | #DIV/0! | 1.09 | #DIV/0! | #DIV/0! | #NUM! | #NUM! |
| Class D | 0.01% |  | -0.01% |  | 0.01% | 0.75 |  | 0.75 |  | 0.27% |  | 0.66 | 0.66 | 0.15% | 0.93 | 0.95 | 2.10% | 1.00 | 1.01 | 1.33% | 112% | 96% |
| Class F | -0.58% |  | -0.21% |  | -0.09% | 0.90 |  | 0.92 |  | 1.75% |  | 0.81 | 0.83 | 2.27% | 0.93 | 0.96 | 3.00% | 1.01 | 1.03 | 2.15% | 107% | 87% |
| Class TGM | -0.17% |  | -0.15% |  | -0.15% | 0.84 |  | 0.84 |  | 0.59% |  | 0.77 | 0.78 | 0.86% | 0.86 | 0.88 | 2.29% | 0.93 | 0.95 | 1.85% | 101% | 83% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  | Max - Bin2Bit Ratio |  |  |  |  |  | TRF of Max - Bin2Bit Ratio |  |  | Avg TRF - Bin2Bit Ratio |  |  | Avg - Bin2Bit Ratio |  |  |  |  |
|  | Y | U |  | V |  | Ref | Test |  |  | Diff (%) |  | Ref | Test | Diff (%) | Ref | Test | Diff (%) | Ref | Test | Diff (%) | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.01% |  | 0.02% |  | -0.01% | 0.93 | 0.93 |  |  | 0.10% |  | 0.86 | 0.86 | 0.15% | 1.03 | 1.03 | 0.05% | 1.10 | 1.10 | 0.04% | 98% | 88% |
| Class C | 0.00% |  | 0.01% |  | 0.01% | 0.82 | 0.82 |  |  | 0.07% |  | 0.73 | 0.73 | 0.13% | 0.98 | 0.98 | 0.05% | 1.05 | 1.05 | 0.04% | 102% | 91% |
| Class E | -0.01% |  | -0.01% |  | 0.00% | 1.01 | 1.02 |  |  | 0.66% |  | 0.94 | 0.95 | 0.79% | 1.18 | 1.18 | 0.19% | 1.24 | 1.24 | 0.17% | 103% | 90% |
| Overall | 0.00% |  | 0.01% |  | 0.00% | 0.92 | 0.92 |  |  | 0.22% |  | 0.84 | 0.84 | 0.29% | 1.06 | 1.06 | 0.08% | 1.12 | 1.12 | 0.07% | 101% | 90% |
| Class D | 0.00% |  | 0.00% |  | 0.00% | 0.79 | 0.79 |  |  | 0.22% |  | 0.71 | 0.71 | 0.22% | 0.97 | 0.97 | 0.10% | 1.04 | 1.04 | 0.06% | 89% | 85% |
| Class F | -0.21% |  | -0.22% |  | -0.28% | 0.92 | 0.93 |  |  | 1.31% |  | 0.84 | 0.85 | 1.71% | 1.03 | 1.03 | 0.67% | 1.09 | 1.10 | 0.43% | 101% | 79% |
| Class TGM | -0.15% |  | -0.23% |  | -0.18% | 0.85 | 0.86 |  |  | 0.68% |  | 0.79 | 0.79 | 0.90% | 0.91 | 0.92 | 0.87% | 0.99 | 1.00 | 0.57% | 102% | 73% |

Table 10. Summary of CABAC bin to bit results for Test2 under CTCs.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  | Max - Bin2Bit Ratio |  |  |  |  |  | TRF of Max - Bin2Bit Ratio |  |  | Avg TRF - Bin2Bit Ratio |  |  | Avg - Bin2Bit Ratio |  |  |  |  |
|  | Y |  | U |  | V | Ref |  |  | Test |  | Diff (%) | Ref | Test | Diff (%) | Ref | Test | Diff (%) | Ref | Test | Diff (%) | EncT | DecT |
| Class A1 | 0.00% |  | 0.00% |  | -0.01% | 1.19 |  |  | 1.19 |  | 0.21% | 1.15 | 1.16 | 0.27% | 1.16 | 1.16 | 0.00% | 1.19 | 1.19 | 0.00% | 104% | 103% |
| Class A2 | 0.00% |  | 0.00% |  | 0.00% | 1.18 |  |  | 1.18 |  | -0.01% | 1.16 | 1.16 | -0.01% | 1.16 | 1.16 | 0.00% | 1.18 | 1.18 | 0.00% | 104% | 105% |
| Class B | 0.00% |  | 0.01% |  | -0.03% | 1.15 |  |  | 1.15 |  | 0.03% | 1.13 | 1.13 | 0.03% | 1.13 | 1.13 | 0.00% | 1.15 | 1.15 | 0.00% | 106% | 105% |
| Class C | 0.01% |  | -0.06% |  | -0.02% | 1.08 |  |  | 1.08 |  | -0.26% | 1.09 | 1.09 | -0.30% | 1.09 | 1.09 | 0.05% | 1.07 | 1.07 | 0.04% | 105% | 106% |
| Class E | 0.00% |  | 0.04% |  | -0.03% | 1.11 |  |  | 1.11 |  | -0.03% | 1.10 | 1.10 | -0.06% | 1.10 | 1.10 | 0.00% | 1.10 | 1.10 | 0.01% | 106% | 105% |
| Overall | 0.00% |  | -0.01% |  | -0.02% | 1.14 |  |  | 1.14 |  | -0.03% | 1.13 | 1.13 | -0.02% | 1.13 | 1.13 | 0.01% | 1.14 | 1.14 | 0.01% | 105% | 105% |
| Class D | -0.01% |  | -0.02% |  | 0.05% | 1.06 |  |  | 1.05 |  | -0.06% | 1.07 | 1.07 | 0.09% | 1.08 | 1.08 | 0.13% | 1.04 | 1.04 | 0.07% | 105% | 108% |
| Class F | -0.05% |  | -0.06% |  | 0.04% | 1.16 |  |  | 1.16 |  | 0.21% | 1.17 | 1.17 | 0.28% | 1.18 | 1.18 | 0.25% | 1.16 | 1.17 | 0.17% | 88% | 94% |
| Class TGM | -0.09% |  | -0.06% |  | -0.06% | 1.20 |  |  | 1.21 |  | 0.40% | 1.33 | 1.34 | 0.71% | 1.33 | 1.34 | 0.47% | 1.20 | 1.20 | 0.33% | 88% | 91% |
|  | Random access Main10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  | Max - Bin2Bit Ratio |  |  |  |  |  | TRF of Max - Bin2Bit Ratio |  |  | Avg TRF - Bin2Bit Ratio |  |  | Avg - Bin2Bit Ratio |  |  |  |  |
|  | Y | U |  | V |  | Ref | Test |  |  | Diff (%) |  | Ref | Test | Diff (%) | Ref | Test | Diff (%) | Ref | Test | Diff (%) | EncT | DecT |
| Class A1 | 0.01% |  | 0.01% |  | 0.03% | 1.18 |  | 1.18 |  | 0.02% |  | 1.14 | 1.14 | 0.05% | 1.18 | 1.18 | -0.01% | 1.22 | 1.22 | -0.01% | 103% | 105% |
| Class A2 | 0.00% |  | 0.08% |  | 0.09% | 1.17 |  | 1.17 |  | 0.05% |  | 1.14 | 1.14 | 0.02% | 1.21 | 1.21 | 0.00% | 1.22 | 1.22 | -0.01% | 103% | 106% |
| Class B | -0.01% |  | 0.11% |  | 0.06% | 1.14 |  | 1.14 |  | -0.01% |  | 1.10 | 1.10 | -0.02% | 1.17 | 1.17 | 0.01% | 1.19 | 1.19 | 0.01% | 98% | 99% |
| Class C | 0.00% |  | 0.08% |  | -0.09% | 1.05 |  | 1.05 |  | 0.07% |  | 1.03 | 1.03 | 0.08% | 1.13 | 1.13 | 0.03% | 1.09 | 1.09 | 0.00% | 101% | 93% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.00% |  | 0.07% |  | 0.02% | 1.13 |  | 1.13 |  | 0.03% |  | 1.10 | 1.10 | 0.03% | 1.17 | 1.17 | 0.01% | 1.17 | 1.17 | 0.00% | 101% | 100% |
| Class D | -0.04% |  | -0.22% |  | 0.09% | 1.02 |  | 1.02 |  | 0.11% |  | 1.01 | 1.01 | 0.22% | 1.14 | 1.14 | 0.09% | 1.02 | 1.02 | 0.03% | 101% | 86% |
| Class F | -0.06% |  | -0.04% |  | 0.09% | 1.11 |  | 1.12 |  | 0.30% |  | 1.08 | 1.09 | 0.47% | 1.16 | 1.17 | 0.25% | 1.15 | 1.15 | 0.14% | 104% | 96% |
| Class TGM | -0.07% |  | -0.06% |  | -0.10% | 1.15 |  | 1.15 |  | 0.49% |  | 1.22 | 1.23 | 0.64% | 1.36 | 1.37 | 0.36% | 1.22 | 1.23 | 0.24% | 98% | 99% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  | Max - Bin2Bit Ratio |  |  |  |  |  | TRF of Max - Bin2Bit Ratio |  |  | Avg TRF - Bin2Bit Ratio |  |  | Avg - Bin2Bit Ratio |  |  |  |  |
|  | Y | U |  | V |  | Ref | Test |  |  | Diff (%) |  | Ref | Test | Diff (%) | Ref | Test | Diff (%) | Ref | Test | Diff (%) | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.01% |  | 0.10% |  | 0.12% | 1.18 | 1.18 |  |  | -0.02% |  | 1.16 | 1.16 | -0.11% | 1.29 | 1.29 | 0.02% | 1.24 | 1.24 | 0.01% | 103% | 102% |
| Class C | -0.05% |  | 0.02% |  | -0.15% | 1.09 | 1.09 |  |  | 0.04% |  | 1.11 | 1.11 | 0.05% | 1.27 | 1.27 | -0.01% | 1.15 | 1.15 | 0.00% | 105% | 108% |
| Class E | -0.07% |  | -0.28% |  | -0.28% | 1.11 | 1.11 |  |  | 0.00% |  | 1.11 | 1.11 | -0.07% | 1.36 | 1.36 | 0.00% | 1.18 | 1.18 | 0.00% | 98% | 93% |
| Overall | -0.03% |  | -0.02% |  | -0.07% | 1.13 | 1.13 |  |  | 0.00% |  | 1.13 | 1.13 | -0.06% | 1.30 | 1.30 | 0.01% | 1.19 | 1.19 | 0.00% | 102% | 101% |
| Class D | 0.01% |  | -0.17% |  | 0.45% | 1.05 | 1.05 |  |  | 0.16% |  | 1.08 | 1.08 | 0.41% | 1.31 | 1.31 | 0.10% | 1.10 | 1.10 | 0.05% | 102% | 98% |
| Class F | 0.05% |  | 0.03% |  | 0.60% | 1.19 | 1.19 |  |  | 0.11% |  | 1.21 | 1.22 | 0.29% | 1.30 | 1.30 | 0.15% | 1.21 | 1.21 | 0.07% | 88% | 97% |
| Class TGM | -0.09% |  | -0.07% |  | 0.00% | 1.19 | 1.19 |  |  | 0.43% |  | 1.29 | 1.29 | 0.53% | 1.49 | 1.49 | 0.11% | 1.30 | 1.30 | 0.07% | 89% | 93% |

Table 11. Summary of CABAC bin to bit results for Test2 under Low QP conditions..

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  | Max - Bin2Bit Ratio |  |  |  |  |  | TRF of Max - Bin2Bit Ratio |  |  | Avg TRF - Bin2Bit Ratio |  |  | Avg - Bin2Bit Ratio |  |  |  |  |
|  | Y |  | U |  | V | Ref |  |  | Test |  | Diff (%) | Ref | Test | Diff (%) | Ref | Test | Diff (%) | Ref | Test | Diff (%) | EncT | DecT |
| Class A1 | -0.02% |  | -0.03% |  | -0.04% | 1.02 |  |  | 1.02 |  | 0.08% | 0.95 | 0.95 | 0.09% | 0.96 | 0.96 | 0.12% | 1.02 | 1.02 | 0.09% | 91% | 84% |
| Class A2 | -0.01% |  | 0.00% |  | 0.01% | 0.90 |  |  | 0.90 |  | -0.03% | 0.84 | 0.84 | -0.03% | 0.84 | 0.84 | 0.01% | 0.91 | 0.91 | 0.01% | 86% | 83% |
| Class B | -0.01% |  | -0.01% |  | 0.02% | 0.93 |  |  | 0.93 |  | 0.05% | 0.86 | 0.86 | 0.06% | 0.87 | 0.87 | 0.06% | 0.94 | 0.94 | 0.05% | 87% | 81% |
| Class C | 0.00% |  | 0.00% |  | 0.01% | 0.83 |  |  | 0.83 |  | 0.15% | 0.74 | 0.74 | 0.17% | 0.75 | 0.75 | 0.11% | 0.83 | 0.83 | 0.09% | 82% | 80% |
| Class E | -0.03% |  | -0.03% |  | -0.03% | 1.01 |  |  | 1.01 |  | 0.37% | 0.94 | 0.94 | 0.43% | 0.94 | 0.95 | 0.41% | 1.01 | 1.01 | 0.35% | 86% | 80% |
| Overall | -0.01% |  | -0.01% |  | 0.00% | 0.93 |  |  | 0.93 |  | 0.11% | 0.86 | 0.86 | 0.12% | 0.87 | 0.87 | 0.13% | 0.94 | 0.94 | 0.10% | 86% | 81% |
| Class D | 0.00% |  | -0.02% |  | -0.02% | 0.80 |  |  | 0.80 |  | 0.01% | 0.71 | 0.71 | 0.02% | 0.73 | 0.73 | 0.09% | 0.81 | 0.81 | 0.05% | 88% | 85% |
| Class F | -0.18% |  | -0.12% |  | -0.13% | 0.96 |  |  | 0.96 |  | 0.74% | 0.87 | 0.88 | 0.98% | 0.85 | 0.86 | 0.94% | 0.94 | 0.94 | 0.71% | 82% | 78% |
| Class TGM | -0.11% |  | -0.09% |  | -0.08% | 0.87 |  |  | 0.87 |  | 0.47% | 0.80 | 0.81 | 0.52% | 0.80 | 0.81 | 0.68% | 0.87 | 0.87 | 0.51% | 93% | 83% |
|  | Random access Main10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  | Max - Bin2Bit Ratio |  |  |  |  |  | TRF of Max - Bin2Bit Ratio |  |  | Avg TRF - Bin2Bit Ratio |  |  | Avg - Bin2Bit Ratio |  |  |  |  |
|  | Y | U |  | V |  | Ref | Test |  |  | Diff (%) |  | Ref | Test | Diff (%) | Ref | Test | Diff (%) | Ref | Test | Diff (%) | EncT | DecT |
| Class A1 | #VALUE! |  | #VALUE! |  | #VALUE! | 0.96 |  | #DIV/0! |  | #DIV/0! |  | 0.89 | #DIV/0! | #DIV/0! | 1.09 | #DIV/0! | #DIV/0! | 1.15 | #DIV/0! | #DIV/0! | #NUM! | #NUM! |
| Class A2 | #VALUE! |  | #VALUE! |  | #VALUE! | 0.84 |  | #DIV/0! |  | #DIV/0! |  | 0.77 | #DIV/0! | #DIV/0! | 1.03 | #DIV/0! | #DIV/0! | 1.09 | #DIV/0! | #DIV/0! | #NUM! | #NUM! |
| Class B | 0.02% |  | 0.03% |  | 0.00% | 0.87 |  | 0.88 |  | 0.08% |  | 0.80 | 0.80 | 0.11% | 1.03 | 1.04 | 0.94% | 1.11 | 1.12 | 0.78% | 95% | 88% |
| Class C | 0.01% |  | 0.02% |  | 0.00% | 0.77 |  | 0.77 |  | 0.06% |  | 0.68 | 0.68 | 0.00% | 0.96 | 0.97 | 1.88% | 1.03 | 1.04 | 1.40% | 93% | 87% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall | #VALUE! |  | #VALUE! |  | #VALUE! | 0.86 |  | #DIV/0! |  | #DIV/0! |  | 0.78 | #DIV/0! | #DIV/0! | 1.02 | #DIV/0! | #DIV/0! | 1.09 | #DIV/0! | #DIV/0! | #NUM! | #NUM! |
| Class D | -0.01% |  | -0.07% |  | 0.04% | 0.75 |  | 0.75 |  | 0.07% |  | 0.66 | 0.66 | -0.07% | 0.93 | 0.95 | 2.08% | 1.00 | 1.01 | 1.30% | 93% | 87% |
| Class F | -0.35% |  | -0.15% |  | -0.10% | 0.90 |  | 0.91 |  | 1.04% |  | 0.81 | 0.83 | 1.32% | 0.93 | 0.96 | 2.63% | 1.01 | 1.03 | 1.87% | 87% | 77% |
| Class TGM | -0.14% |  | -0.12% |  | -0.10% | 0.84 |  | 0.84 |  | 0.46% |  | 0.77 | 0.78 | 0.70% | 0.86 | 0.88 | 2.16% | 0.93 | 0.95 | 1.73% | 89% | 77% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  | Max - Bin2Bit Ratio |  |  |  |  |  | TRF of Max - Bin2Bit Ratio |  |  | Avg TRF - Bin2Bit Ratio |  |  | Avg - Bin2Bit Ratio |  |  |  |  |
|  | Y | U |  | V |  | Ref | Test |  |  | Diff (%) |  | Ref | Test | Diff (%) | Ref | Test | Diff (%) | Ref | Test | Diff (%) | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.01% |  | 0.00% |  | -0.02% | 0.93 | 0.93 |  |  | 0.06% |  | 0.86 | 0.86 | 0.10% | 1.03 | 1.03 | 0.03% | 1.10 | 1.10 | 0.02% | 87% | 80% |
| Class C | -0.01% |  | -0.01% |  | 0.02% | 0.82 | 0.82 |  |  | 0.03% |  | 0.73 | 0.73 | 0.08% | 0.98 | 0.98 | 0.04% | 1.05 | 1.05 | 0.03% | 88% | 83% |
| Class E | -0.02% |  | 0.00% |  | 0.03% | 1.01 | 1.02 |  |  | 0.49% |  | 0.94 | 0.95 | 0.57% | 1.18 | 1.18 | 0.13% | 1.24 | 1.24 | 0.12% | 99% | 89% |
| Overall | -0.01% |  | 0.00% |  | 0.00% | 0.92 | 0.92 |  |  | 0.16% |  | 0.84 | 0.84 | 0.21% | 1.06 | 1.06 | 0.05% | 1.12 | 1.12 | 0.05% | 90% | 83% |
| Class D | 0.01% |  | 0.01% |  | 0.00% | 0.79 | 0.79 |  |  | 0.10% |  | 0.71 | 0.71 | 0.11% | 0.97 | 0.97 | 0.10% | 1.04 | 1.04 | 0.05% | 88% | 85% |
| Class F | -0.17% |  | -0.25% |  | -0.19% | 0.92 | 0.93 |  |  | 0.85% |  | 0.84 | 0.85 | 1.12% | 1.03 | 1.03 | 0.54% | 1.09 | 1.10 | 0.35% | 85% | 72% |
| Class TGM | -0.11% |  | -0.19% |  | -0.16% | 0.85 | 0.86 |  |  | 0.54% |  | 0.79 | 0.79 | 0.75% | 0.91 | 0.92 | 0.77% | 0.99 | 1.00 | 0.50% | 85% | 65% |

# Conclusions

This contribution proposes to unify CCB count method between transform residual and transform skip residual coding by aligning the number of CCB per coefficient for transform residual coding and TS residual coding by assigning 1.75 bin/coefficient at TB-level, and excluding coeff_sign_flag from CCB count in TS residual. On top of this unification, setting another CCB_SIGN limit for coeff_sign_flag is also proposed to control the CABAC throughput. There are some gains in Class F and TGM screen content sequences, especially under Low QP conditions, and no coding losses in all classes by this unification. We suggest to adopt these proposals into the test model.

# References

- B. Bross, J. Chen, and S. Liu “Versatile Video Coding (Draft 6),” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-O2001, 15th Meeting: Gothenburg, SE, 3–12 July 2019.

- VTM6.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM6.0

- H. Schwarz and M. Coban, “Description of Core Experiment 7 (CE7): Quantization and coefficient coding,” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-O2027, 15th Meeting: Gothenburg, SE, 3–12 July 2019.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).