| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0403 |
| --- | --- |

| Title: | AHG8/AHG12: Subpicture-specific reference picture resampling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela, Alireza Aminlou, Kashyap Kammachi-Sreedhar Nokia Technologies, Tampere, Finland | Email: | firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution aims at enabling subpicture-specific reference picture resampling. It assumes that subpicture layout is indicated in the PPS and the subpicture IDs are included in slice headers, as proposed in JVET-P0126. The contribution contains the following proposed items:

- The subpicture boundaries used for clipping the sample locations when subpic_treated_as_pic_flag is equal to 1 are derived from the subpictures of the reference picture (rather than from the subpictures of the current picture).

- When subpics_present_flag is equal to 1, subpic_scaling_flag is included in the same parameter set. subpic_scaling_flag equal to 1 causes the following:

  - The constraints on having the same subpicture layout and the same values of subpic_treated_as_pic_flag[ i ] and loop_filter_across_subpic_enabled_flag[ i ] throughout a CLVS are not imposed. Instead, when a reference picture is used for prediction, it is required that it has a subpicture with the same ID as the subpicture ID of the current slice.

  - The horizontal and vertical scaling factors are derived from the width and height ratios of the respective subpictures (with the same subpicture ID) in the current picture and in the reference picture.

  - The sample locations of the current coding subblock are relative to the top-left corner of the subpicture for the scaling by the scaling factors. After scaling, the top-left position of the subpicture within the reference picture is added to the scaled subpicture-relative sample location in order to obtain the sample locations within the reference picture.

# Introduction

JVET-P0126 proposes indicating subpicture layout in the PPS and subpicture IDs in the slice header. Furthermore, it is proposed that the subpicture positions and sizes as well as the values of subpic_treated_as_pic_flag[ i ] and loop_filter_across_subpic_enabled_flag[ i ] shall be unchanged throughout a CLVS.

This contribution enables subpicture-specific reference picture resampling (RPR). The scaling ratios for resampling are derived from the width and height of the subpictures that have the same subpicture IDs.

The motivation for enabling subpicture-specific RPR and motion compensation includes the following:

- There are video services where the encoded source content for each time originates from multiple sources (e.g. multiple cameras), which are spatially arranged to pictures being encoded and where the layout and sizes of the sources within the pictures may change dynamically.

- For example, in eSports the video stream may contain one or more viewpoints of the game as well as camera feeds of the players, commentators, and audience. The spatial size of each source may be dynamically changing depending on its importance. See more discussion in MPEG M48085.

- Another example is multi-party conferencing with a multipoint control unit. The active speaker may receive a larger spatial area of the entire picture compared to the other participants of the conference.

# Summary of the proposal

This contribution takes JVET-P0126 as the basis. The specific features of JVET-P0126 that are needed in this contribution are the following:

- Indicating the subpicture layout in the PPS instead of the SPS so that the subpicture sizes can change within a CLVS.

- Labeling subpictures with a subpicture ID, and hence enabling the association of subpictures of the same subpicture ID together.

The specification text changes proposed in this contribution can be summarized as follows:

- The subpicture boundaries used for clipping the sample locations when subpic_treated_as_pic_flag is equal to 1 are derived from the subpictures of the reference picture (rather than from the subpictures of the current picture).

- When subpics_present_flag is equal to 1, subpic_scaling_flag is included in the same parameter set. subpic_scaling_flag equal to 1 causes the following:

  - The constraints on having the same subpicture layout and the same values of subpic_treated_as_pic_flag[ i ] and loop_filter_across_subpic_enabled_flag[ i ] throughout a CLVS are not imposed. Instead, when a reference picture is used for prediction, it is required that it has a subpicture with the same ID as the subpicture ID of the current slice.

  - The horizontal and vertical scaling factors are derived from the width and height ratios of the respective subpictures (with the same subpicture ID) in the current picture and in the reference picture.

  - The sample locations of the current coding subblock are relative to the top-left corner of the subpicture for the scaling by the scaling factors. After scaling, the top-left position of the subpicture within the reference picture is added to the scaled subpicture-relative sample location in order to obtain the sample locations within the reference picture.

The detailed specification text changes are indicated with change marks in the accompanying specification text document, which is based on VVC Draft 6 text but assumes that these design choices of JVET-P0126 are available too. Specifically, the proposed specification text changes make use of SubPicLeft[ i ], SubPicRight[ i ], SubPicTop[ i ] and SubPicBot[ i ] variables derived for the i-th subpicture. The text for subpic_scaling_flag is currently not included, since the containing syntax structure is in JVET-P0126.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).