| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0403 |
| --- | --- |

| Title: | AHG8/AHG12: Subpicture-specific reference picture resampling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela, Alireza Aminlou, Kashyap Kammachi-Sreedhar Nokia Technologies, Tampere, Finland | Email: | firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution aims at enabling subpicture-specific reference picture resampling. The contribution contains the following proposed items:

- The subpicture boundaries used for clipping the sample locations when subpic_treated_as_pic_flag is equal to 1 are derived from the subpictures of the reference picture (rather than from the subpictures of the current picture).

- When subpics_present_flag is equal to 1:

  - The horizontal and vertical scaling factors are derived from the width and height ratios of the respective subpictures (with the same subpicture ID) in the current picture and in the reference picture.

  - The sample locations of the current coding subblock are relative to the top-left corner of the subpicture for the scaling by the scaling factors. After scaling, the top-left position of the subpicture within the reference picture is added to the scaled subpicture-relative sample location in order to obtain the sample locations within the reference picture.

Version 2 updates the contribution for alignment with the JVET meeting decisions by the morning of 3 October 2019.

# Introduction

In the 16th JVET meeting (JVET-P), subpicture IDs were agreed to be included in the slice headers.

This contribution enables subpicture-specific reference picture resampling (RPR). The scaling ratios for resampling are derived from the width and height of the subpictures that have the same subpicture IDs.

The motivation for enabling subpicture-specific RPR and motion compensation includes the following:

- There are video services where the encoded source content for each time originates from multiple sources (e.g. multiple cameras), which are spatially arranged to pictures being encoded and where the positions and sizes of the sources within the pictures may change dynamically.

- For example, in eSports the video stream may contain one or more viewpoints of the game as well as camera feeds of the players, commentators, and audience. The spatial size of each source may be dynamically changing depending on its importance. See more discussion in MPEG M48085.

- Another example is multi-party conferencing with a multipoint control unit. The active speaker may receive a larger spatial area of the entire picture compared to the other participants of the conference.

Figure 1 presents an example where three sources are encoded, each in its own subpicture. In the reference picture the source having subpicture ID 1 had the greatest width and height, whereas in the current picture the source has a smaller width and height. The opposite happened for subpicture ID 2, and the width and height of the subpicture with ID 3 remained unchanged.

Figure 1. Example for subpicture-wise reference picture resampling.

# Summary of the proposal

The specification text changes proposed in this contribution can be summarized as follows:

- The subpicture boundaries used for clipping the sample locations when subpic_treated_as_pic_flag is equal to 1 are derived from the subpictures of the reference picture (rather than from the subpictures of the current picture).

- When subpics_present_flag is equal to 1:

  - The horizontal and vertical scaling factors are derived from the width and height ratios of the respective subpictures (with the same subpicture ID) in the current picture and in the reference picture.

  - The sample locations of the current coding subblock are relative to the top-left corner of the subpicture for the scaling by the scaling factors. After scaling, the top-left position of the subpicture within the reference picture is added to the scaled subpicture-relative sample location in order to obtain the sample locations within the reference picture.

The detailed specification text changes are indicated with change marks in the accompanying specification text document, which is based on VVC Draft 6 text but assumes that SubPicLeft[ i ], SubPicRight[ i ], SubPicTop[ i ] and SubPicBot[ i ] variables derived for the i-th subpicture are available from other adoptions (see e.g. JVET-P0126 as an example for deriving the variables).

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).