| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0404 |
| --- | --- |

| Title: | SEI message for MPEG-I Part 7 metadata |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Technologies, Tampere, Finland | Email: | firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution requests JVET's opinion on having a single generic syntax structure containing any specific MPEG-I Part 7 metadata structure, where the generic syntax structure is specified in MPEG-I Part 7 in a manner that can be directly included in the syntax of an SEI message. If such a generic syntax structure is included in MPEG-I Part 7, the contribution proposes to have a single MPEG-I Part 7 SEI message in ISO/IEC 23002-7, rather than specifying separate SEI messages for each MPEG-I Part 7 metadata structure. The syntax and semantics of the MPEG-I Part 7 metadata SEI message are also proposed.

# Introduction

## MPEG-I Part 7 metadata

A key idea of MPEG-I Part 7 is to collect metadata structures and specifications applicable to immersive media so that the same metadata structures and specifications can be used by multiple standards. The use of the same structures and specifications is believed to:

- reduce the overall effort of standardizing the metadata for immersive media,

- avoid cumbersome conversions from one metadata format to another,

- avoid implementation and testing overhead for implementing multiple metadata structures for the same functionality.

Since MPEG-I Part 7 was not in place when OMAF version 1 was developed, similar metadata structures were specified as SEI messages of AVC and HEVC as well as for file format metadata in OMAF version 1. A substantial amount of effort was required for coordination between MPEG Systems (and specifically its OMAF breakout group), JVET, and JCT-VC, as well as for aligning the omnidirectional video metadata structures between the three standards and for authoring the specification text for all the three standards. Such a process is both effort-taking and vulnerable for technical errors and inconsistencies.

The ongoing standardization projects that could reference MPEG-I Part 7 instead of specifying their own immersive media metadata include the following:

- OMAF version 2

- ISO/IEC 23002-7 Supplemental enhancement information messages for coded video bitstreams

It is envisioned that MPEG-I Part 7 metadata structures could be specified as follows in order to make their integration into SEI messages of ISO/IEC 23002-7 straightforward:

- Specifying the metadata structures with the tabular form syntax of ISO/IEC 23002-7 in addition to the syntax description language syntax used in ISOBMFF

- Specifying ID values for each metadata structure of MPEG-I Part 7 metadata and a generic container structure that takes an ID value as input and includes the metadata structure corresponding to the ID value.

## Tabular form syntax for MPEG-I Part 7 metadata

The current MPEG-I Part 7 draft specifies syntax structures using the ISOBMFF conventions, i.e. the syntax description language (SDL) specified in ISO/IEC 14496-1. To make by-reference inclusion of a syntax structure specified in MPEG-I Part 7 into a syntax structure specified in ISO/IEC 23002-7 unambiguous, it is envisioned that equivalent syntax of all the syntax structures of MPEG-I Part 7 could be provided in both ISOBMFF conventions and in the tabular form syntax of ISO/IEC 23002-7.

It is envisioned that the same syntax element names can be used in both ISOBMFF and tabular form syntax. Thus, no repetitive text is needed for specifying the semantics.

As an example for clarifying the idea, the rotation structure is included below in both syntax forms.

### 5.3.1	Rotation structure

#### 5.3.1.1	Definition

The fields in this structure provides the yaw, pitch, and roll angles, respectively, of the rotation to be applied to convert the local coordinate axes to the global coordinate axes. In the case of stereoscopic omnidirectional video, the fields apply to each view individually.

#### 5.3.1.2	Syntax in syntax description language

aligned(8) class RotationStruct() {
	signed int(32) rotation_yaw;
	signed int(32) rotation_pitch;
	signed int(32) rotation_roll;
}

#### 5.3.1.3	Syntax in tabular form

| rotation_struct( ) { | Descriptor |
| --- | --- |
| rotation_yaw | s(32) |
| rotation_pitch | s(32) |
| rotation_roll | s(32) |
| } |  |

## ID for each metadata structure in MPEG-I Part 7

It is envisioned that as a part of MPEG-I Part 7 the following could be specified:

- an identifier value for each MPEG-I Part 7 metadata structure, and

- a container structure that inputs an ID value and includes the metadata structure corresponding to the ID value.

The following syntax illustrates how ID values and a single container structure could be specified in MPEG-I Part 7:

| mpeg_i_part7_payload( metadataId ) { | Descriptor |
| --- | --- |
| if( metadataId = = 0 ) |  |
| rotation_struct( ) |  |
| else if( metadataId = = 1 ) |  |
| content_coverage_struct( ) |  |
| … |  |
| } |  |

Consequently, with the mpeg_i_part7_payload( metadataId ) syntax structure, a derived specification needs only one mechanism or codepoint to include any type of MPEG-I Part 7 metadata, rather than separate mechanisms or codepoints for each MPEG-I Part 7 metadata structure.

# Proposal

## General

With this contribution, we would like to assess JVET's opinion on having a single generic syntax structure containing any specific MPEG-I Part 7 metadata structure, where the generic syntax structure is specified in MPEG-I Part 7 in a manner that can be directly included in the syntax of an SEI message. If such a generic syntax structure is included in MPEG-I Part 7, the contribution proposes to have a single MPEG-I Part 7 SEI message in ISO/IEC 23002-7, rather than specifying separate SEI messages for each MPEG-I Part 7 metadata structure.

## Proposed specification text

The following syntax and semantics for the MPEG-I Part 7 SEI message are proposed.

| mpeg_i_part7_metadata( payloadSize ) { | Descriptor |
| --- | --- |
| metadata_id_6bits | u(6) |
| metadataId = metadata_id_6bits |  |
| if( metadata_id_6bits = = 63 ) |  |
| do { |  |
| metadata_id_byte | u(8) |
| metadataId += metadata_id_byte |  |
| } while( metadata_id_byte = = 255 ) |  |
| metadata_cancel_flag | u(1) |
| if( !metadata_cancel_flag ) { |  |
| metadata_persistence_flag | u(1) |
| mpeg_i_part7_payload( metadataId ) |  |
| } |  |
| } |  |

The MPEG-I Part 7 metadata SEI message carries metadata specified in ISO/IEC 23090-7.

metadata_id_6bits and metadata_id_byte, when present, affect the derivation of metadataId variable, which specifies the identifier of the contained MPEG-I Part 7 structure and the syntax of the payload of this SEI message. It is a requirement of bitstream conformance that the value of metadataId derived from metadata_id_6bits and metadata_id_byte, when present, shall be in the range allowed by ISO/IEC 23090-7. Decoders shall allow and skip MPEG-I Part 7 metadata SEI messages with metadataId value outside of the range specified by ISO/IEC 23090-7.

metadata_cancel_flag equal to 1 specifies that the SEI message cancels the persistence of any previous MPEG-I Part 7 metadata SEI message with the same metadataId value in output order.

metadata_persistence_flag specifies the persistence of the MPEG-I Part 7 metadata SEI message for the current layer.

metadata_persistence_flag equal to 0 specifies that the MPEG-I Part 7 metadata SEI message applies to the current decoded picture only.

Let picA be the current picture. metadata_persistence_flag equal to 1 specifies that the MPEG-I Part 7 metadata SEI message persists for the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture picB in the current layer in an access unit containing MPEG-I Part 7 metadata SEI message with the same metadataId value that is applicable to the current layer is output for which PicOrderCnt( picB ) is greater than PicOrderCnt( picA ), where PicOrderCnt( picB ) and PicOrderCnt( picA ) are the PicOrderCntVal values of picB and picA, respectively, immediately after the invocation of the decoding process for picture order count for picB.

The syntax and semantics of mpeg_i_part7_payload( metadataId ) are specified in ISO/IEC 23090-7.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).