| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0405 |
| --- | --- |

| Title: | Non-CE6: Cleanups of maximum transform size related syntax elements |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhipin Deng, Li Zhang, Kai Zhang, Hongbin Liu, Yang Wang Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China | Tel: Email: | +1(858)999-7093 zhipin.deng@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

In current VVC, the maximum transform size is signalled regardless the size of maximum CTU size. In addition, the maximum SBT size is signalled regardless the size of maximum transform sizes. This contribution proposes to conditionally signal the maximum transform size and maximum SBT size. More specifically, two aspects are proposed as follows:

Aspect #1: sps_max_luma_transform_size_64_flag is signalled only when the CTU size is larger than or equal to 64.

Aspect #2: sps_sbt_max_size_64_flag is signalled only when the maximum transform size is equal to 64.

# Introduction

There are two issues related to the maximum transform size in VVC.

Issue #1: The signalling of sps_max_luma_transform_size_64_flag is meaningless when the CTU size is smaller than 64.

Issue #2: The signalling of sps_sbt_max_size_64_flag is meaningless when the maximum transform size is smaller than 64.

# Proposal

Two aspects are proposed to address the two issues.

Aspect #1: The signalling of sps_max_luma_transform_size_64_flag is conditioned on whether the CTU size is larger than or equal to 64.

Aspect #2: The signalling of sps_sbt_max_size_64_flag is conditioned on whether the maximum transform size is equal to 64.

With the proposed methods, the syntax table becomes more concise and meaningful.

# Simulation results

No performance changes are observed under the common test conditions.

# Proposed spec changes

The proposed changes on top of VVC draft 6 [1] are described as below. Newly added changes are highlighted in yellow and deleted texts are marked with strikethrough.

#### Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_decoding_parameter_set_id | u(4) |
| … |  |
| if( qtbtt_dual_tree_intra_flag ) { |  |
| sps_log2_diff_min_qt_min_cb_intra_slice_chroma | ue(v) |
| sps_max_mtt_hierarchy_depth_intra_slice_chroma | ue(v) |
| if ( sps_max_mtt_hierarchy_depth_intra_slice_chroma != 0 ) { |  |
| sps_log2_diff_max_bt_min_qt_intra_slice_chroma | ue(v) |
| sps_log2_diff_max_tt_min_qt_intra_slice_chroma | ue(v) |
| } |  |
| } |  |
| if( log2_ctu_size_minus5 ) |  |
| sps_max_luma_transform_size_64_flag | u(1) |
| … |  |
| sps_sbt_enabled_flag | u(1) |
| if( sps_sbt_enabled_flag && sps_max_luma_transform_size_64_flag ) |  |
| sps_sbt_max_size_64_flag | u(1) |
| … |  |

#### 7.4.3.3. Sequence parameter set RBSP semantics

sps_max_luma_transform_size_64_flag equal to 1 specifies that the maximum transform size in luma samples is equal to 64. sps_max_luma_transform_size_64_flag equal to 0 specifies that the maximum transform size in luma samples is equal to 32.

When CtbSizeY is less than 64, sps_max_luma_transform_size_64_flag is not present, the value of sps_max_luma_transform_size_64_flag shall is inferred to be equal to 0.

The variables MinTbLog2SizeY, MaxTbLog2SizeY, MinTbSizeY, and MaxTbSizeY are derived as follows:

MinTbLog2SizeY = 2		(7-27)

MaxTbLog2SizeY = sps_max_luma_transform_size_64_flag  ?  6  :  5	(7-28)

MinTbSizeY = 1  <<  MinTbLog2SizeY		(7-29)

MaxTbSizeY = 1  <<  MaxTbLog2SizeY	                                        (7-30)

sps_sbt_max_size_64_flag equal to 0 specifies that the maximum CU width and height for allowing subblock transform is 32 luma samples. sps_sbt_max_size_64_flag equal to 1 specifies that the maximum CU width and height for allowing subblock transform is 64 luma samples.

When not present, the value of sps_sbt_max_size_64_flag is inferred to be equal to 0.

MaxSbtSize = Min( MaxTbSizeY, sps_sbt_max_size_64_flag ? 64 : 32 )	(7-32)

# Conclusions

It is proposed to cleanup the signalling of sps_max_luma_transform_size_64_flag and sps_sbt_max_size_64_flag to maintain a concise syntax table. It is recommended adopting the proposed methods.

# References

B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 6)”, JVET-O2001, July 2019, Gothenburg, SE.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).