| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0406-v3 |
| --- | --- |

| Title: | Non-CE3: Generalization of SCIPU for different YUV formats |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Luong Pham Van, Geert Van der Auwera, Vadim Seregin, Han Huang, Adarsh K. Ramasubramonian, Marta Karczewicz 5775 Morehouse Dr San Diego, CA 92121 | Email: | {lphamvan, geertv, vseregin, hanhuang, aramasub, martak}@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This document proposes to fix a bug of SCIPU (smallest chroma intra PU) regarding the different color formats. Firstly, SCIPU is disabled for YUV 4:0:0. Secondly, the block size of the chroma component is used to decide the SCIPU mode constraint. The proposed fix achieves an average (Y, U, V) BD-rate gain of (-0.21%, -0.47%, -0.40%) in the RA configuration for the YUV 4:2:2 tested sequences with a negligible running time change. (Version3: Update the name of tested sequences and parameters).

# Introduction

In the last meeting, local dual tree has been adopted into VTM6.0 to remove chroma 2x2/2x4/4x2 intra blocks for single tree. This adoption introduces smallest chroma intra prediction unit (SCIPU). In an inter SCIPU, all the luma blocks are encoded using inter mode and the chroma components inherit the split of the luma component. On the other hand, in a non-inter SCIPU, all the luma blocks are encoded using non-inter mode (intra or IBC) while the chroma components are not split. In order to decide the mode type constraint for a SCIPU, the luma block size and the luma split are considered. The following two cases can be considered as a SCIPU: 1) If the size of the luma area is 64 pixels and the luma is split OR 2) if the size of the area is 128 pixels and the luma split is ternary split. These cases are SCIPU because if the chroma components inherit the split of the luma component, 2x2/2x4/4x2 blocks will occur.

It should be noted that the chroma components do not exist in the YUV 4:0:0 format. Therefore, there is no need to impose the SCIPU constraint for this color format.

In addition, the current SCIPU constraint check works correctly for YUV 4:2:0 but not for YUV 4:2:2. For example, in some cases the chroma components use the luma split and 2x2/2x4/4x2 blocks occur in YUV 4:2:0 but not in YUV 4:2:2 (as illustrated in Figure 1). The reason for this difference is that the height of the chroma and luma components in YUV 4:2:2 is identical while the chroma height is half of the luma height in YUV 4:2:0.

This proposal proposes to solve the above problems of SCIPU.

Figure 1 Examples of 2x4/4x2 occurring in YUV 4:2:0 but not in YUV 4:2:2

# Proposed method

In this proposal, two fixes are proposed to solve the problems of SCIPU.

In the first one, SCIPU is disabled for YUV 4:0:0.

In the second one, the chroma size is considered in the SCIPU constraint check in stead of using the luma size as in the current VTM6.0. In the other words, the chroma scaling factors (SubWidthC and SubHeightC) are used when checking the SCIPU constraint for a partition.

Method 1: The specification text is accordingly modified as follows:

#### 7.4.9.4 Coding tree semantics ….. The variable modeTypeCondition is derived as follows: - If one of the following conditions is true, modeTypeCondition is set equal to 0 - slice_type = = I and qtbtt_dual_tree_intra_flag is equal to 1 - modeTypeCurr is not equal to MODE_TYPE_ALL - chroma_format_idc == 0 - chroma_format_idc == 3 - Otherwise, if one of the following conditions is true, modeTypeCondition is set equal to 1 - (cbWidth / SubWidthC) * (cbHeight / SubHeightC) is equal to 6416 and split_qt_flag is equal to 1 - (cbWidth / SubWidthC) * (cbHeight / SubHeightC) is equal to 6416 and MttSplitMode[ x0 ][ y0 ][ mttDepth ] is equal to SPLIT_TT_HOR or SPLIT_TT_VER - (cbWidth / SubWidthC) * (cbHeight / SubHeightC) is equal to 328 and MttSplitMode[ x0 ][ y0 ][ mttDepth ] is equal to SPLIT_BT_HOR or SPLIT_BT_VER - Otherwise, if one of the following conditions is true, modeTypeCondition is set equal to 1 + (slice_type != I ? 1 : 0) - (cbWidth / SubWidthC) * (cbHeight / SubHeightC) is equal to 6416 and MttSplitMode[ x0 ][ y0 ][ mttDepth ] is equal to SPLIT_BT_HOR or SPLIT_BT_VER - (cbWidth / SubWidthC) * (cbHeight / SubHeightC) is equal to 12832 and MttSplitMode[ x0 ][ y0 ][ mttDepth ] is equal to SPLIT_TT_HOR or SPLIT_TT_VER Otherwise, modeTypeCondition is set equal to 0

Method 2: Another specification text change can be written as follows. This modification aligns with the original definition of SCIPU constraint and work for any YUV format (This alternative was written in version 2 of the proposal).

#### 7.4.9.4 Coding tree semantics ….. The variable modeTypeCondition is derived as follows: - If one of the following conditions is true, modeTypeCondition is set equal to 0 - slice_type = = I and qtbtt_dual_tree_intra_flag is equal to 1 - modeTypeCurr is not equal to MODE_TYPE_ALL - chroma_format_idc == 0 - chroma_format_idc == 3 - Otherwise, if one of the following conditions is true, modeTypeCondition is set equal to 1derived as follows: A variable minLuma is set equal to : - (cbWidth*cbHeight) >> 2 if MttSplitMode[ x0 ][ y0 ][ mttDepth ] is equal to SPLIT_TT_HOR or SPLIT_TT_VER or split_qt_flag is equal to 1 - (cbWidth*cbHeight) >> 1 if MttSplitMode[ x0 ][ y0 ][ mttDepth ] is equal to SPLIT_BT_HOR or SPLIT_BT_VER - cbWidth * cbHeight, otherwise. - modeTypeCondition is set equal to 0 if minLuma / (SubWidthC * SubHeightC) is higher than 8. - Otherwise, modeTypeCondition is set equal to 1 + (slice_type == I ¦¦ minLuma == 16?) 0 : 1 - cbWidth * cbHeight is equal to 6416 and split_qt_flag is equal to 1 - cbWidth * cbHeight is equal to 6416 and MttSplitMode[ x0 ][ y0 ][ mttDepth ] is equal to SPLIT_TT_HOR or SPLIT_TT_VER - cbWidth * cbHeight is equal to 328 and MttSplitMode[ x0 ][ y0 ][ mttDepth ] is equal to SPLIT_BT_HOR or SPLIT_BT_VER - Otherwise, if one of the following conditions is true, modeTypeCondition is set equal to 1 + (slice_type != I ? 1 : 0) - cbWidth * cbHeight is equal to 6416 and MttSplitMode[ x0 ][ y0 ][ mttDepth ] is equal to SPLIT_BT_HOR or SPLIT_BT_VER - cbWidth * cbHeight is equal to 12832 and MttSplitMode[ x0 ][ y0 ][ mttDepth ] is equal to SPLIT_TT_HOR or SPLIT_TT_VER Otherwise, modeTypeCondition is set equal to 0

# Experimental results

The proposed method has been implemented on top of VTM6.0 and has been tested under the RA conditions defined in JVET-N1010. Eight sequences of YUV 4:2:2 have been tested. Table 1 shows the test results of the proposed method 2 while the parameters if each sequence are presented in Table 2.

The proposed fix achieves an average (Y, U, V) BD-rate gain of (-0.21%, -0.47%, -0.40%) in the RA configuration with a negligible runtime change.

Table 1 Experimental results

| Sequence | Random Access Main 10 Over VTM-6.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Traffic | -0.08% | -0.08% | -0.02% | 99% | 100% |
| Seeking | -0.12% | -0.99% | -0.49% | 99% | 100% |
| Kimono | -0.05% | -0.10% | -0.04% | 101% | 101% |
| EBUKidsSoccer | -0.05% | -0.07% | -0.08% | 99% | 100% |
| EBUHorse | -0.10% | -0.20% | -0.22% | 100% | 99% |
| EBUGraphics | -0.81% | -1.05% | -0.98% | 100% | 100% |
| DucksAndLegs | -0.10% | -0.10% | -0.25% | 99% | 100% |
| CrowdRun | -0.37% | -1.16% | -1.08% | 98% | 102% |
| Average | -0.21% | -0.47% | -0.40% | 99% | 100% |

Table 2 Sequence parameters

| Sequence | Frame rate | Number of Frame | Intra period | QP |
| --- | --- | --- | --- | --- |
| Traffic | 30 | 150 | 32 | 22, 27, 32, 37 |
| Seeking | 50 | 500 | 48 | 22, 27, 32, 37 |
| Kimono | 24 | 240 | 32 | 22, 27, 32, 37 |
| EBUKidsSoccer | 50 | 500 | 48 | 22, 27, 32, 37 |
| EBUHorse | 50 | 500 | 48 | 22, 27, 32, 37 |
| EBUGraphics | 50 | 500 | 48 | 22, 27, 32, 37 |
| DucksAndLegs | 60 | 600 | 64 | 22, 27, 32, 37 |
| CrowdRun | 50 | 500 | 48 | 22, 27, 32, 37 |

# Conclusion

This proposal proposes to fix the bug of SCIPU regarding the different color formats. It is recommended to adopt the proposed technique 2 to be included in the next VTM.

# Patent rights declaration

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).