| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0411 |
| --- | --- |

| Title: | Non-CE5: Fixes for long luma deblocking filter decision |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | K. Andersson J. Enhorn | Tel: Email: | +46730371390 kenneth.r.andersson@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

The long luma deblocking filter in VTM-6.0 is very good in supressing artifacts from boundaries between large luma blocks. However, in some cases it can happen that wrong long luma deblocking filter decisions are made due to not checking all samples before deblocking. In some other cases natural edges are filtered due to too high beta threshold for long luma deblocking filter decision check of samples close to the boundary. This contribution proposes to fix the problems with additional gradient calculations for a side of the boundary which has max filter length equal to 7 and to reduce the beta threshold for long luma deblocking filter decision check of samples close to the boundary. It is asserted that the fix improves visual quality compared to VTM-6.0 for both SDR and HDR content.

The BDR impact versus VTM-6.0 is -0.02%, -0.07%, -0.12% for AI/RA/LDB and versus VTM-6.0 with ALF turned off -0.01%, -0.06%, -0.14% for AI/RA/LDB. Encoding and decoding times are similar as for the anchor.

For HDR, the BDR impact versus VTM-6.0 is -0.08%/-0.16%/-0.09%/-0.07% for deltaE/PSNR-L100/wPSNRY/PSNRY.

# Introduction

The long luma deblocking filter in VVC draft is very good in supressing blocking artifacts. However, in some cases it can happen that wrong long luma deblocking filter decisions are made due to not checking all samples before deblocking. In some other cases natural edges are filtered due to too high beta threshold for long luma deblocking filter decision check of samples close to the boundary. Section 2 proposes additional gradient calculations to also check edges around p6/q6, e.g. around 7:th sample from boundary in block P and block Q respectively, which currently not is checked, to avoid applying the long filter in cases of natural edges around q6/p6. Section 3 propose to reduce the beta threshold for samples p0 to p5 and q0 to q5 to avoid applying the long filter in case of natural edges somewhere between first and 6:th sample in block P or somewhere between first and 6:th sample in block Q. Results including visual examples are shown in section 4 and conclusions in section 5.

# Fix to check samples p6/q6 used when filtering 7 samples

Consider a line of samples across the boundary between block P and Q, with 8 samples in block P and 8 samples in block Q as follows:

Samples:       p7 p6 p5 p4 p3 p2 p1 p0 | q0 q1 q2 q3 q4 q5 q6 q7

In VVC draft 6 [1], when maxFilterLengtP is equal to 7 sample p6 is never checked and when maxFilterLengthQ is equal to 7 sample q6 is never checked, although those samples can be filtered and thus a line orthogonal to the vertical or the horizontal boundary going through p6 and q6 can never be discovered by the long deblocking decisions and can therefore be filtered by the long deblocking filter.

Same can happen for a wider line orthogonal to the vertical or the horizontal boundary with peak in sample p5 or q5 where p4, p5, p6, q4, q5 and q6 are larger or smaller than neighboring samples as shown below which neither can be detected by the long deblocking decisions and can therefore be filtered by the long deblocking filter:

p7 p6 p5 p4 p3 p2 p1 p0 | q0 q1 q2 q3 q4 q5 q6 q7

10 20 30 20 10 10 10 10 | 10 10 10 10 20 30 20 10  (example values with peak in p5 or q5)

To combat this issue additional ramp insensitive gradient calculations are added to the long luma filter decision when maxFilterLengthP or maxFilterLengthQ is equal to 7 as high-lighted in yellow on top of VVC draft 6[1]:

..

2.  When maxFilterLengthP and maxFilterLengthQ both are equal to or greater than 3 the variables sp0, sq0, spq0, sp3, sq3 and spq3 are derived as follows:

sp0 = maxFilterLengthP==7 ? Abs( p3,0 − p0,0 ) + Abs( p7,0 – p6,0 – p5,0  + p4,0):  Abs( p3,0 − p0,0 )		(8-1081)

sq0 = maxFilterLengthQ==7 ? Abs( q0,0 − q3,0 ) + Abs( q4,0 – q5,0 – q6,0 + q7,0 ): Abs( q0,0 − q3,0 )		(8-1082)

spq0 = Abs( p0,0 − q0,0 )		(8-1083)

sp3 = maxFilterLengthP==7 ? Abs( p3,3 − p0,3 ) + Abs( p7,3 – p6,3 – p5,3 +  p4,3): Abs( p3,3 − p0,3 )		(8-1084)

sq3 = maxFilterLengthQ==7 ? Abs( q0,3 − q3,3 ) + Abs( q4,3 – q5,3 – q6,3 + q7,3 ): Abs( q0,3 − q3,3 )		(8-1085)

spq3 = Abs( p0,3 − q0,3 )		(8-1086)

..

# Fix for beta threshold for samples p0 to p5 and q0 to q5

In VVC draft 6 [1], samples p0 to p5 and q0 to q5 are checked by a beta threshold with same level as used for the shorter filters which makes it possible for the longer luma filters to filter natural edges close to the boundary.

To combat this issue the dpq threshold for the long luma filter decision related to samples closer to the boundary in section 8.8.3.6.5 is reduced as high-lighted in yellow on top of VVC draft 6 [1]:

..

The variable sThr is derived as follows:

- If sidePisLargeBlk is equal to 1 or sideQisLargeBlk is equal to 1, the following applies:

sThr1 = 3 * β  >>  5	(8-1160)

sThr2 =  β  >>  4		(8-1160)

- Otherwise, the following applies:

sThr1 = β  >>  3	(8-1161)

sThr2 = β  >>  2	(8-1161)

The variable dSam is specified as follows:

- If all of the following conditions are true, dSam is set equal to 1:

- dpq is less than ( β  >>  2 )sThr2,

- sp + sq is less than sThr1,

- spq is less than ( 5 * tC + 1 )  >>  1.

- Otherwise, dSam is set equal to 0.

..

# Results

## Visual examples

In most cases it is sufficient to add an additional gradient calculation for the long luma filter decision but in some cases it is also beneficial to reduce the beta threshold for dpq.

Figure 1: POC 111 DayStreet at QP 37 ALF ON, Left: Anchor, Right: Proposed fix with additional gradient to include samples p6/q6 which not are checked in VTM-6.0.

Figure 2: POC 111 DayStreet at QP 37 ALF ON, Left: Proposed fix of dpq threshold, Right: Both proposed fixes for long luma deblocking filter decision

Figure 3: POC 76 EBU_Hurdles at QP 37 ALF ON, Left: Anchor, Right: Proposed fix with additional gradient to include samples p6/q6 which not are checked in VTM-6.0

Figure 4: POC 76 EBU_Hurdles at QP 37 ALF ON, Left: Proposed fix of dpq threshold, Right: Both proposed fixes for long luma deblocking filter decision

## Objective performance SDR

The objective performance for the proposal is shown below in comparison to VTM-6.0 [2] using CTC [3] and also in comparison to VTM-6.0 with ALF turned off for both anchor and test. The objective results show a small gain. Encoding time is measured on simulations run on a cluster with machines with same capability. Decoding time is measured by running test and anchor on same machine without yuv output. Encoding and decoding time is similar as for the anchor.

### Proposed fixes for long luma filter decision vs VTM-6.0

|  | All intra Over VTM-6.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,06% | 0,01% | 0,00% | 104% | 100% |
| Class A2 | -0,01% | 0,00% | 0,00% | 105% | 101% |
| Class B | -0,02% | -0,01% | -0,01% | 104% | 101% |
| Class C | 0,00% | 0,00% | 0,00% | 104% | 101% |
| Class E | 0,00% | 0,00% | 0,00% | 102% | 100% |
| Overall | -0,02% | 0,00% | 0,00% | 104% | 101% |
| Class D | 0,00% | 0,00% | 0,00% | 103% | 103% |
| Class F | -0,01% | 0,00% | 0,01% | 105% | 102% |

|  | Random access Over VTM-6.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,12% | -0,01% | -0,02% | 103% | 99% |
| Class A2 | -0,05% | 0,03% | -0,05% | 101% | 104% |
| Class B | -0,08% | 0,10% | -0,02% | 101% | 100% |
| Class C | -0,04% | 0,02% | -0,05% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0,07% | 0,04% | -0,03% | 101% | 101% |
| Class D | -0,04% | 0,04% | 0,21% | 100% | 102% |
| Class F | -0,05% | 0,04% | 0,02% | 99% | 100% |

|  | Low-delay B Over VTM-6.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,12% | -0,12% | -0,08% | 101% | 100% |
| Class C | -0,08% | 0,13% | 0,02% | 104% | 102% |
| Class E | -0,24% | -0,25% | 0,30% | 98% | 100% |
| Overall | -0,14% | -0,07% | 0,05% | 102% | 101% |
| Class D | -0,01% | 0,03% | 0,22% | 101% | 97% |
| Class F | -0,05% | -0,24% | 0,66% | 100% | 100% |

### Proposed fixes for long luma filter decision vs VTM-6.0 with both ALF=0

|  | All intra Over VTM-6.0 ALF=0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,02% | 0,01% | 0,00% | 100% | 99% |
| Class A2 | -0,01% | 0,00% | 0,00% | 101% | 100% |
| Class B | -0,01% | 0,00% | 0,00% | 101% | 101% |
| Class C | 0,00% | 0,00% | 0,00% | 97% | 100% |
| Class E | 0,01% | 0,00% | 0,00% | 96% | 100% |
| Overall | -0,01% | 0,00% | 0,00% | 99% | 100% |
| Class D | 0,00% | 0,00% | 0,00% | 99% | 96% |
| Class F | 0,00% | -0,01% | 0,00% | 99% | 98% |

|  | Random access Over VTM-6.0 ALF=0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,08% | 0,12% | 0,03% | 101% | 100% |
| Class A2 | -0,06% | 0,03% | 0,05% | 100% | 102% |
| Class B | -0,06% | -0,07% | -0,08% | 100% | 100% |
| Class C | -0,03% | 0,00% | 0,01% | 98% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0,06% | 0,01% | -0,01% | 100% | 100% |
| Class D | -0,04% | -0,01% | 0,05% | 99% | 96% |
| Class F | -0,03% | 0,01% | 0,03% | 92% | 100% |

|  | Low-delay B Over VTM-6.0 ALF=0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,14% | -0,27% | -0,28% | 103% | 98% |
| Class C | -0,06% | 0,03% | -0,04% | 103% | 100% |
| Class E | -0,17% | -0,18% | 0,15% | 105% | 100% |
| Overall | -0,12% | -0,15% | -0,09% | 102% | 99% |
| Class D | 0,01% | 0,15% | -0,16% | 104% | 98% |
| Class F | -0,10% | -0,13% | 0,77% | 102% | 99% |

## Objective performance HDR

HDR metrics DE100 and PSNR-L100 were computed using HDR Tools v 0.17 [4]. Encoding simulations was run on machines with different capabilities, no time reported. Decoding time is measured by running test and anchor on same machine without yuv output.

### Proposed fixes for long luma filter decision vs VTM-6.0

|  | Random Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -0,08% | -0,16% | -0,09% | 0,09% | -0,06% | -0,07% | 0,14% | 0,11% | #VALUE! | 102% |
| Class H2 |  |  |  |  |  | -0,07% | 0,03% | 0,04% | #VALUE! | 98% |
| Overall | -0,08% | -0,16% | -0,09% | 0,09% | -0,06% | -0,07% | 0,10% | 0,08% | #VALUE! | 100% |

# Conclusions

We recommend adoption of both fixes for long luma deblocking filter decision to the VVC draft. This ensures that long luma de-blocking filter avoids filtering of natural edges while combating blocking artifacts from large blocks.

# References

- B. Bross, J. Chen, and S. Liu, "Versatile Video Coding (Draft 6)", Document of Joint Video Experts Team, JVET-O2001.

- VTM-6.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-6.0

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations”, Document of Joint Video Experts Team, JVET-N1010.

- https://gitlab.com/standards/HDRTools.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).