| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0412-v3 |
| --- | --- |

| Title: | CE5-related: On further modification of Hadamard filter from CE5-3.2 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sergey Ikonin Victor Stepin Alexander Karabutov Sofya Nikolaeva | Tel: Email: | +7-495-660-44-59 sergey.ikonin@huawei.com stepin.victor@huawei.com karabutov.alexander@huawei.com nikolaeva.sofya@huawei.com |
| Source: | Huawei Technologies Co., LTD |  |  |

_____________________________

# Abstract

This contribution reports results of modifications of test CE5-3.2 on combination of Hadamard and SAO filter related to encoder in-loop RDO decision fix, bugfix of SAO and Hadamard filter averaging stage, and fix of accessing uninitialized memory at encoder side at the block padding stage. In addition, the effect of applying Hadamard filter to chroma components is reported.

# Introduction

In CE5-3.2.1 Hadamard filter was tested in combination with SAO (JVET-P0078). In this implementation the encoder makes a decision whether to apply Hadamard filter on CTU and slice level by evaluating RDO cost at in-loop filtering stage. The output of SAO and Hadamard filtering is averaged before proceeding to next in-loop filtering stage.

# Filter combination fixes

## Aspect 1

In CE5-3.2.1 at in-loop filtering stage SAO and Hadamard filter has its own RDO to make a decision whether to apply filter at CTU and slice level. However, the result of joint work was not evaluated. In order to compensate that the following modification was made. When cost of SAO filtered frame is above than Hadamard filtered frame then SAO is disabled at the slice level.

## Aspect 2

In implementation tested in CE5-3.2.1 the averaging of SAO and Hadamard output is performed at the frame basis without consideration of whether filter is applied to particular CTU or not. This lead to degradation of joint filtering performance. This bug was eliminated by performing averaging only for those CTUs where both filters are enabled.

## Aspect 3

Aspect 3 is a non-normative fix of a bug identified in CE5-3.2 software. During cost calculation in encoder RDO the padding process may touch uninitialized memory. That may cause mismatch after decoding of concatenated RA streams, although parallel decoding has no issues.

In a table below the simulation results of aspect 1 and aspect 2 applied to CE5-3.2.1 are provided.

Test 1. The test results of Aspect 1 and Aspect 2 applied to CE5-3.2.1.

|  |  | All Intra |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-6.0 |  |  |  |  |  |
|  |  | Y | U | V | EncT |  | DecT |
| Class A1 |  | -0.40% | 0.56% | 0.45% | 108% |  | 108% |
| Class A2 |  | -0.49% | 0.63% | 0.43% | 107% |  | 106% |
| Class B |  | -0.32% | 0.52% | 0.52% | 107% |  | 106% |
| Class C |  | -0.54% | 0.34% | 0.55% | 108% |  | 105% |
| Class E |  | -0.47% | 0.25% | 0.25% | 108% |  | 105% |
| Overall |  | -0.44% | 0.46% | 0.46% | 107% |  | 106% |
| Class D |  | -0.47% | 0.50% | 0.38% | 107% |  | 104% |
| Class F |  | -0.28% | 0.42% | 0.38% | 105% |  | 107% |
|  |  | Random Access |  |  |  |  |  |
|  |  | Over VTM-6.0 |  |  |  |  |  |
|  |  | Y | U | V | EncT |  | DecT |
| Class A1 |  | -0.20% | 0.31% | 0.30% | 103% |  | 105% |
| Class A2 |  | -0.59% | 0.26% | 0.22% | 104% |  | 104% |
| Class B |  | -0.37% | 0.40% | 0.52% | 105% |  | 104% |
| Class C |  | -0.47% | 0.28% | 0.50% | 103% |  | 102% |
| Class E |  |  |  |  |  |  |  |
| Overall |  | -0.41% | 0.32% | 0.41% | 104% |  | 104% |
| Class D |  | -0.59% | 0.07% | 0.12% | 103% |  | 102% |
| Class F |  | -0.19% | 0.20% | 0.36% | 103% |  | 104% |
|  | Low delay B |  |  |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |  |  |
|  | Y |  | U | V | EncT | DecT |  |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.07% |  | 1.00% | 0.77% | 104% | 104% |  |
| Class C | -0.40% |  | 0.53% | 0.87% | 103% | 102% |  |
| Class E | 0.02% |  | 0.94% | 1.24% | 101% | 104% |  |
| Overall | -0.16% |  | 0.83% | 0.92% | 103% | 103% |  |
| Class D | -0.60% |  | 1.51% | 1.28% | 103% | 102% |  |
| Class F | 0.02% |  | 0.60% | 1.09% | 102% | 104% |  |
|  | Low delay P |  |  |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |  |  |
|  | Y |  | U | V | EncT | DecT |  |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.21% |  | 0.89% | 1.29% | 105% | 104% |  |
| Class C | -0.43% |  | 0.55% | 0.84% | 104% | 102% |  |
| Class E | -0.17% |  | 0.19% | 1.11% | 102% | 104% |  |
| Overall | -0.27% |  | 0.60% | 1.09% | 104% | 103% |  |
| Class D | -0.65% |  | 0.67% | 0.92% | 103% | 101% |  |
| Class F | -0.08% |  | 0.37% | 1.37% | 103% | 104% |  |

In a table below the simulation results of all three aspects applied to CE5-3.2.1 are provided.

Test 1b. The test results of Aspect 1, Aspect 2 and Aspect 3 applied to CE5-3.2.1.

|  |  | All Intra |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-6.0 |  |  |  |  |  |
|  |  | Y | U | V | EncT |  | DecT |
| Class A1 |  | -0.40% | 0.63% | 0.43% | 108% |  | 108% |
| Class A2 |  | -0.50% | 0.56% | 0.39% | 107% |  | 106% |
| Class B |  | -0.33% | 0.50% | 0.50% | 109% |  | 106% |
| Class C |  | -0.56% | 0.29% | 0.51% | 108% |  | 106% |
| Class E |  | -0.47% | 0.15% | 0.28% | 108% |  | 106% |
| Overall |  | -0.44% | 0.43% | 0.44% | 108% |  | 106% |
| Class D |  | -0.47% | 0.39% | 0.32% | 108% |  | 104% |
| Class F |  | -0.25% | 0.44% | 0.38% | 106% |  | 106% |
|  |  | Random Access |  |  |  |  |  |
|  |  | Over VTM-6.0 |  |  |  |  |  |
|  |  | Y | U | V | EncT |  | DecT |
| Class A1 |  | -0.23% | 0.38% | 0.19% | 103% |  | 105% |
| Class A2 |  | -0.59% | 0.37% | 0.36% | 103% |  | 104% |
| Class B |  | -0.39% | 0.38% | 0.42% | 104% |  | 103% |
| Class C |  | -0.47% | 0.32% | 0.41% | 103% |  | 102% |
| Class E |  |  |  |  |  |  |  |
| Overall |  | -0.42% | 0.36% | 0.36% | 104% |  | 104% |
| Class D |  | -0.63% | 0.14% | 0.35% | 104% |  | 102% |
| Class F |  | -0.21% | 0.27% | 0.39% | 103% |  | 104% |
|  | Low delay B |  |  |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |  |  |
|  | Y |  | U | V | EncT | DecT |  |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.07%* |  | 1.16% | 0.96% | 104% | 103% |  |
| Class C | -0.40% |  | 0.49% | 0.93% | 103% | 102% |  |
| Class E | -0.05% |  | 1.40% | 1.23% | 101% | 103% |  |
| Overall | -0.17%* |  | 1.00% | 1.02% | 103% | 102% |  |
| Class D | -0.60% |  | 1.03% | 1.29% | 103% | 102% |  |
| Class F | -0.01% |  | 0.13% | 0.81% | 102% | 103% |  |
|  | Low delay P |  |  |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |  |  |
|  | Y |  | U | V | EncT | DecT |  |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.21% |  | 1.25% | 1.25% | 105% | 102% |  |
| Class C | -0.36% |  | 0.75% | 0.94% | 104% | 101% |  |
| Class E | -0.04% |  | 0.85% | 1.02% | 102% | 103% |  |
| Overall | -0.22% |  | 0.98% | 1.09% | 104% | 102% |  |
| Class D | -0.65% |  | 0.89% | 0.93% | 104% | 103% |  |
| Class F | -0.06% |  | 0.33% | 0.15% | 102% | 104% |  |

*) few rate points for LDB are extrapolated

Test 2. Test results of CE5-3.2.1 (for reference).

| . | All Intra |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT | DecT |
| Class A1 | -0.28% | 0.52% |  | 0.49% |  | 108% | 110% |
| Class A2 | -0.38% | 0.40% |  | 0.31% |  | 107% | 109% |
| Class B | -0.26% | 0.50% |  | 0.42% |  | 107% | 108% |
| Class C | -0.42% | 0.21% |  | 0.35% |  | 107% | 107% |
| Class E | -0.33% | 0.34% |  | 0.29% |  | 107% | 108% |
| Overall | -0.33% | 0.39% |  | 0.37% |  | 107% | 108% |
| Class D | -0.36% | 0.48% |  | 0.32% |  | 107% | 106% |
| Class F | -0.17% | 0.17% |  | 0.06% |  | 105% | 107% |
|  | Random Access |  |  |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT | DecT |
| Class A1 | -0.17% | 0.25% |  | 0.17% |  | 103% | 105% |
| Class A2 | -0.60% | 0.16% |  | 0.06% |  | 104% | 104% |
| Class B | -0.34% | 0.38% |  | 0.49% |  | 105% | 103% |
| Class C | -0.38% | 0.33% |  | 0.36% |  | 104% | 103% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.37% | 0.30% |  | 0.31% |  | 104% | 103% |
| Class D | -0.50% | 0.06% |  | 0.18% |  | 103% | 102% |
| Class F | -0.09% | 0.03% |  | 0.15% |  | 103% | 103% |
|  | Low delay B |  |  |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |  |  |
|  | Y |  | U |  | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | 0.03% |  | 1.07% |  | 0.57% | 103% | 103% |
| Class C | -0.25% |  | 0.30% |  | 0.51% | 103% | 102% |
| Class E | -0.05% |  | 0.55% |  | 0.93% | 101% | 103% |
| Overall | -0.08% |  | 0.68% |  | 0.64% | 103% | 103% |
| Class D | -0.36% |  | 0.72% |  | 0.78% | 103% | 102% |
| Class F | 0.18% |  | 0.37% |  | 1.09% | 102% | 104% |
|  | Low delay P |  |  |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |  |  |
|  | Y |  | U |  | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.12% |  | 0.91% |  | 0.85% | 105% | 104% |
| Class C | -0.28% |  | 0.33% |  | 0.55% | 104% | 103% |
| Class E | -0.07% |  | -0.06% |  | 0.76% | 102% | 104% |
| Overall | -0.16% |  | 0.47% |  | 0.73% | 104% | 103% |
| Class D | -0.43% |  | 0.91% |  | 0.09% | 104% | 103% |
| Class F | 0.03% |  | 0.68% |  | 0.33% | 103% | 103% |

Test 3. The test results of Aspect 1 and Aspect 2 applied to CE5-3.2.1. (ALF is off)

| . | All Intra |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0.noALF |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT | DecT |
| Class A1 | -0.68% | 0.69% |  | 1.22% |  | 108% | 110% |
| Class A2 | -0.63% | 0.83% |  | 0.61% |  | 108% | 107% |
| Class B | -0.50% | 0.69% |  | 0.77% |  | 107% | 108% |
| Class C | -0.74% | 0.61% |  | 1.00% |  | 107% | 106% |
| Class E | -0.83% | 0.31% |  | 0.32% |  | 108% | 108% |
| Overall | -0.66% | 0.63% |  | 0.79% |  | 108% | 108% |
| Class D | -0.62% | 0.81% |  | 0.80% |  | 107% | 105% |
| Class F | -0.36% | 0.74% |  | 0.85% |  | 105% | 108% |
|  | Random Access |  |  |  |  |  |  |
|  | Over VTM-6.0.noALF |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT | DecT |
| Class A1 | -0.51% | 0.59% |  | 0.57% |  | 104% | 107% |
| Class A2 | -0.95% | 0.81% |  | 0.45% |  | 104% | 105% |
| Class B | -0.71% | 0.72% |  | 0.57% |  | 105% | 105% |
| Class C | -0.68% | 0.31% |  | 0.51% |  | 104% | 103% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.71% | 0.60% |  | 0.53% |  | 104% | 105% |
| Class D | -0.69% | 0.02% |  | 0.12% |  | 104% | 102% |
| Class F | -0.27% | 0.46% |  | 0.81% |  | 103% | 105% |
|  | Low delay B |  |  |  |  |  |  |
|  | Over VTM-6.0.noALF |  |  |  |  |  |  |
|  | Y |  | U |  | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C | -0.75% |  | 1.61% |  | 2.57% | 104% | 103% |
| Class E | -0.27% |  | 0.95% |  | 1.25% | 102% | 106% |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.80% |  | 2.66% |  | 1.99% | 104% | 103% |
| Class F |  |  |  |  |  |  |  |
|  | Low delay P |  |  |  |  |  |  |
|  | Over VTM-6.0.noALF |  |  |  |  |  |  |
|  | Y |  | U |  | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C | -0.85% |  | 1.74% |  | 2.68% | 105% | 103% |
| Class E | -0.81% |  | 1.11% |  | 0.63% | 103% | 104% |
| Overall |  |  |  |  |  |  |  |
| Class D | -1.17% |  | 2.33% |  | 1.63% | 106% | 102% |
| Class F | -0.19% |  | 0.79% |  | 0.57% | 104% | 107% |

Test 3b. The test results of Aspect 1, Aspect 2 and Aspect 3 applied to CE5-3.2.1. (ALF is off)

| . | All Intra |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0.noALF |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT | DecT |
| Class A1 | -0.67% | 0.77% |  | 1.20% |  | 109% | 110% |
| Class A2 | -0.63% | 0.77% |  | 0.58% |  | 108% | 107% |
| Class B | -0.51% | 0.66% |  | 0.75% |  | 108% | 108% |
| Class C | -0.75% | 0.58% |  | 1.00% |  | 109% | 106% |
| Class E | -0.83% | 0.23% |  | 0.38% |  | 109% | 108% |
| Overall | -0.67% | 0.61% |  | 0.79% |  | 108% | 108% |
| Class D | -0.61% | 0.69% |  | 0.73% |  | 108% | 105% |
| Class F | -0.33% | 0.74% |  | 0.88% |  | 106% | 108% |
|  | Random Access |  |  |  |  |  |  |
|  | Over VTM-6.0.noALF |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT | DecT |
| Class A1 | -0.50% | 0.52% |  | 0.54% |  | 104% | 107% |
| Class A2 | -0.94% | 0.89% |  | 0.44% |  | 104% | 105% |
| Class B | -0.73% | 0.69% |  | 0.47% |  | 105% | 105% |
| Class C | -0.69% | 0.31% |  | 0.50% |  | 104% | 103% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.72% | 0.59% |  | 0.49% |  | 104% | 105% |
| Class D | -0.67% | 0.08% |  | 0.16% |  | 104% | 102% |
| Class F | -0.29% | 0.62% |  | 0.80% |  | 103% | 105% |
|  | Low delay B |  |  |  |  |  |  |
|  | Over VTM-6.0.noALF |  |  |  |  |  |  |
|  | Y |  | U |  | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.52%* |  | 1.96% |  | 1.48% | 104% | 104% |
| Class C | -0.73% |  | 1.62% |  | 2.83% | 104% | 102% |
| Class E | -0.29% |  | 1.10% |  | 1.16% | 101% | 104% |
| Overall | -0.53%* |  | 1.63% |  | 1.85% | 103% | 104% |
| Class D | -0.84% |  | 2.69% |  | 2.22% | 105% | 103% |
| Class F | -0.13% |  | 0.88% |  | 0.15% | 103% | 105% |
|  | Low delay P |  |  |  |  |  |  |
|  | Over VTM-6.0.noALF |  |  |  |  |  |  |
|  | Y |  | U |  | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.65% |  | 2.16% |  | 2.21% | 106% | 105% |
| Class C | -0.83% |  | 1.95% |  | 2.74% | 105% | 103% |
| Class E | -0.59% |  | 1.99% |  | 1.21% | 103% | 105% |
| Overall | -0.69% |  | 2.05% |  | 2.14% | 105% | 104% |
| Class D | -1.18% |  | 2.40% |  | 1.90% | 106% | 100% |
| Class F | -0.21% |  | 1.18% |  | 0.19% | 104% | 105% |

*) A few rate points for LDB are extrapolated

Test 4 Test results of CE5-3.2.1 (for reference) (ALF is off).

| . | All Intra |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0.noALF |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT | DecT |
| Class A1 | -0.39% | 0.56% |  | 0.58% |  | 108% | 112% |
| Class A2 | -0.46% | 0.42% |  | 0.33% |  | 108% | 111% |
| Class B | -0.41% | 0.55% |  | 0.45% |  | 107% | 110% |
| Class C | -0.52% | 0.28% |  | 0.40% |  | 108% | 108% |
| Class E | -0.55% | 0.37% |  | 0.32% |  | 108% | 109% |
| Overall | -0.46% | 0.44% |  | 0.42% |  | 108% | 110% |
| Class D | -0.46% | 0.52% |  | 0.36% |  | 107% | 107% |
| Class F | -0.21% | 0.20% |  | 0.15% |  | 105% | 108% |
|  | Random Access |  |  |  |  |  |  |
|  | Over VTM-6.0.noALF |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT | DecT |
| Class A1 | -0.39% | 0.22% |  | 0.26% |  | 104% | 107% |
| Class A2 | -0.90% | 0.17% |  | 0.15% |  | 104% | 105% |
| Class B | -0.59% | 0.31% |  | 0.43% |  | 105% | 105% |
| Class C | -0.53% | 0.36% |  | 0.36% |  | 104% | 103% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.60% | 0.28% |  | 0.32% |  | 104% | 105% |
| Class D | -0.50% | 0.27% |  | 0.24% |  | 104% | 102% |
| Class F | -0.14% | 0.23% |  | 0.41% |  | 103% | 104% |
|  | Low delay B |  |  |  |  |  |  |
|  | Over VTM-6.0.noALF |  |  |  |  |  |  |
|  | Y |  | U |  | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.42% |  | 0.83% |  | 0.48% | 104% | 105% |
| Class C | -0.65% |  | 0.54% |  | 0.46% | 104% | 103% |
| Class E | -0.39% |  | 0.71% |  | 1.02% | 102% | 105% |
| Overall | -0.49% |  | 0.70% |  | 0.61% | 103% | 104% |
| Class D | -0.61% |  | 1.43% |  | 0.99% | 104% | 103% |
| Class F | 0.16% |  | 0.85% |  | 0.32% | 103% | 105% |
|  | Low delay P |  |  |  |  |  |  |
|  | Over VTM-6.0.noALF |  |  |  |  |  |  |
|  | Y |  | U |  | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.52% |  | 1.24% |  | 1.35% | 106% | 106% |
| Class C | -0.69% |  | 0.65% |  | 0.48% | 105% | 104% |
| Class E | -0.55% |  | 0.93% |  | 0.78% | 103% | 105% |
| Overall | -0.58% |  | 0.97% |  | 0.91% | 105% | 105% |
| Class D | -0.82% |  | 1.08% |  | 0.85% | 105% | 102% |
| Class F | 0.03% |  | 0.66% |  | 0.62% | 104% | 105% |

# Effect of chroma filtering

In CE5-3.2 Hadamard filter is applied to luma component only. The simulation results demonstrated some drop for chroma components. In order to compensate that the filter is applied to chroma components as well. The following results were obtained.

## Chroma filtering based on CE5-3.2 software

Test 5. SAO is off (corresponding to CE5-3.2.2)

|  | All Intra |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0.noSAO |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT | DecT |
| Class A1 | -0.39% | 0.01% |  | 0.01% |  | 109% | 109% |
| Class A2 | -0.37% | -0.01% |  | -0.01% |  | 109% | 109% |
| Class B | -0.32% | 0.02% |  | 0.55% |  | 109% | 109% |
| Class C | -0.55% | -0.34% |  | -0.15% |  | 109% | 109% |
| Class E | -0.51% | -0.24% |  | 0.07% |  | 109% | 109% |
| Overall | -0.42% | -0.11% |  | 0.13% |  | 109% | 109% |
| Class D | -0.50% | -0.22% |  | -0.15% |  | 109% | 108% |
| Class F | -0.51% | -0.48% |  | -0.50% |  | 106% | 109% |
|  | Random Access |  |  |  |  |  |  |
|  | Over VTM-6.0.noSAO |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT | DecT |
| Class A1 | -0.27% | 0.17% |  | -0.03% |  | 105% | 103% |
| Class A2 | -0.69% | -0.24% |  | -0.04% |  | 105% | 102% |
| Class B | -0.46% | 0.06% |  | 0.82% |  | 106% | 102% |
| Class C | -0.46% | -0.06% |  | 0.42% |  | 105% | 101% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.47% | -0.01% |  | 0.37% |  | 105% | 102% |
| Class D | -0.48% | -0.05% |  | 0.28% |  | 104% | 100% |
| Class F | -0.44% | -0.38% |  | 0.12% |  | 104% | 101% |
|  | Low delay B |  |  |  |  |  |  |
|  | Over VTM-6.0.noSAO |  |  |  |  |  |  |
|  | Y |  | U |  | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.14% |  | 0.39% |  | 0.34% | 104% | 102% |
| Class C | -0.33% |  | 0.13% |  | 0.31% | 104% | 102% |
| Class E | 0.21% |  | 0.00% |  | 0.37% | 101% | 101% |
| Overall | -0.22% |  | 0.20% |  | 0.33% | 103% | 102% |
| Class D | -0.54% |  | 0.95% |  | 0.47% | 104% | 101% |
| Class F | -0.15% |  | 0.65% |  | 0.01% | 103% | 102% |
|  | Low delay P |  |  |  |  |  |  |
|  | Over VTM-6.0.noSAO |  |  |  |  |  |  |
|  | Y |  | U |  | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.22% |  | 0.57% |  | 0.62% | 106% | 103% |
| Class C | -0.40% |  | 0.22% |  | 0.34% | 105% | 102% |
| Class E | -0.31% |  | 0.68% |  | 1.86% | 102% | 101% |
| Overall | -0.30% |  | 0.48% |  | 0.84% | 104% | 102% |
| Class D | -0.50% |  | 0.69% |  | 0.45% | 104% | 101% |
| Class F | -0.36% |  | 0.19% |  | -0.29% | 103% | 102% |

Test 6. SAO and ALF are off (corresponding to CE5-3.2.2 NO ALF)

|  | All Intra |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0.noSAO.noALF |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT | DecT |
| Class A1 | -0.87% | -1.34% |  | -2.06% |  | 110% | 112% |
| Class A2 | -0.90% | -0.69% |  | -0.36% |  | 110% | 111% |
| Class B | -0.76% | -0.46% |  | 0.07% |  | 109% | 110% |
| Class C | -1.03% | -0.94% |  | -1.01% |  | 109% | 110% |
| Class E | -1.21% | -0.85% |  | -1.08% |  | 109% | 112% |
| Overall | -0.93% | -0.82% |  | -0.79% |  | 109% | 111% |
| Class D | -0.83% | -0.80% |  | -1.03% |  | 109% | 107% |
| Class F | -1.10% | -1.14% |  | -1.20% |  | 106% | 111% |
|  | Random Access |  |  |  |  |  |  |
|  | Over VTM-6.0.noSAO.noALF |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT | DecT |
| Class A1 | -0.83% | 0.38% |  | 0.14% |  | 106% | 104% |
| Class A2 | -2.02% | -0.21% |  | 0.12% |  | 106% | 103% |
| Class B | -1.60% | 0.24% |  | 1.33% |  | 107% | 103% |
| Class C | -0.99% | 0.07% |  | 0.77% |  | 105% | 102% |
| Class E |  |  |  |  |  |  |  |
| Overall | -1.36% | 0.13% |  | 0.70% |  | 106% | 103% |
| Class D | -0.66% | 0.24% |  | 0.58% |  | 105% | 102% |
| Class F | -1.05% | -0.49% |  | -0.35% |  | 104% | 102% |
|  | Low delay B |  |  |  |  |  |  |
|  | Over VTM-6.0.noSAO.noALF |  |  |  |  |  |  |
|  | Y |  | U |  | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -1.27% |  | 1.00% |  | 1.24% | 105% | 103% |
| Class C | -1.37% |  | 0.02% |  | 0.50% | 104% | 102% |
| Class E | -1.04% |  | 1.42% |  | 0.32% | 102% | 102% |
| Overall | -1.25% |  | 0.78% |  | 0.76% | 104% | 103% |
| Class D | -0.97% |  | 0.53% |  | 1.15% | 105% | 102% |
| Class F | -1.30% |  | -0.14% |  | -0.03% | 103% | 103% |
|  | Low delay P |  |  |  |  |  |  |
|  | Over VTM-6.0.noSAO.noALF |  |  |  |  |  |  |
|  | Y |  | U |  | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -2.84% |  | 0.49% |  | 1.24% | 108% | 104% |
| Class C | -2.29% |  | -0.07% |  | 0.35% | 106% | 103% |
| Class E | -2.40% |  | 0.59% |  | 0.20% | 104% | 102% |
| Overall | -2.55% |  | 0.33% |  | 0.68% | 106% | 103% |
| Class D | -1.96% |  | 0.72% |  | 0.40% | 107% | 102% |
| Class F | -1.54% |  | -0.24% |  | -0.45% | 105% | 102% |

## Chroma filtering based on CE5-3.2 software plus bug fixes

In a following tables effect of chroma filtering applied CE5-3.2 also with fixes mentioned above as Aspects 1, 2, and 3.

Test 5b. SAO is off (corresponding to CE5-3.2.2)

|  | All Intra |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0.noSAO |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT | DecT |
| Class A1 | -0.38% | -0.04% |  | 0.12% |  | 110% | 109% |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.34% | -0.02% |  | 0.41% |  | 110% | 109% |
| Class C | -0.58% | -0.43% |  | -0.18% |  | 110% | 108% |
| Class E | -0.51% | -0.32% |  | 0.16% |  | 109% | 109% |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.52% | -0.31% |  | -0.26% |  | 110% | 108% |
| Class F | -0.49% | -0.39% |  | -0.46% |  | 106% | 109% |
|  | Random Access |  |  |  |  |  |  |
|  | Over VTM-6.0.noSAO |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.44% | 0.01% |  | 0.77% |  | 106% | 101% |
| Class C | -0.46% | -0.04% |  | 0.27% |  | 105% | 101% |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.53% | -0.34% |  | 0.26% |  | 105% | 100% |
| Class F | -0.47% | -0.40% |  | 0.01% |  | 104% | 101% |
|  | Low delay B |  |  |  |  |  |  |
|  | Over VTM-6.0.noSAO |  |  |  |  |  |  |
|  | Y |  | U |  | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C | -0.29% |  | 0.15% |  | 0.48% | 104% | 102% |
| Class E | -0.03% |  | 0.25% |  | 0.29% | 101% | 100% |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.57% |  | 0.98% |  | 0.72% | 104% | 99% |
| Class F |  |  |  |  |  |  |  |
|  | Low delay P |  |  |  |  |  |  |
|  | Over VTM-6.0.noSAO |  |  |  |  |  |  |
|  | Y |  | U |  | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C | -0.35% |  | -0.06% |  | 0.38% | 105% | 102% |
| Class E | -0.20% |  | 0.49% |  | 2.25% | 102% | 100% |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.57% |  | 0.79% |  | 0.39% | 104% | 99% |
| Class F |  |  |  |  |  |  |  |

Test 6b. SAO and ALF are off (corresponding to CE5-3.2.2 NO ALF)

|  | All Intra |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0.noSAO.noALF |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT | DecT |
| Class A1 | -0.86% | -1.37% |  | -1.97% |  | 110% | 111% |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.77% | -0.50% |  | -0.09% |  | 110% | 110% |
| Class C | -1.04% | -1.05% |  | -1.01% |  | 110% | 110% |
| Class E | -1.21% | -0.91% |  | -1.00% |  | 110% | 111% |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.86% | -0.89% |  | -1.13% |  | 110% | 108% |
| Class F |  |  |  |  |  |  |  |
|  | Random Access |  |  |  |  |  |  |
|  | Over VTM-6.0.noSAO.noALF |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -1.59% | 0.17% |  | 1.15% |  | 106% | 102% |
| Class C | -0.99% | 0.01% |  | 0.64% |  | 105% | 102% |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.73% | -0.11% |  | 0.50% |  | 105% | 102% |
| Class F | -1.08% | -0.45% |  | -0.33% |  | 105% | 102% |
|  | Low delay B |  |  |  |  |  |  |
|  | Over VTM-6.0.noSAO.noALF |  |  |  |  |  |  |
|  | Y |  | U |  | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C | -1.35% |  | 0.07% |  | 0.61% | 105% | 102% |
| Class E | -1.01% |  | 0.86% |  | 1.10% | 101% | 101% |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.98% |  | 0.55% |  | 0.55% | 105% | 100% |
| Class F |  |  |  |  |  |  |  |
|  | Low delay P |  |  |  |  |  |  |
|  | Over VTM-6.0.noSAO.noALF |  |  |  |  |  |  |
|  | Y |  | U |  | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C | -2.33% |  | 0.01% |  | 0.44% | 106% | 102% |
| Class E | -2.33% |  | 0.14% |  | 1.06% | 103% | 102% |
| Overall |  |  |  |  |  |  |  |
| Class D | -1.97% |  | 0.47% |  | 0.98% | 106% | 101% |
| Class F |  |  |  |  |  |  |  |

# Summary

## Filter integration fixes

Below is the summary of simulation results of described above fixes applied to CE5-3.2.

Test 7. Simulation summary over VTM6.0

|  | All Intra |  |  |  |  |  | Random Access |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# |  | Y | U | V | EncT | DecT |  | Y | U | V | EncT | DecT |
| CE5-3.2.1 |  | -0.33% | 0.39% | 0.37% | 107% | 108% |  | -0.37% | 0.30% | 0.31% | 104% | 103% |
| CE5-3.2.1 + fixes |  | -0.44% | 0.43% | 0.44% | 108% | 106% |  | -0.42% | 0.36% | 0.36% | 104% | 104% |

|  | Low Delay B |  |  |  |  |  | Low Delay P |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# |  | Y | U | V | EncT | DecT |  | Y | U | V | EncT | DecT |
| CE5-3.2.1 |  | -0.08% | 0.68% | 0.64% | 103% | 103% |  | -0.16% | 0.47% | 0.73% | 104% | 103% |
| CE5-3.2.1 + fixes |  | -0.17%* | 1.00% | 1.02% | 103% | 102% |  | -0.22% | 0.98% | 1.09% | 104% | 102% |

Test 8. Simulation summary over VTM6.0.noALF

|  | All Intra |  |  |  |  |  | Random Access |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# |  | Y | U | V | EncT | DecT |  | Y | U | V | EncT | DecT |
| CE5-3.2.1 |  | -0.46% | 0.44% | 0.42% | 108% | 110% |  | -0.60% | 0.28% | 0.32% | 104% | 105% |
| CE5-3.2.1 + fixes |  | -0.67% | 0.61% | 0.79% | 108% | 108% |  | -0.72% | 0.59% | 0.49% | 104% | 105% |

|  | Low Delay B |  |  |  |  |  | Low Delay P |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# |  | Y | U | V | EncT | DecT |  | Y | U | V | EncT | DecT |
| CE5-3.2.1 |  | -0.49% | 0.70% | 0.61% | 103% | 104% |  | -0.58% | 0.97% | 0.91% | 105% | 105% |
| CE5-3.2.1 + fixes |  | -0.53%* | 1.63% | 1.85% | 103% | 104% |  | -0.69% | 2.05% | 2.14% | 105% | 104% |

## Effect of chroma filtering

The tables below summarise effect of chroma filtering using Hadamard filter applied in addition to CE5-3.2. Effect of SAO based on JVET-P0045 are also given for comparison.

Test 9. Simulation summary over VTM6.0.noSAO

|  | All Intra |  |  |  |  |  | Random Access |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# |  | Y | U | V | EncT | DecT |  | Y | U | V | EncT | DecT |
| SAO |  | 0.00% | -0.13% | -0.15% | 66% | 77% |  | -0.08% | -0.26% | -0.35% | 71% | 72% |
| CE5-3.2.2 |  | -0.42% | 0.39% | 0.36% | 108% | 105% |  | -0.46% | 0.11% | 0.16% | 104% | 101% |
| CE5-3.2.2+chroma |  | -0.42% | -0.11% | 0.13% | 109% | 109% |  | -0.47% | -0.01% | 0.37% | 105% | 102% |

|  | Low Delay B |  |  |  |  |  | Low Delay P |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# |  | Y | U | V | EncT | DecT |  | Y | U | V | EncT | DecT |
| SAO |  | -0.10% | -0.58% | -1.13% | 72% | 70% |  | -0.12% | -0.28% | -0.82% | 72% | 70% |
| CE5-3.2.2 |  | -0.25% | 0.47% | 0.31% | 102% | 101% |  | -0.32% | 0.59% | 0.62% | 103% | 101% |
| CE5-3.2.2+chroma |  | -0.22% | 0.20% | 0.33% | 103% | 102% |  | -0.30% | 0.48% | 0.84% | 104% | 102% |

Test 10. Simulation summary over VTM6.0.noSAO.noALF

|  | All Intra |  |  |  |  |  | Random Access |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# |  | Y | U | V | EncT | DecT |  | Y | U | V | EncT | DecT |
| SAO |  | -0.32% | -0.37% | -0.70% | 100% | 104% |  | -0.79% | -0.80% | -0.72% | 100% | 102% |
| CE5-3.2.2 |  | -0.93% | 0.43% | 0.41% | 108% | 107% |  | -1.35% | 0.13% | 0.14% | 104% | 102% |
| CE5-3.2.2+chroma |  | -0.93% | -0.82% | -0.79% | 109% | 111% |  | -1.36% | 0.13% | 0.70% | 106% | 103% |

|  | Low Delay B |  |  |  |  |  | Low Delay P |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# |  | Y | U | V | EncT | DecT |  | Y | U | V | EncT | DecT |
| SAO |  | -0.95% | -1.71% | -2.82% | 100% | 102% |  | -2.59% | -3.06% | -3.65% | 100% | 102% |
| CE5-3.2.2 |  | -1.34% | 1.00% | 0.83% | 103% | 102% |  | -2.65% | 0.71% | 1.10% | 105% | 103% |
| CE5-3.2.2+chroma |  | -1.25% | 0.78% | 0.76% | 104% | 103% |  | -2.55% | 0.33% | 0.68% | 106% | 103% |

# Conclusion

Applying fixes to in-loop RDO decision and to SAO and Hadamard filter averaging stage provides additionally ~0.11% of coding gain for AI and 0.05% for RA in comparison to test CE5-3.2.1. Additionally, enabling filtering for chroma components allows further increase filter performance.We would like to thank Ericsson for cross-checking.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).