| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0413 |
| --- | --- |

| Title: | CE4-related: Clipping for PROF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wei Chen, Yuwen He 9276 Scranton Road, Suite 300 San Diego, CA 92121, USA | Tel: Email: | 1-858-210-4855 Wei.Chen@InterDigital.com Yuwen.He@InterDigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

Prediction refinement with optical flow (PROF) [1] for affine mode is adopted in current VVC spec [4]. In this contribution, clipping is proposed to PROF refinement offset of each list in order to keep the prediction of each list is within 16-bit range. Two tests for this PROF refinement offset clipping are performed. In the first test, the refinement offset derived from optical flow is clipped within 14-bit range [-8192, 8191], so that the refined prediction signals of each list are still within 16-bit range. In the second test, the refinement offset is clipped based on current CE4-2.1, which proposes to change delta MV precision from 1/64-pel to 1/32-pel and delta MV range is clipped from [-64, 63] to [-32, 31].

Simulation results for test 1 show that the luma BD-rate are 0.00% and 0.00% in RA and LDB cases, respectively, when compared to VTM-6.0.

Simulation results for test 2 show that the luma BD-rate are x.xx% and x.xx% in RA and LDB cases, respectively, when compared to CE 4.2.1.

# Introduction

In the current VVC spec [4], PROF is applied to refine the sub-block based affine motion compensated prediction with optical flow. Specifically, after the sub-block based affine motion compensation is performed, luma prediction sample is refined by adding a difference derived by the optical flow equation defined in [1].

In VTM-6.0, there is no clipping operation applied for PROF refinement offset. Therefore, after adding the refinement offset, the refined prediction signal of each list may exceed the 16-bit range and become 17-bit.

In VTM-6.0, the value range of predicted signals after affine motion compensation is within 2*a*b*255/64 = -15077 and (a2 + b2)*255/64= 31397, where a is the sum of negative filter coefficients and b is the sum of the filter coefficients. Note that 6-tab interpolation filter is used for affine motion compensation, such that |a| is at most 86, while b is at most 22. If the refinement offset is clipped within 14-bit range, the refined prediction signal will have the range at [ -15077 - 8192, 31397 + 8191], which is still within 16-bit range 65536.

# Proposal

In this proposal, two tests are performed to clip the PROF refinement offset in 14-bit: (1) test is based on VTM-6.0; (2) test is based on CE4-2.1.

In the first test, the refinement offset derived from optical flow is clipped within 14-bit range [-8192, 8191], so that the refined prediction signals are still within 16-bit range. This simplification method brings below benefits:

- Reducing about 250 lines of code in current VTM software

  - In current VTM-6.0, without the proposed clipping operation for refinement offset, the code for PROF based uni-prediction and bi-prediction is separated.

- PROF transparently supporting weighted prediction

  - Due to the separate code for PROF, there is a bug in the current VTM-6.0 code that PROF is disabled when weighted prediction is enabled, which is not the intent in the current SPEC and original PROF design. If the proposed clipping operation of refinement offset is applied, this bug does not exist, and PROF would support weighted prediction transparently.

- Improved Hardware efficiency

  - The hardware component may be shared with other tools if the refined prediction signal does not exceed 16-bit range.

- Improved VTM SIMD code efficiency

  - If the refined prediction signal is restricted to be within 16-bit, 16-bit PEL variable can be used instead of 32-bit INT variable, which allows SIMD code working more samples at one time.

In the second test, the refinement offset is clipped in the same way as the first test but based on current CE4-2.1.

# Simulation results

The proposed method was implemented based on the VTM-6.0 reference software [2] and tested using the test condition and evaluation criteria that are used in CE4 [3].

Table 1 Test 1 for PROF

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM 6.0 |  |  |  |  | Over VTM 6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 99% | - | - | - | - | - |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 98% | - | - | - | - | - |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 99% | 0.00% | -0.01% | -0.01% | 100% | 101% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 98% | 0.00% | 0.00% | 0.00% | 100% | 96% |
| Class E | - | - | - | - | - | 0.00% | 0.00% | 0.00% | 100% | 98% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 98% | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class D | 0.00% | 0.00% | 0.00% | 99% | 98% | 0.00% | 0.00% | 0.00% | 100% | 98% |
| Class F | 0.01% | 0.01% | 0.01% | 100% | 99% | 0.07% | 0.20% | 0.00% | 100% | 98% |

Table 2 Test 2 for PROF

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over CE4.2.1 |  |  |  |  | Over CE4.2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | - | - | - | - | - |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | - | - | - | - | - |
| Class B | 0.00% | 0.00% | 0.00% | #DIV/0! | #DIV/0! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class C | 0.00% | 0.00% | 0.00% | #DIV/0! | #DIV/0! | 0.00% | 0.00% | 0.00% | #DIV/0! | #DIV/0! |
| Class E | - | - | - | - | - | 0.00% | 0.00% | 0.00% | #DIV/0! | #DIV/0! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | 0.00% | 0.00% | 0.00% | #DIV/0! | #DIV/0! | 0.00% | 0.00% | 0.00% | #DIV/0! | #DIV/0! |
| Class F | 0.00% | 0.00% | 0.01% | #DIV/0! | #DIV/0! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

# Reference

- J. Luo, Y. He, “CE4-2.1: Prediction refinement with optical flow for affine mode”, JVET-O0070, Gothenburg, SE, July 2019.

- VTM-6.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-6.0.

- C.-C. Chen, H. Yang, X. Xiu, “Description of Core Experiment 4 (CE4): Inter prediction”, JVET-O2024, July 2019, Gothenburg, Sweden.

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 6)”, JVET-O2001, July 2019, Gothenburg, Sweden.

# Patent rights declaration(s)

InterDigital Communications, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).