| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0414 |
| --- | --- |

| Title: | Non-CE4: Simplification of SIF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wei Chen, Yuwen He 9276 Scranton Road, Suite 300 San Diego, CA 92121, USA | Tel: Email: | 1-858-210-4855 Wei.Chen@InterDigital.com Yuwen.He@InterDigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

In the current VVC SPEC [4], when half-luma-sample MVD precision is signaled for a CU, a 6-tap half-pel switchable interpolation filter (SIF) instead of the default 8-tap interpolation filter is used. This contribution proposes to simplify SIF by adding a CU size restriction. In the first test, half-pel MVD precision is disabled if either the width or height of a CU is larger than 32. In the second test, half-pel MVD precision is disabled if the area of a CU is larger than 1024. Simulation results are shown as below:

For test 1, Y-BD-rate=0.03%, EncT=98%, DecT=99% for RA configuration, and Y-BD-rate=0.02%, EncT=99%, DecT=101% for LDB configuration.

For test 2, Y-BD-rate=0.01%, EncT=99%, DecT=100% for RA configuration, and Y-BD-rate=0.01%, EncT=99%, DecT=100% for LDB configuration.

According to most resent tool off test [5], SIF has a 4% encoding time, and this proposed simplification provides a complexity reduction by 2%.

# Introduction

In current VVC SPEC [4], a CU-level adaptive motion vector resolution (AMVR) scheme is introduced. AMVR allows MVD of the CU to be coded in different precision. In the case of half-luma-sample MVD precision, a 6-tap interpolation filter instead of the default 8-tap interpolation filter is used. This 6-tap Gauss filter is known to introduce certain level of smoothing effect. If a CU contains higher frequency content, SIF may cause over-smoothing. For bigger size CU, SIF may not be selected.

# Proposal

This contribution proposes to simplify SIF by adding a CU level size restriction. In the first test, half-pel MVD precision is disabled if a CU has either the width or the height larger than 32. In the second test, half-pel MVD precision is disabled if a CU’s area is larger than 1024.

# Simulation results

The proposed methods were implemented based on the VTM-6.0 reference software [2] and tested using the test condition and evaluation criteria that are used in CE4 [3].

Table 1.  Test 1 SIF does not apply to a CU with (width > 32 or height > 32)

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM 6.0 |  |  |  |  | Over VTM 6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | 0.14% | 0.07% | 97% | 100% | - | - | - | - | - |
| Class A2 | 0.06% | 0.13% | 0.13% | 98% | 100% | - | - | - | - | - |
| Class B | 0.03% | 0.14% | 0.03% | 98% | 99% | 0.00% | -0.12% | -0.06% | 99% | 102% |
| Class C | -0.01% | 0.01% | -0.05% | 99% | 98% | 0.02% | -0.07% | 0.15% | 99% | 100% |
| Class E | - | - | - | - | - | 0.03% | 0.27% | -0.26% | 99% | 101% |
| Overall | 0.03% | 0.10% | 0.03% | 98% | 99% | 0.02% | -0.01% | -0.04% | 99% | 101% |
| Class D | -0.01% | 0.03% | 0.05% | 99% | 100% | -0.01% | 0.12% | 0.03% | 99% | 97% |
| Class F | -0.02% | -0.03% | -0.02% | 99% | 99% | 0.08% | 0.19% | 0.35% | 100% | 99% |

Table 2.  Test 2 SIF does not apply to a CU with (width * height > 1024)

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM 6.0 |  |  |  |  | Over VTM 6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.17% | 0.04% | 98% | 100% | - | - | - | - | - |
| Class A2 | 0.04% | 0.11% | 0.07% | 99% | 100% | - | - | - | - | - |
| Class B | 0.01% | 0.11% | 0.05% | 99% | 99% | 0.01% | 0.00% | -0.08% | 99% | 102% |
| Class C | -0.01% | 0.01% | -0.07% | 100% | 101% | 0.01% | 0.08% | 0.02% | 99% | 98% |
| Class E | - | - | - | - | - | 0.02% | 0.30% | 0.01% | 99% | 99% |
| Overall | 0.01% | 0.10% | 0.02% | 99% | 100% | 0.01% | 0.10% | -0.03% | 99% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 102% | 0.06% | 0.08% | 0.04% | 100% | 98% |
| Class F | 0.01% | -0.02% | -0.03% | 100% | 100% | 0.12% | 0.30% | 1.02% | 100% | 98% |

# Reference

- J. Chen, Y. Ye, S. Kim, “Algorithm description for Versatile Video Coding and Test Model 6 (VTM 6) v2”, JVET-O2002, Gothenburg, SE, July 2019.

- VTM-6.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-6.0.

- C.-C. Chen, H. Yang, X. Xiu, “Description of Core Experiment 4 (CE4): Inter prediction”, JVET-O2024, July 2019, Gothenburg, Sweden.

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 6)”, JVET-O2001, July 2019, Gothenburg, Sweden.

- AHG13 tool off test, https://hevc.hhi.fraunhofer.de/svn/svn_VVCTestConfig/branches/VTM-6.0/Results/.

# Patent rights declaration(s)

InterDigital Communications, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).