| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0415 |
| --- | --- |

| Title: | Non-CE4: Unification of DMVR and BDOF enabling conditions |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wei Chen, Yuwen He 9276 Scranton Road, Suite 300 San Diego, CA 92121, USA | Tel: Email: | 1-858-210-4855 Wei.Chen@InterDigital.com Yuwen.He@InterDigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Proposed specification text change for method 1

### General decoding process for coding units coded in inter prediction mode

- If MergeTriangleFlag[ xCb ][ yCb ], inter_affine_flag[ xCb ][ yCb ] and merge_subblock_flag[ xCb ][ yCb ] are all equal to 0, the following applies:

- The derivation process for motion vector components and reference indices as specified in clause 8.5.2.1 is invoked with the luma coding block location ( xCb, yCb ), the luma coding block width cbWidth and the luma coding block height cbHeight as inputs, and the luma motion vectors mvL0[ 0 ][ 0 ] and mvL1[ 0 ][ 0 ], the reference indices refIdxL0 and refIdxL1 and the prediction list utilization flags predFlagL0[ 0 ][ 0 ] and predFlagL1[ 0 ][ 0 ], the half sample interpolation filter index hpelIfIdx, and the bi-prediction weight index bcwIdx as outputs.

- When all of the following conditions are true, dmvrFlag is set equal to 1:

- sps_dmvr_enabled_flag is equal to 1 and slice_disable_bdof_dmvr_flag is equal to 0

- general_merge_flag[ xCb ][ yCb ] is equal to 1

- both predFlagL0[ 0 ][ 0 ] and predFlagL1[ 0 ][ 0 ] are equal to 1

- mmvd_merge_flag[ xCb ][ yCb ] is equal to 0

- ciip_flag[ xCb ][ yCb ] is equal to 0

- DiffPicOrderCnt( currPic, RefPicList[ 0 ][ refIdxL0 ]) is equal to DiffPicOrderCnt( RefPicList[ 1 ][ refIdxL1 ], currPic )

- BcwIdx[ xCb ][ yCb ] is equal to 0

- Both luma_weight_l0_flag[ refIdxL0 ] and luma_weight_l1_flag[ refIdxL1 ] are equal to 0

- cbWidth is greater than or equal to 8

- cbHeight is greater than or equal to 8

- cbHeight*cbWidth is greater than or equal to 128

- For X being each of 0 and 1, the pic_width_in_luma_samples and pic_height_in_luma_samples of the reference picture refPicLX associated with the refIdxLX are equal to the pic_width_in_luma_samples and pic_height_in_luma_samples of the current picture, respectively.