| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0417 |
| --- | --- |

| Title: | AHG13: Removal of Intra Subblock Partitioning (ISP) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tim Hellman Wade Wan Brian Heng | Email: | firstname.lastname@broadcom.com |
| Source: | Broadcom Inc. |  |  |

_____________________________

# Abstract

This contribution proposes a claimed simplification to VVC by the removal of the Intra Subblock Partitioning (ISP) mode.  It contends that ISP does not provide sufficient coding gain to justify its design complexity, especially at higher resolutions.  It further claims that the unique transform and prediction sizes defined by ISP add an implementation and verification burden to decoders.  The proposal recommends the complete removal of ISP mode from the text.

# Description

The Intra Subblock Partitioning (ISP) tool works by dividing an intra luma CU into 2 or 4 vertical or horizontal partitions.  These partitions determine the transform unit (TU) size and, in most cases, the prediction unit (PU) size as well.  The figure below provides some examples.

Figure 1: Example sub-partitions

ISP substantially increases the number of TU shapes.  Figure 2 enumerates all the allowed luma transform shapes in VVC by X and Y dimension.  A ‘Y’ indicates a supported shape, and the red-shaded boxes are TU shapes unique to ISP.  VVC without ISP has 25 transform shapes – ISP adds 14, or 56% more.  Implementing these exclusive shapes requires different scan patterns and coefficient mapping logic in the decoder’s inverse transform block, and significantly increases the number of shape combinations that must be generated and tested.  In addition, the added sizes are very narrow or short (1 or 2 pixels in the narrow dimension), which can present design cost challenges for transform blocks attempting to meet the required throughput by processing multiple pixels per cycle.

Figure 2: Unique TU sizes introduced by ISP (in red)

ISP also creates new PU shapes by splitting the CU vertically.  As with the transform shapes, the new PU shapes complicate the spatial prediction logic as all 67 intra prediction modes need to be supported for them.  This also potentially affects throughput.  Moreover, ISP mode is the only regular spatial prediction mode where the PU size is different than the CU size.  This is a blemish on the otherwise elegant VVC CU design, which eliminates all the complicated PU shapes present in HEVC.

Finally, ISP introduces significant complexity to the specification. 144 lines of text are dedicated to the description of the ISP function and its interaction with other tools.  ISP also requires 2 unique syntax elements and 5 CABAC contexts.  The associated sample draft text document details the changes.

# Experimental Results

The results listed here are summarized from the AHG13 test results [1].  The tests were carried out using the VTM6.0 reference software with the ISP tool disabled.

When compared to the VTM6.0 anchor with ISP enabled, the average BD-rate increase in AI/RA/LD_B is 0.41%/0.20%/0.10%.  The average BD-rate increase for class A1 and A2 4K sequences is 0.18%/0.15%/0.07%.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.08% | -0.06% | -0.16% | 87% | 98% |
| Class A2 | 0.27% | 0.21% | 0.24% | 87% | 98% |
| Class B | 0.40% | 0.36% | 0.30% | 85% | 97% |
| Class C | 0.55% | 0.32% | 0.53% | 84% | 97% |
| Class E | 0.72% | 0.58% | 0.62% | 85% | 97% |
| Overall | 0.41% | 0.29% | 0.32% | 86% | 97% |
| Class D | 0.48% | 0.45% | 0.52% | 84% | 96% |
| Class F (optional) | 0.54% | 0.25% | 0.16% | 92% | 97% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.13% | 0.14% | 0.10% | 97% | 100% |
| Class A2 | 0.17% | 0.13% | 0.04% | 98% | 100% |
| Class B | 0.20% | 0.35% | 0.28% | 97% | 100% |
| Class C | 0.26% | 0.31% | 0.27% | 97% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.20% | 0.25% | 0.19% | 97% | 100% |
| Class D | 0.22% | -0.31% | 0.25% | 97% | 100% |
| Class F (optional) | 0.36% | 0.10% | 0.41% | 97% | 99% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.10% | 0.43% | 0.05% | 98% | 100% |
| Class C | 0.05% | 0.24% | 0.24% | 98% | 100% |
| Class E | 0.16% | 0.53% | 0.52% | 100% | 101% |
| Overall | 0.10% | 0.39% | 0.23% | 98% | 100% |
| Class D | 0.07% | 0.06% | 0.49% | 98% | 99% |
| Class F (optional) | 0.28% | 0.29% | 0.77% | 99% | 99% |

Table 1. VTM6.0 with ISP disabled vs anchor

# Conclusion

ISP was adopted into the standard at the Marrakech meeting [2].  The variant adopted reported an AI/RA gain of 0.57%/0.27%.  Subsequent tool adoptions and ISP changes have reduced its effectiveness by roughly 25%, to the point where it has the least gain of any non-screen content tool tested in AHG13 [1].   Its benefit for high-resolution video, where its added encoder complexity might be justified, is very close to zero.

VVC already has at least 8 other tools that improve intra coding over HEVC.  The ISP tool is complex, and its benefit is small.  For these reasons, this proposal recommends the removal of ISP mode.

# References

[1] W Chien, J.Boyce et al, “JVET AHG report: Tool reporting procedure (AHG13)”, JVET-P0013-v1, the 16th JVET meeting, October 2019, Geneva, CH

[2] S. De-Luxán-Hernández et al, “CE3: Intra Sub-Partitions Coding Mode (Tests 1.1.1 and 1.1.2)”, JVET-M0102, the 13th JVET meeting, Jan 2019, Marrakech, MA

[3] J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, the 14th JVET meeting, March 2019, Geneva, CH

[4] B. Bross, J. Chen and S. Liu, “Versatile Video Coding (Draft 6)”, JVET-O2001-v14, the 15th JVET meeting, July 2019, Gothenburg, SE

# Patent rights declaration(s)

Broadcom Inc. does not have current or pending patent rights relating to the technology described in this contribution.