| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0424-v1 |
| --- | --- |

| Title: | Non-CE8: Resetting palette predictor at CTU row |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Weijia Zhu, Li Zhang, Jizheng Xu 8910 University Center Lane San Diego, CA, U.S.A. 92122 | Tel: Email: | +1-360-525-0535 weijia.zhu@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

In the palette mode, a palette predictor is used for predicting the current palette. This palette predictor is reset at the beginning of each slice/tile/brick, and it is also reset at the beginning of each CTU row only when WPP is enabled. It is proposed to always reset the palette predictor at the beginning of each CTU row regardless if WPP is enabled. The proposed method is implemented and compared with VTM-6.0 following the CE8 test conditions. Results are summarized as below.

Dual tree on: AI: 0.07%, 99%, 100%; RA: 0.04%, 99%, 100%; LDB: -0.14%, 99%, 100%

Dual tree off: AI: 0.28%, 100%, 98%; RA: -0.13%, 99%, 99%; LDB: -0.38%, 99%, 101%.

# Introduction

In the palette mode, a palette predictor is used for predicting the current palette table. This palette predictor is reset at the beginning of each slice/tile/brick, and it is also reset at the beginning of each CTU row when WPP is enabled. Thus, when WPP is disabled, it is difficult to have parallel encoding among CTU rows.

# Proposed method

In this document, it is proposed to remove the WPP restriction on the reset of palette predictor, i.e. the reset the palette predictor is always reset at the beginning of each CTU row regardless if WPP is enabled.

The benefits are twofold. Firstly, the resetting procedures for palette of WPP on and WPP off cases can be unified. Secondly, it enables parallel encoding among CTU rows.

# Simulation results

The proposed method is evaluated with VTM6. The simulation follows the CE 8 test conditions [1]. The results are summarized in Table 1-6. More detailed results can be found in the attached spreadsheets.

Table 1. The performance of the proposed scheme versus VTM 6 under AI with dual tree on

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | 0.08% | 0.23% | -0.37% | 100% | 101% |
| TGM 720p | 0.32% | -0.51% | -0.61% | 98% | 97% |
| Animation | 0.24% | 0.39% | 0.31% | 100% | 99% |
| Mixed content | -0.42% | 1.05% | 0.63% | 99% | 100% |
| Camera-Captured content | -0.01% | 0.05% | 0.01% | 101% | 101% |
| Overall | 0.07% | 0.21% | -0.07% | 99% | 100% |

Table 2. The performance of the proposed scheme versus VTM 6 under RA with dual tree on

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | 0.00% | -0.19% | -0.20% | 100% | 101% |
| TGM 720p | 0.07% | -0.34% | -0.29% | 99% | 102% |
| Animation | 0.05% | 0.18% | 0.20% | 100% | 99% |
| Mixed content | 0.08% | 1.29% | 1.02% | 99% | 99% |
| Camera-Captured content | -0.01% | 0.03% | 0.09% | 99% | 99% |
| Overall | 0.04% | 0.15% | 0.12% | 99% | 100% |

Table 3. The performance of the proposed scheme versus VTM 6 under LDB with dual tree on

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | -0.29% | -0.62% | -0.60% | 100% | 102% |
| TGM 720p | -0.11% | -0.27% | 0.13% | 97% | 98% |
| Animation | 0.15% | 0.27% | 0.07% | 99% | 100% |
| Mixed content | -0.35% | 1.25% | 0.46% | 99% | 101% |
| Camera-Captured content | 0.01% | -0.01% | -0.05% | 102% | 102% |
| Overall | -0.14% | 0.06% | -0.03% | 99% | 100% |

Table 4. The performance of the proposed scheme versus VTM 6 under AI with dual tree off

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | 0.37% | 0.67% | -0.27% | 99% | 98% |
| TGM 720p | 0.14% | -0.80% | -1.51% | 99% | 95% |
| Animation | 0.85% | 1.24% | 1.48% | 100% | 97% |
| Mixed content | -0.02% | -1.60% | -1.60% | 101% | 100% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Overall | 0.28% | -0.10% | -0.47% | 100% | 98% |

Table 5. The performance of the proposed scheme versus VTM 6 under RA with dual tree off

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | -0.19% | 0.05% | -0.72% | 99% | 100% |
| TGM 720p | -0.36% | -0.83% | -1.18% | 99% | 99% |
| Animation | 0.04% | 0.26% | 0.39% | 99% | 98% |
| Mixed content | 0.00% | -1.93% | -1.77% | 100% | 100% |
| Camera-Captured content | 0.00% | -0.01% | 0.00% | 99% | 101% |
| Overall | -0.13% | -0.51% | -0.73% | 99% | 99% |

Table 6. The performance of the proposed scheme versus VTM 6 under LDB with dual tree off

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | -0.49% | -0.33% | -0.99% | 98% | 101% |
| TGM 720p | -0.69% | -0.47% | -1.31% | 100% | 100% |
| Animation | 0.02% | 0.48% | 0.74% | 100% | 101% |
| Mixed content | -0.46% | -1.20% | -2.03% | 99% | 101% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Overall | -0.38% | -0.34% | -0.82% | 99% | 101% |

# Text changes

The text changes below are on top of JVET-O2001-vE [2]. Changes are highlight by grey. Deleted texts are marked with strikethrough.

7.3.8.1 General slice data syntax

| slice_data( ) { | Descriptor |
| --- | --- |
| for( i = 0; i < NumBricksInCurrSlice; i++ ) { |  |
| CtbAddrInBs = FirstCtbAddrBs[ SliceBrickIdx[ i ] ] |  |
| for( j = 0; j < NumCtusInBrick[ SliceBrickIdx[ i ] ]; j++, CtbAddrInBs++ ) { |  |
| if( ( j % BrickWidth[ SliceBrickIdx[ i ] ] ) = = 0 ) { |  |
| NumHmvpCand = 0 |  |
| NumHmvpIbcCand = 0 |  |
| ResetIbcBuf = 1 |  |
| PredictorPaletteSize = 0 |  |
| } |  |
| CtbAddrInRs = CtbAddrBsToRs[ CtbAddrInBs ] |  |
| coding_tree_unit( ) |  |
| if( entropy_coding_sync_enabled_flag && ( j < NumCtusInBrick[ SliceBrickIdx[ i ] ] – 1 ) && ( ( j + 1 ) % BrickWidth[ SliceBrickIdx[ i ] ] = = 0 ) ) { |  |
| end_of_subset_one_bit /* equal to 1 */ | ae(v) |
| byte_alignment( ) |  |
| } |  |
| } |  |
| end_of_brick_one_bit /* equal to 1 */ | ae(v) |
| if( i < NumBricksInCurrSlice − 1 ) |  |
| byte_alignment( ) |  |
| } |  |
| } |  |

9.3.2.1 General

#### Outputs of this process are initialized CABAC internal variables.

#### …

The context variables of the arithmetic decoding engine are initialized as follows:

–	If the CTU is the first CTU in a brick, the initialization process for context variables is invoked as specified in clause 9.3.2.2 and the variable PredictorPaletteSize is initialized to 0.

–	Otherwise, if entropy_coding_sync_enabled_flag is equal to 1 and either CtbAddrInRs % PicWidthInCtbsY is equal to 0 or BrickId[ CtbAddrInBs ] is not equal to BrickId[ CtbAddrRsToBs[ CtbAddrInRs − 1 ] ], the following applies:

–	The location ( xNbT, yNbT ) of the top-left luma sample of the spatial neighbouring block T (Figure 9-2) is derived using the location ( x0, y0 ) of the top-left luma sample of the current CTB as follows:

( xNbT, yNbT ) = ( x0, y0 − CtbSizeY )	(9-3)

–	The derivation process for neighbouring block availability as specified in clause 6.4.4 is invoked with the location ( xCurr, yCurr ) set equal to ( x0, y0 ), the neighbouring location ( xNbY, yNbY ) set equal to ( xNbT, yNbT ), checkPredModeY set equal to FALSE, and cIdx set equal to 0 as inputs, and the output is assigned to availableFlagT.

–	The synchronization process for context variables is invoked as follows:

–	If availableFlagT is equal to 1, the synchronization process for context variables as specified in clause 9.3.2.4 is invoked with TableStateIdx0Wpp, TableStateIdx1Wpp, TableMpsValWpp as inputs and the variable PredictorPaletteSize is initialized to 0.

–	Otherwise, the initialization process for context variables is invoked as specified in clause 9.3.2.2 and the variable PredictorPaletteSize is initialized to 0.

# Conclusion

It is proposed to reset the palette predictor at each CTU row to enhance the parallel degree. We recommend adopting the proposed method to the next version of VVC draft.

# References

- X. Xu, Y.-H Chao, Y.-C Sun and J. Xu, “Description of Core Experiment 8 (CE8): Screen Content Coding Tools,” JVET-O2028, Jul. 2019

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 6)”, JVET-O2001, Jul. 2019

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).