| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0431 |
| --- | --- |

| Title: | AHG12: On subpicture ID |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yao-Jen Chang Vadim Seregin Muhammed Coban Marta Karczewicz | Tel: Email: | +1 858 845 1779 yjchang@qti.qualcomm.com vseregin@qti.qualcomm.com mcoban@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

The proposal addresses the issues of sub-pictures extraction, which include how to extract subpictures from the original picture, how the subpicture ID is designed to identify which subpictures are extracted, and how to interact between different parameter sets and slice header. The proposed method is summarized as follows:

- Add a PPS flag to specify the number of subpictures extracted from the original picture.

- Add PPS syntax elements to specify the subpicture IDs of the extracted subpictures.

- Add a slice header syntax element to specify the value of subpicture ID for the slice in use.

# Introduction

The subpictures signaling was adopted in the previous meeting, and the syntax design [1] is signaled in SPS and is extracted in the table below:

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| subpics_present_flag | u(1) |
| if( subpics_present_flag ) { |  |
| max_subpics_minus1 | u(8) |
| subpic_grid_col_width_minus1 | u(v) |
| subpic_grid_row_height_minus1 | u(v) |
| for( i = 0; i < NumSubPicGridRows; i++ ) |  |
| for( j = 0; j < NumSubPicGridCols; j++ ) |  |
| subpic_grid_idx[ i ][ j ] | u(v) |
| for( i = 0; i <= NumSubPics; i++ ) { |  |
| subpic_treated_as_pic_flag[ i ] | u(1) |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| } |  |
| } |  |
| … |  |

However, the current design cannot specify how to extract subpictures from the original picture. This proposal addresses the issue by using subpicture ID in PPS and slice headers as elaborated in the following section.

# Signaling of subpicture ID

The proposed method is summarized as follows:

- Add a PPS flag num_subpicture_in_pic_minus1 to specify the number of subpictures extracted from the original picture.

  - The value of num_subpicture_in_pic_minus1 shall be in the range of 0 to the number of subpictures in the picture minus 1, inclusive.

- Add a PPS syntax element subpicture_id[ i ] to specify the subpicture IDs of the extracted subpictures.

  - The maximum value of the index i for subpicture_id shall be num_subpicture_in_pic_minus1.

  - The value of subpicture_id[ i ] shall be in the range of 0 to the number of subpictures in the picture minus 1, inclusive.

- Add a slice header syntax element slice_subpicture_id to specify the value of subpicture_id for the slice in use.

  - slice_subpicture_id must be one of the values of subpicture_id[ i ].

Figure 1. An example of subpicture layout

Figure 1 shows an example of subpicture layout. There are 4 subpictures and 6 slices in the picture. Each subpicture consists of at least one slice. In current design, the SPS syntax can derive the subpicture index for each grid from the syntax element subpic_grid_idx[ i ][ j ]. In the depicted example,  the value of subpic_grid_idx[ i ][ j ] is in the range of 0 to 3, inclusive.

Suppose Subpicture 1 and Subpicture 3 are extracted from the picture referring to the PPS. The proposed syntax elements num_subpicture_in_pic_minus1, subpicture_id[i] and slice_subpicture_id are signaled as illustrated in the table below and Figure 2.

| Suppose subpicture 1 and 3 are extracted |  |
| --- | --- |
| SPS | subpic_grid_idx[NumSubPicGridRows][NumSubPicGridCols] = {0, 1, 2, 3} |
| PPS | num_subpicture_in_pic_minus1 = 1 subpicture_id[0] = 1 subpicture_id[1] = 3 |
| Slice | slice_subpicture_id = 1 in Slice 2 and 3 slice_subpicture_id = 3 in Slice 5 |

Figure 2. Subpicture ID in the case of extraction of Subpicture 1 and Subpicture 3

The syntax changes to VVC Draft 6 are as described in the following table with yellow change mark:

Section 7.3.2.3 in VVC Draft 6 version 14: seq_parameter_set_rbsp( ) {; Descriptor. …. subpics_present_flag; u(1). if( subpics_present_flag ) {. max_subpics_minus1; u(8). subpic_grid_col_width_minus1; u(v). subpic_grid_row_height_minus1; u(v). for( i = 0; i < NumSubPicGridRows; i++ ). for( j = 0; j < NumSubPicGridCols; j++ ). subpic_grid_idx[ i ][ j ]; u(v). …. Section 7.4.3.3 in VVC Draft 6 version 14: subpics_present_flag equal to 1 indicates that subpicture parameters are present in the present in the SPS RBSP syntax. subpics_present_flag equal to 0 indicates that subpicture parameters are not present in the present in the SPS RBSP syntax. NOTE 2 – When a bitstream is the result of a sub-bitstream extraction process and contains only a subset of the subpictures of the input bitstream to the sub-bitstream extraction process, it might be required to set the value of subpics_present_flag equal to 1 in the RBSP of the SPSs. max_subpics_minus1 plus 1 specifies the maximum number of subpictures that may be present in the CVS. max_subpics_minus1 shall be in the range of 0 to 254. The value of 255 is reserved for future use by ITU-T ¦ ISO/IEC. subpic_grid_col_width_minus1 plus 1 specifies the width of each element of the subpicture identifier grid in units of 4 samples. The length of the syntax element is Ceil( Log2( pic_width_max_in_luma_samples / 4) ) bits. The variable NumSubPicGridCols is derived as follows: NumSubPicGridCols = ( pic_width_max_in_luma_samples + subpic_grid_col_width_minus1 * 4 + 3 ) / ( subpic_grid_col_width_minus1 * 4 + 4 ) (7-5) subpic_grid_row_height_minus1 plus 1 specifies the height of each element of the subpicture identifier grid in units of 4 samples. The length of the syntax element is Ceil( Log2( pic_height_max_in_luma_samples / 4) ) bits. The variable NumSubPicGridRows is derived as follows: NumSubPicGridRows = ( pic_height_max_in_luma_samples + subpic_grid_row_height_minus1 * 4 + 3 ) / ( subpic_grid_row_height_minus1 * 4 + 4 ) (7-6) subpic_grid_idx[ i ][ j ] specifies the subpicture index of the grid position (i, j). The length of the syntax element is Ceil( Log2( max_subpics_minus1 + 1 ) ) bits. The variables SubPicTop[ subpic_grid_idx[ i ][ j ] ], SubPicLeft[ subpic_grid_idx[ i ][ j ] ], SubPicWidth[ subpic_grid_idx [ i ][ j ] ], SubPicHeight[ subpic_grid_idx[ i ][ j ] ], and NumSubPics are derived as follows: NumSubPics = 0 for( i = 0; i. < NumSubPicGridRows; i++ ) { for( j = 0; j < NumSubPicGridCols; j++ ) { if ( i = = 0) SubPicTop[ subpic_grid_idx[ i ][ j ] ] = 0 else if( subpic_grid_idx[ i ][ j ] != subpic_grid_idx[ i – 1 ][ j ] ) { SubPicTop[ subpic_grid_idx[ i ][ j ] ] = i SubPicHeight[ subpic_grid_idx[ i − 1][ j ] ] = i − SubPicTop[ subpic_grid_idx[ i − 1 ][ j ] ] } if ( j = = 0) SubPicLeft[ subpic_grid_idx[ i ][ j ] ] = 0 (7-7) else if (subpic_grid_idx[ i ][ j ] != subpic_grid_idx[ i ][ j − 1 ] ) { SubPicLeft[ subpic_grid_idx[ i ][ j ] ] = j SubPicWidth[ subpic_grid_idx[ i ][ j ] ] = j − SubPicLeft[ subpic_grid_idx[ i ][ j − 1 ] ] } if ( i = = NumSubPicGridRows − 1) SubPicHeight[ subpic_grid_idx[ i ][ j ] ] = i − SubPicTop[ subpic_grid_idx[ i − 1 ][ j ] ] + 1 if (j = = NumSubPicGridRows − 1) SubPicWidth[ subpic_grid_idx[ i ][ j ] ] = j − SubPicLeft[ subpic_grid_idx[ i ][ j − 1 ] ] + 1 if( subpic_grid_idx[ i ][ j ] > NumSubPics) NumSubPics = subpic_grid_idx[ i ][ j ] } } Section 7.3.2.4 in VVC Draft 6 version 14: pic_parameter_set_rbsp( ) {; Descriptor. …. num_subpicture_in_pic_minus1; ue(v). for( i = 0; i <= num_subpicture_in_pic_minus1; i++ ) {. subpicture_id[ i ]; ue(v). }. …. Section 7.4.3.4 in VVC Draft 6 version 14: num_subpicture_in_pic_minus1 plus 1 specifies the number of subpictures in each picture referring to the PPS. The value of num_subpicture_in_pic_minus1 shall be in the range of 0 to NumSubPics, inclusive. subpicture_id[ i ] specifies the subpicture ID of the i-th subpicture in the picture referring to the PPS. The value of subpicture_id[ i ] shall be in the range of 0 to NumSubPics, inclusive. Section 7.3.6.1 in VVC Draft 6 version 14: slice_header( ) {; Descriptor. …. if( subpics_present_flag && NumSubPics > 0 ). slice_subpicture_id; u(v). …. Section 7.4.7.1 in VVC Draft 6 version 14: slice_subpicture_id specifies the value of subpicture_id for the slice in use. The value of slice_subpicture_id shall be in the range of 0 to NumSubPics, inclusive.

# Reference

- B. Bross, J. Chen and S. Liu, “Versatile Video Coding (Draft 6)”, JVET-O2001, the 15th JVET meeting, Jul. 2019, Gothenburg, Sweden.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).