| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0433 |
| --- | --- |

| Title: | AHG12: On brick signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yao-Jen Chang Muhammed Coban Marta Karczewicz | Tel: Email: | +1 858 845 1779 yjchang@qti.qualcomm.com mcoban@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

The proposal is summarized as follows:

- Alignment of num_tiles_in_pic_minus1 and RowHeight[ i ] on parsing dependencies issue by removing the signaling of num_tiles_in_pic_minus1 from PPS, and additionally signaling the information of CtbSize in PPS.

- Text typo cleanup by replacing RowHeight[ i ] with TileHeight[ i ] in the syntax

# Introduction

A new brick signaling process in PPS syntax was adopted in Gothenburg meeting by incorporating the following two modifications:

- Add a new syntax element num_tiles_in_pic_minus1 for the purpose of removing the parsing dependency between brick signaling and SPS parameters, i.e., the picture sizes. (JVET-O0236)

- Infer brick_split_flag[ i ] to be 0 if RowHeight is equal to 1, and infer uniform_brick_spacing_flag[ i ] to be 1 if RowHeight is equal to 2 for the purpose of bugfixes. (JVET-O0452)

The syntax design for brick signaling is extracted from VVC 6 [1] in the table below:

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| … |  |
| single_tile_in_pic_flag | u(1) |
| if( !single_tile_in_pic_flag ) { |  |
| uniform_tile_spacing_flag | u(1) |
| if( uniform_tile_spacing_flag ) { |  |
| tile_cols_width_minus1 | ue(v) |
| tile_rows_height_minus1 | ue(v) |
| } else { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| tile_column_width_minus1[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| tile_row_height_minus1[ i ] | ue(v) |
| } |  |
| brick_splitting_present_flag | u(1) |
| if( uniform_tile_spacing_flag && brick_splitting_present_flag ) |  |
| num_tiles_in_pic_minus1 | ue(v) |
| for( i = 0; brick_splitting_present_flag && i <= num_tiles_in_pic_minus1 + 1; i++ ) { |  |
| if( RowHeight[ i ] > 1 ) |  |
| brick_split_flag[ i ] | u(1) |
| if( brick_split_flag[ i ] ) { |  |
| if( RowHeight[ i ] > 2 ) |  |
| uniform_brick_spacing_flag[ i ] | u(1) |
| if( uniform_brick_spacing_flag[ i ] ) |  |
| brick_height_minus1[ i ] | ue(v) |
| else { |  |
| num_brick_rows_minus2[ i ] | ue(v) |
| for( j = 0; j <= num_brick_rows_minus2[ i ]; j++ ) |  |
| brick_row_height_minus1[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |

However, there are issues remained in the syntax:

- NumTilesInPic and RowHeight[ i ] are dependent on SPS signaling, i.e., log2_ctu_size_minus5. While the issue is resolved for NumTilesInPic by signaling the syntax element num_tiles_in_pic_minus1 in PPS as mentioned above, the issue remains unsolved for RowHeight[ i ].

# Alignment of num_tiles_in_pic_minus1 and RowHeight[ i ] on parsing dependencies issue

Remove the signaling of num_tiles_in_pic_minus1 from PPS, and additionally signal the information of CtbSize in PPS.

- pps_log2_ctu_size_minus5 as well as pic_width_in_luma_samples and pic_height_in_luma_samples are required in PPS to derive the values of PicWidthInCtbsY and PicHeightInCtbsY, which are used to infer the values of NumTilesInPic and RowHeight[ i ].

- The signaling of pps_log2_ctu_size_minus5 requires 2 additional bits for, but removing the syntax element of num_tiles_in_pic_minus1 reduces as many as 33 bits in the worst case of 8K content, e.g. 8192 x 4320 and CtbSize = 32. It is even worst if NumTilesInPic and RowHeight[ i ] are explicitly signaled.

The syntax changes to VVC Draft 6 are as described in the following table with yellow change mark:

Section 7.3.2.4 in VVC Draft 6 version 14: pic_parameter_set_rbsp( ) {; Descriptor. …. pic_width_in_luma_samples; ue(v). pic_height_in_luma_samples; ue(v). pps_log2_ctu_size_minus5; u(2). …. single_tile_in_pic_flag; u(1). if( !single_tile_in_pic_flag ) {. uniform_tile_spacing_flag; u(1). if( uniform_tile_spacing_flag ) {. tile_cols_width_minus1; ue(v). tile_rows_height_minus1; ue(v). } else {. num_tile_columns_minus1; ue(v). num_tile_rows_minus1; ue(v). for( i = 0; i < num_tile_columns_minus1; i++ ). tile_column_width_minus1[ i ]; ue(v). for( i = 0; i < num_tile_rows_minus1; i++ ). tile_row_height_minus1[ i ]; ue(v). }. brick_splitting_present_flag; u(1). if( uniform_tile_spacing_flag && brick_splitting_present_flag ). num_tiles_in_pic_minus1; ue(v). for( i = 0; brick_splitting_present_flag && i <= num_tiles_in_pic_minus1 + 1 NumTilesInPic ; i++ ) {. if( RowHeight[ i ] > 1 ). brick_split_flag[ i ]; u(1). if( brick_split_flag[ i ] ) {. if( RowHeight[ i ] > 2 ). uniform_brick_spacing_flag[ i ]; u(1). if( uniform_brick_spacing_flag[ i ] ). brick_height_minus1[ i ]; ue(v). else {. num_brick_rows_minus2[ i ]; ue(v). for( j = 0; j <= num_brick_rows_minus2[ i ]; j++ ). brick_row_height_minus1[ i ][ j ]; ue(v). }. }. }. Section 7.4.3.4 in VVC Draft 6 version 14: pic_width_in_luma_samples specifies the width of each decoded picture referring to the PPS in units of luma samples. pic_width_in_luma_samples shall not be equal to 0, shall be an integer multiple of Max( 8, MinCbSizeY ), and shall be less than or equal to pic_width_max_in_luma_samples. When subpics_present_flag is equal to 1, the value of pic_width_in_luma_samples shall be equal to pic_width_max_in_luma_samples. pic_height_in_luma_samples specifies the height of each decoded picture referring to the PPS in units of luma samples. pic_height_in_luma_samples shall not be equal to 0 and shall be an integer multiple of Max( 8, MinCbSizeY ), and shall be less than or equal to pic_height_max_in_luma_samples. When subpics_present_flag is equal to 1, the value of pic_height_in_luma_samples shall be equal to pic_height_max_in_luma_samples. pps_log2_ctu_size_minus5 plus 5 specifies the luma coding tree block size of each CTU in each picture referring to the PPS. The value of pps_log2_ctu_size_minus5 shall be equal to log2_ctu_size_minus5. CtbSize = 1 << (pps_log2_ctu_size_minus5 + 5) single_tile_in_pic_flag equal to 1 specifies that there is only one tile in each picture referring to the PPS. single_tile_in_pic_flag equal to 0 specifies that there is more than one tile in each picture referring to the PPS. NOTE – In absence of further brick splitting within a tile, the whole tile is referred to as a brick. When a picture contains only a single tile without further brick splitting, it is referred to as a single brick. It is a requirement of bitstream conformance that the value of single_tile_in_pic_flag shall be the same for all PPSs that are referred to by coded pictures within a CVS. uniform_tile_spacing_flag equal to 1 specifies that tile column boundaries and likewise tile row boundaries are distributed uniformly across the picture and signalled using the syntax elements tile_cols_width_minus1 and tile_rows_height_minus1. uniform_tile_spacing_flag equal to 0 specifies that tile column boundaries and likewise tile row boundaries may or may not be distributed uniformly across the picture and signalled using the syntax elements num_tile_columns_minus1 and num_tile_rows_minus1 and a list of syntax element pairs tile_column_width_minus1[ i ] and tile_row_height_minus1[ i ]. When not present, the value of uniform_tile_spacing_flag is inferred to be equal to 1. tile_cols_width_minus1 plus 1 specifies the width of the tile columns excluding the right-most tile column of the picture in units of CTBs when uniform_tile_spacing_flag is equal to 1. The value of tile_cols_width_minus1 shall be in the range of 0 to PicWidthInCtbsY − 1, inclusive. When not present, the value of tile_cols_width_minus1 is inferred to be equal to PicWidthInCtbsY − 1. tile_rows_height_minus1 plus 1 specifies the height of the tile rows excluding the bottom tile row of the picture in units of CTBs when uniform_tile_spacing_flag is equal to 1. The value of tile_rows_height_minus1 shall be in the range of 0 to PicHeightInCtbsY − 1, inclusive. When not present, the value of tile_rows_height_minus1 is inferred to be equal to PicHeightInCtbsY − 1. num_tile_columns_minus1 plus 1 specifies the number of tile columns partitioning the picture when uniform_tile_spacing_flag is equal to 0. The value of num_tile_columns_minus1 shall be in the range of 0 to PicWidthInCtbsY − 1, inclusive. If single_tile_in_pic_flag is equal to 1, the value of num_tile_columns_minus1 is inferred to be equal to 0. Otherwise, when uniform_tile_spacing_flag is equal to 1, the value of num_tile_columns_minus1 is inferred as specified in clause 6.5.1. num_tile_rows_minus1 plus 1 specifies the number of tile rows partitioning the picture when uniform_tile_spacing_flag is equal to 0. The value of num_tile_rows_minus1 shall be in the range of 0 to PicHeightInCtbsY − 1, inclusive. If single_tile_in_pic_flag is equal to 1, the value of num_tile_rows_minus1 is inferred to be equal to 0. Otherwise, when uniform_tile_spacing_flag is equal to 1, the value of num_tile_rows_minus1 is inferred as specified in clause 6.5.1. The variable NumTilesInPic is set equal to ( num_tile_columns_minus1 + 1 ) * ( num_tile_rows_minus1 + 1 ). When single_tile_in_pic_flag is equal to 0, NumTilesInPic shall be greater than 1. tile_column_width_minus1[ i ] plus 1 specifies the width of the i-th tile column in units of CTBs. tile_row_height_minus1[ i ] plus 1 specifies the height of the i-th tile row in units of CTBs. brick_splitting_present_flag equal to 1 specifies that one or more tiles of pictures referring to the PPS may be divided into two or more bricks. brick_splitting_present_flag equal to 0 specifies that no tiles of pictures referring to the PPS are divided into two or more bricks. num_tiles_in_pic_minus1 plus 1 specifies the number of tiles in each picture referring to the PPS. The value of num_tiles_in_pic_minus1 shall be equal to NumTilesInPic – 1. When not present, the value of num_tiles_in_pic_minus1 is inferred to be equal to NumTilesInPic − 1. brick_split_flag[ i ] equal to 1 specifies that the i-th tile is divided into two or more bricks. brick_split_flag[ i ] equal to 0 specifies that the i-th tile is not divided into two or more bricks. When not present, the value of brick_split_flag[ i ] is inferred to be equal to 0. uniform_brick_spacing_flag[ i ] equal to 1 specifies that horizontal brick boundaries are distributed uniformly across the i-th tile and signalled using the syntax element brick_height_minus1[ i ]. uniform_brick_spacing_flag[ i ] equal to 0 specifies that horizontal brick boundaries may or may not be distributed uniformly across i-th tile and signalled using the syntax element num_brick_rows_minus2[ i ] and a list of syntax elements brick_row_height_minus1[ i ][ j ]. When not present, the value of uniform_brick_spacing_flag[ i ] is inferred to be equal to 1. brick_height_minus1[ i ] plus 1 specifies the height of the brick rows excluding the bottom brick in the i-th tile in units of CTBs when uniform_brick_spacing_flag[ i ] is equal to 1. When present, the value of brick_height_minus1 shall be in the range of 0 to RowHeight[ i ] − 2, inclusive. When not present, the value of brick_height_minus1[ i ] is inferred to be equal to RowHeight[ i ] − 1. num_brick_rows_minus2[ i ] plus 2 specifies the number of bricks partitioning the i-th tile when uniform_brick_spacing_flag[ i ] is equal to 0. When present, the value of num_brick_rows_minus2[ i ] shall be in the range of 0 to RowHeight[ i ] − 2, inclusive. If brick_split_flag[ i ] is equal to 0, the value of num_brick_rows_minus2[ i ] is inferred to be equal to −1. Otherwise, when uniform_brick_spacing_flag[ i ] is equal to 1, the value of num_brick_rows_minus2[ i ] is inferred as specified in 6.5.1. brick_row_height_minus1[ i ][ j ] plus 1 specifies the height of the j-th brick in the i-th tile in units of CTBs when uniform_tile_spacing_flag is equal to 0.

# Text cleanup by replacing RowHeight[ i ] with TileHeight[ i ]

The cleanup wants to fix a typo when brick_split_flag[ i ] and uniform_brick_spacing_flag[ i ] are conditioned on RowHeight.

- The index of RowHeight should be RowHeight[ i % (num_tile_columns_minus1 + 1)] instead of RowHeight[ i ] because the value of indexes of RowHeight is in the range of 0 to num_tile_rows_minus1, inclusive.

Proposed: Replace RowHeight[ i ] with a new parameter TileHeight[ i ] in the syntax for typo fix and cleanup purposes

- TileHeight[ i ] is derived as RowHeight[ i % (num_tile_columns_minus1 + 1)] in subclause 6.5.1.

The syntax changes to VVC Draft 6 are as described in the following table with yellow change mark:

Section 6.5.1 in VVC Draft 6 version 14: The list colWidth[ i ] for i ranging from 0 to num_tile_columns_minus1, inclusive, specifying the width of the i-th tile column in units of CTBs, is derived, and when uniform_tile_spacing_flag is equal to 1, the value of num_tile_columns_minus1 is inferred, as follows: if( uniform_tile_spacing_flag ) { remainingWidthInCtbsY = PicWidthInCtbsY i = 0 while( remainingWidthInCtbsY > ( tile_cols_width_minus1 + 1 ) ) { colWidth[ i++ ] = tile_cols_width_minus1 + 1 remainingWidthInCtbsY −= ( tile_cols_width_minus1 + 1 ) } colWidth[ i ] = remainingWidthInCtbsY num_tile_columns_minus1 = i } else { colWidth[ num_tile_columns_minus1 ] = PicWidthInCtbsY (6-1) for( i = 0; i < num_tile_columns_minus1; i++ ) { colWidth[ i ] = tile_column_width_minus1[ i ] + 1 colWidth[ num_tile_columns_minus1 ] −= colWidth[ i ] } } The list RowHeight[ j ] for j ranging from 0 to num_tile_rows_minus1, inclusive, specifying the height of the j-th tile row in units of CTBs, is derived, and when uniform_tile_spacing_flag is equal to 1, the value of num_tile_rows_minus1 is inferred, as follows: if( uniform_tile_spacing_flag ) { remainingHeightInCtbsY = PicHeightInCtbsY i = 0 while( remainingHeightInCtbsY > ( tile_rows_height_minus1 + 1 ) ) { RowHeight[ i++ ] = tile_rows_height_minus1 + 1 remainingHeightInCtbsY −= ( tile_rows_height_minus1 + 1 ) } RowHeight[ i ] = remainingHeightInCtbsY num_tile_rows_minus1 = i } else { RowHeight[ num_tile_rows_minus1 ] = PicHeightInCtbsY (6-2) for( j = 0; j < num_tile_rows_minus1; j++ ) { RowHeight[ j ] = tile_row_height_minus1[ j ] + 1 RowHeight[ num_tile_rows_minus1 ] −= RowHeight[ j ] } } The list TileHeight [ k ] for k ranging from 0 to NumTilesInPic – 1, inclusive, specifying the height of the k-th tile in units of CTBs, is derived as follows: for( k = 0; k < NumTilesInPic; k++ ) { TileHeight[ k ] = RowHeight[ k % (num_tile_columns_minus1 + 1) ] } Section 7.3.2.4 in VVC Draft 6 version 14: pic_parameter_set_rbsp( ) {; Descriptor. …. single_tile_in_pic_flag; u(1). if( !single_tile_in_pic_flag ) {. uniform_tile_spacing_flag; u(1). if( uniform_tile_spacing_flag ) {. tile_cols_width_minus1; ue(v). tile_rows_height_minus1; ue(v). } else {. num_tile_columns_minus1; ue(v). num_tile_rows_minus1; ue(v). for( i = 0; i < num_tile_columns_minus1; i++ ). tile_column_width_minus1[ i ]; ue(v). for( i = 0; i < num_tile_rows_minus1; i++ ). tile_row_height_minus1[ i ]; ue(v). }. brick_splitting_present_flag; u(1). if( uniform_tile_spacing_flag && brick_splitting_present_flag ). num_tiles_in_pic_minus1; ue(v). for( i = 0; brick_splitting_present_flag && i <= num_tiles_in_pic_minus + 1; i++ ) {. if( RowHeight[ i ] > 1 ) if( TileHeight[ i ] > 1 ). brick_split_flag[ i ]; u(1). if( brick_split_flag[ i ] ) {. if( RowHeight[ i ] > 2 ) if( TileHeight[ i ] > 2 ). uniform_brick_spacing_flag[ i ]; u(1). if( uniform_brick_spacing_flag[ i ] ). brick_height_minus1[ i ]; ue(v). else {. num_brick_rows_minus2[ i ]; ue(v). for( j = 0; j <= num_brick_rows_minus2[ i ]; j++ ). brick_row_height_minus1[ i ][ j ]; ue(v). }. }. }. Section 7.4.3.4 in VVC Draft 6 version 14: single_tile_in_pic_flag equal to 1 specifies that there is only one tile in each picture referring to the PPS. single_tile_in_pic_flag equal to 0 specifies that there is more than one tile in each picture referring to the PPS. NOTE – In absence of further brick splitting within a tile, the whole tile is referred to as a brick. When a picture contains only a single tile without further brick splitting, it is referred to as a single brick. It is a requirement of bitstream conformance that the value of single_tile_in_pic_flag shall be the same for all PPSs that are referred to by coded pictures within a CVS. uniform_tile_spacing_flag equal to 1 specifies that tile column boundaries and likewise tile row boundaries are distributed uniformly across the picture and signalled using the syntax elements tile_cols_width_minus1 and tile_rows_height_minus1. uniform_tile_spacing_flag equal to 0 specifies that tile column boundaries and likewise tile row boundaries may or may not be distributed uniformly across the picture and signalled using the syntax elements num_tile_columns_minus1 and num_tile_rows_minus1 and a list of syntax element pairs tile_column_width_minus1[ i ] and tile_row_height_minus1[ i ]. When not present, the value of uniform_tile_spacing_flag is inferred to be equal to 1. tile_cols_width_minus1 plus 1 specifies the width of the tile columns excluding the right-most tile column of the picture in units of CTBs when uniform_tile_spacing_flag is equal to 1. The value of tile_cols_width_minus1 shall be in the range of 0 to PicWidthInCtbsY − 1, inclusive. When not present, the value of tile_cols_width_minus1 is inferred to be equal to PicWidthInCtbsY − 1. tile_rows_height_minus1 plus 1 specifies the height of the tile rows excluding the bottom tile row of the picture in units of CTBs when uniform_tile_spacing_flag is equal to 1. The value of tile_rows_height_minus1 shall be in the range of 0 to PicHeightInCtbsY − 1, inclusive. When not present, the value of tile_rows_height_minus1 is inferred to be equal to PicHeightInCtbsY − 1. num_tile_columns_minus1 plus 1 specifies the number of tile columns partitioning the picture when uniform_tile_spacing_flag is equal to 0. The value of num_tile_columns_minus1 shall be in the range of 0 to PicWidthInCtbsY − 1, inclusive. If single_tile_in_pic_flag is equal to 1, the value of num_tile_columns_minus1 is inferred to be equal to 0. Otherwise, when uniform_tile_spacing_flag is equal to 1, the value of num_tile_columns_minus1 is inferred as specified in clause 6.5.1. num_tile_rows_minus1 plus 1 specifies the number of tile rows partitioning the picture when uniform_tile_spacing_flag is equal to 0. The value of num_tile_rows_minus1 shall be in the range of 0 to PicHeightInCtbsY − 1, inclusive. If single_tile_in_pic_flag is equal to 1, the value of num_tile_rows_minus1 is inferred to be equal to 0. Otherwise, when uniform_tile_spacing_flag is equal to 1, the value of num_tile_rows_minus1 is inferred as specified in clause 6.5.1. The variable NumTilesInPic is set equal to ( num_tile_columns_minus1 + 1 ) * ( num_tile_rows_minus1 + 1 ). When single_tile_in_pic_flag is equal to 0, NumTilesInPic shall be greater than 1. tile_column_width_minus1[ i ] plus 1 specifies the width of the i-th tile column in units of CTBs. tile_row_height_minus1[ i ] plus 1 specifies the height of the i-th tile row in units of CTBs. brick_splitting_present_flag equal to 1 specifies that one or more tiles of pictures referring to the PPS may be divided into two or more bricks. brick_splitting_present_flag equal to 0 specifies that no tiles of pictures referring to the PPS are divided into two or more bricks. num_tiles_in_pic_minus1 plus 1 specifies the number of tiles in each picture referring to the PPS. The value of num_tiles_in_pic_minus1 shall be equal to NumTilesInPic – 1. When not present, the value of num_tiles_in_pic_minus1 is inferred to be equal to NumTilesInPic − 1. brick_split_flag[ i ] equal to 1 specifies that the i-th tile is divided into two or more bricks. brick_split_flag[ i ] equal to 0 specifies that the i-th tile is not divided into two or more bricks. When not present, the value of brick_split_flag[ i ] is inferred to be equal to 0. uniform_brick_spacing_flag[ i ] equal to 1 specifies that horizontal brick boundaries are distributed uniformly across the i-th tile and signalled using the syntax element brick_height_minus1[ i ]. uniform_brick_spacing_flag[ i ] equal to 0 specifies that horizontal brick boundaries may or may not be distributed uniformly across i-th tile and signalled using the syntax element num_brick_rows_minus2[ i ] and a list of syntax elements brick_row_height_minus1[ i ][ j ]. When not present, the value of uniform_brick_spacing_flag[ i ] is inferred to be equal to 1. brick_height_minus1[ i ] plus 1 specifies the height of the brick rows excluding the bottom brick in the i-th tile in units of CTBs when uniform_brick_spacing_flag[ i ] is equal to 1. When present, the value of brick_height_minus1 shall be in the range of 0 to RowHeight[ i ] − 2, inclusive. When not present, the value of brick_height_minus1[ i ] is inferred to be equal to RowHeight[ i ] − 1. num_brick_rows_minus2[ i ] plus 2 specifies the number of bricks partitioning the i-th tile when uniform_brick_spacing_flag[ i ] is equal to 0. When present, the value of num_brick_rows_minus2[ i ] shall be in the range of 0 to RowHeight[ i ] − 2, inclusive. If brick_split_flag[ i ] is equal to 0, the value of num_brick_rows_minus2[ i ] is inferred to be equal to −1. Otherwise, when uniform_brick_spacing_flag[ i ] is equal to 1, the value of num_brick_rows_minus2[ i ] is inferred as specified in 6.5.1. brick_row_height_minus1[ i ][ j ] plus 1 specifies the height of the j-th brick in the i-th tile in units of CTBs when uniform_tile_spacing_flag is equal to 0.

# Reference

- B. Bross, J. Chen and S. Liu, “Versatile Video Coding (Draft 6)”, JVET-O2001, the 15th JVET meeting, Jul. 2019, Gothenburg, Sweden.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).