| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0120-v1 |
| --- | --- |

| Title: | AHG17: On Constraints for ALF APS |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jianle Chen Hendry San Diego, CA, USA | Tel: Email: | - jianle.chen@futurewei.com fhendry@futurewei.com |
| Source: | Futurewei Technologies Inc. |  |  |

_____________________________

# Abstract

In this contribution it is asserted that the current specification on ALF APS and referencing from slice header lacks constraints to ensure the referencing of ALF parameters from slice header to ALF APS works. To overcome the lacking, the following constraints are proposed:

- It is requirement of bitstream conformance that the values of alf_luma_filter_signal_flag and alf_chroma_filter_signal_flag shall not both be equal to 0.

- The value of alf_luma_filter_signal_flag of the APS NAL unit having aps_params_type equal to ALF_APS and adaptation_parameter_set_id equal to slice_alf_aps_id_luma[ i ] shall be equal to 1.

- The value of alf_chroma_filter_signal_flag of the APS NAL unit having aps_params_type equal to ALF_APS and adaptation_parameter_set_id equal to slice_alf_aps_id_chroma shall be equal to 1.

# Problem description

An ALF APS contains ALF parameters that are expressed in alf_data( ). Within alf_data( ), there are two flags alf_luma_filter_signal_flag and alf_chroma_filter_signal_flag which specify whether ALF parameters for luma component and ALF parameters for chroma components, respectively, are present.

The following problems are asserted exist in the current specification related to those two flags:

- The values of these two flags are currently not constrained to not be equal to 0 at the same time to avoid having ALF APS with empty alf_data( ).

- When ALF is enabled for luma component of a slice, ALF parameters needed for invoking ALF process for luma component of the slice shall be available in the ALF APS with APS Id specified by slice_alf_aps_id_luma[ i ]. However currently there is no constraint to ensure that the ALF APS referred to by slice contains ALF parameters for luma component.

- When ALF is enabled for chroma components of a slice, ALF parameters needed for invoking ALF process for chroma components of the slice shall be available in the ALF APS with APS Id specified by slice_alf_aps_id_chroma. However currently there is no constraint to ensure that the ALF APS referred to by slice contains ALF parameters for chroma components.

# Proposal

To solve the problems described above, we propose three constraints as shown below.

In Adaptive loop filter data semantics, add the following constraint after the semantics of alf_chroma_filter_signal_flag:

It is a requirement of bitstream conformance that the values of alf_luma_filter_signal_flag and alf_chroma_filter_signal_flag shall not both be equal to 0.

In General slice header semantics, add the following constraint after the semantics of slice_alf_aps_id_luma[ i ]:

The value of alf_luma_filter_signal_flag of the APS NAL unit having aps_params_type equal to ALF_APS and adaptation_parameter_set_id equal to slice_alf_aps_id_luma[ i ] shall be equal to 1.

In General slice header semantics, add the following constraint after the semantics of slice_alf_aps_id_chroma:

The value of alf_chroma_filter_signal_flag of the APS NAL unit having aps_params_type equal to ALF_APS and adaptation_parameter_set_id equal to slice_alf_aps_id_chroma shall be equal to 1.

# Patent rights declaration(s)

Futurewei Technologies, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).