| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0448-v2 |
| --- | --- |

| Title: | AHG17: Sub-picture level info for extraction and merging |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Robert Skupin Yago Sanchez Karsten Suehring | Email: | {firstname.lastname}@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

The document proposes syntax for indicating the level of sub-pictures in a more fine-granular way than just a level_idc by adding reference level and percentage syntax elements into PTL or VUI syntax.

It is argued that for merging use cases with varying target level (e.g. streaming 4K and 8K sub-pic mosaics), traditional level_idc signalling does not allow accurately judging a sub-picture level contribution to the composite bitstream.

It is further argued that implicit derivation of such a level percentage, e.g. by sub-picture to picture size proportion, is not adequate as it may change in merging use cases and an encoder/decoder mismatches would occur.

It is further argued that the presence of such conformance information is vital for merging use-cases, so that placement in parameter set contained syntax such as PTL or even VUI is preferable over SEI.

# Introduction

At the 15th meeting of JVET in Gothenburg, level signalling for the purpose of merging and extracting sub-pictures was proposed. While document JVET-O0555 addressed the extraction use case with a sub-picture level derivation depending on the ratio of sub-picture and picture size, document JVET-0492 addressed the merging use case by explicit signalling a reference level and percentage in the PTL syntax of the complete picture.

The following comments where received during BoG review as noted in JVET-O1076 with added author responses:

Comment 1: Isn't the actual level of the current bitstream as well as other parameters like spatial resolution can be used for the same purpose as the additional information herein?

Response:  Such a solution would be similar to JVET-O0555 and is not a complete solution as it does not cover merging use cases as explained in the following.

The JVET-O555 proposal is reproduced here for convenience:

SubPicSetLevelIdc = level_idc

for (k = level_idc; k > 0; k --)

if (  SubPicSetSizeY <= MaxLumaPs[ k ]  &&

MaxLumaSr[ level_idc ] * SubPicSetSizeY / PicSizeInSamplesY <= MaxLumaSr[ k ] )

SubPicSetLevelIdc = k

The derivation is based on a reference level ( level_idc of current bitstream) and a percentage (SubPicSetSizeY / PicSizeInSamplesY).

While an encoder can ensure to obey the above derived SubPicSetLevelIdc for a sub-picture set that it currently encodes, the derived SubPicSetLevelIdc for a sub-picture may change, when merged in another bitstream, since PicSizeInSamplesY and level_idc might change.

Both, PicSizeInSamplesY and the MaxLumaSr[ level_idc ] may vary independently from each other, especially due to the other level limit aspects apart from picture size. Hence, depending on the merged bitstream, another level k unknown by the original encoder could fulfill the above condition resulting in encoder/decoder mismatches.

This is an issue in many scenarios in which sub-pictures are encoded independently in parallel and later combined in varying ways, e.g. varying the amount of high and low-resolution tiles in 360-degree video streaming.

Therefore, the current proposal re-iterates explicit signalling of a reference level and percentage to ensure correctness of the information independent of decoding context (composite bitstream as a merging result).

Comment 2: It was commented that if this is to be included into the specification, it should be SEI message instead.

Response: It is argued that the presence of such conformance information is vital for merging use-cases, so that placement in parameter set contained syntax such as PTL or even VUI is preferable over SEI.

Comment 3: Is it possible to have multiple target level?

Response: It is possible to have multiple target levels and percentages but it is asserted that in most of the envisioned use-cases a single one is typically useful. It is also asserted that it would be possible to extrapolate the signalled single level percentage of a given target reference level to another reference level in a relatively good approximation. However, if it is desired to accurately signal the properties of the stream for different levels the syntax could be extended. In the following section two solutions are proposed one for a level percentage for a single target level and another for multiple ones.

Furthermore, the proposal is adjusted to address the extraction use case as well similarly to JVET-O0555, by indicating the reference level and percentage per sub-picture.

# Proposed solution

## Proposal 1: Single ref_level_idc

| vui_parameters() / profile_tier_level( maxNumSubLayersMinus1 ) { | Descriptor |
| --- | --- |
| […] |  |
| sub_pic_level_present_flag | u(1) |
| if( sub_pic_level_present_flag ) { |  |
| num_sub_pictures_minus1 | u(7) |
| ref_level_idc | u(8) |
| for( i = 0; i <= num_sub_pictures_minus1; i++ ){ |  |
| ref_level_percentage[ i ] | u(8) |
| } |  |
| } |  |
| […] |  |
| } |  |

sub_pic_level_present_flag equal to 1 specifies that the syntax elements num_sub_pictures_minus1, ref_level_idc and ref_level_percentage are present. sub_pic_level_present_flag equal to 0 specifies that the syntax elements num_sub_pictures_minus1, ref_level_idc and ref_level_percentage are not present.

num_sub_pictures_minus1 specifies the number of sub-pictures in the current bitstream. The value of num_sub_picutre_minus1 shall be in the range of 0 to 127 [Ed. To be adjusted].

ref_level_idc indicates a level to which each sub-picture conforms as specified in Annex A. Bitstreams shall not contain values of ref_level_idc other than those specified in Annex A. Other values of ref_level_idc are reserved for future use by ITU-T | ISO/IEC.

ref_level_percentage[ i ] specifies the percentage of the level limits associated with ref_level_idc that the i-th sub-picture conforms to as specified in Annex A.4.1. The value of ref_level_percentage[ i ] shall be in the range of 0 to 255, inclusive.

Relevance for HRD level conformance

The following is based on text of HEVC level definitions from Annex A.4.1 with changes highlighted in yellow:

When sub_pic_level_present_flag is equal to 1, an additional conformance point test is enabled for each of the 1 to num_sub_picture_minus1 plus 1 sub-pictures by setting level equal to ref_level_idc and LevelPercentage equal to ref_level_percentage[ i ] ÷ 256. Otherwise, for all other conformance tests, LevelPercentage is set equal to 1.

The bitstreams conforming to a profile at a specified tier and level shall obey the following constraints for each bitstream conformance test as specified in Annex C:

- PicSizeInSamplesY shall be less than or equal to Floor( MaxLumaPs * LevelPercentage ), where MaxLumaPs is specified in Table A.6.

- The value of pic_width_in_luma_samples shall be less than or equal to Sqrt( MaxLumaPs * LevelPercentage * 8 ).

- The value of pic_height_in_luma_samples shall be less than or equal to Sqrt( MaxLumaPs * LevelPercentage * 8 ).

- For level 5 and higher levels, the value of CtbSizeY shall be equal to 32 or 64.

- The value of NumPicTotalCurr shall be less than or equal to 8.

- The value of num_tile_columns_minus1 shall be less than Floor( MaxTileCols * LevelPercentage ) and num_tile_rows_minus1 shall be less than Floor( MaxTileRows * LevelPercentage ), where MaxTileCols and MaxTileRows are specified in Table A.6.

- The value of NumBricksInPic shall be less than or equal to MaxBricksPerPicture, where MaxBricksPerPicture is specified in Table A.6.

[Ed. (RS): It is expected that MaxBricksPerPicture will be limited by level definitions as well in the future.]

- For the VCL HRD parameters, CpbSize[ i ] shall be less than or equal to Floor( CpbVclFactor * MaxCPB* LevelPercentage ) for at least one value of i in the range of 0 to cpb_cnt_minus1[ HighestTid ], inclusive, where CpbSize[ i ] is specified in clause E.3.3 based on parameters selected as specified in clause C.1, CpbVclFactor is specified in Table A.8, and MaxCPB is specified in Table A.6 in units of CpbVclFactor bits.

- For the NAL HRD parameters, CpbSize[ i ] shall be less than or equal to Floor( CpbNalFactor * MaxCPB* LevelPercentage ) for at least one value of i in the range of 0 to cpb_cnt_minus1[ HighestTid ], inclusive, where CpbSize[ i ] is specified in clause E.3.3 based on parameters selected as specified in clause C.1, CpbNalFactor is specified in Table A.8, and MaxCPB is specified in Table A.6 in units of CpbNalFactor bits.

## Proposal 2: Multiple ref_level_idc

| vui_parameters() / profile_tier_level( maxNumSubLayersMinus1 ) { | Descriptor |
| --- | --- |
| […] |  |
| sub_pic_level_present_flag | u(1) |
| if( sub_pic_level_present_flag ) { |  |
| num_sub_pictures_minus1 | u(7) |
| num_target_level_minus1 | u(8) |
| for( i = 0; i <= num_target_level_minus1; i++ ){ |  |
| ref_level_idc[ i ] | u(8) |
| for( j = 0; j <= num_sub_pictures_minus1; j++ ){ |  |
| ref_level_percentage[ i ][ j ] | u(8) |
| } |  |
| } |  |
| } |  |
| […] |  |
| } |  |

sub_pic_level_present_flag equal to 1 specifies that the syntax elements num_sub_pictures_minus1, ref_level_idc and ref_level_percentage are present. sub_pic_level_present_flag equal to 0 specifies that the syntax elements num_sub_pictures_minus1, ref_level_idc and ref_level_percentage are not present.

num_sub_pictures_minus1 specifies the number of sub-pictures in the current bitstream. The value of num_sub_picutre_minus1 shall be in the range of 0 to 127 [Ed. To be adjusted].

ref_level_idc[ i ] indicates the i-th level to which each sub-picture conforms as specified in Annex A. Bitstreams shall not contain values of ref_level_idc[ i ] other than those specified in Annex A. Other values of ref_level_idc[ i ] are reserved for future use by ITU-T | ISO/IEC.

ref_level_percentage[ i ][ j ] specifies the percentage of the level limits associated with ref_level_idc[ i ] that each sub-picture conforms to as specified in Annex A.4.1. The value of ref_level_percentage[ i ][ j ] shall be in the range of 0 to 255, inclusive.

Relevance for HRD level conformance

The following is based on text of HEVC level definitions from Annex A.4.1 with changes highlighted in yellow:

When sub_pic_level_present_flag is equal to 1, an additional conformance point test is enabled for each of the 1 to num_sub_picture_minus1 plus 1 sub-pictures and each of the 1 to num_target_level_minus1 plus 1 by setting level equal to ref_level_idc[ i ] and LevelPercentage equal to ref_level_percentage[ i ][ j ] ÷ 256. Otherwise, for all other conformance tests, LevelPercentage is set equal to 1.

The bitstreams conforming to a profile at a specified tier and level shall obey the following constraints for each bitstream conformance test as specified in Annex C:

- PicSizeInSamplesY shall be less than or equal to Floor( MaxLumaPs * LevelPercentage ), where MaxLumaPs is specified in Table A.6.

- The value of pic_width_in_luma_samples shall be less than or equal to Sqrt( MaxLumaPs * LevelPercentage * 8 ).

- The value of pic_height_in_luma_samples shall be less than or equal to Sqrt( MaxLumaPs * LevelPercentage * 8 ).

- For level 5 and higher levels, the value of CtbSizeY shall be equal to 32 or 64.

- The value of NumPicTotalCurr shall be less than or equal to 8.

- The value of num_tile_columns_minus1 shall be less than Floor( MaxTileCols * LevelPercentage ) and num_tile_rows_minus1 shall be less than Floor( MaxTileRows * LevelPercentage ), where MaxTileCols and MaxTileRows are specified in Table A.6.

- The value of NumBricksInPic shall be less than or equal to MaxBricksPerPicture, where MaxBricksPerPicture is specified in Table A.6.

[Ed. (RS): It is expected that MaxBricksPerPicture will be limited by level definitions as well in the future.]

- For the VCL HRD parameters, CpbSize[ i ] shall be less than or equal to Floor( CpbVclFactor * MaxCPB* LevelPercentage ) for at least one value of i in the range of 0 to cpb_cnt_minus1[ HighestTid ], inclusive, where CpbSize[ i ] is specified in clause E.3.3 based on parameters selected as specified in clause C.1, CpbVclFactor is specified in Table A.8, and MaxCPB is specified in Table A.6 in units of CpbVclFactor bits.

- For the NAL HRD parameters, CpbSize[ i ] shall be less than or equal to Floor( CpbNalFactor * MaxCPB* LevelPercentage ) for at least one value of i in the range of 0 to cpb_cnt_minus1[ HighestTid ], inclusive, where CpbSize[ i ] is specified in clause E.3.3 based on parameters selected as specified in clause C.1, CpbNalFactor is specified in Table A.8, and MaxCPB is specified in Table A.6 in units of CpbNalFactor bits.

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).