| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0449 |
| --- | --- |

| Title: | CE-4 related: Thin Partition Mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Krit Panusopone Seungwook Hong Limin Wang | Tel: Email: | +1 858-674-6883 krit.panusopone@nokia.com seungwook.hong@nokia.com limin.2.wang@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution proposes thin partition mode. When using this mode, a CU is split into 2 rectangular partitions; each partition has its own uni-direction temporal prediction MV. Thin partition mode can be split along 90-degree (vertical split), or 0-degree (horizontal split). Blending applies to pixels in the CU along horizontal/vertical split by weighted averaging MC predictors from both rectangular partitions. The results show that replacing triangular partition mode with thin partition mode when CU width or height equals 4 have luma BD-rate of approximately -0.01% and 0.00% for RA and LB configurations, respectively.

# Introduction

In the current VVC design [1], triangle partition mode is one of merge modes. When using triangle partition mode, a CU is split into 2 triangle partitions. Each triangle partition has its own uni-direction temporal prediction MV, derived from merge candidate index. Triangle partition mode can be split along 45-degree (from top right corner to bottom left corner), or 135-degree (from top left corner to bottom right corner) , as shown in Fig. 1. Blending applies to pixels in the CU around triangle split by weighted averaging MC predictors from both triangle partitions.

Fig. 1. CU partitions with triangular partition mode

# Proposed Solution

Thin partition mode splits a CU into 2 rectangular partitions; i.e., a CU is either split along 90-degree (vertical split), or 0-degree (horizontal split). Like triangle partition mode, each partition in thin partition mode has its own uni-direction temporal prediction MV.

In VVC framework, thin partition mode can be used when CU width or CU height is 4. Additionally, thin partition mode splits CU according to CU dimension. Specifically, a CU with 4-pixel wide is split horizontally while a CU with 4-pixel high is split vertically. The split happens in the middle of the CU so that two split rectangular partitions have the same dimension. Fig. 2 shows two examples of thin partition mode for 4xN and Nx4 CU, respectively.

Fig. 2. Two examples of CU partitions with thin partition mode.

Coding efficiency of thin partition mode can be improved with blending process. This contribution uses blending weights of [7/8 5/8 3/8 1/8] for luma block and [5/8 3/8] for chroma block.

No new syntax is required to support thin partition mode in this contribution. However, syntax for thin partition mode is slightly different because split direction is not needed. The related syntax is shown in the following merge data syntax table.

| merge_data( x0, y0, cbWidth, cbHeight, chType ) { | Descriptor |
| --- | --- |
| if ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_IBC ) { |  |
| if( MaxNumIbcMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( MaxNumSubblockMergeCand > 0 && cbWidth >= 8 && cbHeight >= 8 ) |  |
| merge_subblock_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_subblock_flag[ x0 ][ y0 ] = = 1 ) { |  |
| if( MaxNumSubblockMergeCand > 1 ) |  |
| merge_subblock_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( ( cbWidth * cbHeight ) >= 64 && ( (sps_ciip_enabled_flag && cu_skip_flag[ x0 ][ y0 ] = = 0 && cbWidth < 128 && cbHeight < 128) ¦ ¦ ( sps_triangle_enabled_flag && MaxNumTriangleMergeCand > 1 && slice_type = = B ) ) ) |  |
| regular_merge_flag[ x0 ][ y0 ] | ae(v) |
| if ( regular_merge_flag[ x0 ][ y0 ] = = 1 ){ |  |
| if( sps_mmvd_enabled_flag ) |  |
| mmvd_merge_flag[ x0 ][ y0 ] | ae(v) |
| if( mmvd_merge_flag[ x0 ][ y0 ] = = 1 ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| mmvd_cand_flag[ x0 ][ y0 ] | ae(v) |
| mmvd_distance_idx[ x0 ][ y0 ] | ae(v) |
| mmvd_direction_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| } else { |  |
| if( sps_ciip_enabled_flag && sps_triangle_enabled_flag && MaxNumTriangleMergeCand > 1 && slice_type = = B && cu_skip_flag[ x0 ][ y0 ] = = 0 && ( cbWidth * cbHeight ) >= 64 && cbWidth < 128 && cbHeight < 128 ) { |  |
| ciip_flag[ x0 ][ y0 ] | ae(v) |
| if( ciip_flag[ x0 ][ y0 ] && MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| if( !ciip_flag[ x0 ][ y0 ] && MaxNumTriangleMergeCand > 1 ) { |  |
| if ( cbWidth > 4 && cbHeight > 4 ) |  |
| merge_triangle_split_dir[ x0 ][ y0 ] | ae(v) |
| merge_triangle_idx0[ x0 ][ y0 ] | ae(v) |
| merge_triangle_idx1[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

MV derived for thin partition mode is stored for later use as neighbor MV (AMVP, TMVP). In the current VVC design, MV is stored at 4x4 block granularity. For thin partition mode, each 4x4 block stores MV of triangle partition that it belongs to. 4x4 blocks along horizontal/vertical split (blending block) that have their final predictors come from blending of two rectangular partitions, should only store MV from the second rectangular partition, partition that does not include pixel at top-left position.

# Experimental Results

The proposed thin partition mode is implemented in VTM-6.0 [2]. Experimental results are shown in Tables 1 and 2 against the VTM-6.0 under RA and LB CTC [3], respectively.

Table 1. Simulation result of thin partition mode for RA

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.05% | 0.00% | 99% | 98% |
| Class A2 | 0.02% | 0.09% | 0.05% | 99% | 97% |
| Class B | -0.01% | 0.08% | -0.01% | 98% | 95% |
| Class C | -0.04% | 0.08% | 0.15% | 99% | 96% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.05% | 0.04% | 99% | 96% |
| Class D | -0.05% | 0.20% | 0.06% | 99% | 98% |
| Class F | -0.03% | -0.05% | -0.06% | 100% | 98% |

Table 2. Simulation result of thin partition mode for LB

|  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.11% | -0.11% | 99% | 91% |
| Class C | -0.03% | 0.06% | 0.11% | 98% | 93% |
| Class E | 0.03% | 0.62% | -0.11% | 98% | 90% |
| Overall | 0.00% | 0.22% | -0.04% | 98% | 92% |
| Class D | 0.00% | -0.13% | 0.19% | 98% | 94% |
| Class F | 0.00% | -0.14% | 0.48% | 99% | 91% |

# Conclusions

Thin partition mode is proposed in this contribution. Unlike triangle partition mode, thin partition mode splits a CU either horizontally or vertically into two rectangular partitions. CU dimension is used to determine whether thin partition mode can be used or not and to decide split direction when thin partition mode is used. The results show a BD-rate performance of -0.01% and 0.00% for thin partition mode, under RA and LB coding condition, respectively.

# References

- B. Bross, J. Chen and S. Liu, “Versatile Video Coding”, JVET- JVET-O2001-vE, July 2019.

- VTM-6.0 reference software, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

- F. Bossen, J. Boyce, X. Li, V. Seregin and K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, Mar. 2019.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).