| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0452-v2 |
| --- | --- |

| Title: | AHG17: Low-delay ALF syntax |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Karsten Suehring Robert Skupin Yago Sanchez Thomas Schierl | Email: | {firstname.lastname}@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

In low-delay coding, ALF processing currently cannot be used because the ALF filter parameters of a picture are estimated after the picture is completely encoded. The ALF APS still needs to be inserted into the bitstream before the picture and the per-CTU data interspersed with the coded CTUs in the slice payload. This causes unnecessary processing delays in writing the bitstream, because the decoder also runs the ALF at the end of the decoding process of a picture. This contribution proposes to move ALF related data (ALF APS, ALF per-CTU data) towards the end of the picture, or behind the picture.

In the HLS BoG, it was suggested to use a VCL NAL unit instead of a new APS type for coding CTU-related ALF information. This variant is added as option 2c in version 2 of this contribution. Additionally an enable-flag is proposed in the SPS and a missing initialization indication for CABAC is added in options 2a and 2c.

# Introduction

Figure 1: Order of processing steps at the encoder

ALF processing is one of last coding steps in both encoder and decoder. After running intra- or inter prediction and residual coding on all CTUs, the ALF parameters are estimated (see Figure 1). In contrast to being a late step in the encoding process, the ALF coefficient data needs to be encoded in bitstream before the picture (as ALF APS). CTU based flags are interspersed in the slice_data() syntax for each CTU, so that the CABAC of the slice coding needs to be run after the ALF estimation (see Figure 2).

Figure 2: ALF data interspersed with other CTU data

In low-delay coding the encoder wants to start sending a picture already, before encoding of all parts is finished. This makes it impossible to use the ALF coefficients that are estimated for a picture in the same picture.

At the Gothenburg meeting it was discussed that in ultra-low-delay coding it may be useful to estimate ALF coefficients for the current picture and only send them after the picture for use in future pictures. If temporal sub-layers are used, this may turn out complicated, because the following access unit may be associated with a different sub-layer.

To allow ALF usage in ultra-low-delay coding, the following techniques are proposed:

- Proposal 1: suffix APS NAL unit: an APS that can follow the VCL NAL units in the access unit.

- Proposal 2a: ALF CTU data APS: an APS which contains CABAC coded per-CTU ALF parameters

- Proposal 2b: moving the per-CTU ALF syntax to the end of the slice

- Proposal 2c: moving the per-CTU ALF syntax to a new slice ALF VCL NAL unit type

# Proposal 1: Suffix APS

In this part, it is proposed to move the ALF APS behind the VCL NAL units of a picture.

Therefore, a new NAL unit type is introduced:

| 18 | PREFIX_APS_NUT | Adaptation parameter set adaptation_parameter_set_rbsp( ) | non-VCL |
| --- | --- | --- | --- |
| 25 | SUFFIX_APS_NUT | Adaptation parameter set adaptation_parameter_set_rbsp( ) | non-VCL |

Note: Using a previously reserved value here. An appropriate value needs to be selected to avoid conflict with other adoptions and NAL unit types could be renumbered to consecutive numbers.

The following constraints are added:

- Only ALF APS and ALF CTU data APS (see proposal 2a) may use SUFFIX_APS_NUT

- In bitstream order SUFFIX_APS_NUT is allowed after the first VLC NAL unit of an access unit and before any EOS_NUT or EOB_NUT, if present.

Optionally an SPS indication could be added, which could constrain the use of only PREFIX_APS_NUT or only SUFFIX_APS_NUT for ALF APSs within the coded video sequence (not included in specification text).

# Proposal 2a: ALF CTU data APS

To allow slice_data() to be written and processed independent of ALF usage, all ALF related CTU parameters are moved into an own APS:

| coding_tree_unit( ) { | Descriptor |
| --- | --- |
| xCtb = ( CtbAddrInRs % PicWidthInCtbsY ) << CtbLog2SizeY |  |
| yCtb = ( CtbAddrInRs / PicWidthInCtbsY ) << CtbLog2SizeY |  |
| if( slice_sao_luma_flag ¦ ¦ slice_sao_chroma_flag ) |  |
| sao( xCtb >> CtbLog2SizeY, yCtb >> CtbLog2SizeY ) |  |
| if( slice_alf_enabled_flag ){ |  |
| alf_ctb_flag[ 0 ][ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] | ae(v) |
| if( alf_ctb_flag[ 0 ][ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] ) { |  |
| if( slice_num_alf_aps_ids_luma > 0 ) |  |
| alf_ctb_use_first_aps_flag | ae(v) |
| if( !alf_ctb_use_first_aps_flag ) { |  |
| if( slice_num_alf_aps_ids_luma > 1 ) |  |
| alf_use_aps_flag | ae(v) |
| if( alf_use_aps_flag ) { |  |
| if( slice_num_alf_aps_ids_luma > 2 ) |  |
| alf_luma_prev_filter_idx_minus1 | ae(v) |
| } else |  |
| alf_luma_fixed_filter_idx | ae(v) |
| } |  |
| } |  |
| if( slice_alf_chroma_idc = = 1 ¦ ¦ slice_alf_chroma_idc = = 3 ) { |  |
| alf_ctb_flag[ 1 ][ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] | ae(v) |
| if( alf_ctb_flag[ 1 ][ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] && aps_alf_chroma_num_alt_filters_minus1 > 0 ) |  |
| alf_ctb_filter_alt_idx[ 0 ][ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] | ae(v) |
| } |  |
| if( slice_alf_chroma_idc = = 2 ¦ ¦ slice_alf_chroma_idc = = 3 ) { |  |
| alf_ctb_flag[ 2 ][ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] | ae(v) |
| if( alf_ctb_flag[ 2 ][ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] && aps_alf_chroma_num_alt_filters_minus1 > 0 ) |  |
| alf_ctb_filter_alt_idx[ 1 ][ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] | ae(v) |
| } |  |
| } |  |
| if( slice_type = = I && qtbtt_dual_tree_intra_flag ) |  |
| dual_tree_implicit_qt_split ( xCtb, yCtb, CtbSizeY, 0 ) |  |
| else |  |
| coding_tree( xCtb, yCtb, CtbSizeY, CtbSizeY, 1, 1, 0, 0, 0, 0, 0, SINGLE_TREE, MODE_TYPE_ALL ) |  |
| } |  |

The new APS type is added to adaptation_parameter_set_rbsp( ):

| adaptation_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| adaptation_parameter_set_id | u(5) |
| aps_params_type | u(3) |
| if( aps_params_type = = ALF_APS ) |  |
| alf_data( ) |  |
| if( aps_params_type = = ALF_CTU_APS ) |  |
| alf_ctu_data( ) |  |
| else if( aps_params_type = = LMCS_APS ) |  |
| lmcs_data( ) |  |
| else if( aps_params_type = = SCALING_APS ) |  |
| scaling_list_data( ) |  |
| aps_extension_flag | u(1) |
| if( aps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| aps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

And defined as follows:

| alf_ctu_data() { |  |
| --- | --- |
| aps_num_alf_aps_ids_luma | u(3) |
| for( i = 0; i < aps_num_alf_aps_ids_luma; i++ ) |  |
| aps_alf_aps_id_luma[ i ] | u(3) |
| aps_alf_chroma_idc | u(2) |
| if( aps_alf_chroma_idc ) |  |
| aps_alf_aps_id_chroma | u(3) |
| num_ctbs_hor | ue(v) |
| num_ctbs_ver | ue(v) |
| cabac_init_type | u(2) |
| byte_alignment( ) |  |
| for ( xCtb=0; xCtb < num_ctbs_hor, xCtb++ ) |  |
| for ( yCtb=0; yCtb < num_ctbs_ver, yCtb++ ) { |  |
| alf_ctb_flag[ 0 ][ xCtb ][ yCtb ] | ae(v) |
| if( alf_ctb_flag[ 0 ][ xCtb ][ yCtb ] ) { |  |
| if( slice_num_alf_aps_ids_luma > 0 ) |  |
| alf_ctb_use_first_aps_flag | ae(v) |
| if( !alf_ctb_use_first_aps_flag ) { |  |
| if( slice_num_alf_aps_ids_luma > 1 ) |  |
| alf_use_aps_flag | ae(v) |
| if( alf_use_aps_flag ) { |  |
| if( aps_num_alf_aps_ids_luma > 2 ) |  |
| alf_luma_prev_filter_idx_minus1 | ae(v) |
| } else |  |
| alf_luma_fixed_filter_idx | ae(v) |
| } |  |
| } |  |
| if( aps_alf_chroma_idc = = 1 ¦ ¦ aps_alf_chroma_idc = = 3 ) { |  |
| alf_ctb_flag[ 1 ][ xCtb ][ yCtb ] | ae(v) |
| if( alf_ctb_flag[ 1 ][ xCtb ][ yCtb ] && aps_alf_chroma_num_alt_filters_minus1 > 0 ) |  |
| alf_ctb_filter_alt_idx[ 0 ][ xCtb ][ yCtb ] | ae(v) |
| } |  |
| if( aps_alf_chroma_idc = = 2 ¦ ¦ aps_alf_chroma_idc = = 3 ) { |  |
| alf_ctb_flag[ 2 ][ xCtb ][ yCtb ] | ae(v) |
| if( alf_ctb_flag[ 2 ][ xCtb ][ yCtb ] && aps_alf_chroma_num_alt_filters_minus1 > 0 ) |  |
| alf_ctb_filter_alt_idx[ 1 ][ xCtb ][ yCtb ] | ae(v) |
| } |  |
| } |  |
| } |  |

Note: the part after the byte alignment is CABAC coded. The CABAC initialization is derived by the syntax element cabac_init_type, which directly signals the value of initType.

The slice header is changed as follows:

| slice_header( ) { | Descriptor |
| --- | --- |
| […] | ue(v) |
| if( sps_alf_enabled_flag ) { |  |
| slice_alf_enabled_flag | u(1) |
| if( slice_alf_enabled_flag ) { |  |
| slice_ctu_alf_aps_id | u(3) |
| slice_num_alf_aps_ids_luma | u(3) |
| for( i = 0; i < slice_num_alf_aps_ids_luma; i++ ) |  |
| slice_alf_aps_id_luma[ i ] | u(3) |
| if( ChromaArrayType != 0 ) |  |
| slice_alf_chroma_idc | u(2) |
| if( slice_alf_chroma_idc ) |  |
| slice_alf_aps_id_chroma | u(3) |
| } |  |
| } |  |
| […] |  |
| } |  |

Note that the slice header now only points to the CTU ALF APS, which then points to one or more ALF APSs.

# Proposal 2b: ALF CTU data at end of slice

The coding_tree_unit( ) syntax is modified as in 2a, i.e. all CTU based ALF parameters are removed.

Those are added into a new stucture alf_ctu_data( )

| slice_data( ) { | Descriptor |
| --- | --- |
| for( i = 0; i < NumBricksInCurrSlice; i++ ) { |  |
| CtbAddrInBs = FirstCtbAddrBs[ SliceBrickIdx[ i ] ] |  |
| for( j = 0; j < NumCtusInBrick[ SliceBrickIdx[ i ] ]; j++, CtbAddrInBs++ ) { |  |
| if( ( j % BrickWidth[ SliceBrickIdx[ i ] ] ) = = 0 ) { |  |
| NumHmvpCand = 0 |  |
| NumHmvpIbcCand = 0 |  |
| ResetIbcBuf = 1 |  |
| } |  |
| CtbAddrInRs = CtbAddrBsToRs[ CtbAddrInBs ] |  |
| coding_tree_unit( ) |  |
| if( entropy_coding_sync_enabled_flag && ( j < NumCtusInBrick[ SliceBrickIdx[ i ] ] – 1 ) && ( ( j + 1 ) % BrickWidth[ SliceBrickIdx[ i ] ] = = 0 ) ) { |  |
| end_of_subset_one_bit /* equal to 1 */ | ae(v) |
| byte_alignment( ) |  |
| } |  |
| } |  |
| end_of_brick_one_bit /* equal to 1 */ | ae(v) |
| if( i < NumBricksInCurrSlice − 1 ) |  |
| byte_alignment( ) |  |
| } |  |
| for( i = 0; i < NumBricksInCurrSlice; i++ ) { |  |
| CtbAddrInBs = FirstCtbAddrBs[ SliceBrickIdx[ i ] ] |  |
| for( j = 0; j < NumCtusInBrick[ SliceBrickIdx[ i ] ]; j++, CtbAddrInBs++ ) |  |
| alf_ctu_data( ) |  |
| } |  |
| } |  |

| alf_ctu_data( ) { | Descriptor |
| --- | --- |
| xCtb = ( CtbAddrInRs % PicWidthInCtbsY ) |  |
| yCtb = ( CtbAddrInRs / PicWidthInCtbsY ) |  |
| if( slice_alf_enabled_flag ){ |  |
| alf_ctb_flag[ 0 ][ xCtb ][ yCtb ] | ae(v) |
| if( alf_ctb_flag[ 0 ][ xCtb ][ yCtb ] ) { |  |
| if( slice_num_alf_aps_ids_luma > 0 ) |  |
| alf_ctb_use_first_aps_flag | ae(v) |
| if( !alf_ctb_use_first_aps_flag ) { |  |
| if( slice_num_alf_aps_ids_luma > 1 ) |  |
| alf_use_aps_flag | ae(v) |
| if( alf_use_aps_flag ) { |  |
| if( slice_num_alf_aps_ids_luma > 2 ) |  |
| alf_luma_prev_filter_idx_minus1 | ae(v) |
| } else |  |
| alf_luma_fixed_filter_idx | ae(v) |
| } |  |
| } |  |
| if( slice_alf_chroma_idc = = 1 ¦ ¦ slice_alf_chroma_idc = = 3 ) { |  |
| alf_ctb_flag[ 1 ][ xCtb ][ yCtb ] | ae(v) |
| if( alf_ctb_flag[ 1 ][ xCtb ][ yCtb ] && aps_alf_chroma_num_alt_filters_minus1 > 0 ) |  |
| alf_ctb_filter_alt_idx[ 0 ][ xCtb ][ yCtb ] | ae(v) |
| } |  |
| if( slice_alf_chroma_idc = = 2 ¦ ¦ slice_alf_chroma_idc = = 3 ) { |  |
| alf_ctb_flag[ 2 ][ xCtb ][ yCtb ] | ae(v) |
| if( alf_ctb_flag[ 2 ][ xCtb ][ yCtb ] && aps_alf_chroma_num_alt_filters_minus1 > 0 ) |  |
| alf_ctb_filter_alt_idx[ 1 ][ xCtb ][ yCtb ] | ae(v) |
| } |  |
| } |  |
| } |  |

# Proposal 2c: ALF CTU data in a new VCL NAL unit

This approach is similar to 2a, where all ALF related CTU parameters are moved into an own APS. The HLS BoG suggested to use a new VCL NAL unit type instead of an APS as shown in Figure 3.

Figure 3: ALF CTU data in own NAL unit

The NAL new NAL unit is sent after the coded slices of the picture and after the ALF APS:

Figure 4: NAL unit order with proposal 2c

The new NAL unit is defined as follows:

| xx | SLICE_ALF_NUT | Slice ALF data slice_alf_rbsp( ) | VCL |
| --- | --- | --- | --- |

Changes to the coding_tree_unit( ) are the same as in 2a above:

| coding_tree_unit( ) { | Descriptor |
| --- | --- |
| xCtb = ( CtbAddrInRs % PicWidthInCtbsY ) << CtbLog2SizeY |  |
| yCtb = ( CtbAddrInRs / PicWidthInCtbsY ) << CtbLog2SizeY |  |
| if( slice_sao_luma_flag ¦ ¦ slice_sao_chroma_flag ) |  |
| sao( xCtb >> CtbLog2SizeY, yCtb >> CtbLog2SizeY ) |  |
| if( slice_alf_enabled_flag ){ |  |
| alf_ctb_flag[ 0 ][ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] | ae(v) |
| if( alf_ctb_flag[ 0 ][ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] ) { |  |
| if( slice_num_alf_aps_ids_luma > 0 ) |  |
| alf_ctb_use_first_aps_flag | ae(v) |
| if( !alf_ctb_use_first_aps_flag ) { |  |
| if( slice_num_alf_aps_ids_luma > 1 ) |  |
| alf_use_aps_flag | ae(v) |
| if( alf_use_aps_flag ) { |  |
| if( slice_num_alf_aps_ids_luma > 2 ) |  |
| alf_luma_prev_filter_idx_minus1 | ae(v) |
| } else |  |
| alf_luma_fixed_filter_idx | ae(v) |
| } |  |
| } |  |
| if( slice_alf_chroma_idc = = 1 ¦ ¦ slice_alf_chroma_idc = = 3 ) { |  |
| alf_ctb_flag[ 1 ][ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] | ae(v) |
| if( alf_ctb_flag[ 1 ][ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] && aps_alf_chroma_num_alt_filters_minus1 > 0 ) |  |
| alf_ctb_filter_alt_idx[ 0 ][ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] | ae(v) |
| } |  |
| if( slice_alf_chroma_idc = = 2 ¦ ¦ slice_alf_chroma_idc = = 3 ) { |  |
| alf_ctb_flag[ 2 ][ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] | ae(v) |
| if( alf_ctb_flag[ 2 ][ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] && aps_alf_chroma_num_alt_filters_minus1 > 0 ) |  |
| alf_ctb_filter_alt_idx[ 1 ][ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] | ae(v) |
| } |  |
| } |  |
| if( slice_type = = I && qtbtt_dual_tree_intra_flag ) |  |
| dual_tree_implicit_qt_split ( xCtb, yCtb, CtbSizeY, 0 ) |  |
| else |  |
| coding_tree( xCtb, yCtb, CtbSizeY, CtbSizeY, 1, 1, 0, 0, 0, 0, 0, SINGLE_TREE, MODE_TYPE_ALL ) |  |
| } |  |

The new NAL unit is defined as follows:

| slice_alf_rbsp() { |  |
| --- | --- |
| num_alf_aps_ids_luma | u(3) |
| for( i = 0; i < num_alf_aps_ids_luma; i++ ) |  |
| aps_alf_aps_id_luma[ i ] | u(3) |
| alf_chroma_idc | u(2) |
| if(alf_chroma_idc ) |  |
| alf_aps_id_chroma | u(3) |
| num_ctbs_hor | ue(v) |
| num_ctbs_ver | ue(v) |
| cabac_init_type | u(2) |
| byte_alignment( ) |  |
| for ( xCtb=0; xCtb < num_ctbs_hor, xCtb++ ) |  |
| for ( yCtb=0; yCtb < num_ctbs_ver, yCtb++ ) { |  |
| alf_ctb_flag[ 0 ][ xCtb ][ yCtb ] | ae(v) |
| if( alf_ctb_flag[ 0 ][ xCtb ][ yCtb ] ) { |  |
| if(num_alf_aps_ids_luma > 0 ) |  |
| alf_ctb_use_first_aps_flag | ae(v) |
| if( !alf_ctb_use_first_aps_flag ) { |  |
| if(num_alf_aps_ids_luma > 1 ) |  |
| alf_use_aps_flag | ae(v) |
| if( alf_use_aps_flag ) { |  |
| if( num_alf_aps_ids_luma > 2 ) |  |
| alf_luma_prev_filter_idx_minus1 | ae(v) |
| } else |  |
| alf_luma_fixed_filter_idx | ae(v) |
| } |  |
| } |  |
| if( alf_chroma_idc = = 1 ¦ ¦ alf_chroma_idc = = 3 ) { |  |
| alf_ctb_flag[ 1 ][ xCtb ][ yCtb ] | ae(v) |
| if( alf_ctb_flag[ 1 ][ xCtb ][ yCtb ] && alf_chroma_num_alt_filters_minus1 > 0 ) |  |
| alf_ctb_filter_alt_idx[ 0 ][ xCtb ][ yCtb ] | ae(v) |
| } |  |
| if( alf_chroma_idc = = 2 ¦ ¦ alf_chroma_idc = = 3 ) { |  |
| alf_ctb_flag[ 2 ][ xCtb ][ yCtb ] | ae(v) |
| if( alf_ctb_flag[ 2 ][ xCtb ][ yCtb ] && alf_chroma_num_alt_filters_minus1 > 0 ) |  |
| alf_ctb_filter_alt_idx[ 1 ][ xCtb ][ yCtb ] | ae(v) |
| } |  |
| } |  |
| } |  |

Note: the part after the byte alignment is CABAC coded. The CABAC initialization is derived by the syntax element cabac_init_type, which directly signals the value of initType. The syntax table can be split into two sections similar to the slice, which is split into slice_header() for header data and slice_data for CABAC coded data.

The slice header is changed as follows:

| slice_header( ) { | Descriptor |
| --- | --- |
| […] | ue(v) |
| if( sps_alf_enabled_flag ) { |  |
| slice_alf_enabled_flag | u(1) |
| if( slice_alf_enabled_flag ) { |  |
| slice_num_alf_aps_ids_luma | u(3) |
| for( i = 0; i < slice_num_alf_aps_ids_luma; i++ ) |  |
| slice_alf_aps_id_luma[ i ] | u(3) |
| if( ChromaArrayType != 0 ) |  |
| slice_alf_chroma_idc | u(2) |
| if( slice_alf_chroma_idc ) |  |
| slice_alf_aps_id_chroma | u(3) |
| } |  |
| } |  |
| […] |  |
| } |  |

Note: There is no ID in this variant, by which the slice would be linked to the new SLICE_APS_NUT NAL unit. An association is achieved solely by being present in the same access unit.

A rule shall be added to the order of NAL units indicating that SLICE_ALF_NUT shall follow all slice NAL units in the bitstream.

# Enable-flag signalling in SPS

Separating the ALF NAL CTU data from the slice may not be desirable for all applications. Thus the proposed functionality could be controlled by a SPS enable-flag.

# Discussion of proposals 2a vs. 2b vs. 2.c

The authors prefer adopting Proposal 1 with Proposal 2a.

Advantages of 2a (new APS type)

- APS can be written after the picture is completely encoded

- The picture can be split into multiple slices, which are sent to the decoder directly after encoding them. ALF estimation can still be performed after sending all slices of the picture. There is no need to re-encode any slice.

Disadvantages of 2a (new APS type)

- CABAC coding in an APS

- Some overhead for NAL unit and APS headers

Advantages of 2b (moving syntax to the end of slice)

- Simple move of syntax elements, no change of coding performance

- No new APS required

Disadvantages of 2b (moving syntax to the end of slice)

- When multiple slices are used, at least partial ALF estimation has to be performed after the first slice. This may lead to sub-optimal results compared to 2a

Advantages of 2c (new VCL NAL unit type):

- the ALF data NAL unit can be written after the picture is completely encoded

- The picture can be split into multiple slices, which are sent to the decoder directly after encoding them. ALF estimation can still be performed after sending all slices of the picture. There is no need to re-encode any slice.

Disadvantages of 2c (new VCL NAL unit type):

- - requires assigning a new NAL unit type

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).