| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0459-v1 |
| --- | --- |

| Title: | AHG17: Mergeability identifier for subpictures |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Robert Skupin Yago Sanchez Karsten Suehring Thomas Schierl | Email: | {firstname.lastname}@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution proposes signaling to support merging of sub-pictures, specifically identification of mergeable bitstreams and the merging method to be applied.

An SEI message is proposed that carries one or more mergeability_id and a mergeability_type per sub-picture describing which NAL unit types and syntax structures are required to be rewritten in a merging operation. In addition, if the sub-pictures to be merged are not included into the same bitstream a variable is identified by external means such as an application specification or a service operator that indicates that the mergeability_id and a mergeability_type are consistent across bitstreams.

# Introduction

At the 14th meeting of JVET in Gothenburg, a sub-picture design was adopted into the VVC draft text. Merging of such sub-pictures into a composite bitstream is valid operation choice for many applications where a single decoder is required, 360-degree video streaming to constrained devices.

In JVET-O0492, an SEI message was proposed to support merging of sub-pictures, by specifically identification of mergeable bitstreams and the merging method to be applied.

A merging method is identified by the necessary NAL units to be rewritten during merging, e.g.

- DPS, VPS, SPS (I)

- DPS, VPS, SPS, PPS

- DPS, VPS, SPS, PPS, APS

- All parameter sets and slice headers

The merging method specific identification of mergeable bitstreams is achieved by matching identifier values.

The following comments where received regarding the proposal in JVET-O0492 during BoG review as noted in JVET-O1076 with added author responses:

Comment 2: It was commented that such information could also be left for systems, e.g., file format or DASH to carry.

Response: It is argued that the information is most accessible at the encoder. Specifically, when low level characteristics are to be regarded, compiling/verifying the information would potentially require low level bitstream parsing. However, carriage of the information in a higher layer such as file format or DASH would benefit from having the compiled/verified information available in an SEI message.

Comment 3: It was commented that it is possible to just use the ID itself without a type field, if the semantics of the type field is incorporated into the semantics of the ID.

Response: It is asserted that any more bit efficient solution beyond concatenation would not be a good solution as it potentially leads to conflicts/collisions of ID values, e.g. a certain ID value of a certain incorporated merging method could collide with another ID value of another merging method.

Comment 1: It was commented that using 6 bits for mergeability_id might not be enough to avoid potential conflict of IDs, particularly if the semantics of the ID is NOT defined and controlled by a single organization.

Response: The updated proposal reflects the comment with a larger value space of the identifier. Further, the proposal includes alternative options on how to control ID assignment and avoid conflicts/collisions.

- identifier scope determined by external means, or

- identifier scope determined by bitstream

The proposed design has further been updated with respect to the initial proposal in JVET-O0492 by allowing sub-picture specific indication

# Proposal

Mergeability SEI message

The mergeability SEI message carries information that allows identification of mergeable sub-pictures and the required merging operation.

| mergeability( payloadSize ) { | Descriptor |
| --- | --- |
| num_sub_pictures_minus1 | u(8) |
| for( i = 0; i <= num_sub_pictures_minus1; i++) { |  |
| num_mergeability_ids_minus1[ i ] | u(8) |
| for( j = 0; j <= num_mergeability_ids_minus1[ i ]; j ++) |  |
| mergeability_id[ i ][ j ] | u(30) |
| mergeability_type[ i ][ j ] | u(2) |
| } |  |
| } |  |
| } |  |

num_sub_pictures_minus1 plus 1 specifies the number of sub-pictures in the bitstream. The value of num_sub_pictures_minus1 shall be in the range of 0 to 255, inclusive.

num_mergeability_ids_minus1[ i ] plus 1 specifies the number of merge idenfiers in the SEI message. The value of num_mergeability_ids_minus1[ i ] shall be in the range of 0 to 255, inclusive.

mergeability_id[ i ][ j ] contains the j-th identifying number of the i-th sub-picture that may be used to identify the sub-pictures with matching value of mergeability_id[ i ][ j ] which can be merged with the i-th sub-picture into an conformant joint bitstream according to the method associated with mergeability_type[ i ][ j ]. The value of mergeability_id[ i ][ j ] shall be in the range of 0 to 227 − 1, inclusive.

Option a) If some external means, not specified in this Specification, is available to set the value of MergeabilityIdSpace, it is a bitstream conformance that sub-picture of more than one bitstream carrying the same value of mergeability_id[ i ][ j ], mergeability_type[ i ][ j ] and MergeabilityIdSpace can be merged accoding to the merging method identifed by mergeability_type[ i ][ j ].

Option b) It is a requirement of bitstream conformance that sub-pictures in the bitstream carrying the same value of mergeability_id[ i ][ j ] and mergeability_type[ i ][ j ] can be merged accoding to the merging method identifed by mergeability_type[ i ][ j ].

Values of mergeability_id[ i ][ j ] from 227 to 230 − 1, inclusive, are reserved for future use by ITU-T | ISO/IEC and bitstreams shall not contain such values. Decoders encountering a value of mergeability_id[ i ][ j ] in the range of 227 to 230 − 1, inclusive, shall ignore it.

mergeability_type[ i ][ j ] equal to 0 specifies that merging of the i-th sub-picture with other sub-pictures with matching values of the associated mergeability_id[ i ][ j ] into a single joint bitstream does not require changes of any active PPS, active APS, slice headers or slice payloads in the bitstream. mergeability_type[ i ][ j ] equal to 1 specifies that merging of the i-th sub-picture with other sub-pictures with matching values of the associated mergeability_id[ i ][ j ] into a single joint bitstream does not require changes of any active APS or NAL unit slice headers or payloads in the bitstream. mergeability_type[ i ][ j ] equal to 2 specifies that merging of the i-th sub-picture with other sub-pictures with matching values of the associated mergeability_id[ i ][ j ] into a single joint bitstream does not require changes of any slice header or slice payloads in the bitstream. mergeability_type[ i ][ j ] equal to 3 specifies specifies that merging of the i-th sub-picture with other sub-pictures with matching values of the associated mergeability_id[ i ][ j ] into a single joint bitstream does not require changes of any slice payloads in the bitstream. The value of mergeability_type[ i ][ j ] shall be in the range of 0 to 3, inclusive.

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).