| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0462-v1 |
| --- | --- |

| Title: | AHG6: 360-degree video related SEI messages |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Robert Skupin Yago Sanchez Karsten Suehring Thomas Schierl | Email: | {firstname.lastname}@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

At the 15th JVET meeting in Gothenburg, proposals were made regarding new 360-degree video signalling. This document identifies issues of changing some of the design decisions of the respective signalling in in AVC/HEVC and proposes to adopt the following SEI messages into the CD text of “Supplemental enhancement information messages for coded video bitstreams (Draft 1)” in JVET-O2007:

- the equirectangular Projection SEI message, and

- the cubemap projection SEI message, and

- the sphere rotation SEI message, and

- the region-wise packing SEI message, and

- the omnidirectional viewport SEI message.

# Discussion

At the 15th JVET meeting in Gothenburg, proposals were made regarding new 360-degree video signalling. In AVC/HEVC, the related signalling entails

- the equirectangular Projection SEI message, and

- the cubemap projection SEI message, and

- the sphere rotation SEI message, and

- the region-wise packing SEI message, and

- the omnidirectional viewport SEI message.

The AVC/HEVC SEI messages and related sample location derivation processes were designed in close coordination with the relevant MPEG systems groups in parallel. This was done to ensure mostly equivalent signalling capabilities of SEI messages and ISO Base Media File Format and even mostly aligned syntax which significantly eases implementation, e.g. for MPEG OMAF

The AVC and HEVC design decouples general projection characteristics (e.g. rotation), projection and packing format which has numerous advantages:

- It significantly simplifies the projection SEI message syntax so 360-degree video can be supported with a lower burden (ignoring rotation or packing)

- Packing formats and syntax does not have to be re-invented for every new projection and newly added projection formats, e.g. advanced cubemap variants, can use the existing packing formats.

- [list advantages]

During specification of the respective amendments to AVC/HEVC, designs that mixed projection and packing formats have been controversially discussed at the time which resulted in a conscious decision for the present separate design.

On the other hand, there are concerns that the RWP design in AVC/HEVC is too flexible and that the syntax is not bit-efficient. However, it is the opinion of the authors that application specifications such as MPEG OMAF are the right entity to define constraints as has been done for instance regarding temporal changes of RWP. It is further thought that bit-efficiency can be seen as a neglectable issue when considered in context of the bitrates in typical 360-degree applications.

Would JVET go on to re-define new signalling in VVC for 360-degree video, it would either

- require the respective MPEG systems groups to do much more integration work into existing specifications, or

- adopted the 360-degree video signalling of AVC/HEVC in parallel to ease integration.

# Proposal

Therefore, the document proposes to adopt an AVC/HEVC aligned signalling for 360-degree video and possibly add new projection format in an aligned fashion.

The attached specification text integrates the respective sections from Annex E of the HEVC specification as a whole into the CD text of “Supplemental enhancement information messages for coded video bitstreams (Draft 1)” in JVET-O2007.

The respective HEVC text also contains references to the frame packing arrangement SEI message and the segmented rectangular frame packing arrangement SEI message of HEVC. It is proposed to discuss how to progress with these two SEI messages as well.

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).