| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O2007-v1 |
| --- | --- |

| Title: | Supplemental enhancement information messages for coded video bitstreams (Draft 1) |  |  |
| --- | --- | --- | --- |
| Status: | Output document approved by JVET |  |  |
| Purpose: | Draft text |  |  |
| Author(s) or Contact(s): | Jill Boyce Gary Sullivan Ye-Kui Wang | Email: | jill.boyce@intel.com garysull@miscrosoft.com ye-kui.wang@futurewei.com |
| Source: | Editors |  |  |

_____________________________

# Abstract

This document is Draft 1 of a standard to specify the syntax and semantics of supplemental enhancement information (SEI) messages for use with coded video bitstreams. The SEI messages defined in this draft standard may be conveyed within coded video bitstreams in a manner specified in a video coding specification or may be conveyed by other means as determined by the specifications for systems that make use of such coded video bitstreams. This draft standard is particularly intended for use with coded video bitstreams as specified by Rec. ITU-T H.VVC | ISO/IEC 23090-3, although it is drafted in a manner intended to be sufficiently generic that it may also be used with other types of coded video bitstreams.

Editors’ Notes:

Draft 1:

- Migration from JVET-N1001-v10 Annex F: Decoded picture hash SEI message and dependent random access point indication SEI message

CONTENTS

Page

Abstract	i

1	Scope	1

2	Normative references	1

2.1	Identical Recommendations | International Standards	1

2.2	Paired Recommendations | International Standards equivalent in technical content	1

2.3	Additional references	1

3	Definitions	1

4	Abbreviations	4

5	Conventions	4

5.1	General	4

5.2	Arithmetic operators	4

5.3	Logical operators	5

5.4	Relational operators	5

5.5	Bit-wise operators	5

5.6	Assignment operators	5

5.7	Range notation	6

5.8	Mathematical functions	6

5.9	Order of operation precedence	7

5.10	Variables, syntax elements and tables	8

5.11	Text description of logical operations	9

5.12	Processes	10

6	Syntax and semantics	11

6.1	Method of specifying syntax in tabular form	11

6.2	Specification of syntax functions and descriptors	12

7	SEI message payload specifications	13

7.1	General	13

7.2	SEI payload syntax	13

7.2.1	General	13

7.2.2	Decoded picture hash SEI message syntax	13

7.2.3	Dependent random access point indication SEI message syntax	13

7.2.4	Reserved SEI message syntax	14

7.3	SEI payload semantics	14

7.3.1	General	14

7.3.2	Decoded picture hash SEI message semantics	14

7.3.3	Dependent random access point indication SEI message semantics	16

7.3.4	Reserved SEI message semantics	16

INTERNATIONAL  STANDARD

Draft ISO/IEC 23002-7

Draft Rec. ITU-T H.SEI

ITU-T  RECOMMENDATION

# Supplemental enhancement information messages for coded video bitstreams

# Scope

This Recommendation | International Standard specifies the syntax and semantics of supplemental enhancement information (SEI) messages. The SEI messages defined in this standard may be conveyed within coded video bitstreams in a manner specified in a video coding specification or may be conveyed by other means as determined by the specifications for systems that make use of such coded video bitstreams. This document is particularly intended for use with coded video bitstreams as specified by Rec. ITU-T H.VVC | ISO/IEC 23090-3, although it is drafted in a manner intended to be sufficiently generic that it may also be used with other types of coded video bitstreams.

This document is written in a manner such that it is intended to be referenced by other technical specifications. Such other technical specifications are to be written in a manner to specify certain necessary elements to enable the use of these SEI messages.

SEI messages can assist in processes related to decoding, display or other purposes. However, unless otherwise specified in a referencing specification, the interpretation and use of the SEI messages specified in this document is not a required functionality of a video decoder or receiving video system. Although semantics are specified for these SEI messages, decoders and receiving video systems may simply ignore the content of these SEI messages or may use them in some manner not specified in this document.

# Normative references

The following Recommendations and International Standards contain provisions which, through reference in this text, constitute provisions of this Recommendation | International Standard. At the time of publication, the editions indicated were valid. All Recommendations and Standards are subject to revision, and parties to agreements based on this Recommendation | International Standard are encouraged to investigate the possibility of applying the most recent edition of the Recommendations and Standards listed below. Members of IEC and ISO maintain registers of currently valid International Standards. The Telecommunication Standardization Bureau of the ITU maintains a list of currently valid ITU-T Recommendations.

## Identical Recommendations | International Standards

–	None

## Paired Recommendations | International Standards equivalent in technical content

–	None

## Additional references

–	IETF RFC 1321 (in force), The MD5 Message-Digest Algorithm.

# Definitions

For the purposes of this Recommendation | International Standard, the following definitions apply.

  - byte-aligned: A position in a coded video bitstream or syntax structure is byte-aligned when the position is an integer multiple of 8 bits from the position of the first bit in the coded video bitstream or syntax structure, and a bit or byte or syntax element is said to be byte-aligned when the position at which it appears in a coded video bitstream or syntax structure is byte-aligned.

  - byte stream: An encapsulation of a NAL unit stream containing start code prefixes and NAL units.

  - chroma: An adjective, represented by the symbols Cb and Cr, specifying that a sample array or single sample is representing one of the two colour difference signals related to the primary colours.

NOTE – The term chroma is used rather than the term chrominance in order to avoid the implication of the use of linear light transfer characteristics that is often associated with the term chrominance.

  - coded picture: A coded representation of a picture containing all CTUs of the picture.

  - coded video bitstream: A sequence of bits that forms the representation of coded pictures and associated data forming one or more coded video sequences (CVSs).

  - coded video sequence (CVS): A sequence of access units that consists, in decoding order, of an IRAP access unit, followed by zero or more access units that are not IRAP access units, including all subsequent access units up to but not including any subsequent access unit that is an IRAP access unit.

  - component: An array or single sample from one of the three arrays (luma and two chroma) that compose a picture in 4:2:0, 4:2:2, or 4:4:4 colour format or the array or a single sample of the array that compose a picture in monochrome format.

  - decoded picture: A decoded picture is derived by decoding a coded picture.

  - decoder: An embodiment of a decoding process.

  - decoding order: The order in which syntax elements are conveyed in the coded video bitstream and are processed by a decoding process.

  - decoding process: The process that reads a coded video bitstream and derives decoded pictures from it.

  - emulation prevention byte: A byte equal to 0x03 that is present within a NAL unit when the syntax elements of the coded video bitstream form certain patterns of byte values in a manner that ensures that no sequence of consecutive byte-aligned bytes in the NAL unit can contain a start code prefix.

  - encoder: An embodiment of an encoding process.

  - encoding process: A process that produces a coded video bitstream.

  - flag: A variable or single-bit syntax element that can take one of the two possible values: 0 and 1.

  - informative: A term used to refer to content provided in this Specification that does not establish any mandatory requirements for conformance to this Specification and thus is not considered an integral part of this Specification.

  - intra random access point (IRAP) access unit: An access unit in which the coded picture is an IRAP picture.

  - intra random access point (IRAP) picture: A coded picture that can be decoded without first decoding any other picture in the coded video bitstream.

  - luma: An adjective, represented by the symbol or subscript Y or L, specifying that a sample array or single sample is representing the monochrome signal related to the primary colours.

NOTE – The term luma is used rather than the term luminance in order to avoid the implication of the use of linear light transfer characteristics that is often associated with the term luminance. The symbol L is sometimes used instead of the symbol Y to avoid confusion with the symbol y as used for vertical location.

  - may: A term that is used to refer to behaviour that is allowed, but not necessarily required.

NOTE – In some places where the optional nature of the described behaviour is intended to be emphasized, the phrase "may or may not" is used to provide emphasis.

  - must: A term that is used in expressing an observation about a requirement or an implication of a requirement that is specified elsewhere in this Specification (used exclusively in an informative context).

  - network abstraction layer (NAL) unit: A syntax structure containing an indication of the type of data to follow and bytes containing that data in the form of an RBSP interspersed as necessary with emulation prevention bytes.

  - network abstraction layer (NAL) unit stream: A sequence of NAL units.

  - non-VCL NAL unit: A NAL unit that is not a VCL NAL unit.

  - note: A term that is used to prefix informative remarks (used exclusively in an informative context).

  - output order: The order in which the decoded pictures are output from the decoded picture buffer (for the decoded pictures that are to be output from the decoded picture buffer).

  - parameter: A syntax element of a sequence parameter set (SPS) or picture parameter set (PPS), or the second word of the defined term quantization parameter.

  - partitioning: The division of a set into subsets such that each element of the set is in exactly one of the subsets.

  - picture: An array of luma samples in monochrome format or an array of luma samples and two corresponding arrays of chroma samples in 4:2:0, 4:2:2, and 4:4:4 colour format.

NOTE – A picture may be either a frame or a field. However, in one CVS, either all pictures are frames or all pictures are fields.

  - picture parameter set (PPS): A syntax structure containing syntax elements that apply to zero or more entire coded pictures as determined by a syntax element found in each slice header.

  - picture order count (POC): A variable that is associated with each picture, uniquely identifies the associated picture among all pictures in the CVS, and, when the associated picture is to be output from the decoded picture buffer, indicates the position of the associated picture in output order relative to the output order positions of the other pictures in the same CVS that are to be output from the decoded picture buffer.

  - profile: A specified subset of the syntax of this Specification.

  - random access: The act of starting the decoding process for a coded video bitstream at a point other than the beginning of the stream.

  - raw byte sequence payload (RBSP): A syntax structure containing an integer number of bytes that is encapsulated in a NAL unit and that is either empty or has the form of a string of data bits containing syntax elements followed by an RBSP stop bit and zero or more subsequent bits equal to 0.

  - raw byte sequence payload (RBSP) stop bit: A bit equal to 1 present within a raw byte sequence payload (RBSP) after a string of data bits, for which the location of the end within an RBSP can be identified by searching from the end of the RBSP for the RBSP stop bit, which is the last non-zero bit in the RBSP.

  - reference picture: A picture that is a short-term reference picture.

NOTE – A reference picture contains samples that may be used for inter prediction in the decoding process of subsequent pictures in decoding order.

  - reference picture list: A list of reference pictures that is used for inter prediction of a P or B slice.

NOTE – For the decoding process of a P slice, there is one reference picture list – reference picture list 0. For the decoding process of a B slice, there are two reference picture lists – reference picture list 0 and reference picture list 1.

  - reserved: A term that may be used to specify that some values of a particular syntax element are for future use by ITU-T | ISO/IEC and shall not be used in syntax structures conforming to this version of this Specification, but may be used in syntax structures conforming to future extensions of this Specification by ITU-T | ISO/IEC.

  - sequence parameter set (SPS): A syntax structure containing syntax elements that apply to zero or more entire CVSs as determined by the content of a syntax element found in the PPS referred to by a syntax element found in each slice header.

  - shall: A term used to express mandatory requirements for conformance to this Specification.

NOTE – When used to express a mandatory constraint on the values of syntax elements or on the results obtained by operation of the specified decoding process, it is the responsibility of the encoder to ensure that the constraint is fulfilled. When used in reference to operations performed by the decoding process, any decoding process that produces identical cropped decoded pictures to those output from the decoding process described in this Specification conforms to the decoding process requirements of this Specification.

  - short-term reference picture: A picture that is marked as "used for short-term reference".

  - should: A term used to refer to behaviour of an implementation that is encouraged to be followed under anticipated ordinary circumstances, but is not a mandatory requirement for conformance to this Specification.

  - source: A term used to describe the video material or some of its attributes before encoding.

  - start code prefix: A unique sequence of three bytes equal to 0x000001 embedded in the byte stream as a prefix to each NAL unit.

NOTE – The location of a start code prefix can be used by a decoder to identify the beginning of a new NAL unit and the end of a previous NAL unit. Emulation of start code prefixes is prevented within NAL units by the inclusion of emulation prevention bytes.

  - string of data bits (SODB): A sequence of some number of bits representing syntax elements present within a raw byte sequence payload prior to the raw byte sequence payload stop bit, where the left-most bit is considered to be the first and most significant bit, and the right-most bit is considered to be the last and least significant bit.

  - syntax element: An element of data represented in a syntax structure.

  - syntax structure: Zero or more syntax elements that are present together in a specified order.

  - temporal sublayer identifier: A number greater than or equal to 0 such that pictures of all temporal sublayers have a specified temporal output order relative to each other and pictures with a lower temporal identifier can be decoded without reference to pictures with a higher temporal identifier.

  - trailing picture: A coded picture that follows an IRAP picture in both decoding order and output order.

  - unspecified: A term that may be used to specify some values of a particular syntax element to indicate that the values have no specified meaning in this Specification and will not have a specified meaning in the future as an integral part of future versions of this Specification.

  - video coding layer (VCL) NAL unit: A collective term for coded slice NAL units and the subset of NAL units that have reserved values of nal_unit_type that are classified as VCL NAL units in this Specification.

# Abbreviations

CRC	Cyclic Redundancy Check

CVS	Coded Video Sequence

DRAP	Dependent Random Access Point

IDR	Instantaneous Decoding Refresh

IRAP	Intra Random Access Point

NAL	Network Abstraction Layer

POC	Picture Order Count

PPS	Picture Parameter Set

RPS	Reference Picture Set

SEI	Supplemental Enhancement Information

SPS	Sequence Parameter Set

VCL	Video Coding Layer

VPS	Video Parameter Set

# Conventions

## General

NOTE – The mathematical operators used in this Specification are similar to those used in the C programming language. However, the results of integer division and arithmetic shift operations are defined more precisely, and additional operations are defined, such as exponentiation and real-valued division. Numbering and counting conventions generally begin from 0, e.g., "the first" is equivalent to the 0-th, "the second" is equivalent to the 1-th, etc.

## Arithmetic operators

The following arithmetic operators are defined as follows:

| + | Addition |
| --- | --- |
| − | Subtraction (as a two-argument operator) or negation (as a unary prefix operator) |
| * | Multiplication, including matrix multiplication |
| xy | Exponentiation. Specifies x to the power of y. In other contexts, such notation is used for superscripting not intended for interpretation as exponentiation. |
| / | Integer division with truncation of the result toward zero. For example, 7 / 4 and −7 / −4 are truncated to 1 and −7 / 4 and 7 / −4 are truncated to −1. |
| ÷ | Used to denote division in mathematical equations where no truncation or rounding is intended. |
| xy | Used to denote division in mathematical equations where no truncation or rounding is intended. |
| i = xyf( i ) | The summation of f( i ) with i taking all integer values from x up to and including y. |
| x % y | Modulus. Remainder of x divided by y, defined only for integers x and y with x >= 0 and y > 0. |

## Logical operators

The following logical operators are defined as follows:

x  &&  y	Boolean logical "and" of x and y

x  | |  y	Boolean logical "or" of x and y

!	Boolean logical "not"

x ? y : z	If x is TRUE or not equal to 0, evaluates to the value of y; otherwise, evaluates to the value of z.

## Relational operators

The following relational operators are defined as follows:

>	Greater than

>=	Greater than or equal to

<	Less than

<=	Less than or equal to

= =	Equal to

!=	Not equal to

When a relational operator is applied to a syntax element or variable that has been assigned the value "na" (not applicable), the value "na" is treated as a distinct value for the syntax element or variable. The value "na" is considered not to be equal to any other value.

## Bit-wise operators

The following bit-wise operators are defined as follows:

&	Bit-wise "and". When operating on integer arguments, operates on a two's complement representation of the integer value. When operating on a binary argument that contains fewer bits than another argument, the shorter argument is extended by adding more significant bits equal to 0.

|	Bit-wise "or". When operating on integer arguments, operates on a two's complement representation of the integer value. When operating on a binary argument that contains fewer bits than another argument, the shorter argument is extended by adding more significant bits equal to 0.

^	Bit-wise "exclusive or". When operating on integer arguments, operates on a two's complement representation of the integer value. When operating on a binary argument that contains fewer bits than another argument, the shorter argument is extended by adding more significant bits equal to 0.

x  >>  y	Arithmetic right shift of a two's complement integer representation of x by y binary digits. This function is defined only for non-negative integer values of y. Bits shifted into the most significant bits (MSBs) as a result of the right shift have a value equal to the MSB of x prior to the shift operation.

x  <<  y	Arithmetic left shift of a two's complement integer representation of x by y binary digits. This function is defined only for non-negative integer values of y. Bits shifted into the least significant bits (LSBs) as a result of the left shift have a value equal to 0.

## Assignment operators

The following arithmetic operators are defined as follows:

=	Assignment operator

+ +	Increment, i.e., x+ + is equivalent to x = x + 1; when used in an array index, evaluates to the value of the variable prior to the increment operation.

− −	Decrement, i.e., x− − is equivalent to x = x − 1; when used in an array index, evaluates to the value of the variable prior to the decrement operation.

+=	Increment by amount specified, i.e., x  +=  3 is equivalent to x = x + 3, and x  +=  (−3) is equivalent to x = x + (−3).

−=	Decrement by amount specified, i.e., x  −=  3 is equivalent to x = x − 3, and x  −=  (−3) is equivalent to x = x − (−3).

## Range notation

The following notation is used to specify a range of values:

x = y..z	x takes on integer values starting from y to z, inclusive, with x, y, and z being integer numbers and z being greater than y.

## Mathematical functions

The following mathematical functions are defined:

Abs( x ) =  x;x  >=  0-x;x < 0		(1)

Asin( x )	the trigonometric inverse sine function, operating on an argument x that is
in the range of −1.0 to 1.0, inclusive, with an output value in the range of 
−π÷2 to π÷2, inclusive, in units of radians		(2)

Atan( x )	the trigonometric inverse tangent function, operating on an argument x, with
an output value in the range of −π÷2 to π÷2, inclusive, in units of radians	(3)

Atan2( y, x ) =  Atan yx ;x > 0Atan yx +πAtan yx -π;;x < 0  &&  y  >=  0x < 0  &&  y  <  0+π2-π2;;x = =  0  &&  y  >=  0otherwise	(4)

Ceil( x )	the smallest integer greater than or equal to x.	(5)

Clip1Y( x ) = Clip3( 0, ( 1  <<  BitDepthY ) − 1, x )		(6)

Clip1C( x ) = Clip3( 0, ( 1  <<  BitDepthC ) − 1, x )		(7)

Clip3( x, y, z ) =  x;z < xy;z > yz;otherwise		(8)

Cos( x )	the trigonometric cosine function operating on an argument x in units of radians.	(9)

Floor( x )	the largest integer less than or equal to x.		(10)

GetCurrMsb( a, b, c, d ) =  c+d;b-a  >=  d / 2  c-d;a-b  >  d / 2c;otherwise	(11)

Ln( x )	the natural logarithm of x (the base-e logarithm, where e is the natural logarithm base constant 2.718 281 828...).		(12)

Log2( x )	the base-2 logarithm of x.		(13)

Log10( x ) the base-10 logarithm of x.		(14)

Min( x, y ) =   x;x <= yy;x > y		(15)

Max( x, y ) =   x;x >= yy;x < y		(16)

Round( x ) = Sign( x ) * Floor( Abs( x ) + 0.5 )		(17)

Sign( x ) = 1;x > 00;x  = =  0-1;x  <  0		(18)

Sin( x )	the trigonometric sine function operating on an argument x in units of radians	(19)

Sqrt( x )	the square root of x		(20)

Swap( x, y ) equivalent to setting a temporary variable t equal to x, then x = y and y = t	(21)

Tan( x )	the trigonometric tangent function operating on an argument x in units of radians	(22)

## Order of operation precedence

When order of precedence in an expression is not indicated explicitly by use of parentheses, the following rules apply:

–	Operations of a higher precedence are evaluated before any operation of a lower precedence.

–	Operations of the same precedence are evaluated sequentially from left to right.

Table 1 specifies the precedence of operations from highest to lowest; a higher position in the table indicates a higher precedence.

NOTE – For those operators that are also used in the C programming language, the order of precedence used in this Specification is the same as used in the C programming language.

Table 1 – Operation precedence from highest (at top of table) to lowest (at bottom of table)

operations (with operands x, y, and z)

"x++", "x− −"

"!x", "−x" (as a unary prefix operator)

xy

"x * y", "x / y", "x ÷ y", "xy", "x % y"

"x + y", "x − y" (as a two-argument operator), ""

"x << y", "x >> y"

"x < y", "x <= y", "x > y", "x >= y"

"x = = y", "x != y"

"x & y"

"x ¦ y"

"x && y"

"x ¦ ¦ y"

"x ? y : z"

"x..y"

"x = y", "x += y", "x −= y"

## Variables, syntax elements and tables

Syntax elements in the syntax tables are represented in bold type. Each syntax element is described by its name (all lower case letters with underscore characters), and one descriptor for its method of coded representation. The decoding process behaves according to the value of the syntax element and to the values of previously decoded syntax elements. When a value of a syntax element is used in the syntax tables or the text, it appears in regular (i.e., not bold) type.

In some cases the syntax tables may use the values of other variables derived from syntax elements values. Such variables appear in the syntax tables, or text, named by a mixture of lower case and upper case letter and without any underscore characters. Variables starting with an upper case letter are derived for the decoding of the current syntax structure and all depending syntax structures. Variables starting with an upper case letter may be used in the decoding process for later syntax structures without mentioning the originating syntax structure of the variable. Variables starting with a lower case letter are only used within the clause in which they are derived.

In some cases, "mnemonic" names for syntax element values or variable values are used interchangeably with their numerical values. Sometimes "mnemonic" names are used without any associated numerical values. The association of values and names is specified in the text. The names are constructed from one or more groups of letters separated by an underscore character. Each group starts with an upper case letter and may contain more upper case letters.

NOTE – The syntax is described in a manner that closely follows the C-language syntactic constructs.

Functions that specify properties of the current position in the SEI message payload data are referred to as syntax functions. These functions are specified in clause 6.2 and assume the existence of a pointer with an indication of the position of the next bit to be read by the decoding process from the payload data. Syntax functions are described by their names, which are constructed as syntax element names and end with left and right round parentheses including zero or more variable names (for definition) or values (for usage), separated by commas (if more than one variable).

Functions that are not syntax functions (including mathematical functions specified in clause 5.8) are described by their names, which start with an upper case letter, contain a mixture of lower and upper case letters without any underscore character, and end with left and right parentheses including zero or more variable names (for definition) or values (for usage) separated by commas (if more than one variable).

A one-dimensional array is referred to as a list. A two-dimensional array is referred to as a matrix. Arrays can either be syntax elements or variables. Subscripts or square parentheses are used for the indexing of arrays. In reference to a visual depiction of a matrix, the first subscript is used as a row (vertical) index and the second subscript is used as a column (horizontal) index. The indexing order is reversed when using square parentheses rather than subscripts for indexing. Thus, an element of a matrix s at horizontal position x and vertical position y may be denoted either as s[ x ][ y ] or as syx. A single column of a matrix may be referred to as a list and denoted by omission of the row index. Thus, the column of a matrix s at horizontal position x may be referred to as the list s[ x ].

A specification of values of the entries in rows and columns of an array may be denoted by { {...} {...} }, where each inner pair of brackets specifies the values of the elements within a row in increasing column order and the rows are ordered in increasing row order. Thus, setting a matrix s equal to { { 1  6 } { 4 9 }} specifies that s[ 0 ][ 0 ] is set equal to 1, s[ 1 ][ 0 ] is set equal to 6, s[ 0 ][ 1 ] is set equal to 4, and s[ 1 ][ 1 ] is set equal to 9.

Binary notation is indicated by enclosing the string of bit values by single quote marks. For example, '01000001' represents an eight-bit string having only its second and its last bits (counted from the most to the least significant bit) equal to 1.

Hexadecimal notation, indicated by prefixing the hexadecimal number by "0x", may be used instead of binary notation when the number of bits is an integer multiple of 4. For example, 0x41 represents an eight-bit string having only its second and its last bits (counted from the most to the least significant bit) equal to 1.

Numerical values not enclosed in single quotes and not prefixed by "0x" are decimal values.

A value equal to 0 represents a FALSE condition in a test statement. The value TRUE is represented by any value different from zero.

## Text description of logical operations

In the text, a statement of logical operations as would be described mathematically in the following form:

if( condition 0 )
  statement 0
else if( condition 1 )
  statement 1
...
else /* informative remark on remaining condition */
  statement n

may be described in the following manner:

... as follows / ... the following applies:

–	If condition 0, statement 0

–	Otherwise, if condition 1, statement 1

–	...

–	Otherwise (informative remark on remaining condition), statement n

Each "If ... Otherwise, if ... Otherwise, ..." statement in the text is introduced with "... as follows" or "... the following applies" immediately followed by "If ... ". The last condition of the "If ... Otherwise, if ... Otherwise, ..." is always an "Otherwise, ...". Interleaved "If ... Otherwise, if ... Otherwise, ..." statements can be identified by matching "... as follows" or "... the following applies" with the ending "Otherwise, ...".

In the text, a statement of logical operations as would be described mathematically in the following form:

if( condition 0a  &&  condition 0b )
  statement 0
else if( condition 1a  | |  condition 1b )
  statement 1
...
else
  statement n

may be described in the following manner:

... as follows / ... the following applies:

–	If all of the following conditions are true, statement 0:

–	condition 0a

–	condition 0b

–	Otherwise, if one or more of the following conditions are true, statement 1:

–	condition 1a

–	condition 1b

–	...

–	Otherwise, statement n

In the text, a statement of logical operations as would be described mathematically in the following form:

if( condition 0 )
  statement 0
if( condition 1 )
  statement 1

may be described in the following manner:

When condition 0, statement 0

When condition 1, statement 1

## Processes

Processes are used to describe the decoding of syntax elements. A process has a separate specification and invoking. All syntax elements and upper case variables that pertain to the current syntax structure and depending syntax structures are available in the process specification and invoking. A process specification may also have a lower case variable explicitly specified as input. Each process specification has explicitly specified an output. The output is a variable that can either be an upper case variable or a lower case variable.

When invoking a process, the assignment of variables is specified as follows:

–	If the variables at the invoking and the process specification do not have the same name, the variables are explicitly assigned to lower case input or output variables of the process specification.

–	Otherwise (the variables at the invoking and the process specification have the same name), assignment is implied.

# Syntax and semantics

## Method of specifying syntax in tabular form

Technical specifications that reference this document shall specify some means to carry the payload syntax of each specified SEI message, to identify which SEI message is conveyed, and to identify the length in bits of the SEI message payload data. The length of the SEI message payload data is referred to herein by the variable name payloadSize.

For example, each SEI message may be carried as a string of data bits (SODB) that is prefixed with an SEI message payload type indication derived as a payloadType variable within a NAL unit that may contain emulation prevention bytes as specified in Rec. ITU-T H.VVC | ISO/IEC 23090-3.

The syntax tables in this document specify a superset of the syntax of all allowed SEI message payloads. Additional constraints on the syntax may be specified, either directly or indirectly, in other clauses.

The following table lists examples of the syntax specification format. When syntax_element appears, it specifies that a syntax element is parsed from the SEI message payload data and the data pointer is advanced to the next position beyond the syntax element in the syntax parsing process.

|  | Descriptor |
| --- | --- |
| /* A statement can be a syntax element with an associated descriptor or can be an expression used to specify conditions for the existence, type and quantity of syntax elements, as in the following two examples */ |  |
| syntax_element | ue(v) |
| conditioning statement |  |
| /* A group of statements enclosed in curly brackets is a compound statement and is treated functionally as a single statement. */ |  |
| { |  |
| statement |  |
| statement |  |
| ... |  |
| } |  |
| /* A "while" structure specifies a test of whether a condition is true, and if true, specifies evaluation of a statement (or compound statement) repeatedly until the condition is no longer true */ |  |
| while( condition ) |  |
| statement |  |
| /* A "do ... while" structure specifies evaluation of a statement once, followed by a test of whether a condition is true, and if true, specifies repeated evaluation of the statement until the condition is no longer true */ |  |
| do |  |
| statement |  |
| while( condition ) |  |
| /* An "if ... else" structure specifies a test of whether a condition is true and, if the condition is true, specifies evaluation of a primary statement, otherwise, specifies evaluation of an alternative statement. The "else" part of the structure and the associated alternative statement is omitted if no alternative statement evaluation is needed */ |  |
| if( condition ) |  |
| primary statement |  |
| else |  |
| alternative statement |  |
| /* A "for" structure specifies evaluation of an initial statement, followed by a test of a condition, and if the condition is true, specifies repeated evaluation of a primary statement followed by a subsequent statement until the condition is no longer true. */ |  |
| for( initial statement; condition; subsequent statement ) |  |
| primary statement |  |

## Specification of syntax functions and descriptors

The functions presented here are used in the syntactical description. These functions are expressed in terms of the value of an SEI message payload data pointer that indicates the position of the next bit to be read by the decoding process from the payload data.

byte_aligned( ) is specified as follows:

–	If the current position in the payload data is on a byte boundary, i.e., the next bit in the payload data is the first bit in a byte, the return value of byte_aligned( ) is equal to TRUE.

–	Otherwise, the return value of byte_aligned( ) is equal to FALSE.

more_data_in_payload( ) is specified as follows:

–	If byte_aligned( ) is equal to TRUE and the current position in the sei_payload( ) syntax structure is 8 * payloadSize bits from the beginning of the sei_payload( ) syntax structure, the return value of more_data_in_payload( ) is equal to FALSE.

–	Otherwise, the return value of more_data_in_payload( ) is equal to TRUE.

more_rbsp_data( ) is specified as follows:

–	If there is no more data in the raw byte sequence payload (RBSP), the return value of more_rbsp_data( ) is equal to FALSE.

–	Otherwise, the RBSP data are searched for the last (least significant, right-most) bit equal to 1 that is present in the RBSP. Given the position of this bit, which is the first bit (rbsp_stop_one_bit) of the rbsp_trailing_bits( ) syntax structure, the following applies:

–	If there is more data in an RBSP before the rbsp_trailing_bits( ) syntax structure, the return value of more_rbsp_data( ) is equal to TRUE.

–	Otherwise, the return value of more_rbsp_data( ) is equal to FALSE.

The method for enabling determination of whether there is more data in the RBSP is specified by the application.

more_rbsp_trailing_data( ) is specified as follows:

–	If there is more data in an RBSP, the return value of more_rbsp_trailing_data( ) is equal to TRUE.

–	Otherwise, the return value of more_rbsp_trailing_data( ) is equal to FALSE.

read_bits( n ) reads the next n bits from the payload data and advances the data pointer by n bit positions. When n is equal to 0, read_bits( n ) is specified to return a value equal to 0 and to not advance the data pointer.

The following descriptors specify the parsing process of each syntax element:

–	b(8): byte having any pattern of bit string (8 bits). The parsing process for this descriptor is specified by the return value of the function read_bits( 8 ).

–	f(n): fixed-pattern bit string using n bits written (from left to right) with the left bit first. The parsing process for this descriptor is specified by the return value of the function read_bits( n ).

–	i(n): signed integer using n bits. When n is "v" in the syntax table, the number of bits varies in a manner dependent on the value of other syntax elements. The parsing process for this descriptor is specified by the return value of the function read_bits( n ) interpreted as a two's complement integer representation with most significant bit written first.

–	st(v): null-terminated string encoded as universal coded character set (UCS) transmission format-8 (UTF-8) characters as specified in ISO/IEC 10646. The parsing process is specified as follows: st(v) begins at a byte-aligned position in the payload data and reads and returns a series of bytes from the payload data, beginning at the current position and continuing up to but not including the next byte-aligned byte that is equal to 0x00, and advances the data pointer by ( stringLength + 1 ) * 8 bit positions, where stringLength is equal to the number of bytes returned.

NOTE – The st(v) syntax descriptor is only used in this Specification when the current position in the payload data is a byte-aligned position.

–	u(n): unsigned integer using n bits. When n is "v" in the syntax table, the number of bits varies in a manner dependent on the value of other syntax elements. The parsing process for this descriptor is specified by the return value of the function read_bits( n ) interpreted as a binary representation of an unsigned integer with most significant bit written first.

[Ed. (GJS): ue(v) and se(v) are not defined, since they are not used in the currently specified SEI messages. If additional SEI messages are defined that need them, their specification would need to be added.]

# SEI message payload specifications

## General

This clause specifies the syntax and semantics for SEI message payloads.

[Ed. (GJS): It might be nice to reorganize the subclauses such that for each SEI message there is one subclause, and the syntax and semantics are two subordinate subclauses of that, instead of grouping together the syntax of all SEI messages. This could be more friendly to readers by avoiding the need to use split screen viewing or to jump back and forth in the document when studying a particular SEI message.]

## SEI payload syntax

### General

The syntax of the container of the SEI messages as well as the method of identifying which SEI message is outside the scope of this document. The syntax specified in this subclause is for the payload data of each associated SEI message. For SEI messages for which the specified syntax structure is empty, such as the dependent random access point SEI message, there is no payload data and the mere indication that the SEI message is present (e.g., as indicated by a payload type indicator with no payload data) is sufficient to convey the associated information (e.g., by indicating that a set of specified constraints are fulfilled).

### Decoded picture hash SEI message syntax

| decoded_picture_hash( payloadSize ) { | Descriptor |
| --- | --- |
| hash_type | u(8) |
| for( cIdx = 0; cIdx < ( chroma_format_idc = = 0 ? 1 : 3 ); cIdx++ ) |  |
| if( hash_type = = 0 ) |  |
| for( i = 0; i < 16; i++) |  |
| picture_md5[ cIdx ][ i ] | b(8) |
| else if( hash_type = = 1 ) |  |
| picture_crc[ cIdx ] | u(16) |
| else if( hash_type = = 2 ) |  |
| picture_checksum[ cIdx ] | u(32) |
| } |  |

### Dependent random access point indication SEI message syntax

| dependent_rap_indication( payloadSize ) { | Descriptor |
| --- | --- |
| } |  |

### Syntax of omnidirectional video specific SEI messages

#### Equirectangular projection SEI message syntax

| equirectangular_projection( payloadSize ) { | Descriptor |
| --- | --- |
| erp_cancel_flag | u(1) |
| if( !erp_cancel_flag ) { |  |
| erp_persistence_flag | u(1) |
| erp_guard_band_flag | u(1) |
| erp_reserved_zero_2bits | u(2) |
| if( erp_guard_band_flag = = 1 ) { |  |
| erp_guard_band_type | u(3) |
| erp_left_guard_band_width | u(8) |
| erp_right_guard_band_width | u(8) |
| } |  |
| } |  |
| } |  |

#### Cubemap projection SEI message syntax

| cubemap_projection( payloadSize ) { | Descriptor |
| --- | --- |
| cmp_cancel_flag | u(1) |
| if( !cmp_cancel_flag ) |  |
| cmp_persistence_flag | u(1) |
| } |  |

#### Sphere rotation SEI message syntax

| sphere_rotation( payloadSize ) { | Descriptor |
| --- | --- |
| sphere_rotation_cancel_flag | u(1) |
| if( !sphere_rotation_cancel_flag ) { |  |
| sphere_rotation_persistence_flag | u(1) |
| sphere_rotation_reserved_zero_6bits | u(6) |
| yaw_rotation | i(32) |
| pitch_rotation | i(32) |
| roll_rotation | i(32) |
| } |  |
| } |  |

#### Region-wise packing SEI message syntax

| regionwise_packing( payloadSize ) { | Descriptor |
| --- | --- |
| rwp_cancel_flag | u(1) |
| if( !rwp_cancel_flag ) { |  |
| rwp_persistence_flag | u(1) |
| constituent_picture_matching_flag | u(1) |
| rwp_reserved_zero_5bits | u(5) |
| num_packed_regions | u(8) |
| proj_picture_width | u(32) |
| proj_picture_height | u(32) |
| packed_picture_width | u(16) |
| packed_picture_height | u(16) |
| for( i = 0; i < num_packed_regions; i++ ) { |  |
| rwp_reserved_zero_4bits[ i ] | u(4) |
| rwp_transform_type[ i ] | u(3) |
| rwp_guard_band_flag[ i ] | u(1) |
| proj_region_width[ i ] | u(32) |
| proj_region_height[ i ] | u(32) |
| proj_region_top[ i ] | u(32) |
| proj_region_left[ i ] | u(32) |
| packed_region_width[ i ] | u(16) |
| packed_region_height[ i ] | u(16) |
| packed_region_top[ i ] | u(16) |
| packed_region_left[ i ] | u(16) |
| if( rwp_guard_band_flag[ i ] ) { |  |
| rwp_left_guard_band_width[ i ] | u(8) |
| rwp_right_guard_band_width[ i ] | u(8) |
| rwp_top_guard_band_height[ i ] | u(8) |
| rwp_bottom_guard_band_height[ i ] | u(8) |
| rwp_guard_band_not_used_for_pred_flag[ i ] | u(1) |
| for( j = 0; j < 4; j++ ) |  |
| rwp_guard_band_type[ i ][ j ] | u(3) |
| rwp_guard_band_reserved_zero_3bits[ i ] | u(3) |
| } |  |
| } |  |
| } |  |
| } |  |

#### Omnidirectional viewport SEI message syntax

| omni_viewport( payloadSize ) { | Descriptor |
| --- | --- |
| omni_viewport_id | u(10) |
| omni_viewport_cancel_flag | u(1) |
| if( !omni_viewport_cancel_flag ) { |  |
| omni_viewport_persistence_flag | u(1) |
| omni_viewport_cnt_minus1 | u(4) |
| for( i = 0; i <= omni_viewport_cnt_minus1; i++ ) { |  |
| omni_viewport_azimuth_centre[ i ] | i(32) |
| omni_viewport_elevation_centre[ i ] | i(32) |
| omni_viewport_tilt_centre[ i ] | i(32) |
| omni_viewport_hor_range[ i ] | u(32) |
| omni_viewport_ver_range[ i ] | u(32) |
| } |  |
| } |  |
| } |  |

### Reserved SEI message syntax

| reserved_sei_message( payloadSize ) { | Descriptor |
| --- | --- |
| for( i = 0; i < payloadSize; i++ ) |  |
| reserved_sei_message_payload_byte | u(8) |
| } |  |

## SEI payload semantics

### General

The semantics and persistence scope for each SEI message are specified in the semantics specification for each particular SEI message.

NOTE – Persistence information for SEI messages is informatively summarized in Table 2 –

Table 2 –  Persistence scope of SEI messages (informative)

| SEI message | Persistence scope |
| --- | --- |
| Decoded picture hash | The access unit containing the SEI message |
| DRAP indication | The access unit containing the SEI message |
| Equirectangular projection | Specified by the syntax of the SEI message |
| Cubemap projection | Specified by the syntax of the SEI message |
| Sphere rotation | Specified by the syntax of the SEI message |
| Region-wise packing | Specified by the syntax of the SEI message |
| Omnidirectional viewport | Specified by the syntax of the SEI message |

### Decoded picture hash SEI message semantics

This message provides a hash for each colour component of the current decoded picture.

Use of this SEI message requires the definition of the following parameters:

–	A picture width and picture height in units of luma samples, denoted herein by pic_width_in_luma_samples and pic_height_in_luma_samples.

–	A chroma format indicator, denoted herein by chroma_format_idc, such that the value 0 indicates that the picture has only a luma component and other values indicate that the picture has three colour components that consist of a luma component and two associated chroma components, such that the width and height of each chroma component are pic_width_in_luma_samples / SubWidthC and pic_height_in_luma_samples / SubHeightC, respectively, where SubWidthC and SubHeightC are determined from chroma_format_idc as specified by Table 3.

–	A bit depth for the samples of the luma component, denoted herein as BitDepthY, and when chroma_format_idc is not equal to 0, a bit depth for the samples of the two associated chroma components, denoted herein as BitDepthC.

–	For each colour component cIdx, an array component[ cIdx ][ i ] in raster scan order of decoded sample values in two's complement representation

Table 3 – SubWidthC and SubHeightC values derived from
chroma_format_idc

| chroma_format_idc | Chroma format | SubWidthC | SubHeightC |
| --- | --- | --- | --- |
| 0 | Monochrome | 1 | 1 |
| 1 | 4:2:0 | 2 | 2 |
| 2 | 4:2:2 | 2 | 1 |
| 3 | 4:4:4 | 1 | 1 |

Prior to computing the hash, the decoded picture data are arranged into one or three strings of bytes called pictureData[ cIdx ] of lengths dataLen[ cIdx ] as follows:

for( cIdx = 0; cIdx < ( chroma_format_idc  = =  0 ) ? 1 : 3; cIdx++ ) {
	if( cIdx  = =  0 ) {
		compWidth[ cIdx ] = pic_width_in_luma_samples
		compHeight[ cIdx ] = pic_height_in_luma_samples
		compDepth[ cIdx ] = BitDepthY
	} else {
		compWidth[ cIdx ] = pic_width_in_luma_samples / SubWidthC
		compHeight[ cIdx ] = pic_height_in_luma_samples / SubHeightC
		compDepth[ cIdx ] = BitDepthC		(23)
	}
	iLen = 0
	for( i = 0; i < compWidth[ cIdx ] * compHeight[ cIdx ]; i++ ) {
		pictureData[ cIdx ][ iLen++ ] = component[ cIdx ][ i ] & 0xFF
		if( compDepth[ cIdx ] > 8 )
			pictureData[ cIdx ][ iLen++ ] = component[ cIdx ][ i ]  >>  8
	}
	dataLen[ cIdx ] = iLen
}

where component[ cIdx ][ i ] is an array in raster scan of decoded sample values in two's complement representation.

hash_type indicates the method used to calculate the checksum as specified in Table 7-4. Values of hash_type that are not listed in in Table 7-4 are reserved for future use by ITU-T | ISO/IEC and shall not be present in payload data conforming to this version of this Specification. Decoders shall ignore decoded picture hash SEI messages that contain reserved values of hash_type.

Table 7-4 –  Interpretation of hash_type

| hash_type | Method |
| --- | --- |
| 0 | MD5 (IETF RFC 1321) |
| 1 | CRC |
| 2 | Checksum |

picture_md5[ cIdx ][ i ] is the 16-byte MD5 hash of the cIdx-th colour component of the decoded picture. The value of picture_md5[ cIdx ][ i ] shall be equal to the value of digestVal[ cIdx ] obtained as follows, using the MD5 functions defined in IETF RFC 1321:

MD5Init( context )
MD5Update( context, pictureData[ cIdx ], dataLen[ cIdx ] )	(24)
MD5Final( digestVal[ cIdx ], context )

picture_crc[ cIdx ] is the cyclic redundancy check (CRC) of the colour component cIdx of the decoded picture. The value of picture_crc[ cIdx ] shall be equal to the value of crcVal[ cIdx ] obtained as follows:

crc = 0xFFFF
pictureData[ cIdx ][  dataLen[ cIdx ] ] = 0
pictureData[ cIdx ][  dataLen[ cIdx ] + 1 ] = 0
for( bitIdx = 0; bitIdx < ( dataLen[ cIdx ]  + 2 ) * 8; bitIdx++ ) {	(25)
	dataByte = pictureData[ cIdx ][ bitIdx  >>  3 ]
	crcMsb = ( crc  >>  15 ) & 1
	bitVal = ( dataByte  >>  ( 7 − ( bitIdx & 7 ) ) ) & 1
	crc = ( ( ( crc  <<  1 ) + bitVal ) & 0xFFFF ) ^ ( crcMsb * 0x1021 )
}
crcVal[ cIdx ] = crc

NOTE – The same CRC specification is found in Rec. ITU-T H.271.

picture_checksum[ cIdx ] is the checksum of the colour component cIdx of the decoded picture. The value of picture_checksum[ cIdx ] shall be equal to the value of checksumVal[ cIdx ] obtained as follows:

sum = 0
for( y = 0; y < compHeight[ cIdx ]; y++ )
	for( x = 0; x < compWidth[ cIdx ]; x++ ) {
		xorMask = ( x & 0xFF ) ^ ( y & 0xFF ) ^ ( x  >>  8 ) ^ ( y  >>  8 )
		sum = ( sum + ( ( component[ cIdx ][ y * compWidth[ cIdx ] + x ] & 0xFF ) ^	(26)
				xorMask ) ) & 0xFFFFFFFF
		if( compDepth[ cIdx ] > 8 )
			sum = ( sum + ( ( component[ cIdx ][ y * compWidth[ cIdx ] + x ]  >>  8 ) ^
					xorMask ) ) & 0xFFFFFFFF
	}
checksumVal[ cIdx ] = sum

### Dependent random access point indication SEI message semantics

The picture associated with a dependent random access point (DRAP) indication SEI message is referred to as a DRAP picture.

The presence of the DRAP indication SEI message indicates that the constraints on picture order and picture referencing specified in this subclause apply. These constraints can enable a decoder to properly decode the DRAP picture and the pictures that follow it in both decoding order and output order without needing to decode any other pictures except the associated IRAP picture.

The constraints indicated by the presence of the DRAP indication SEI message are as follows:

–	The DRAP picture shall be a trailing picture.

–	The DRAP picture shall have a temporal sublayer identifier equal to 0.

–	The DRAP picture shall not include any pictures in the active entries of its reference picture lists except the preceding IRAP picture in decoding order.

–	Any picture that follows the DRAP picture in both decoding order and output order shall not include, in the active entries of its reference picture lists, any picture that precedes the DRAP picture in decoding order or output order, with the exception of the preceding IRAP picture in decoding order.

### Semantics of omnidirectional video specific SEI messages

#### Equirectangular projection SEI message semantics

The equirectangular projection SEI message provides information to enable remapping (through an equirectangular projection) of the colour samples of the projected pictures onto a sphere coordinate space in sphere coordinates (ϕ, θ) for use in omnidirectional video applications for which the viewing perspective is from the origin looking outward toward the inside of the sphere. The sphere coordinates are defined so that ϕ is the azimuth (longitude, increasing eastward) and θ is the elevation (latitude, increasing northward).

When an equirectangular projection SEI message is present for any picture of a CLVS of a particular layer, an equirectangular projection SEI message shall be present for the first picture of the CLVS and no SEI message indicating a different type of projection shall be present for any picture of the CLVS.

When general_non_packed_constraint_flag is equal to 1 in the active SPS for the current layer, there shall be no equirectangular projection SEI messages applicable for any picture of the CLVS of the current layer.

When aspect_ratio_idc is present and greater than 1 in the active SPS for the current layer, there should be no equirectangular projection SEI messages applicable for any picture of the CLVS of the current layer.

A frame packing arrangement SEI message for which all the following conditions are true is referred to as an effectively applicable frame packing arrangement SEI message:

–	The value of frame_packing_arrangement_cancel_flag is equal to 0.

–	The value of frame_packing_arrangement_type is equal to 3, 4, or 5.

–	The value of quincunx_sampling_flag is equal to 0.

–	The value of spatial_flipping_flag is equal to 0.

–	The value of field_views_flag is equal to 0.

–	The value of frame0_grid_position_x is equal to 0.

–	The value of frame0_grid_position_y is equal to 0.

–	The value of frame1_grid_position_x is equal to 0.

–	The value of frame1_grid_position_y is equal to 0.

[Ed. (RS): VVC currently does not include the above mentioned frame packing signalling. Also affects furhter projection SEI messages.]

When a frame packing arrangement SEI message with frame_packing_arrangement_cancel_flag equal to 0 that applies to the picture is present that is not an effectively applicable frame packing arrangement SEI message, an equirectangular projection SEI message with erp_cancel_flag equal to 0 that applies to the picture shall not be present. Decoders shall ignore equirectangular projection SEI messages when a frame packing arrangement SEI message with frame_packing_arrangement_cancel_flag equal to 0 that applies to the picture is present that is not an effectively applicable frame packing arrangement SEI message.

When a segmented rectangular frame packing arrangement SEI message with segmented_rect_frame_packing_arrangement_cancel_flag equal to 0 is present that applies to the picture, an equirectangular projection SEI message with erp_cancel_flag equal to 0 shall not be present that applies to the picture. Decoders shall ignore equirectangular projection SEI messages when a segmented rectangular frame packing arrangement SEI message with segmented_rect_frame_packing_arrangement_cancel_flag equal to 0 is present that applies to the picture.

[Ed. (RS): VVC currently does not include the above mentioned segmented rectangular frame packing arrangement signalling. Also affects furhter projection SEI messages]

erp_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous equirectangular projection SEI message in output order. erp_cancel_flag equal to 0 indicates that equirectangular projection information follows.

erp_persistence_flag specifies the persistence of the equirectangular projection SEI message for the current layer.

erp_persistence_flag equal to 0 specifies that the equirectangular projection SEI message applies to the current decoded picture only.

Let picA be the current picture. erp_persistence_flag equal to 1 specifies that the equirectangular projection SEI message persists for the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture picB in the current layer in an access unit containing an equirectangular projection SEI message that is applicable to the current layer is output for which PicOrderCnt( picB ) is greater than PicOrderCnt( picA ), where PicOrderCnt( picB ) and PicOrderCnt( picA ) are the PicOrderCntVal values of picB and picA, respectively, immediately after the invocation of the decoding process for picture order count for picB.

erp_guard_band_flag equal to 1 indicates that the constituent picture contains guard band areas for which the sizes are specified by the syntax elements erp_left_guard_band_width and erp_right_guard_band_width. erp_guard_band_flag equal to 0 indicates that the constituent picture does not contains guard band areas for which the sizes are specified by the syntax elements erp_left_guard_band_width and erp_right_guard_band_width.

erp_reserved_zero_2bits shall be equal to 0 in bitstreams conforming to this version of this Specification. Other values for erp_reserved_zero_2bits are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore the value of erp_reserved_zero_2bits.

erp_guard_band_type indicates the type of the guard bands as follows:

–	erp_guard_band_type equal to 0 indicates that the content of the guard band in relation to the content of the constituent picture is unspecified.

–	erp_guard_band_type equal to 1 indicates that the content of the guard band suffices for interpolation of sample values at sub-pel sample fractional locations within the constituent picture.

NOTE – erp_guard_band_type equal to 1 could be used when the boundary samples of a constituent picture have been copied horizontally to the guard band.

–	erp_guard_band_type equal to 2 indicates that the content of the guard band represents actual picture content at a quality that gradually changes from the picture quality of the constituent picture to that of the spherically adjacent region.

–	erp_guard_band_type equal to 3 indicates that the content of the guard bands represents actual picture content at a similar level of quality as the constituent picture.

–	erp_guard_band_type values greater than 3 are reserved for future use by ITU-T | ISO/IEC. Decoders shall treat the value of erp_guard_band_type when the value is greater than 3 as equivalent to the value 0.

erp_left_guard_band_width specifies the width of the guard band on the left side of the constituent picture in units of luma samples. When erp_guard_band_flag is equal to 0, the value of erp_left_guard_band_width is inferred to be equal to 0. When chroma_format_idc is equal to 1 (4:2:0 chroma format) or 2 (4:2:2 chroma format), erp_left_guard_band_width shall be an even number.

erp_right_guard_band_width specifies the width of the guard band on the right side of the constituent picture in units of luma samples. When erp_guard_band_flag is equal to 0, the value of erp_right_guard_band_width is inferred to be equal to 0. When chroma_format_idc is equal to 1 (4:2:0 chroma format) or 2 (4:2:2 chroma format), erp_right_‌guard_band_width shall be an even number.

#### Cubemap projection SEI message semantics

The cubemap projection SEI message provides information to enable remapping (through a cubemap projection) of the colour samples of the projected pictures onto a sphere coordinate space in sphere coordinates (ϕ, θ) for use in omnidirectional video applications for which the viewing perspective is from the origin looking outward toward the inside of the sphere. The sphere coordinates are defined so that ϕ is the azimuth (longitude, increasing eastward) and θ is the elevation (latitude, increasing northward).

When a cubemap projection SEI message is present for any picture of a CLVS of a particular layer, a cubemap projection SEI message shall be present for the first picture of the CLVS and no SEI message indicating a different type of projection shall be present for any picture of the CLVS.

When general_non_packed_constraint_flag is equal to 1 in the active SPS for the current layer, there shall be no cubemap projection SEI messages applicable for any picture of the CLVS of the current layer.

When aspect_ratio_idc is present and greater than 1 in the active SPS for the current layer, there should be no cubemap projection SEI messages applicable for any picture of the CLVS of the current layer.

A frame packing arrangement SEI message for which all the following conditions are true is referred to as an effectively applicable frame packing arrangement SEI message:

–	The value of frame_packing_arrangement_cancel_flag is equal to 0.

–	The value of frame_packing_arrangement_type is equal to 3, 4, or 5.

–	The value of quincunx_sampling_flag is equal to 0.

–	The value of spatial_flipping_flag is equal to 0.

–	The value of field_views_flag is equal to 0.

–	The value of frame0_grid_position_x is equal to 0.

–	The value of frame0_grid_position_y is equal to 0.

–	The value of frame1_grid_position_x is equal to 0.

–	The value of frame1_grid_position_y is equal to 0.

When a frame packing arrangement SEI message with frame_packing_arrangement_cancel_flag equal to 0 that applies to the picture is present that is not  an effectively applicable frame packing arrangement SEI message, a cubemap projection SEI message with cmp_cancel_flag equal to 0 that applies to the picture shall not be present. Decoders shall ignore cubemap projection SEI messages when a frame packing arrangement SEI message with frame_packing_arrangement_cancel_flag equal to 0 that applies to the picture is present that is not an effectively applicable frame packing arrangement SEI message.

When a segmented rectangular frame packing arrangement SEI message with segmented_rect_frame_packing_arrangement_cancel_flag equal to 0 is present that applies to the picture, a cubemap projection SEI message with cmp_cancel_flag equal to 0 shall not be present that applies to the picture. Decoders shall ignore cubemap projection SEI messages when a segmented rectangular frame packing arrangement SEI message with segmented_rect_frame_packing_arrangement_cancel_flag equal to 0 is present that applies to the picture.

cmp_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous cubemap projection SEI message in output order. cmp_cancel_flag equal to 0 indicates that cubemap projection information follows.

cmp_persistence_flag specifies the persistence of the cubemap projection SEI message for the current layer.

cmp_persistence_flag equal to 0 specifies that the cubemap projection SEI message applies to the current decoded picture only.

Let picA be the current picture. cmp_persistence_flag equal to 1 specifies that the cubemap projection SEI message persists for the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture picB in the current layer in an access unit containing a cubemap projection SEI message that is applicable to the current layer is output for which PicOrderCnt( picB ) is greater than PicOrderCnt( picA ), where PicOrderCnt( picB ) and PicOrderCnt( picA ) are the PicOrderCntVal values of picB and picA, respectively, immediately after the invocation of the decoding process for picture order count for picB.

#### Sphere rotation SEI message semantics

The sphere rotation SEI message provides information on rotation angles yaw (α), pitch (β), and roll (γ) that are used for conversion between the global coordinate axes and the local coordinate axes.

Relative to an (x, y, z) Cartesian coordinate system, yaw expresses a rotation around the z (vertical, up) axis, pitch rotates around the y (lateral, side-to-side) axis, and roll rotates around the x (back-to-front) axis. Rotations are extrinsic, i.e., around x, y, and z fixed reference axes. The angles increase clockwise when looking from the origin towards the positive end of an axis.

sphere_rotation_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous sphere rotation SEI message in output order. sphere_rotation_cancel_flag equal to 0 indicates that sphere rotation information follows.

sphere_rotation_persistence_flag specifies the persistence of the sphere rotation SEI message for the current layer.

sphere_rotation_persistence_flag equal to 0 specifies that the sphere rotation SEI message applies to the current decoded picture only.

Let picA be the current picture. sphere_rotation_persistence_flag equal to 1 specifies that the sphere rotation SEI message persists for the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture picB in the current layer in an access unit containing a sphere rotation SEI message that is applicable to the current layer is output for which PicOrderCnt( picB ) is greater than PicOrderCnt( picA ), where PicOrderCnt( picB ) and PicOrderCnt( picA ) are the PicOrderCntVal values of picB and picA, respectively, immediately after the invocation of the decoding process for picture order count for picB.

When an equirectangular projection SEI message with erp_cancel_flag equal to 0 or a cubemap projection SEI message with cmp_cancel_flag equal to 0 is not present in the CLVS that applies to the current picture and precedes the sphere rotation SEI message in decoding order, a sphere rotation SEI message with sphere_rotation_cancel_flag equal to 0 shall not be present in the CLVS that applies to the current picture. Decoders shall ignore sphere rotation SEI messages with sphere_rotation_cancel_flag equal to 0 that do not follow, in decoding order, an equirectangular projection SEI message with erp_cancel_flag equal to 0 or a cubemap projection SEI message with cmp_cancel_flag equal to 0 in the CLVS that applies to the current picture.

sphere_rotation_reserved_zero_6bits shall be equal to 0 in bitstreams conforming to this version of this Specification. Other values for sphere_rotation_reserved_zero_6bits are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore the value of sphere_rotation_reserved_zero_6bits.

yaw_rotation specifies the value of the yaw rotation angle, in units of 2−16 degrees. The value of yaw_rotation shall be in the range of −180 * 216 (i.e., −11 796 480) to 180 * 216 − 1 (i.e., 11 796 479), inclusive. When not present, the value of yaw_rotation is inferred to be equal to 0.

pitch_rotation specifies the value of the pitch rotation angle, in units of 2−16 degrees. The value of pitch_rotation shall be in the range of −90 * 216 (i.e., −5 898 240) to 90 * 216 (i.e., 5 898 240), inclusive. When not present, the value of pitch_rotation is inferred to be equal to 0.

roll_rotation specifies the value of the roll rotation angle, in units of 2−16 degrees. The value of roll_rotation shall be in the range of −180 * 216 (i.e., −11 796 480) to 180 * 216 − 1 (i.e., 11 796 479), inclusive. When not present, the value of roll_rotation is inferred to be equal to 0.

#### Region-wise packing SEI message semantics

The region-wise packing SEI message provides information to enable remapping of the colour samples of the cropped decoded pictures onto projected pictures as well as information on the location and size of the guard bands, if any.

rwp_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous region-wise packing SEI message in output order. rwp_cancel_flag equal to 0 indicates that region-wise packing information follows.

rwp_persistence_flag specifies the persistence of the region-wise packing SEI message for the current layer.

rwp_persistence_flag equal to 0 specifies that the region-wise packing SEI message applies to the current decoded picture only.

Let picA be the current picture. rwp_persistence_flag equal to 1 specifies that the region-wise packing SEI message persists for the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture picB in the current layer in an access unit containing a region-wise packing SEI message that is applicable to the current layer is output for which PicOrderCnt( picB ) is greater than PicOrderCnt( picA ), where PicOrderCnt( picB ) and PicOrderCnt( picA ) are the PicOrderCntVal values of picB and picA, respectively, immediately after the invocation of the decoding process for picture order count for picB.

When an equirectangular projection SEI message with erp_cancel_flag equal to 0 and erp_guard_band_flag equal to 0 or a cubemap projection SEI message with cmp_cancel_flag equal to 0 is not present in the CLVS that applies to the current picture and precedes the region-wise packing SEI message in decoding order, a region-wise packing SEI message with rwp_cancel_flag equal to 0 shall not be present in the CLVS that applies to the current picture. Decoders shall ignore region-wise packing SEI messages with rwp_cancel_flag equal to 0 that do not follow, in decoding order, an equirectangular projection SEI message with erp_cancel_flag equal to 0 or a cubemap projection SEI message with cmp_cancel_flag equal to 0 in the CLVS that applies to the current picture.

For the frame packing arrangement scheme indicated by a frame packing arrangement SEI message that applies to the current picture, if a region-wise packing SEI message with rwp_cancel_flag equal to 0 is present that applies to the current picture, the frame packing arrangement scheme applies to the projected picture, otherwise, the frame packing arrangement scheme applies to the cropped decoded picture.

If a frame packing arrangement SEI message with frame_packing_arrangement_cancel_flag equal to 0, frame_packing_arrangement_type equal to 3, 4, or 5, and quincunx_sampling_flag equal to 0 is not present that applies to the current picture, the variables StereoFlag, TopBottomFlag, SideBySideFlag, and TempInterleavingFlag are all set equal to 0, the variables HorDiv1 and VerDiv1 are both set equal to 1. Otherwise the following applies:

–	StereoFlag is set equal to 1.

–	When the frame_packing_arrangement_type is equal to 3, SideBySideFlag is set equal to 1, TopBottomFlag and TempInterleavingFlag are both set equal to 0, HorDiv1 is set equal to 2 and VerDiv1 is set equal to 1.

–	When the frame_packing_arrangement_type is equal to 4, TopBottomFlag is set equal to 1, SideBySideFlag and TempInterleavingFlag are both set equal to 0, HorDiv1 is set equal to 1 and VerDiv1 is set equal to 2.

–	When the frame_packing_arrangement_type is equal to 5, TempInterleavingFlag is set equal to 1, TopBottomFlag and SideBySideFlag are both set equal to 0, HorDiv1 and VerDiv1 are both set equal to 1.

constituent_picture_matching_flag equal to 1 specifies that the projected region information, packed region information, and guard band region information in this SEI message apply individually to each constituent picture and that the packed picture and the projected picture have the same stereoscopic frame packing format indicated by the frame packing arrangement SEI message. constituent_picture_matching_flag equal to 0 specifies that the projected region information, packed region information, and guard band region information in this SEI message apply to the projected picture.

When either of the following conditions is true, the value of constituent_picture_matching_flag shall be equal to 0:

–	StereoFlag is equal to 0.

–	StereoFlag is equal to 1 and frame_packing_arrangement_type is equal to 5.

rwp_reserved_zero_5bits shall be equal to 0 in bitstreams conforming to this version of this Specification. Other values for rwp_reserved_zero_56bits[ i ] are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore the value of rwp_reserved_zero_5bits[ i ].

num_packed_regions specifies the number of packed regions when constituent_picture_matching_flag is equal to 0. The value of num_packed_regions shall be greater than 0. When constituent_picture_matching_flag is equal to 1, the total number of packed regions is equal to num_packed_regions * 2, and the information in each entry of the loop of num_packed_regions entries applies to each constituent picture of the projected picture and the packed picture.

proj_picture_width and proj_picture_height specify the width and height, respectively, of the projected picture, in relative projected picture sample units.

The values of proj_picture_width and proj_picture_height shall both be greater than 0.

packed_picture_width and packed_picture_height specify the width and height, respectively, of the packed picture, in relative packed picture sample units.

The values of packed_picture_width and packed_picture_height shall both be greater than 0.

It is a requirement of bitstream conformance that packed_picture_width and packed_picture_height shall have such values that packed_picture_width is an integer multiple of cropPicWidth and packed_picture_height is an integer multiple of cropPicHeight, where cropPicWidth and cropPicHeight are the width and height, respectively, of the cropped decoded picture.

rwp_reserved_zero_4bits[ i ] shall be equal to 0 in bitstreams conforming to this version of this Specification. Other values for rwp_reserved_zero_4bits[ i ] are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore the value of rwp_reserved_zero_4bits[ i ].

rwp_transform_type[ i ] specifies the rotation and mirroring to be applied to the i-th packed region to remap to the i-th projected region. When rwp_transform_type[ i ] specifies both rotation and mirroring, rotation applies before mirroring. The values of rwp_transform_type[ i ] are specified in Table D.22.

Table D.22 – rwp_transform_type[ i ] values

| Value | Description |
| --- | --- |
| 0 | no transform |
| 1 | mirroring horizontally |
| 2 | rotation by 180 degrees (anticlockwise) |
| 3 | rotation by 180 degrees (anticlockwise) before mirroring horizontally |
| 4 | rotation by 90 degrees (anticlockwise) before mirroring horizontally |
| 5 | rotation by 90 degrees (anticlockwise) |
| 6 | rotation by 270 degrees (anticlockwise) before mirroring horizontally |
| 7 | rotation by 270 degrees (anticlockwise) |

rwp_guard_band_flag[ i ] equal to 0 specifies that the i-th packed region does not have a guard band. rwp_guard_band_flag[ i ] equal to 1 specifies that the i-th packed region has a guard band.

proj_region_width[ i ], proj_region_height[ i ], proj_region_top[ i ] and proj_region_left[ i ] specify the width, height, top sample row, and the left-most sample column, respectively, of the i-th projected region, either within the projected picture (when constituent_picture_matching_flag is equal to 0) or within the constituent picture of the projected picture (when constituent_picture_matching_flag is equal to 1).

proj_region_width[ i ], proj_region_height[ i ], proj_region_top[ i ], and proj_region_left[ i ] are indicated in relative projected picture sample units.

NOTE 1 – Two projected regions may partially or entirely overlap with each other.

packed_region_width[ i ], packed_region_height[ i ], packed_region_top[ i ], and packed_region_left[ i ] specify the width, height, the top luma sample row, and the left-most luma sample column, respectively, of the packed region, either within the region-wise packed picture (when constituent_picture_matching_flag is equal to 0) or within each constituent picture of the region-wise packed picture (when constituent_picture_matching_flag is equal to 1).

packed_region_width[ i ], packed_region_height[ i ], packed_region_top[ i ], and packed_region_left[ i ] are indicated in relative region-wise packed picture sample units. packed_region_width[ i ], packed_region_height[ i ], packed_region_top[ i ], and packed_region_left[ i ] shall represent integer horizontal and vertical coordinates of luma sample units within the cropped decoded pictures.

NOTE 2 – Two packed regions may partially or entirely overlap with each other.

rwp_left_guard_band_width[ i ] specifies the width of the guard band on the left side of the i-th packed region in relative region-wise packed picture sample units. When chroma_format_idc is equal to 1 (4:2:0 chroma format) or 2 (4:2:2 chroma format), rwp_left_guard_band_width[ i ] shall correspond to an even number of luma samples within the cropped decoded picture.

rwp_right_guard_band_width[ i ] specifies the width of the guard band on the right side of the i-th packed region in relative region-wise packed picture sample units. When chroma_format_idc is equal to 1 (4:2:0 chroma format) or 2 (4:2:2 chroma format), rwp_right_guard_band_width[ i ] shall correspond to an even number of luma samples within the cropped decoded picture.

rwp_top_guard_band_height[ i ] specifies the height of the guard band above the i-th packed region in relative region-wise packed picture sample units. When chroma_format_idc is equal to 1 (4:2:0 chroma format), rwp_top_guard_band_height[ i ] shall correspond to an even number of luma samples within the cropped decoded picture.

rwp_bottom_guard_band_height[ i ] specifies the height of the guard band below the i-th packed region in relative region-wise packed picture sample units. When chroma_format_idc is equal to 1 (4:2:0 chroma format), rwp_bottom_guard_band_height[ i ] shall correspond to an even number of luma samples within the cropped decoded picture.

When rwp_guard_band_flag[ i ] is equal to 1, rwp_left_guard_band_width[ i ], rwp_right_guard_band_width[ i ], rwp_‌top_‌guard_band_height[ i ], or rwp_bottom_guard_band_height[ i ] shall be greater than 0.

The i-th packed region as specified by this SEI message shall not overlap with any other packed region specified by the same SEI message or any guard band specified by the same SEI message.

The guard bands associated with the i-th packed region, if any, as specified by this SEI message shall not overlap with any packed region specified by the same SEI message or any other guard bands specified by the same SEI message.

rwp_guard_band_not_used_for_pred_flag[ i ] equal to 0 specifies that the guard bands may or may not be used in the inter prediction process. rwp_guard_band_not_used_for_pred_flag[ i ] equal to 1 specifies that the sample values of the guard bands are not used in the inter prediction process.

NOTE 3 – When rwp_guard_band_not_used_for_pred_flag[ i ] is equal to 1, the sample values within guard bands in cropped decoded pictures can be rewritten even if the cropped decoded pictures were used as references for inter prediction of subsequent pictures to be decoded. For example, the content of a packed region can be seamlessly expanded to its guard band with decoded and re-projected samples of another packed region.

rwp_guard_band_type[ i ][ j ] indicates the type of the guard bands for the i-th packed region as follows, with j equal to 0, 1, 2, or 3 indicating that the semantics below apply to the left, right, top, or bottom edge, respectively, of the packed region:

–	rwp_guard_band_type[ i ][ j ] equal to 0 indicates that the content of the guard bands in relation to the content of the packed regions is unspecified. When rwp_guard_band_not_used_for_pred_flag[ i ] is equal to 0, rwp_guard_band_type[ i ][ j ] shall not be equal to 0.

–	rwp_guard_band_type[ i ][ j ] equal to 1 indicates that the content of the guard bands suffices for interpolation of sample values at sub-pel sample fractional locations within the packed region and less than one sample outside of the boundary of the packed region.

NOTE 4 – rwp_guard_band_type[ i ][ j ] equal to 1 can be used when the boundary samples of a packed region have been copied horizontally or vertically to the guard band.

–	rwp_guard_band_type[ i ][ j ] equal to 2 indicates that the content of the guard bands represents actual picture content that is spherically adjacent to the content in the packed region and is on the surface of the packed region at quality that gradually changes from the picture quality of the packed region to that of the spherically adjacent packed region.

–	rwp_guard_band_type[ i ][ j ] equal to 3 indicates that the content of the guard bands represents actual picture content that is spherically adjacent to the content in the packed region and is on the surface of the packed region at a similar picture quality as within the packed region.

–	rwp_guard_band_type[ i ][ j ] values greater than 3 are reserved for future use by ITU-T | ISO/IEC. Decoders shall treat the value of rwp_guard_band_type[ i ][ j ] when the value is greater than 3 as equivalent to the value 0.

rwp_guard_band_reserved_zero_3bits[ i ] shall be equal to 0 in bitstreams conforming to this version of this Specification. Other values for rwp_guard_band_reserved_zero_3bits[ i ] are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore the value of rwp_guard_band_reserved_zero_3bits[ i ].

The variables NumPackedRegions, PackedRegionLeft[ n ], PackedRegionTop[ n ], PackedRegionWidth[ n ], PackedRegionHeight[ n ], ProjRegionLeft[ n ], ProjRegionTop[ n ], ProjRegionWidth[ n ], ProjRegionHeight[ n ], and TransformType[ n ] are derived as follows:

–	For n in the range of 0 to num_packed_regions − 1, inclusive, the following applies:

- PackedRegionLeft[ n ] is set equal to packed_region_left[ n ].

- PackedRegionTop[ n ] is set equal to packed_region_top[ n ].

- PackedRegionWidth[ n ] is set equal to packed_region_width[ n ].

- PackedRegionHeight[ n ] is set equal to packed_region_height[ n ].

- ProjRegionLeft[ n ] is set equal to proj_region_left[ n ].

- ProjRegionTop[ n ] is set equal to proj_region_top[ n ].

- ProjRegionWidth[ n ] is set equal to proj_region_width[ n ].

- ProjRegionHeight[ n ] is set equal to proj_region_height[ n ].

- TransformType[ n ] is set equal to rwp_transform_type[ n ].

–	If constituent_picture_matching_flag is equal to 0, the following applies:

- NumPackedRegions is set equal to num_packed_regions.

–	Otherwise (constituent_picture_matching_flag is equal to 1), the following applies:

- NumPackedRegions is set equal to 2 * num_packed_regions.

- When TopBottomFlag is equal to 1, the following applies:

  - projLeftOffset and packedLeftOffset are both set equal to 0.

  - projTopOffset is set equal to proj_picture_height / 2 and packedTopOffset is set equal to packed_picture_height / 2.

- When SideBySideFlag is equal to 1, the following applies:

  - projLeftOffset is set equal to proj_picture_width / 2 and packedLeftOffset is set equal to packed_picture_width / 2.

  - projTopOffset and packedTopOffset are both set equal to 0.

- For n in the range of NumPackedRegions / 2 to NumPackedRegions − 1, inclusive, the following applies:

  - nIdx is set equal to n − NumPackedRegions / 2.

  - PackedRegionLeft[ n ] is set equal to packed_region_left[ nIdx ] + packedLeftOffset.

  - PackedRegionTop[ n ] is set equal to packed_region_top[ nIdx ] + packedTopOffset.

  - PackedRegionWidth[ n ] is set equal to packed_region_width[ nIdx ].

  - PackedRegionHeight[ n ] is set equal to packed_region_height[ nIdx ].

  - ProjRegionLeft[ n ] is set equal to proj_region_left[ nIdx ] + projLeftOffset.

  - ProjRegionTop[ n ] is set equal to proj_region_top[ nIdx ] + projTopOffset.

  - ProjRegionWidth[ n ] is set equal to proj_region_width[ nIdx ].

  - ProjRegionHeight[ n ] is set equal to proj_region_height[ nIdx ].

  - TransformType[ n ] is set equal to rwp_transform_type[ nIdx ].

For each value of n in the range of 0 to NumPackedRegions − 1, inclusive, the values of ProjRegionWidth[ n ], ProjRegionHeight[ n ], ProjRegionTop[ n ], and ProjRegionLeft[ n ] are constrained as follows:

–	ProjRegionWidth[ n ] shall be in the range of 1 to proj_picture_width, inclusive.

–	ProjRegionHeight[ n ] shall be in the range of 1 to proj_picture_height, inclusive.

–	ProjRegionLeft[ n ] shall be in the range of 0 to proj_picture_width − 1, inclusive.

–	ProjRegionTop[ n ] shall be in the range of 0 to proj_picture_height − 1, inclusive.

–	If ProjRegionTop[ n ] is less than proj_picture_height / VerDiv1, the sum of ProjRegionTop[ n ] and ProjRegionHeight[ n ] shall be less than or equal to proj_picture_height / VerDiv1. Otherwise, the sum of ProjRegionTop[ n ] and ProjRegionHeight[ n ] shall be less than or equal to proj_picture_height / VerDiv1 * 2.

For each value of n in the range of 0 to NumPackedRegions − 1, inclusive, the values of PackedRegionWidth[ n ], PackedRegionHeight[ n ], PackedRegionTop[ n ], and PackedRegionLeft[ n ] are constrained as follows:

–	PackedRegionWidth[ n ] shall be in the range of 1 to packed_picture_width, inclusive.

–	ProjRegionHeight[ n ] shall be in the range of 1 to packed_picture_height, inclusive.

–	PackedRegionLeft[ n ] shall be in the range of 0 to packed_picture_width − 1, inclusive.

–	PackedRegionTop[ n ] shall be in the range of 0 to packed_picture_height − 1, inclusive.

–	If PackedRegionLeft[ n ] is less than packed_picture_width / HorDiv1, the sum of PackedRegionLeft[ n ] and PackedRegionWidth[ n ] shall be less than or equal to packed_picture_width / HorDiv1. Otherwise, the sum of PackedRegionLeft[ n ] and PackedRegionWidth[ n ] shall be less than or equal to packed_picture_width / HorDiv1 * 2.

–	If PackedRegionTop[ n ] is less than packed_picture_height / VerDiv1, the sum of PackedRegionTop[ n ] and PackedRegionHeight[ n ] shall be less than or equal to packed_picture_height / VerDiv1. Otherwise, the sum of PackedRegionTop[ n ] and PackedRegionHeight[ n ] shall be less than or equal to packed_picture_height / VerDiv1 * 2.

–	When chroma_format_idc is equal to 1 (4:2:0 chroma format) or 2 (4:2:2 chroma format), PackedRegionLeft[ n ] shall correspond to an even horizontal coordinate value of luma sample units, and PackedRegionWidth[ n ] shall correspond to an even number of luma samples, both within the decoded picture.

–	When chroma_format_idc is equal to 1 (4:2:0 chroma format), PackedRegionTop[ n ] shall correspond to an even vertical coordinate value of luma sample units, and ProjRegionHeight[ n ] shall correspond to an even number of luma samples, both within the decoded picture.

#### Omnidirectional viewport SEI message semantics

The omnidirectional viewport SEI message specifies the coordinates of one or more regions of spherical-coordinate geometry, bounded by four great circles, corresponding to viewports recommended for display when the user does not have control of the viewing orientation or has released control of the viewing orientation.

When an effectively applicable frame packing arrangement SEI message, as specified in clause D.3.41.1 or D.3.41.2, that applies to the picture is present, the information indicated by the omnidirectional viewport SEI message applies to both views.

omni_viewport_id contains an identifying number that may be used to identify the purpose of the one or more recommended viewport regions.

omni_viewport_id equal to 0 indicates that the recommended viewports are per "director’s cut", i.e., a viewport suggested according to the creative intent of the content author or content provider. omni_viewport_id equal to 1 indicates that the recommended viewports are selected based on measurements of viewing statistics.

Values of omni_viewport_id from 2 to 511, inclusive, may be used as determined by the application. Values of omni_viewport_id from 512 to 1023 are reserved for future use by ITU-T | ISO/IEC. Decoders encountering a value of omni_viewport_id in the range of 512 to 1023, inclusive, shall ignore it.

omni_viewport_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous omnidirectional viewport SEI message in output order. omni_viewport_cancel_flag equal to 0 indicates that omnidirectional viewport information follows.

omni_viewport_persistence_flag specifies the persistence of the omnidirectional viewport SEI message for the current layer.

omni_viewport_persistence_flag equal to 0 specifies that the omnidirectional viewport SEI message applies to the current decoded picture only.

Let picA be the current picture. omni_viewport_persistence_flag equal to 1 specifies that the omnidirectional viewport SEI message persists for the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture picB in the current layer in an access unit containing an omnidirectional viewport SEI message that is applicable to the current layer is output for which PicOrderCnt( picB ) is greater than PicOrderCnt( picA ), where PicOrderCnt( picB ) and PicOrderCnt( picA ) are the PicOrderCntVal values of picB and picA, respectively, immediately after the invocation of the decoding process for picture order count for picB.

When an equirectangular projection SEI message with erp_cancel_flag equal to 0 or a cubemap projection SEI message with cmp_cancel_flag equal to 0 is not present in the CLVS that applies to the current picture and precedes the omnidirectional viewport SEI message in decoding order, an omnidirectional viewport SEI message with omni_viewport_cancel_flag equal to 0 shall not be present in the CLVS that applies to the current picture. Decoders shall ignore omnidirectional viewport SEI messages with omni_viewport_cancel_flag equal to 0 that do not follow, in decoding order, an equirectangular projection SEI message with erp_cancel_flag equal to 0 or a cubemap projection SEI message with cmp_cancel_flag equal to 0 in the CLVS that applies to the current picture.

omni_viewport_cnt_minus1 plus 1 specifies the number of recommended viewport regions that are indicated by the SEI message.

When omni_viewport_cnt_minus1 is greater than 0 and there is no information provided by external means not specified in this Specification on which recommended viewport is suggested to be displayed, the following applies:

- When omni_viewport_id is equal to 0 or 1, the 0-th recommended viewport is suggested to be displayed when the user does not have control of the viewing orientation or has released control of the viewing orientation.

- When omni_viewport_id is equal to 0, between any two recommended viewports per director's cut, the i-th recommended viewport has higher priority than the j-th recommended viewport for any values of i and j when i is less than j. The 0-th recommended viewport per director's cut has the highest priority.

- When omni_viewport_id is equal to 1, between any two recommended viewports, the i-th recommended viewport has higher popularity, among some selection of candidate viewports, than the j-th recommended viewport for any values of i and j when i is less than j. The 0-th most-viewed recommended viewport has the highest popularity. The selection of the candidate viewports is outside the scope of this Specification.

omni_viewport_azimuth_centre[ i ] and omni_viewport_elevation_centre[ i ] indicate the centre of the i-th recommended viewport region, in units of 2−16 degrees relative to the global coordinate axes. The value of omni_viewport_azimuth_centre[ i ] shall be in the range of −180 * 216 (i.e., −11 796 480) to 180 * 216 − 1 (i.e., 11 796 479), inclusive. The value of omni_viewport_elevation_centre[ i ] shall be in the range of −90 * 216 (i.e., −5 898 240) to 90 * 216 (i.e., 5 898 240), inclusive.

omni_viewport_tilt_centre[ i ] indicates the tilt angle of the i-th recommended viewport region, in units of 2−16 degrees. The value of omni_viewport_tilt_centre[ i ] shall be in the range of −180 * 216 (i.e., −11 796 480) to  216 − 1 (i.e., 11 796 479), inclusive.

omni_viewport_hor_range[ i ] indicates the azimuth range of the i-th recommended viewport region, in units of 2−16 degrees. The value of omni_viewport_hor_range[ i ] shall be in the range of 1 to 360 * 216 (i.e., 23 592 960), inclusive.

omni_viewport_ver_range[ i ] indicates the elevation range of the i-th recommended viewport region, in units of 2−16 degrees. The value of omni_viewport_ver_range[ i ] shall be in the range of 1 to 180 * 216 (i.e., 11 796 480), inclusive.

#### Sample location remapping process

##### General

To remap colour sample locations of a region-wise packed picture to a unit sphere, the following ordered steps are applied:

- A region-wise packed picture is obtained as the cropped decoded picture by decoding a coded picture. For purposes of interpretation of chroma samples, the input to the indicated remapping process is the set of decoded sample values after applying an (unspecified) upsampling conversion process to the 4:4:4 colour sampling format as necessary when chroma_format_idc is equal to 1 (4:2:0 chroma format) or 2 (4:2:2 chroma format). This (unspecified) upsampling process should account for the relative positioning relationship between the luma and chroma samples as indicated by chroma_sample_loc_type_top_field and chroma_sample_loc_type_bottom_field, when present.

- If region-wise packing is indicated, the sample locations of the region-wise packed picture are converted to sample locations of the respective projected picture as specified in clause D.3.41.6.4. Otherwise, the projected picture is identical to the region-wise packed picture.

- If frame packing is indicated, the sample locations of the projected picture are converted to sample locations of the respective constituent picture of the projected picture, as specified in clause D.3.41.6.5. Otherwise, the constituent picture of the projected picture is identical to the projected picture.

- The sample locations of a constituent picture of the projected picture are converted to sphere coordinates relative to the local coordinate axes, as specified in clause D.3.41.6.2.

- If rotation is indicated, the sphere coordinates relative to the local coordinate axes are converted to sphere coordinates relative to the global coordinate axes, as specified in clause D.3.41.6.3. Otherwise, the global coordinate axes are identical to the local coordinate axes.

The overall process for mapping of luma sample locations within a region-wise packed picture to sphere coordinates relative to the global coordinate axes is normatively specified in clause D.3.41.6.5.

For each region-wise packed picture corresponding to a decoded picture, the following applies:

–	When an equirectangular projection SEI message with erp_cancel_flag equal to 0 that applies to the picture is present, ErpFlag is set equal to 1, and CmpFlag is set equal to 0.

–	When a cubemap projection SEI message with cmp_cancel_flag equal to 0 that applies to the picture is present, CmpFlag is set equal to 1, and ErpFlag is set equal to 0.

–	If a sphere rotation SEI message with sphere_rotation_cancel_flag equal to 0 that applies to the picture is present, RotationFlag is set equal to 1, and RotationYaw, RotationPitch, and RotationRoll are set equal to yaw_rotation ÷ 216, pitch_rotation ÷ 216, and roll_rotation ÷ 216, respectively.

–	Otherwise, RotationFlag is set equal to 0.

–	If a frame packing arrangement SEI message with frame_packing_arrangement_cancel_flag equal to 0 that applies to the picture is not present, StereoFlag, TopBottomFlag, and SideBySideFlag are all set equal to 0, HorDiv1 is set equal to 1, and VerDiv1 is set equal to 1.

–	Otherwise, the following applies:

–	StereoFlag is set equal to 1.

–	If the value of frame_packing_arrangement_type of the frame packing arrangement SEI message is equal to 3, TopBottomFlag is set equal to 0, SideBySideFlag is set equal to 1, HorDiv1 is set equal to 2 and VerDiv1 is set equal to 1.

–	Otherwise, if the value of frame_packing_arrangement_type of the frame packing arrangement SEI message is equal to 4, TopBottomFlag is set equal to 1, SideBySideFlag is set equal to 0, HorDiv1 is set equal to 1, and VerDiv1 is set equal to 2.

–	Otherwise, TopBottomFlag is set equal to 0, SideBySideFlag is set equal to 0, HorDiv1 is set equal to 1, and VerDiv1 is set equal to 1.

–	If a region-wise packing SEI message with rwp_cancel_flag equal to 0 that applies to the picture is not present, RegionWisePackingFlag is set equal to 0, and ConstituentPicWidth and ConstituentPicHeight are set to be equal to cropPicWidth / HorDiv1 and cropPicHeight / VerDiv1, respectively, where cropPicWidth and cropPicHeight are the width and height, respectively, of the cropped decoded picture.

–	Otherwise, RegionWisePackingFlag is set equal to 1, and ConstituentPicWidth and ConstituentPicHeight are set equal to proj_picture_width / HorDiv1 and proj_picture_height / VerDiv1, respectively.

##### Projection for one sample location

Inputs to this process are:

–	pictureWidth and pictureHeight, which are the width and height, respectively, of a monoscopic projected luma picture, in relative projected picture sample units, and

–	the centre point of a sample location (hPos, vPos) along the horizontal and vertical axes, respectively, in relative projected picture sample units, where hPos and vPos may have non-integer real values.

Outputs of this process are:

–	sphere coordinates (ϕ, θ) for the sample location in degrees relative to the local coordinate axes.

The projection for a sample location is derived as follows:

–	If ErpFlag is equal to 1, the following applies:

–	If RegionWisePackingFlag is equal to 0 and erp_guard_band_flag is equal to 1, the following applies:

hPos′ = hPos − erp_left_guard_band_width	(D-54)
pictureWidth = pictureWidth − erp_left_guard_band_width − erp_right_guard_band_width

–	Otherwise, the following applies:

hPos′ = hPos		(D-55)

–	The following applies:

ϕ = 180 − hPos′ * ( 360 ÷ pictureWidth )		(D-56)
θ = 90 − vPos * ( 180 ÷ pictureHeight )

–	Otherwise (CmpFlag is equal to 1), it is a requirement of bitstream conformance that pictureWidth shall be a multiple of 3 and pictureHeight shall be a multiple of 2, and that pictureWidth / 3 shall be equal to pictureHeight / 2, and the following applies:

lw = pictureWidth / 3
lh = pictureHeight / 2
w = Floor( hPos ÷ lw )
h = Floor( vPos ÷ lh )
tmpHorVal = hPos − w * lw
tmpVerVal = vPos − h * lh
hPos′ = −( 2 * tmpHorVal ÷ lw ) + 1
vPos′ = −( 2 * tmpVerVal ÷ lh ) + 1
if( w  = =  1  &&  h  = =  0 ) { /* positive x front face */
	x = 1.0
	y = hPos′
	z = vPos′
} else if( w  = =  1  &&  h  = =  1 ) { /* negative x back face */
	x = −1.0
	y = −vPos′
	z = −hPos′
} else if( w  = =  2  &&  h  = =  1 ) { /* positive z top face */	(D-57)
	x = −hPos′
	y = −vPos′
	z = 1.0
} else if( w  = =  0  &&  h  = =  1 ) { /* negative z bottom face */
	x = hPos′
	y = −vPos′
	z = −1.0
} else if( w  = =  0  &&  h  = =  0 ) { /* positive y left face */
	x = −hPos′
	y = 1.0
	z = vPos′
} else { /* ( w  = =  2  &&  h  = =  0 ), negative y right face */
	x = hPos′
	y = −1.0
	z = vPos′
}
ϕ = Atan2( y, x ) * 180 ÷ π
θ = Asinz ÷ x2 + y2 + z2 * 180 ÷ π

##### Conversion from the local coordinate axes to the global coordinate axes

Inputs to this process are:

–	rotation_yaw (αd), rotation_pitch (βd), rotation_roll (γd), all in units of degrees, and

–	sphere coordinates (ϕd, θd) relative to the local coordinate axes.

Outputs of this process are:

–	sphere coordinates (ϕ′, θ′) relative to the global coordinate axes.

The outputs are derived as follows:

ϕ = ϕd * π ÷ 180
θ = θd * π ÷ 180
α = αd * π ÷ 180
β = βd * π ÷ 180
γ = γd * π ÷ 180
x1 = Cos( ϕ ) * Cos( θ )
y1 = Sin( ϕ ) * Cos( θ )
z1 = Sin( θ )
x2 = Cos( β ) * Cos ( γ ) * x1 − Cos( β ) * Sin( γ ) * y1 + Sin( β ) * z1	(D-58)
y2 = ( Cos( α ) * Sin( γ ) + Sin( α ) * Sin( β ) * Cos( γ ) ) * x1 +
	( Cos( α ) * Cos( γ ) − Sin( α ) * Sin( β ) * Sin( γ ) ) * y1 −
	Sin( α ) * Cos( β ) * z1
z2 = ( Sin( α ) * Sin( γ ) − Cos( α ) * Sin( β ) * Cos( γ ) ) * x1 +
	( Sin( α ) * Cos( γ ) + Cos( α ) * Sin( β ) * Sin( γ ) ) * y1 +
	Cos( α ) * Cos( β ) * z1
ϕ′ = Atan2( y2, x2 ) * 180 ÷ π
θ′ = Asin( z2 ) * 180 ÷ π

##### Conversion of sample locations for rectangular region-wise packing

Inputs to this process are:

–	sample location (x, y) within the packed region, where x and y are in relative packed picture sample units, while the sample location is at an integer sample location within the packed picture,

–	the width and the height (projRegWidth, projRegHeight) of the projected region, in relative projected picture sample units,

–	the width and the height (packedRegWidth, packedRegHeight) of the packed region, in relative packed picture sample units,

–	transform type (transformType), and

–	offset values for the sampling position (offsetX, offsetY) in the range of 0, inclusive, to 1, exclusive, in horizontal and vertical relative packed picture sample units, respectively.

NOTE – offsetX and offsetY both equal to 0.5 indicate a sampling position that is in the centre point of a sample in packed picture sample units.

Outputs of this process are:

–	the centre point of the sample location (hPos, vPos) within the projected region in relative projected picture sample units, where hPos and vPos may have non-integer real values.

The outputs are derived as follows:

if( transformType  = =  0  | |  transformType  = =  1  | |  transformType  = =  2  | |  transformType  = =  3 ) {
	horRatio = projRegWidth ÷ packedRegWidth
	verRatio = projRegHeight ÷ packedRegHeight
} else if ( transformType  = =  4  | |  transformType  = =  5  | |  transformType  = =  6  | |
	transformType  = =  7 ) {
	horRatio = projRegWidth ÷ packedRegHeight
	verRatio = projRegHeight ÷ packedRegWidth
}
if( transformType  = =  0 ) {
	hPos = horRatio * ( x + offsetX )
	vPos = verRatio * ( y + offsetY )
} else if ( transformType  = =  1 ) {
	hPos = horRatio * ( packedRegWidth − x − offsetX )
	vPos = verRatio * ( y + offsetY )
} else if ( transformType  = =  2 ) {
	hPos = horRatio * ( packedRegWidth − x − offsetX )
	vPos = verRatio * ( packedRegHeight − y − offsetY )	(D-59)
} else if ( transformType  = =  3 ) {
	hPos = horRatio * ( x + offsetX )
	vPos = verRatio * ( packedRegHeight − y − offsetY )
} else if ( transformType  = =  4 ) {
	hPos = horRatio * ( y + offsetY )
	vPos = verRatio * ( x + offsetX )
} else if ( transformType  = =  5 ) {
	hPos = horRatio * ( y + offsetY )
	vPos = verRatio * ( packedRegWidth − x − offsetX )
} else if ( transformType  = = 6 ) {
	hPos = horRatio * ( packedRegHeight − y − offsetY )
	vPos = verRatio * ( packedRegWidth − x − offsetX )
} else if ( transformType  = =  7 ) {
	hPos = horRatio * ( packedRegHeight − y − offsetY )
	vPos = verRatio * ( x + offsetX )
}

##### Mapping of luma sample locations within a cropped decoded picture to sphere coordinates relative to the global coordinate axes

This clause specifies the semantics of luma sample locations within a cropped decoded picture to sphere coordinates relative to the global coordinate axes.

offsetX is set equal to 0.5 and offsetY is set equal to 0.5.

If RegionWisePackingFlag is equal to 1, the following applies for each packed region n in the range of 0 to NumPackedRegions − 1, inclusive:

–	For each sample location (xPackedPicture, yPackedPicture) belonging to the n-th packed region, the following applies:

–	The corresponding sample location (xProjPicture, yProjPicture) of the projected picture is derived as follows:

–	x is set equal to xPackedPicture − PackedRegionLeft[ n ].

–	y is set equal to yPackedPicture − PackedRegionTop[ n ].

–	Clause D.3.41.6.4 is invoked with x, y, PackedRegionWidth[ n ], PackedRegionHeight[ n ], ProjRegionWidth[ n ], ProjRegionHeight[ n ], TransformType[ n ], offsetX and offsetY as inputs, and the output is assigned to sample location (hPos, vPos).

–	xProjPicture is set equal to ProjRegionLeft[ n ] + hPos.

–	When StereoFlag is equal to 0 or TopBottomFlag is equal to 1, and when xProjPicture is greater than or equal to proj_picture_width, xProjPicture is set equal to xProjPicture − proj_picture_width.

–	When SideBySideFlag is equal to 1, the following applies:

–	When ProjRegionLeft[ n ] is less than proj_picture_width / 2 and xProjPicture is greater than or equal to proj_picture_width / 2, xProjPicture is set equal to xProjPicture − proj_picture_width / 2.

–	When ProjRegionLeft[ n ] is greater than or equal to proj_picture_width / 2 and xProjPicture is greater than or equal to proj_picture_width, xProjPicture is set equal to xProjPicture − proj_picture_width / 2.

–	yProjPicture is set equal to ProjRegionTop[ n ] + vPos.

–	Clause D.3.41.6.6 is invoked with xProjPicture, yProjPicture, ConstituentPicWidth, and ConstituentPicHeight as inputs, and the outputs indicating the sphere coordinates and the constituent picture index (for frame-packed stereoscopic video) for the luma sample location (xPackedPicture, yPackedPicture) belonging to the n-th packed region in the decoded picture.

Otherwise (RegionWisePackingFlag is equal 0), the following applies for each sample location (x, y) that is not an equirectangular projection guard band sample within the cropped decoded picture, where a sample location (x, y) is an equirectangular projection guard band sample when and only when ErpFlag is equal to 1, x is in the range of 0 to erp_left_guard_band_width − 1, inclusive, or ConstituentPicWidth − erp_right_guard_band_width to ConstituentPicWidth − 1, inclusive, and y is in the range of 0 to ConstituentPicHeight − 1, inclusive:

–	xProjPicture is set equal to x + offsetX.

–	yProjPicture is set equal to y + offsetY.

–	If ErpFlag is equal to 0, projPicWidth is set equal to ConstituentPicWidth. Otherwise (ErpFlag is equal to 1), projPicWidth is set equal to ConstituentPicWidth − ( erp_left_guard_band_width + erp_right_guard_band_width ).

–	Clause D.3.41.6.6 is invoked with xProjPicture, yProjPicture, projPicWidth, and ConstituentPicHeight as inputs, and the outputs indicating the sphere coordinates and the constituent picture index (for frame-packed stereoscopic video) for the sample location (x, y) within the region-wise packed picture.

##### Conversion from a sample location in a projected picture to sphere coordinates relative to the global coordinate axes

Inputs to this process are:

–	the centre point of a sample location (xProjPicture, yProjPicture) within a projected picture, where xProjPicture and yProjPicture are in relative projected picture sample units and may have non-integer real values, and

–	pictureWidth and pictureHeight, which are the width and height, respectively, of a monoscopic projected luma picture, in relative projected picture sample units.

Outputs of this process are:

–	sphere coordinates (azimuthGlobal, elevationGlobal), in units of degrees relative to the global coordinate axes, and

–	when StereoFlag is equal to 1, the index of the constituent picture (constituentPicture) equal to 0 or 1.

The outputs are derived with the following ordered steps:

- constituentPicture, xProjPicture, and yProjPicture are conditionally set as follows:

–	If xProjPicture is greater than or equal to pictureWidth or yProjPicture is greater than or equal to pictureHeight, the following applies:

–	constituentPicture is set equal to 1.

–	When xProjPicture is greater than or equal to pictureWidth, xProjPicture is set to xProjPicture − pictureWidth.

–	When yProjPicture is greater than or equal to pictureHeight, yProjPicture is set to yProjPicture − pictureHeight.

–	Otherwise, constituentPicture is set equal to 0.

- Clause D.3.41.6.2 is invoked with pictureWidth, pictureHeight, xProjPicture, and yProjPicture as inputs, and the output is assigned to azimuthLocal, elevationLocal.

- azimuthGlobal and elevationGlobal are set as follows:

–	If RotationFlag is equal to 1, clause D.3.41.6.3 is invoked with azimuthLocal, elevationLocal, RotationYaw, RotationPitch, and RotationRoll as inputs, and the output is assigned to azimuthGlobal and elevationGlobal.

–	Otherwise, azimuthGlobal is set equal to azimuthLocal and elevationGlobal is set equal to elevationLocal.

### Reserved SEI message semantics

The reserved SEI message consists of data reserved for future backward-compatible use by ITU-T | ISO/IEC. Unless otherwise specified by a referencing specification, coded video bitstreams shall not contain reserved SEI messages and systems that make use of such coded video bitstreams shall not otherwise send reserved SEI messages until and unless the use of such messages has been specified by ITU-T | ISO/IEC. Decoders shall ignore reserved SEI messages.

# Bibliography

- Recommendation ITU-T H.VVC (in force), Versatile video coding.

ISO/IEC 23090-3 (in force), Information technology – Coded representation of immersive media – Part 3: Versatile video coding.

- Recommendation ITU-T H.271 (in force), Video back-channel messages for conveyance of status information and requests from a video receiver to a video sender.

