| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0466 |
| --- | --- |

| Title: | AHG18 : Residual coding for lossless video coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jungah Choi Hyeongmun Jang Jin Heo Sunmi Yoo Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Seung Hwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Tel: Email: | +82-2-6912-6561 jungah.choi@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

In this contribution, residual coding method for lossless coding is proposed. VVC includes two residual coding methods: transform skip (TS) residual coding and normal residual coding. AHG18 released initial lossless software where BDPCM blocks are coded by TS residual coding and other blocks are coded by normal residual coding. In this contribution, we suggest to use TS residual coding for intra and IBC modes in lossless coding. In addition, we tested VTM-6.0 TS residual coding, CE7-1.2d, and CE7-1.2d-alternative methods.

Overall

- Test #1: 7.84% (AI), X.XX% (RA)

- Test #2: -0.37% (AI), X.XX% (RA)

- Test #3: -0.74% (AI), X.XX% (RA)

Class F

- Test #1: 2.17% (AI), X.XX% (RA)

- Test #2: -2.28% (AI), X.XX% (RA)

- Test #3: -2.51% (AI), X.XX% (RA)

TGM

- Test #1: 0.32% (AI), X.XX% (RA)

- Test #2: -3.35% (AI), X.XX% (RA)

- Test #3: -3.34% (AI), X.XX% (RA)

Proposed method

In AHG18, the initial lossless software is released to investigate lossless coding tools, and it uses transform skip (TS) residual coding for BDPCM blocks and normal residual coding for other blocks. In this contribution, we suggest to use TS residual coding for intra and IBC modes in lossless coding. Using the proposed method, intra (including BDPCM) and IBC modes are coded by TS residual coding and others are coded by normal residual coding.

transform_skip_flag[ x0 ][ y0 ] specifies whether a transform is applied to the luma transform block or not. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered transform block relative to the top-left luma sample of the picture. transform_skip_flag[ x0 ][ y0 ] equal to 1 specifies that no transform is applied to the luma transform block. transform_skip_flag[ x0 ][ y0 ] equal to 0 specifies that the decision whether transform is applied to the luma transform block or not depends on other syntax elements. When transform_skip_flag[ x0 ][ y0 ] is not present, it is inferred as follows: - If cu_transquant_bypass_flag[ x0 ][ y0 ] is equal to 1 and CuPredMode[ chType ][ x0 ][ y0 ] ! = MODE_INTER, transform_skip_flag[ x0 ][ y0 ] is inferred to be equal to 1. - Otherwise if cu_transquant_bypass_flag[ x0 ][ y0 ] is equal to 1 and CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTER, transform_skip_flag[ x0 ][ y0 ] is inferred to be equal to 0. - Otherwise (cu_transquant_bypass_flag[ x0 ][ y0 ] is equal to 0), transform_skip_flag[ x0 ][ y0 ] is inferred to be equal to BdcpmFlag[ x0 ][ x0 ].

In addition, we tested several TS residual coding methods for lossless coding:

- Test #1: VTM-6.0 TS residual coding

- Test #2: CE7-1.2d TS residual coding [1]

- Test #3: CE7-1.2d-alternative TS residual coding [1]

Experimental results

The experimental results are shown in following tables.

Table I. Test results (Test #1)

|  | All Intra |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM6 | Proposed |  | VTM6 | Proposed |  |
| Class A1 | 2.2 | 2.0 | 11.07% |  |  |  |
| Class A2 | 1.7 | 1.5 | 13.17% |  |  |  |
| Class B | 2.1 | 1.9 | 7.12% |  |  |  |
| Class C | 2.1 | 2.0 | 6.07% |  |  |  |
| Class D | 1.9 | 1.8 | 6.90% |  |  |  |
| Class E | 2.9 | 2.9 | 2.88% |  |  |  |
| Class F | 5.5 | 5.4 | 2.17% |  |  |  |
| TGM | 12.3 | 12.3 | 0.32% |  |  |  |
| Overall | 2.2 | 2.0 | 7.84% |  |  |  |
| Enc Time[%] |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |

Table II. Test results (Test #2)

|  | All Intra |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM6 | Proposed |  | VTM6 | Proposed |  |
| Class A1 | 2.2 | 2.2 | 0.12% |  |  |  |
| Class A2 | 1.7 | 1.7 | 0.45% |  |  |  |
| Class B | 2.1 | 2.1 | -0.91% |  |  |  |
| Class C | 2.1 | 2.1 | -0.98% |  |  |  |
| Class D | 1.9 | 2.0 | -1.88% |  |  |  |
| Class E | 2.9 | 2.9 | 0.05% |  |  |  |
| Class F | 5.5 | 5.6 | -2.28% |  |  |  |
| TGM | 12.3 | 12.7 | -3.35% |  |  |  |
| Overall | 2.2 | 2.2 | -0.37% |  |  |  |
| Enc Time[%] |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |

Table III. Test results (Test #3)

|  | All Intra |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM6 | Proposed |  | VTM6 | Proposed |  |
| Class A1 | 2.2 | 2.2 | -0.14% |  |  |  |
| Class A2 | 1.7 | 1.7 | 0.08% |  |  |  |
| Class B | 2.1 | 2.1 | -1.09% |  |  |  |
| Class C | 2.1 | 2.1 | -1.29% |  |  |  |
| Class D | 1.9 | 2.0 | -2.04% |  |  |  |
| Class E | 2.9 | 3.0 | -0.84% |  |  |  |
| Class F | 5.5 | 5.6 | -2.51% |  |  |  |
| TGM | 12.3 | 12.7 | -3.34% |  |  |  |
| Overall | 2.2 | 2.2 | -0.74% |  |  |  |
| Enc Time[%] | 111% |  |  |  |  |  |
| Dec Time[%] | 113% |  |  |  |  |  |

References

- M. Karczewicz et. al, “CE7: Modifications to transform-skip residual coding (CE7-1.2),” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-P0079, 16th meeting, Gevena, CH, 1-11 Oct. 2019. Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).