| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0466-v3 |
| --- | --- |

| Title: | AHG18 : Residual coding for lossless video coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jungah Choi Hyeongmun Jang Jin Heo Sunmi Yoo Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Seung Hwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Tel: Email: | +82-2-6912-6561 jungah.choi@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

In this contribution, residual coding method for lossless coding is proposed. VVC includes two residual coding methods: transform skip (TS) residual coding and normal residual coding. AHG18 released initial lossless software where BDPCM blocks are coded by TS residual coding and other blocks are coded by normal residual coding. In this contribution, TS residual coding for (i) intra and IBC modes (ii) IBC mode (iii) entire modes are suggested in lossless coding. In addition, we tested VTM-6.0 TS residual coding, CE7-1.2d, and CE7-1.2d-alternative methods.

Class F

- Test #3: -4.53% (AI), -2.06% (RA)

- Test #4: -4.17% (AI), -1.87% (RA)

- Test #5: -4.53% (AI), X.XX% (RA)

TGM

- Test #3: -4.36% (AI), -2.32% (RA)

- Test #4: -4.11% (AI), -2.17% (RA)

- Test #5: -4.36% (AI), -3.62% (RA)

Overall

- Test #3: -1.51% (AI), -0.35% (RA)

- Test #4: -1.60% (AI), -0.33% (RA)

- Test #5: -1.51% (AI), X.XX% (RA)

Proposed method I

In AHG18, the initial lossless software is released to investigate lossless coding tools, and it uses transform skip (TS) residual coding for BDPCM blocks and normal residual coding for other blocks. In this contribution, we suggest to use TS residual coding for intra and IBC modes in lossless coding. Using the proposed method, intra (including BDPCM) and IBC modes are coded by TS residual coding and others are coded by normal residual coding.

transform_skip_flag[ x0 ][ y0 ] specifies whether a transform is applied to the luma transform block or not. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered transform block relative to the top-left luma sample of the picture. transform_skip_flag[ x0 ][ y0 ] equal to 1 specifies that no transform is applied to the luma transform block. transform_skip_flag[ x0 ][ y0 ] equal to 0 specifies that the decision whether transform is applied to the luma transform block or not depends on other syntax elements. When transform_skip_flag[ x0 ][ y0 ] is not present, it is inferred as follows: - If cu_transquant_bypass_flag[ x0 ][ y0 ] is equal to 1 and CuPredMode[ chType ][ x0 ][ y0 ] ! = MODE_INTER, transform_skip_flag[ x0 ][ y0 ] is inferred to be equal to 1. - Otherwise if cu_transquant_bypass_flag[ x0 ][ y0 ] is equal to 1 and CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTER, transform_skip_flag[ x0 ][ y0 ] is inferred to be equal to 0. - Otherwise (cu_transquant_bypass_flag[ x0 ][ y0 ] is equal to 0), transform_skip_flag[ x0 ][ y0 ] is inferred to be equal to BdcpmFlag[ x0 ][ x0 ].

In addition, we tested several TS residual coding methods for lossless coding:

- Test #1: VTM-6.0 TS residual coding + BDPCM bug-fix [1]

- Test #2: CE7-1.2d TS residual coding [2] + BDPCM bug-fix [1]

- Test #3: CE7-1.2d-alternative TS residual coding [2] + BDPCM bug-fix [1]

Proposed method II

In the initial lossless software, transform_skip_flag is not coded, and IBC mode is coded by normal residual coding. In this contribution, we suggest to use TS residual coding for IBC mode in lossless coding. Using the proposed method, TS residual coding is applied to IBC modes as well as BDPCM modes. The proposed method is implemented on top of CE7-1.2d-alternative [2] and BDPCM bug-fix [1] (Test #4).

transform_skip_flag[ x0 ][ y0 ] specifies whether a transform is applied to the luma transform block or not. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered transform block relative to the top-left luma sample of the picture. transform_skip_flag[ x0 ][ y0 ] equal to 1 specifies that no transform is applied to the luma transform block. transform_skip_flag[ x0 ][ y0 ] equal to 0 specifies that the decision whether transform is applied to the luma transform block or not depends on other syntax elements. When transform_skip_flag[ x0 ][ y0 ] is not present, it is inferred as follows: - If cu_transquant_bypass_flag[ x0 ][ y0 ] is equal to 1 and CuPredMode[ chType ][ x0 ][ y0 ] == MODE_IBC, transform_skip_flag[ x0 ][ y0 ] is inferred to be equal to 1. - Otherwise, transform_skip_flag[ x0 ][ y0 ] is inferred to be equal to BdcpmFlag[ x0 ][ x0 ].

Proposed method III

There are two residual coding methods in VTM-6.0: TS residual coding and normal residual coding. In the proposed method, TS residual coding is always used for lossless coding. The proposed method is implemented on top of CE7-1.2d-alternative [2] and BDPCM bug-fix [1] (Test #5).

transform_skip_flag[ x0 ][ y0 ] specifies whether a transform is applied to the luma transform block or not. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered transform block relative to the top-left luma sample of the picture. transform_skip_flag[ x0 ][ y0 ] equal to 1 specifies that no transform is applied to the luma transform block. transform_skip_flag[ x0 ][ y0 ] equal to 0 specifies that the decision whether transform is applied to the luma transform block or not depends on other syntax elements. When transform_skip_flag[ x0 ][ y0 ] is not present, it is inferred as follows: - If cu_transquant_bypass_flag is equal to 1, transform_skip_flag[ x0 ][ y0 ] is inferred to be equal to 1. - Otherwise, if BdcpmFlag[ x0 ][ x0 ] is equal to 1, transform_skip_flag[ x0 ][ y0 ] is inferred to be equal to 1. - Otherwise (BdcpmFlag[ x0 ][ x0 ] is equal to 0), transform_skip_flag[ x0 ][ y0 ] is inferred to be equal to 0.

Experimental results

The experimental results are shown in following tables. In all tests, the anchor is AHG18 initial lossless software (VTM6.0 Lossless branch). For anchor and proposed methods, we used following settings: --TransquantBypassEnable=1 –CUTransquantBypassFlagForce=1 – CostMode=lossless –InternalBitDepth=0 –qp 0. For the proposed method, we additionally set BDPCM to 1 for all test sequences.

Table I. Test results (Test #1)

Table II. Test results (Test #2)

Table III. Test results (Test #3)

Table IV. Test results (Test #4)

Table V. Test results (Test #5)

References

- M. Sarwer et. al, “AHG18: Residual coding method for lossless mode,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-P0463, 16th meeting, Gevena, CH, 1-11 Oct. 2019. Patent rights declaration(s)

- M. Karczewicz et. al, “CE7: Modifications to transform-skip residual coding (CE7-1.2),” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-P0079, 16th meeting, Gevena, CH, 1-11 Oct. 2019.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).