| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0469-v1 |
| --- | --- |

| Title: | AHG15: efficient coding of qp_out_val |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Frank Bossen Andrew Segall | Tel: Email: | fbossen@sharpsec.com |
| Source: | Sharp Electronics of Canada Ltd., Sharp Labs of America |  |  |

_____________________________

# Abstract

The current VVC draft includes the ability to encode custom chroma QP mapping tables using a set of qpY and qpC pairs. Such pairs are delta coded as syntax elements delta_qp_in_val_minus1 and delta_qp_out_val. Two modified encodings of delta_qp_out_val are proposed. It is asserted that these modifications reduce the number of bits required to encode the mapping table by about 25-30% on average, while preserving a built-in constraint that restricts qpC to monotonically increase with qpY.

# Introduction

The delta_qp_out_val syntax element is directly coded as ue(v). This tends to be inefficient as the value 0 is not the most commonly used.

# Proposal

Two alternatives are proposed:

- Instead of directly coding the difference dy = qpOutVal[ i ][ j ] - qpOutVal[ i ][ j − 1 ], the value (dy <= delta_qp_in_val_minus1[ i ][ j ] ? delta_qp_in_val_minus1[ i ][ j ] – dy : dy) is encoded. This is similar to the method previously proposed in JVET-O1166

- Instead of directly coding the difference dy = qpOutVal[ i ][ j ] - qpOutVal[ i ][ j − 1 ], the value (dy ^ delta_qp_in_val_minus1[ i ][ j ]) is encoded

Text modifications are restricted to changing equation (7-31), where three instances of delta_qp_out_val[i][*] are replaced by either:

- (delta_qp_out_val[i][*] <= delta_qp_in_val_minus1[i][*] ? delta_qp_in_val_minus1[i][*] - delta_qp_out_val[i][*] : delta_qp_out_val[i][*]); or

- (delta_qp_out_val[i][*] ^ delta_qp_in_val_minus1[i][*])

# Bit counts

Bit counts are reported for a number of tables previously considered by JVET. Note that VTM6 results include the fix for rounding issue #438.

| Table | Pivot points | VTM6 |
| --- | --- | --- |
| CTC: All-INTRA | (0, 0), (31, 32), (43, 41) | 51 |
| CTC: LDP, LDB, RA | (32, 32), (44, 41) | 39 |
| CTC: Class H | (6, 6), (25, 26), (32, 32), (42, 37) | 63 |
| VTM5 | (26, 26), (33, 32), (43, 37) | 47 |
| JVET-O0433 (HLG) | (29, 29), (30, 29), (34, 33), (45, 37), (60, 49), (62, 51), (63, 51) | 73 |
| Sum |  | 273 |

| Table | Pivot points | Proposal 1 |
| --- | --- | --- |
| CTC: All-INTRA | (0, 0), (31, 32), (43, 41) | 41 |
| CTC: LDP, LDB, RA | (32, 32), (44, 41) | 25 |
| CTC: Class H | (6, 6), (25, 26), (32, 32), (42, 37) | 51 |
| VTM5 | (26, 26), (33, 32), (43, 37) | 33 |
| JVET-O0433 (HLG) | (24, 24), (34, 33), (45, 37), (60, 49), (61, 50), (63, 51) | 55 |
| Sum |  | 205 (-25%) |

| Table | Pivot points | Proposal 2 |
| --- | --- | --- |
| CTC: All-INTRA | (1, 1), (31, 32), (43, 41) | 33 |
| CTC: LDP, LDB, RA | (32, 32), (44, 41) | 25 |
| CTC: Class H | (9, 9), (23, 24), (33, 33), (34, 33), (41, 37), (42, 37) | 45 |
| VTM5 | (26, 26), (33, 32), (43, 37) | 35 |
| JVET-O0433 (HLG) | (24, 24), (34, 33), (45, 37), (60, 49), (61, 50), (63, 51) | 57 |
| Sum |  | 195 (-29%) |

## Comparison with JVET-P0410

JVET-P0410 proposes a number of changes to the coding of the mapping tables. The alternatives proposed are compared within the JVET-P0410 framework. It should be noted that results reported here are slightly better than in JVET-P0410 given the pivot point optimisation that was used.

| Table | Pivot points | JVET-P0410 |
| --- | --- | --- |
| CTC: All-INTRA | (0, 0), (31, 32), (43, 41) | 40 |
| CTC: LDP, LDB, RA | (32, 32), (44, 41) | 20 |
| CTC: Class H | (6, 6), (25, 26), (32, 32), (42, 37) | 48 |
| VTM5 | (26, 26), (33, 32), (43, 37) | 24 |
| JVET-O0433 (HLG) | (25, 25), (34, 33), (45, 37), (60, 49), (61, 50), (63, 51) | 50 |
| Sum |  | 182 (-33%) |

| Table | Pivot points | Proposal 1 |
| --- | --- | --- |
| CTC: All-INTRA | (15, 15), (16, 17), (31, 32), (43, 41) | 42 |
| CTC: LDP, LDB, RA | (32, 32), (44, 41) | 18 |
| CTC: Class H | (6, 6), (25, 26), (32, 32), (42, 37) | 50 |
| VTM5 | (26, 26), (33, 32), (43, 37) | 22 |
| JVET-O0433 (HLG) | (25, 25), (34, 33), (45, 37), (60, 49), (61, 50), (63, 51) | 46 |
| Sum |  | 178 (-35%) |

| Table | Pivot points | Proposal 1 |
| --- | --- | --- |
| CTC: All-INTRA | (1, 1), (31, 32), (43, 41) | 36 |
| CTC: LDP, LDB, RA | (32, 32), (44, 41) | 18 |
| CTC: Class H | (7, 7), (25, 26), (32, 32), (42, 37) | 46 |
| VTM5 | (26, 26), (33, 32), (43, 37) | 24 |
| JVET-O0433 (HLG) | (25, 25), (34, 33), (45, 37), (60, 49), (61, 50), (63, 51) | 48 |
| Sum |  | 172 (-37%) |

# Patent rights declaration(s)

Sharp Labs of America may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).