| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0473 |
| --- | --- |

| Title: | Non-CE8: Implicit block partitioning in palette mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yung-Hsuan Chao, Ted Hsieh, Vadim Seregin, Marta Karczewicz | Tel: Email: | +1-858-8452163 yunghsua@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document proposes two block partition approaches to address the buffer issue in the palette mode in VTM6.0 for CU size larger than 32x32. In the first method, a CU of size 64x64, 64x32, and 32x64 is split vertically or horizontally into multiple rectangular subblocks, each contains 1024 samples. The split direction depends on the rotation flag of traverse scan in palette index map coding. In the second method, a CU of size 64x64, 64x32, and 32x64 is split into multiple 32x32 subblocks. The signaling/parsing of palette index values, palette run type, run length, and quantizaed escape colors are done in each subblock sequentially. As a result, pixels within a subblock can be reconstructed after the syntax elements of the subblocks are parsed, reducing the buffer needed to store palette index values and index map in the current design in VTM6.0.

The results of the proposed methods on YUV444 sequences show only minor losses compared to VTM6.0:

Method 1 –

Dual tree OFF: 0.06% AI, 0.01% RA, 0.10% LB

Method 2 –

Dual tree OFF: 0.10% AI, 0.02% RA, 0.13% LB

# Introduction

In the hardware pipeline on the decoder side, decoded transform coefficients are stored in the coefficient buffer right after parsing stage, before inverse quantization and inverse transform stages. In the palette mode in VTM6.0, palette syntax elements are parsed in CU level, instead of coefficient group (CG) level as in blocks using transform or transform skip modes. Since the maximum run length, indicating the number of consecutive pixels that are coded using the same palette type, is equal to number of pixels in a CU, and since the parsing of quantized escape colors are grouped at the end of a CU, in the worst case, index map of the whole CU have to be stored before reconstructing the pixels in a CU. Therefore, buffer size for 64x64 pixels must be maintained and extra buffer will also be needed for the index values grouped and parsed at the beginning of each CU.

In trasnform unit (TU) using transform, due to the zero out of transformed coefficients, the maximum buffer size for coefficient/transform skip residual is 32x32 in VVC. Therefore, the memory requirement for palette mode after parsing stage is a burden for hardware implementation.

# Proposed method

In this proposal, two methods are proposed to address the buffer issue in palette mode in VTM6.0.

## Method 1

In the first method, for CU size larger than 32x32, the block is implicitly split vertically and horizontally into multiple rectangular blocks, each contains 1024 samples, as illustrated in Figure 1.

Figure 1: Split of a palette coded CUs based on scanning direction if CU size is larger than 32x32

The palette syntax, i.e., palette index values, run lengths, run types, and quantized escape colors, are signaled/parsed in each sub-block separately. The parsing of other syntax elements, i.e., palette table, re-use flags for palette predictor, and rotation flag is done in CU level as in VTM6.0, namely, the palette table and traverse scan rotation will be the same for different subblocks within a CU.

## Method 2

In the second method, for CU size larger than 32x32, the block is implicitly split into multiple 32x32 blocks, as illustrated in Figure 2.

Figure 1: quad split for palette coded CUs if CU size is larger than 32x32 in VVC

The processing order for each subblock is done as: top-left -> top right -> bottom left -> bottom right. The palette syntax, i.e., palette index values, run lengths, run types, and quantized escape colors, are signaled/parsed in each sub-block separately. The parsing of other syntax elements, i.e., palette table, re-use flags for palette predictor, and rotation flag is done in CU level as in VTM6.0, namely, the palette table and traverse scan rotation will be the same for different subblocks within a CU.

# Simulation Results

The results are reported using the test conditions defined in CE8 description [2]. IBC, BDPCM, HashME, and PLT are turned ON for all the YUV4:4:4 sequences. The results for method 1 and method 2 are shown in Table 1 and 2.

Table 1: Results of method 1 (horizontal/vertical split) for DualITree=0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.07% | 0.07% | 0.08% | 100% | 99% |
| TGM 720p | 0.12% | 0.25% | 0.17% | 99% | 99% |
| Animation | 0.04% | 0.09% | 0.12% | 101% | 102% |
| Mixed content | 0.01% | 0.03% | -0.01% | 102% | 100% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Overall | 0.06% | 0.10% | 0.08% | 101% | 100% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.03% | 0.00% | 0.08% | 100% | 101% |
| TGM 720p | 0.02% | 0.10% | 0.11% | 99% | 99% |
| Animation | 0.02% | -0.03% | -0.02% | 102% | 102% |
| Mixed content | -0.03% | 0.11% | 0.14% | 100% | 100% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Overall | 0.01% | 0.04% | 0.07% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.22% | 0.29% | 0.28% | 101% | 100% |
| TGM 720p | 0.04% | 0.32% | 0.46% | 102% | 101% |
| Animation | -0.05% | 0.05% | -0.02% | 101% | 98% |
| Mixed content | 0.25% | 0.18% | 0.03% | 101% | 98% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 101% | 102% |
| Overall | 0.10% | 0.20% | 0.19% | 101% | 100% |

Table 2: Results of method 2 (quad-tree split) for DualITree=0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.20% | 0.23% | 0.26% | 101% | 97% |
| TGM 720p | 0.14% | 0.20% | 0.21% | 100% | 102% |
| Animation | 0.04% | 0.08% | 0.11% | 101% | 101% |
| Mixed content | 0.04% | 0.13% | 0.14% | 102% | 102% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Overall | 0.10% | 0.15% | 0.16% | 101% | 100% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.15% | 0.23% | 0.30% | 100% | 101% |
| TGM 720p | -0.05% | 0.02% | 0.04% | 101% | 101% |
| Animation | 0.03% | -0.03% | -0.02% | 101% | 102% |
| Mixed content | -0.05% | 0.12% | 0.12% | 101% | 101% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Overall | 0.02% | 0.08% | 0.10% | 101% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.28% | 0.37% | 0.38% | 100% | 102% |
| TGM 720p | 0.19% | 0.47% | 0.86% | 102% | 100% |
| Animation | -0.06% | 0.06% | 0.05% | 101% | 100% |
| Mixed content | 0.13% | 0.05% | 0.09% | 102% | 101% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 101% | 103% |
| Overall | 0.13% | 0.23% | 0.34% | 101% | 101% |

# Reference

- Y.-H. Chao, C.-H. Hung, W.-J. Chien, V. Seregin, M. Karczewicz, Y.-C. Sun, T.-S. Chang, J. Lou ,W. Zhu, L. Zhang, J. Xu, K. Zhang, H. Liu, “CE8-2.1: Palette mode in HEVC” , JVET-O0119, 15th meeting, Gothenburg, SE, July 2019.

- X. Xu, Y.-H. Chao, Y.-C. Sun, J. Xu, “Description of Core Experiment 8 (CE8): 4:4:4 Screen Content Coding Tools”, JVET-O2028, 15th meeting, Gothenburg, SE, July 2019.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form)