| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0476 |
| --- | --- |

| Title: | Non-CE8: Palette mode and prediction mode signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yung-Hsuan Chao, Chao-Hsiung Hung, Wei-Jung Chien, Marta Karczewicz | Tel: Email: | +1-858-8452163 yunghsua@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

7.3.8.5	Coding unit syntax

| coding_unit( x0, y0, cbWidth, cbHeight, cqtDepth, treeType, modeType ) { | Descriptor |
| --- | --- |
| chType = treeType = = DUAL_TREE_CHROMA? 1 : 0 |  |
| if( slice_type != I ¦ ¦ sps_ibc_enabled_flag ¦ ¦ sps_palette_enabled_flag) { |  |
| if( treeType != DUAL_TREE_CHROMA && !( ( ( cbWidth = = 4 && cbHeight = = 4 ) ¦ ¦ modeType = = MODE_TYPE_INTRA ) && !sps_ibc_enabled_flag ) ) { |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| if( cu_skip_flag[ x0 ][ y0 ] = = 0 ) |  |
| pred_mode_flag | ae(v) |
| } |  |
| if( slice_type != I && !( cbWidth = = 4 && cbHeight = = 4 ) && CuPredMode[ chType ][ x0 ][ y0 ] != MODE_INTRA && cbWidth <= 64 && cbHeight <= 64 && modeType = = MODE_TYPE_ALL && sps_ibc_enabled_flag && treeType != DUAL_TREE_CHROMA ) |  |
| pred_mode_ibc_flag | ae(v) |
| if( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA && sps_palette_enabled_flag && cbWidth <= 64 && cbHeight <= 64 && && cu_skip_flag[ x0 ][ y0 ] = = 0 && modeType != MODE_INTER ) |  |
| pred_mode_plt_flag | ae(v) |
| } |  |
| if( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA ¦ ¦ CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_PLT ) { |  |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_LUMA ) { |  |
| if( pred_mode_plt_flag ) { |  |
| if( treeType = = DUAL_TREE_LUMA ) |  |
| palette_coding( x0, y0, cbWidth, cbHeight, 0, 1 ) |  |
| else /* SINGLE_TREE */ |  |
| palette_coding( x0, y0, cbWidth, cbHeight, 0, 3 ) |  |
| } else { |  |
| if( sps_bdpcm_enabled_flag && cbWidth <= MaxTsSize && cbHeight <= MaxTsSize ) |  |
| intra_bdpcm_flag | ae(v) |
| if( intra_bdpcm_flag ) |  |
| intra_bdpcm_dir_flag | ae(v) |
| else { |  |
| if( sps_mip_enabled_flag && ( Abs( Log2( cbWidth ) − Log2( cbHeight ) ) <= 2 ) && cbWidth <= MaxTbSizeY && cbHeight <= MaxTbSizeY ) |  |
| intra_mip_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_mip_flag[ x0 ][ y0 ] ) |  |
| intra_mip_mode[ x0 ][ y0 ] | ae(v) |
| else { |  |
| if( sps_mrl_enabled_flag && ( ( y0 % CtbSizeY ) > 0 ) ) |  |
| intra_luma_ref_idx[ x0 ][ y0 ] | ae(v) |
| if ( sps_isp_enabled_flag && intra_luma_ref_idx[ x0 ][ y0 ] = = 0 && ( cbWidth <= MaxTbSizeY && cbHeight <= MaxTbSizeY ) && ( cbWidth * cbHeight > MinTbSizeY * MinTbSizeY ) ) |  |
| intra_subpartitions_mode_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_subpartitions_mode_flag[ x0 ][ y0 ] = = 1 ) |  |
| intra_subpartitions_split_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_luma_ref_idx[ x0 ][ y0 ] = = 0 ) |  |
| intra_luma_mpm_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_luma_mpm_flag[ x0 ][ y0 ] ) { |  |
| if( intra_luma_ref_idx[ x0 ][ y0 ] = = 0 ) |  |
| intra_luma_not_planar_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_luma_not_planar_flag[ x0 ][ y0 ] ) |  |
| intra_luma_mpm_idx[ x0 ][ y0 ] | ae(v) |
| } else |  |
| intra_luma_mpm_remainder[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| if( ( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_CHROMA ) && ChromaArrayType != 0 ) { |  |
| if ( pred_mode_plt_flag && treeType = = DUAL_TREE_CHROMA ) |  |
| palette_coding( x0, y0, cbWidth / SubWidthC, cbHeight / SubHeightC, 1, 2 ) |  |
| else { |  |
| if( CclmEnabled ) |  |
| cclm_mode_flag | ae(v) |
| if( cclm_mode_flag ) |  |
| cclm_mode_idx | ae(v) |
| else |  |
| intra_chroma_pred_mode | ae(v) |
| } |  |
| } |  |
| } else if( treeType != DUAL_TREE_CHROMA ) { /* MODE_INTER or MODE_IBC */ |  |
| if( cu_skip_flag[ x0 ][ y0 ] = = 0 ) |  |
| general_merge_flag[ x0 ][ y0 ] | ae(v) |
| if( general_merge_flag[ x0 ][ y0 ] ) { |  |
| merge_data( x0, y0, cbWidth, cbHeight, chType ) |  |
| } else if ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_IBC ) { |  |
| mvd_coding( x0, y0, 0, 0 ) |  |
| if( MaxNumIbcMergeCand > 1 ) |  |
| mvp_l0_flag[ x0 ][ y0 ] | ae(v) |
| if( sps_amvr_enabled_flag && ( MvdL0[ x0 ][ y0 ][ 0 ] != 0 ¦ ¦ MvdL0[ x0 ][ y0 ][ 1 ] != 0 ) ) { |  |
| amvr_precision_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| } else { |  |
| if( slice_type = = B ) |  |
| inter_pred_idc[ x0 ][ y0 ] | ae(v) |
| if( sps_affine_enabled_flag && cbWidth >= 16 && cbHeight >= 16 ) { |  |
| inter_affine_flag[ x0 ][ y0 ] | ae(v) |
| if( sps_affine_type_flag && inter_affine_flag[ x0 ][ y0 ] ) |  |
| cu_affine_type_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( sps_smvd_enabled_flag && !mvd_l1_zero_flag && inter_pred_idc[ x0 ][ y0 ] = = PRED_BI && !inter_affine_flag[ x0 ][ y0 ] && RefIdxSymL0 > −1 && RefIdxSymL1 > −1 ) |  |
| sym_mvd_flag[ x0 ][ y0 ] | ae(v) |
| if( inter_pred_idc[ x0 ][ y0 ] != PRED_L1 ) { |  |
| if( NumRefIdxActive[ 0 ] > 1 && !sym_mvd_flag[ x0 ][ y0 ] ) |  |
| ref_idx_l0[ x0 ][ y0 ] | ae(v) |
| mvd_coding( x0, y0, 0, 0 ) |  |
| if( MotionModelIdc[ x0 ][ y0 ] > 0 ) |  |
| mvd_coding( x0, y0, 0, 1 ) |  |
| if(MotionModelIdc[ x0 ][ y0 ] > 1 ) |  |
| mvd_coding( x0, y0, 0, 2 ) |  |
| mvp_l0_flag[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| MvdL0[ x0 ][ y0 ][ 0 ] = 0 |  |
| MvdL0[ x0 ][ y0 ][ 1 ] = 0 |  |
| } |  |
| if( inter_pred_idc[ x0 ][ y0 ] != PRED_L0 ) { |  |
| if( NumRefIdxActive[ 1 ] > 1 && !sym_mvd_flag[ x0 ][ y0 ] ) |  |
| ref_idx_l1[ x0 ][ y0 ] | ae(v) |
| if( mvd_l1_zero_flag && inter_pred_idc[ x0 ][ y0 ] = = PRED_BI ) { |  |
| MvdL1[ x0 ][ y0 ][ 0 ] = 0 |  |
| MvdL1[ x0 ][ y0 ][ 1 ] = 0 |  |
| MvdCpL1[ x0 ][ y0 ][ 0 ][ 0 ] = 0 |  |
| MvdCpL1[ x0 ][ y0 ][ 0 ][ 1 ] = 0 |  |
| MvdCpL1[ x0 ][ y0 ][ 1 ][ 0 ] = 0 |  |
| MvdCpL1[ x0 ][ y0 ][ 1 ][ 1 ] = 0 |  |
| MvdCpL1[ x0 ][ y0 ][ 2 ][ 0 ] = 0 |  |
| MvdCpL1[ x0 ][ y0 ][ 2 ][ 1 ] = 0 |  |
| } else { |  |
| if( sym_mvd_flag[ x0 ][ y0 ] ) { |  |
| MvdL1[ x0 ][ y0 ][ 0 ] = −MvdL0[ x0 ][ y0 ][ 0 ] |  |
| MvdL1[ x0 ][ y0 ][ 1 ] = −MvdL0[ x0 ][ y0 ][ 1 ] |  |
| } else |  |
| mvd_coding( x0, y0, 1, 0 ) |  |
| if( MotionModelIdc[ x0 ][ y0 ] > 0 ) |  |
| mvd_coding( x0, y0, 1, 1 ) |  |
| if(MotionModelIdc[ x0 ][ y0 ] > 1 ) |  |
| mvd_coding( x0, y0, 1, 2 ) |  |
| mvp_l1_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| } else { |  |
| MvdL1[ x0 ][ y0 ][ 0 ] = 0 |  |
| MvdL1[ x0 ][ y0 ][ 1 ] = 0 |  |
| } |  |
| if( ( sps_amvr_enabled_flag && inter_affine_flag[ x0 ][ y0 ] = = 0 && ( MvdL0[ x0 ][ y0 ][ 0 ] != 0 ¦ ¦ MvdL0[ x0 ][ y0 ][ 1 ] != 0 ¦ ¦ MvdL1[ x0 ][ y0 ][ 0 ] != 0 ¦ ¦ MvdL1[ x0 ][ y0 ][ 1 ] != 0 ) ) ¦ ¦ ( sps_affine_amvr_enabled_flag && inter_affine_flag[ x0 ][ y0 ] = = 1 && ( MvdCpL0[ x0 ][ y0 ][ 0 ] [ 0 ] != 0 ¦ ¦ MvdCpL0[ x0 ][ y0 ][ 0 ] [ 1 ] != 0 ¦ ¦ MvdCpL1[ x0 ][ y0 ][ 0 ] [ 0 ] != 0 ¦ ¦ MvdCpL1[ x0 ][ y0 ][ 0 ] [ 1 ] != 0 ¦ ¦ MvdCpL0[ x0 ][ y0 ][ 1 ] [ 0 ] != 0 ¦ ¦ MvdCpL0[ x0 ][ y0 ][ 1 ] [ 1 ] != 0 ¦ ¦ MvdCpL1[ x0 ][ y0 ][ 1 ] [ 0 ] != 0 ¦ ¦ MvdCpL1[ x0 ][ y0 ][ 1 ] [ 1 ] != 0 ¦ ¦ MvdCpL0[ x0 ][ y0 ][ 2 ] [ 0 ] != 0 ¦ ¦ MvdCpL0[ x0 ][ y0 ][ 2 ] [ 1 ] != 0 ¦ ¦ MvdCpL1[ x0 ][ y0 ][ 2 ] [ 0 ] != 0 ¦ ¦ MvdCpL1[ x0 ][ y0 ][ 2 ] [ 1 ] != 0 ) ) { |  |
| amvr_flag[ x0 ][ y0 ] | ae(v) |
| if( amvr_flag[ x0 ][ y0 ] ) |  |
| amvr_precision_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( sps_bcw_enabled_flag && inter_pred_idc[ x0 ][ y0 ] = = PRED_BI && luma_weight_l0_flag[ ref_idx_l0 [ x0 ][ y0 ] ] = = 0 && luma_weight_l1_flag[ ref_idx_l1 [ x0 ][ y0 ] ] = = 0 && chroma_weight_l0_flag[ ref_idx_l0 [ x0 ][ y0 ] ] = = 0 && chroma_weight_l1_flag[ ref_idx_l1 [ x0 ][ y0 ] ] = = 0 && cbWidth * cbHeight >= 256 ) |  |
| bcw_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| if( CuPredMode[ chType ][ x0 ][ y0 ] != MODE_INTRA && !pred_mode_plt_flag && general_merge_flag[ x0 ][ y0 ] = = 0 ) |  |
| cu_cbf | ae(v) |
| if( cu_cbf ) { |  |
| if( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTER && sps_sbt_enabled_flag && !ciip_flag[ x0 ][ y0 ] && !MergeTriangleFlag[ x0 ][ y0 ] ) { |  |
| if( cbWidth <= MaxSbtSize && cbHeight <= MaxSbtSize ) { |  |
| allowSbtVerH = cbWidth >= 8 |  |
| allowSbtVerQ = cbWidth >= 16 |  |
| allowSbtHorH = cbHeight >= 8 |  |
| allowSbtHorQ = cbHeight >= 16 |  |
| if( allowSbtVerH ¦ ¦ allowSbtHorH ¦ ¦ allowSbtVerQ ¦ ¦ allowSbtHorQ ) |  |
| cu_sbt_flag | ae(v) |
| } |  |
| if( cu_sbt_flag ) { |  |
| if( ( allowSbtVerH ¦ ¦ allowSbtHorH ) && ( allowSbtVerQ ¦ ¦ allowSbtHorQ) ) |  |
| cu_sbt_quad_flag | ae(v) |
| if( ( cu_sbt_quad_flag && allowSbtVerQ && allowSbtHorQ ) ¦ ¦ ( !cu_sbt_quad_flag && allowSbtVerH && allowSbtHorH ) ) |  |
| cu_sbt_horizontal_flag | ae(v) |
| cu_sbt_pos_flag | ae(v) |
| } |  |
| } |  |
| LfnstDcOnly = 1 |  |
| LfnstZeroOutSigCoeffFlag = 1 |  |
| transform_tree( x0, y0, cbWidth, cbHeight, treeType ) |  |
| lfnstWidth = ( treeType = = DUAL_TREE_CHROMA ) ? cbWidth / SubWidthC : cbWidth |  |
| lfnstHeight = ( treeType = = DUAL_TREE_CHROMA ) ? cbHeight / SubHeightC : cbHeight |  |
| if( Min( lfnstWidth, lfnstHeight ) >= 4 && sps_lfnst_enabled_flag = = 1 && CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && IntraSubPartitionsSplitType = = ISP_NO_SPLIT && ( !intra_mip_flag[ x0 ][ y0 ] ¦ ¦ Min( lfnstWidth, lfnstHeight ) >= 16 ) && tu_mts_idx[ x0 ][ y0 ] = = 0 && Max( cbWidth, cbHeight ) <= MaxTbSizeY) { |  |
| if( LfnstDcOnly = = 0 && LfnstZeroOutSigCoeffFlag = = 1 ) |  |
| lfnst_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |

# 7.4.9.5	Coding unit semantics

The following assignments are made for x = x0..x0 + cbWidth − 1 and y = y0..y0 + cbHeight − 1:

CbPosX[ chType ][ x ][ y ] = x0		(7-134)

CbPosY[ chType ][ x ][ y ] = y0		(7-135)

CbWidth[ chType ][ x ][ y ] = cbWidth		(7-136)

CbHeight[ chType ][ x ][ y ] = cbHeight		(7-137)

CqtDepth[ chType ][ x ][ y ] = cqtDepth		(7-138)

The variable CclmEnabled is derived by invoking the cross-component chroma intra prediction mode checking process specified in clause 8.4.4 with the luma location ( xCb, yCb ) set equal to ( x0, y0 ) as input.

cu_skip_flag[ x0 ][ y0 ] equal to 1 specifies that for the current coding unit, when decoding a P or B slice, no more syntax elements except one or more of the following are parsed after cu_skip_flag[ x0 ][ y0 ]: the IBC mode flag pred_mode_ibc_flag [ x0 ][ y0 ], and the merge_data( ) syntax structure; when decoding an I slice, no more syntax elements except merge_idx[ x0 ][ y0 ] are parsed after cu_skip_flag[ x0 ][ y0 ]. cu_skip_flag[ x0 ][ y0 ] equal to 0 specifies that the coding unit is not skipped. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

When cu_skip_flag[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

pred_mode_flag equal to 0 specifies that the current coding unit is coded in inter prediction mode. pred_mode_flag equal to 1 specifies that the current coding unit is coded in intra prediction mode.

When pred_mode_flag is not present, it is inferred as follows:

- If modeType is equal to MODE_TYPE_INTER, pred_mode_flag is inferred to be equal to 0.

- Oherwise, if cu_skip_flag[ x0 ][ y0 ] is equal to 1, pred_mode_flag is inferred to be equal to 0.

- Otherwise, pred_mode_flag is inferred to be equal to 1.

The variable CuPredMode[ chType ][ x ][ y ] is derived as follows for x = x0..x0 + cbWidth − 1 and y = y0..y0 + cbHeight − 1:

- If pred_mode_flag is equal to 0, CuPredMode[ chType ][ x ][ y ] is set equal to MODE_INTER.

- Otherwise (pred_mode_flag is equal to 1), CuPredMode[ chType ][ x ][ y ] is set equal to MODE_INTRA.

pred_mode_ibc_flag equal to 1 specifies that the current coding unit is coded in IBC prediction mode. pred_mode_ibc_flag equal to 0 specifies that the current coding unit is not coded in IBC prediction mode.

When pred_mode_ibc_flag is not present, it is inferred as follows:

- If cu_skip_flag[ x0 ][ y0 ] is equal to 1, and cbWidth is equal to 4, and cbHeight is equal to 4, pred_mode_ibc_flag is inferred to be equal 1.

- Otherwise, if both cbWidth and cbHeight are equal to 128, pred_mode_ibc_flag is inferred to be equal to 0.

- Otherwise, if modeType is equal to MODE_TYPE_INTER, pred_mode_ibc_flag is inferred to be equal to 0.

- Otherwise, if treeType is equal to DUAL_TREE_CHROMA, pred_mode_ibc_flag is inferred to be equal to 0.

- Otherwise, if sps_ibc_enabled_flag is equal to 0, pred_mode_ibc_flag is inferred to be equal to 0.

- Otherwise, pred_mode_ibc_flag is infered to be equal to 1 if pred_mode_flag is equal to 0, and 0 when pred_mode_flag is equal to 1

When pred_mode_ibc_flag is equal to 1, the variable CuPredMode[ chType ][ x ][ y ] is set to be equal to MODE_IBC for x = x0..x0 + cbWidth − 1 and y = y0..y0 + cbHeight − 1.

pred_mode_plt_flag specifies the use of palette mode in the current coding unit. pred_mode_plt_flag equal to 1 indicates that palette mode is applied in the current coding unit. pred_mode_plt_flag equal to 0 indicates that palette mode is not applied in the current coding unit. When pred_mode_plt_flag is not present, it is inferred to be equal to 0.

When pred_mode_plt_flag is equal to 1, the variable CuPredMode[ x ][ y ] is set to be equal to MODE_PLT for x = x0..x0 + cbWidth − 1 and y = y0..y0 + cbHeight − 1.

intra_bdpcm_flag equal to 1 specifies that BDPCM is applied to the current luma coding block at the location ( x0, y0 ), i.e. the transform is skipped, the intra luma prediction mode is specified by intra_bdpcm_dir_flag. intra_bdpcm_flag equal to 0 specifies that BDPCM is not applied to the current luma coding block at the location ( x0, y0 ).

When intra_bdpcm_flag is not present it is inferred to be equal to 0.

The variable BdpcmFlag[ x ][ y ] is set equal to intra_bdpcm_flag for x = x0..x0 + cbWidth − 1 and y = y0..y0 + cbHeight − 1.

intra_bdpcm_dir_flag equal to 0 specifies that the BDPCM prediction direction is horizontal. intra_bdpcm_dir_flag equal to 1 specifies that the BDPCM prediction direction is vertical.

The variable BdpcmDir[ x ][ y ] is set equal to intra_bdpcm_dir_flag for x = x0..x0 + cbWidth − 1 and y = y0..y0 + cbHeight − 1.

intra_mip_flag[ x0 ][ y0 ] equal to 1 specifies that the intra prediction type for luma samples is matrix-based intra prediction. intra_mip_flag[ x0 ][ y0 ] equal to 0 specifies that the intra prediction type for luma samples is not matrix-based intra prediction.

When intra_mip_flag[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

intra_mip_mode[ x0 ][ y0 ] specifies the matrix-based intra prediction mode for luma samples. The array indices x0, y0 specify the location ( x0 , y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

intra_luma_ref_idx[ x0 ][ y0 ] specifies the intra prediction reference line index IntraLumaRefLineIdx[ x ][ y ] for x = x0..x0 + cbWidth − 1 and y = y0..y0 + cbHeight − 1 as specified in Table 7-15.

When intra_luma_ref_idx[ x0 ][ y0 ] is not present it is inferred to be equal to 0.

Table 7-15 – Specification of IntraLumaRefLineIdx[ x ][ y ] based on intra_luma_ref_idx[ x0 ][ y0 ].

| intra_luma_ref_idx[ x0 ][ y0 ] | IntraLumaRefLineIdx[ x ][ y ] x = x0..x0 + cbWidth − 1 y = y0..y0 + cbHeight − 1 |
| --- | --- |
| 0 | 0 |
| 1 | 1 |
| 2 | 3 |

intra_subpartitions_mode_flag[ x0 ][ y0 ] equal to 1 specifies that the current intra coding unit is partitioned into NumIntraSubPartitions[ x0 ][ y0 ] rectangular transform block subpartitions. intra_subpartitions_mode_flag[ x0 ][ y0 ] equal to 0 specifies that the current intra coding unit is not partitioned into rectangular transform block subpartitions.

When intra_subpartitions_mode_flag[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

intra_subpartitions_split_flag[ x0 ][ y0 ] specifies whether the intra subpartitions split type is horizontal or vertical. When intra_subpartitions_split_flag[ x0 ][ y0 ] is not present, it is inferred as follows:

- If cbHeight is greater than MaxTbSizeY, intra_subpartitions_split_flag[ x0 ][ y0 ] is inferred to be equal to 0.

- Otherwise (cbWidth is greater than MaxTbSizeY), intra_subpartitions_split_flag[ x0 ][ y0 ] is inferred to be equal to 1.

The variable IntraSubPartitionsSplitType specifies the type of split used for the current luma coding block as illustrated in Table 7-16. IntraSubPartitionsSplitType is derived as follows:

- If intra_subpartitions_mode_flag[ x0 ][ y0 ] is equal to 0, IntraSubPartitionsSplitType is set equal to 0.

- Otherwise, the IntraSubPartitionsSplitType is set equal to 1 + intra_subpartitions_split_flag[ x0 ][ y0 ].

Table 7-16 – Name association to IntraSubPartitionsSplitType

| IntraSubPartitionsSplitType | Name of IntraSubPartitionsSplitType |
| --- | --- |
| 0 | ISP_NO_SPLIT |
| 1 | ISP_HOR_SPLIT |
| 2 | ISP_VER_SPLIT |

The variable NumIntraSubPartitions specifies the number of transform block subpartitions into which an intra luma coding block is divided. NumIntraSubPartitions is derived as follows:

- If IntraSubPartitionsSplitType is equal to ISP_NO_SPLIT, NumIntraSubPartitions is set equal to 1.

- Otherwise, if one of the following conditions is true, NumIntraSubPartitions is set equal to 2:

  - cbWidth is equal to 4 and cbHeight is equal to 8,

  - cbWidth is equal to 8 and cbHeight is equal to 4.

- Otherwise, NumIntraSubPartitions is set equal to 4.

The syntax elements intra_luma_mpm_flag[ x0 ][ y0 ], intra_luma_not_planar_flag[ x0 ][ y0 ], intra_luma_mpm_idx[ x0 ][ y0 ] and intra_luma_mpm_remainder[ x0 ][ y0 ] specify the intra prediction mode for luma samples. The array indices x0, y0 specify the location ( x0 , y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture. When intra_luma_mpm_flag[ x0 ][ y0 ] is equal to 1, the intra prediction mode is inferred from a neighbouring intra-predicted coding unit according to clause 8.4.2.

When intra_luma_mpm_flag[ x0 ][ y0 ] is not present, it is inferred to be equal to 1.

When intra_luma_not_planar_flag[ x0 ][ y0 ] is not present, it is inferred to be equal to 1.

cclm_mode_flag equal to 1 specifies that one of the INTRA_LT_CCLM, INTRA_L_CCLM and INTRA_T_CCLM intra chroma prediction modes is applied. cclm_mode_flag equal to 0 specifies that none of the INTRA_LT_CCLM, INTRA_L_CCLM and INTRA_T_CCLM intra chroma prediction modes is applied.

When cclm_mode_flag is not present, it is inferred to be equal to 0.

cclm_mode_idx specifies which one of the INTRA_LT_CCLM, INTRA_L_CCLM and INTRA_T_CCLM intra chroma prediction modes is applied.

intra_chroma_pred_mode specifies the intra prediction mode for chroma samples. When intra_chroma_pred_mode is not present, it is inferred to be equal to 0.

general_merge_flag[ x0 ][ y0 ] specifies whether the inter prediction parameters for the current coding unit are inferred from a neighbouring inter-predicted partition. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

When general_merge_flag[ x0 ][ y0 ] is not present, it is inferred as follows:

- If cu_skip_flag[ x0 ][ y0 ] is equal to 1, general_merge_flag[ x0 ][ y0 ] is inferred to be equal to 1.

- Otherwise, general_merge_flag[ x0 ][ y0 ] is inferred to be equal to 0.

mvp_l0_flag[ x0 ][ y0 ] specifies the motion vector predictor index of list 0 where x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

When mvp_l0_flag[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

mvp_l1_flag[ x0 ][ y0 ] has the same semantics as mvp_l0_flag, with l0 and list 0 replaced by l1 and list 1, respectively.

inter_pred_idc[ x0 ][ y0 ] specifies whether list0, list1, or bi-prediction is used for the current coding unit according to Table 7-17. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

Table 7-17 – Name association to inter prediction mode

| inter_pred_idc | Name of inter_pred_idc |  |  |
| --- | --- | --- | --- |
|  | ( cbWidth + cbHeight ) > 12 | ( cbWidth + cbHeight ) = = 12 | ( cbWidth + cbHeight ) = = 8 |
| 0 | PRED_L0 | PRED_L0 | n.a. |
| 1 | PRED_L1 | PRED_L1 | n.a. |
| 2 | PRED_BI | n.a. | n.a. |

When inter_pred_idc[ x0 ][ y0 ] is not present, it is inferred to be equal to PRED_L0.

sym_mvd_flag[ x0 ][ y0 ] equal to 1 specifies that the syntax elements ref_idx_l0[ x0 ][ y0 ] and ref_idx_l1[ x0 ][ y0 ], and the mvd_coding( x0, y0, refList ,cpIdx ) syntax structure for refList equal to 1 are not present. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

When sym_mvd_flag[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

ref_idx_l0[ x0 ][ y0 ] specifies the list 0 reference picture index for the current coding unit. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

When ref_idx_l0[ x0 ][ y0 ] is not present it is inferred as follows:

- If sym_mvd_flag[ x0 ][ y0 ] is equal to 1, ref_idx_l0[ x0 ][ y0 ] is inferred to be equal to RefIdxSymL0.

- Otherwise (sym_mvd_flag[ x0 ][ y0 ] is equal to 0), ref_idx_l0[ x0 ][ y0 ] is inferred to be equal to 0.

ref_idx_l1[ x0 ][ y0 ] has the same semantics as ref_idx_l0, with l0, L0 and list 0 replaced by l1, L1 and list 1, respectively.

inter_affine_flag[ x0 ][ y0 ] equal to 1 specifies that for the current coding unit, when decoding a P or B slice, affine model based motion compensation is used to generate the prediction samples of the current coding unit. inter_affine_flag[ x0 ][ y0 ] equal to 0 specifies that the coding unit is not predicted by affine model based motion compensation. When inter_affine_flag[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

cu_affine_type_flag[ x0 ][ y0 ] equal to 1 specifies that for the current coding unit, when decoding a P or B slice, 6-parameter affine model based motion compensation is used to generate the prediction samples of the current coding unit. cu_affine_type_flag[ x0 ][ y0 ] equal to 0 specifies that 4-parameter affine model based motion compensation is used to generate the prediction samples of the current coding unit.

MotionModelIdc[ x ][ y ] represents motion model of a coding unit as illustrated in Table 7-18. The array indices x, y specify the luma sample location ( x, y ) relative to the top-left luma sample of the picture.

The variable MotionModelIdc[ x ][ y ] is derived as follows for x = x0..x0 + cbWidth − 1 and y = y0..y0 + cbHeight − 1:

- If general_merge_flag[ x0 ][ y0 ] is equal to 1, the following applies:

MotionModelIdc[ x ][ y ] = merge_subblock_flag[ x0 ][ y0 ]	(7-139)

- Otherwise (general_merge_flag[ x0 ][ y0 ] is equal to 0), the following applies:

MotionModelIdc[ x ][ y ] = inter_affine_flag[ x0 ][ y0 ] + cu_affine_type_flag[ x0 ][ y0 ]	(7-140)

Table 7-18 – Interpretation of MotionModelIdc[ x0 ][ y0 ]

| MotionModelIdc[ x ][ y ] | Motion model for motion compensation |
| --- | --- |
| 0 | Translational motion |
| 1 | 4-parameter affine motion |
| 2 | 6-parameter affine motion |

amvr_flag[ x0 ][ y0 ] specifies the resolution of motion vector difference. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture. amvr_flag[ x0 ][ y0 ] equal to 0 specifies that the resolution of the motion vector difference is 1/4 of a luma sample. amvr_flag[ x0 ][ y0 ] equal to 1 specifies that the resolution of the motion vector difference is further specified by amvr_precision_flag[ x0 ][ y0 ].

When amvr_flag[ x0 ][ y0 ] is not present, it is inferred as follows:

- If CuPredMode[ chType ][ x0 ][ y0 ] is equal to MODE_IBC, amvr_flag[ x0 ][ y0 ] is inferred to be equal to 1.

- Otherwise ( CuPredMode[ chType ][ x0 ][ y0 ] is not equal to MODE_IBC ), amvr_flag[ x0 ][ y0 ] is inferred to be equal to 0.

amvr_precision_idx[ x0 ][ y0 ] equal to 0 specifies that the resolution of the motion vector difference with AmvrShift as defined in Table 7-19. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

When amvr_precision_flag[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

The motion vector differences are modified as follows:

- If inter_affine_flag[ x0 ][ y0 ] is equal to 0, the the variables MvdL0[ x0 ][ y0 ][ 0 ], MvdL0[ x0 ][ y0 ][ 1 ], MvdL1[ x0 ][ y0 ][ 0 ], MvdL1[ x0 ][ y0 ][ 1 ] are modified as follows:

MvdL0[ x0 ][ y0 ][ 0 ] = MvdL0[ x0 ][ y0 ][ 0 ]  << AmvrShift	(7-141)

MvdL0[ x0 ][ y0 ][ 1 ] = MvdL0[ x0 ][ y0 ][ 1 ]  <<  AmvrShift	(7-142)

MvdL1[ x0 ][ y0 ][ 0 ] = MvdL1[ x0 ][ y0 ][ 0 ]  <<  AmvrShift	(7-143)

MvdL1[ x0 ][ y0 ][ 1 ] = MvdL1[ x0 ][ y0 ][ 1 ]  <<  AmvrShift	(7-144)

- Otherwise (inter_affine_flag[ x0 ][ y0 ] is equal to 1), the variables MvdCpL0[ x0 ][ y0 ][ 0 ][ 0 ], MvdCpL0[ x0 ][ y0 ][ 0 ][ 1 ], MvdCpL0[ x0 ][ y0 ][ 1 ][ 0 ], MvdCpL0[ x0 ][ y0 ][ 1 ][ 1 ], MvdCpL0[ x0 ][ y0 ][ 2 ][ 0 ] and MvdCpL0[ x0 ][ y0 ][ 2 ][ 1 ] are modified as follows:

MvdCpL0[ x0 ][ y0 ][ 0 ][ 0 ] = MvdCpL0[ x0 ][ y0 ][ 0 ][ 0 ]  <<  AmvrShift	(7-145)

MvdCpL1[ x0 ][ y0 ] [ 0 ][ 1 ] = MvdCpL1[ x0 ][ y0 ][ 0 ][ 1 ]  <<  AmvrShift	(7-146)

MvdCpL0[ x0 ][ y0 ][ 1 ][ 0 ] = MvdCpL0[ x0 ][ y0 ][ 1 ][ 0 ]  <<  AmvrShift	(7-147)

MvdCpL1[ x0 ][ y0 ] [ 1 ][ 1 ] = MvdCpL1[ x0 ][ y0 ][ 1 ][ 1 ]  <<  AmvrShift	(7-148)

MvdCpL0[ x0 ][ y0 ][ 2 ][ 0 ] = MvdCpL0[ x0 ][ y0 ][ 2 ][ 0 ]  <<  AmvrShift	(7-149)

MvdCpL1[ x0 ][ y0 ] [ 2 ][ 1 ] = MvdCpL1[ x0 ][ y0 ][ 2 ][ 1 ]  <<  AmvrShift	(7-150)

Table 7-19 – Specification of AmvrShift.

| amvr_flag | amvr_precision_idx | AmvrShift |  |  |
| --- | --- | --- | --- | --- |
|  |  | inter_affine_flag = =1 | CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_IBC ) | inter_affine_flag = =0 && CuPredMode[ chType ][ x0 ][ y0 ] ! = MODE_IBC |
| 0 | - | 2 (1/4 luma sample) | - | 2 (1/4 luma sample) |
| 1 | 0 | 0 (1/16 luma sample) | 4 (1 luma sample) | 3 (1/2 luma sample) |
| 1 | 1 | 4 (1 luma sample) | 6 (4 luma samples) | 4 (1 luma sample) |
| 1 | 2 | - | - | 6 (4 luma samples) |

bcw_idx[ x0 ][ y0 ] specifies the weight index of bi-prediction with CU weights. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

When bcw_idx[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

cu_cbf equal to 1 specifies that the transform_tree( ) syntax structure is present for the current coding unit. cu_cbf equal to 0 specifies that the transform_tree( ) syntax structure is not present for the current coding unit.

When cu_cbf is not present, it is inferred as follows:

- If cu_skip_flag[ x0 ][ y0 ] is equal to 1 or pred_mode_plt_flag is equal to 1, cu_cbf is inferred to be equal to 0.

- Otherwise, cu_cbf is inferred to be equal to 1.

cu_sbt_flag equal to 1 specifies that for the current coding unit, subblock transform is used. cu_sbt_flag equal to 0 specifies that for the current coding unit, subblock transform is not used.

When cu_sbt_flag is not present, its value is inferred to be equal to 0.

NOTE – : When subblock transform is used, a coding unit is split into two transform units; one transform unit has residual data, the other does not have residual data.

cu_sbt_quad_flag equal to 1 specifies that for the current coding unit, the subblock transform includes a transform unit of 1/4 size of the current coding unit. cu_sbt_quad_flag equal to 0 specifies that for the current coding unit the subblock transform includes a transform unit of 1/2 size of the current coding unit.

When cu_sbt_quad_flag is not present, its value is inferred to be equal to 0.

cu_sbt_horizontal_flag equal to 1 specifies that the current coding unit is split horizontally into 2 transform units. cu_sbt_horizontal_flag[ x0 ][ y0 ] equal to 0 specifies that the current coding unit is split vertically into 2 transform units.

When cu_sbt_horizontal_flag is not present, its value is derived as follows:

- If cu_sbt_quad_flag is equal to 1, cu_sbt_horizontal_flag is set to be equal to allowSbtHorQ.

- Otherwise (cu_sbt_quad_flag is equal to 0), cu_sbt_horizontal_flag is set to be equal to allowSbtHorH.

cu_sbt_pos_flag equal to 1 specifies that the tu_cbf_luma, tu_cbf_cb and tu_cbf_cr of the first transform unit in the current coding unit are not present in the bitstream. cu_sbt_pos_flag equal to 0 specifies that the tu_cbf_luma, tu_cbf_cb and tu_cbf_cr of the second transform unit in the current coding unit are not present in the bitstream.

The variable SbtNumFourthsTb0 is derived as follows:

sbtMinNumFourths = cu_sbt_quad_flag  ?  1  :  2		(7-151)

SbtNumFourthsTb0 = cu_sbt_pos_flag  ?  ( 4 − sbtMinNumFourths )  :  sbtMinNumFourths	(7-152)

lfnst_idx[ x0 ][ y0 ] specifies whether and which one of the two low frequency non-separable transform kernels in a selected transform set is used. lfnst_idx[ x0 ][ y0 ] equal to 0 specifies that the low frequency non-separable transform is not used. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left sample of the considered transform block relative to the top-left sample of the picture.

When lfnst_idx[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

When ResetIbcBuf is equal to 1, the following applies:

- For x = 0..IbcBufWidthY − 1 and y = 0..CtbSizeY − 1, the following assignments are made:

IbcVirBuf[ 0 ][ x ][ y ] = −1		(7-153)

- The variable ResetIbcBuf is set equal to 0.

When x0 % VSize is equal to 0 and y0 % VSize is equal to 0, the following assignments are made for x = x0..x0 + VSize − 1 and y = y0..y0 + VSize − 1:

IbcVirBuf[ 0 ][ x % IbcBufWidthY ][ y % CtbSizeY ] = −1	(7-154)

##### 9.3.4.2.2	Derivation process of ctxInc using left and above syntax elements

Input to this process is the luma location ( x0, y0 ) specifying the top-left luma sample of the current luma block relative to the top-left sample of the current picture, the colour component cIdx, the current coding quadtree depth cqtDepth, the dual tree channel type chType, the width and the height of the current coding block in luma samples cbWidth and cbHeight, and the variables allowSplitBtVer, allowSplitBtHor, allowSplitTtVer, allowSplitTtHor, and allowSplitQt as derived in the coding tree semantics in clause 7.4.9.4.

Output of this process is ctxInc.

The location ( xNbL, yNbL ) is set equal to ( x0 − 1, y0 ) and the derivation process for neighbouring block availability as specified in clause 6.4.4 is invoked with the location ( xCurr, yCurr ) set equal to ( x0, y0 ), the neighbouring location ( xNbY, yNbY ) set equal to ( xNbL, yNbL ), checkPredModeY set equal to FALSE, and cIdx as inputs, and the output is assigned to availableL.

The location ( xNbA, yNbA ) is set equal to ( x0, y0 − 1 ) and the derivation process for neighbouring block availability as specified in clause 6.4.4 is invoked with the location ( xCurr, yCurr ) set equal to ( x0, y0 ), the neighbouring location ( xNbY, yNbY ) set equal to ( xNbA, yNbA ), checkPredModeY set equal to FALSE, and cIdx as inputs, and the output is assigned to availableA.

The assignment of ctxInc is specified as follows with condL and condA specified in Table 9-83:

- For the syntax elements alf_ctb_flag[ x0 ][ y0 ][ cIdx ], split_qt_flag, split_cu_flag, cu_skip_flag[ x0 ][ y0 ], pred_mode_ibc_flag[ x0 ][ y0 ], intra_mip_flag[ x0 ][ y0 ], inter_affine_flag[ x0 ][ y0 ] and merge_subblock_flag[ x0 ][ y0 ]:

ctxInc = ( condL  &&  availableL ) + ( condA  &&  availableA ) + ctxSetIdx * 3	(9-30)

- For the syntax elements pred_mode_flag[ x0 ][ y0 ] and mode_constraint_flag:

ctxInc = ( condL  &&  availableL ) | | ( condA  &&  availableA )	(9-31)

Table 9-83 – Specification of ctxInc using left and above syntax elements

| Syntax element | condL | condA | ctxSetIdx |
| --- | --- | --- | --- |
| alf_ctb_flag[ x0 ][ y0 ][ cIdx ] | alf_ctb_flag[ xNbL ][ yNbL ][ cIdx ] | alf_ctb_flag[ xNbA ][ yNbA ][cIdx ] | cIdx |
| split_qt_flag | CqtDepth[ chType ][ xNbL ][ yNbL ] > cqtDepth | CqtDepth[ chType ][ xNbA ][ yNbA ] > cqtDepth | ( cqtDepth < 2) ? 0 : 1 |
| split_cu_flag | CbHeight[ chType ][ xNbL ][ yNbL ] < cbHeight | CbWidth[ chType ][ xNbA ][ yNbA ] < cbWidth | ( allowSplitBtVer + allowSplitBtHor + allowSplitTtVer + allowSplitTtHor + 2 * allowSplitQt − 1 ) / 3 |
| mode_constraint_flag | CuPredMode[ chType ][ xNbL ][ yNbL ] = = MODE_INTRA | CuPredMode[ chType ][ xNbA ][ yNbA ] = = MODE_INTRA | 0 |
| cu_skip_flag[ x0 ][ y0 ] | cu_skip_flag[ xNbL ][ yNbL ] | cu_skip_flag[ xNbA ][ yNbA ] | 0 |
| pred_mode_flag[ x0 ][ y0 ] | CuPredMode[ chType ][ xNbL ][ yNbL ] = = MODE_INTRA ¦ ¦ MODE_PLT | CuPredMode[ chType ][ xNbA ][ yNbA ] = = MODE_INTRA ¦ ¦ MODE_PLT | 0 |
| pred_mode_ibc_flag[ x0 ][ y0 ] | CuPredMode[ chType ][ xNbL ][ yNbL ] = = MODE_IBC | CuPredMode[ chType ][ xNbA ][ yNbA ] = = MODE_IBC | 0 |
| intra_mip_flag[ x0 ][ y0 ] | intra_mip_flag[ xNbL ][ yNbL ] | intra_mip_flag[ xNbA ][ yNbA ] | 0 |
| merge_subblock_flag[ x0 ][ y0 ] | merge_subblock_flag[ xNbL ][ yNbL ] ¦ inter_affine_flag[ xNbL ][ yNbL ] | merge_subblock_flag[ xNbA ][ yNbA ] ¦ inter_affine_flag[ xNbA ][ yNbA ] | 0 |
| inter_affine_flag [ x0 ][ y0 ] | merge_subblock_flag[ xNbL ][ yNbL ] ¦ inter_affine_flag[ xNbL ][ yNbL ] | merge_subblock_flag[ xNbA ][ yNbA ] ¦ inter_affine_flag[ xNbA ][ yNbA ] | 0 |

# Reference

- X. Xu, Y.-H. Chao, Y.-C. Sun, J. Xu, “Description of Core Experiment 8 (CE8): 4:4:4 Screen Content Coding Tools”, JVET-O2028, 15th meeting, Gothenburg, SE, July 2019.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).