| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0476 |
| --- | --- |

| Title: | Non-CE8: Palette mode and prediction mode signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yung-Hsuan Chao, Chao-Hsiung Hung, Wei-Jung Chien, Marta Karczewicz | Tel: Email: | +1-858-8452163 yunghsua@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In this document, a cleanup of prediction mode signaling, i.e. signaling MODE_INTRA, MODE_INTER, MODE_IBC, and MODE_PLT, is proposed. The proposed signaling unifies the signaling condition of prediction mode for I slice and P/B slice, and IBC enabled and disabled. The results on YUV4:4:4 sequences show minor loss compared to VTM6.0:

IBC ON:

Dual Tree OFF: 0.08 % AI, -0.06 % RA, 0.08 % LB

Dual Tree ON: 0.01 % AI, 0.02 % RA, 0.08% LB

IBC OFF:

Dual Tree OFF: 0.00 % AI, -0.01 % RA, -0.06 % LB

Dual Tree ON: 0.00 % AI, -0.03 % RA, 0.00% LB

The results on YUV4:2:0 sequences compared to VTM6.0 are as follows:

Overall: 0.00 % AI, 0.00 % RA, -0.01 % LB

ClassF: 0.01 % AI, 0.03 % RA, 0.10 % LB

ClassSCC: 0.02 % AI, -0.01 % RA, -0.21 % LB

# Introduction

In VTM6.0, the conditions for prediction mode signaling are different for I slice and P/B slice, and for IBC enabled and disabled. When PLT and IBC are enabled, e.g., PLT and IBC are enabled in SPS and for CU size smaller or euqal to 64x64, Figure 1 shows the CU signaling flow chart in I slice.

Figure 1: Prediction mode signaling of VTM6.0 in I slice when IBC and PLT are enabled

The first bin is signaled to indicate if the current cu is skip mode. If it is skip mode, this CU must be MODE_IBC (intra block copy mode). Otherwise, another bin is signaled to indicate whether the current prediction mode is MODE_IBC. If it is not MODE_IBC, one bin is then signaled to indicate whether the current prediction mode is MODE_PLT (palette mode). If the mode is not MODE_PLT, the block is of MODE_INTRA.

Figure 2 shows the CU signaling flow chart in P/B slice when PLT and IBC are enabled.

Figure 2: Prediction mode signaling of VTM6.0 in P/B slice when IBC and PLT are enabled

Same as in Figure 1, The flag for CU_SKIP will be signaled first. If current mode is skip mode, the second bin is signaled to indicate the current prediction mode is MODE_IBC or not (MODE_INTER, regular inter prediction mode). If current mode is not skip mode, however, instead of signaling IBC flag as in Figure 1, the second bin is signaled to indicate whether the current prediction mode is intra mode or palette mode. If the condition is TRUE, the third bin is signaled to indicate the current mode is MODE_PLT or not (MODE_INTRA). Otherwise, the third bin is signaled to indicate the current mode is MODE_IBC or not (MODE_INTER).

Figure 3 shows CU signaling flow chart in P/B tiles/slice when IBC is disabled. CU_SKIP will also be signaled first. If the current prediction mode is skip mode, it must be MODE_INTER. If the current prediction mode is not skip mode, instead of signaling (INTRA or PLT) flag as in Figure 2. the second bin is signaled to indicate whether the current prediction mode is MODE_INTRA. If prediction mode is not intra mode, the third bin is signaled to indicate the current prediction mode is MODE_PLT or not (MODE_INTER).

Figure 3: Prediction mode signaling of VTM6.0 in P/B slice when IBC is disabled

The conditions of mode signaling and the semantics are different depending on different configurations, which complicates the implementation and specification text.

# Proposed Method

In this proposal, a unified prediction mode signaling scheme is proposed, as shown in Figure 4.

Figure 4: Proposed unification for prediction mode signaling. The path marked in red is enabled when palette mode is enabled. The path marked in blue is enabled when IBC is enabled.

The flag for CU_SKIP will be signaled first. If current mode is skip mode, the second bin is signaled to indicate the current prediction mode is MODE_IBC or not (MODE_INTER, regular inter prediction mode). If current mode is not skip mode, the second bin is signaled to indicate whether the current prediction mode is intra mode or palette mode. If the condition is TRUE, the third bin is signaled to indicate the current mode is MODE_PLT or not (MODE_INTRA). Otherwise, the third bin is signaled to indicate the current mode is MODE_IBC or not (MODE_INTER).

When IBC is disabled, e.g. when IBC is disabled in SPS or when CU size is larger than 64x64, the proposed mode signaling in Figure 4 with IBC flag signaling and MODE_IBC (marked as blue) removed can be applied.

When palette is disabled, e.g. when PLT is disabled in SPS or when CU size is larger than 64x64, the proposed mode signaling in Figure 4 with PLT flag signaling and MODE_PLT (marked as red) removed can be applied.

When inter prediction is disabled, e.g. for I slice, the proposed mode signaling in Figure 4 with IBC flag signaling and MODE_INTER removed can be applied.

# Simulation Results

3.1. YUV4:4:4 IBC ON

The proposed method is evaluated according to the CE8 description [2] for the intra-only (AI), random-access (RA) and low delay B slice (LB) configurations. When IBC is turned on, the IBC, HashME, BDPCM, and PLT are turned on for all Classes. The results with DualITree OFF and DualITree ON are shown in Table 1 and Table 2.

Table 1: Results of IBC = 1 and DualITree = 0

|  | All Intra Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0_PLT1_IBC1_DT0 |  |  |  |  |  |  |  |  |
|  | Y |  | U |  | V |  | EncT |  | DecT |
| TGM 1080p | 0.07% |  | 0.00% |  | 0.02% |  | 100% |  | 96% |
| TGM 720p | 0.19% |  | 0.01% |  | 0.02% |  | 100% |  | 100% |
| Animation | 0.00% |  | 0.01% |  | 0.01% |  | 99% |  | 101% |
| Mixed content | 0.04% |  | -0.09% |  | -0.03% |  | 99% |  | 98% |
| Camera-Captured content | 0.01% |  | -0.07% |  | 0.02% |  | 100% |  | 102% |
| Overall | 0.08% |  | -0.02% |  | 0.01% |  | 100% |  | 99% |
|  | Random access Main10 |  |  |  |  |  |  |  |  |
|  | Over VTM-6.0_PLT1_IBC1_DT0 |  |  |  |  |  |  |  |  |
|  | Y |  | U |  | V |  | EncT |  | DecT |
| TGM 1080p | -0.09% |  | -0.05% |  | 0.04% |  | 98% |  | 100% |
| TGM 720p | -0.08% |  | 0.02% |  | 0.20% |  | 100% |  | 100% |
| Animation | 0.01% |  | -0.03% |  | -0.09% |  | 101% |  | 102% |
| Mixed content | -0.10% |  | 0.04% |  | -0.18% |  | 99% |  | 100% |
| Camera-Captured content | -0.02% |  | 0.13% |  | -0.17% |  | 100% |  | 100% |
| Overall | -0.06% |  | 0.01% |  | -0.01% |  | 99% |  | 100% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |
|  | Over VTM-6.0_PLT1_IBC1_DT0 |  |  |  |  |  |  |  |  |
|  | Y |  | U |  | V |  | EncT |  | DecT |
| TGM 1080p | 0.02% |  | 0.05% |  | 0.10% |  | 102% |  | 100% |
| TGM 720p | 0.22% |  | -0.09% |  | 0.75% |  | 102% |  | 100% |
| Animation | 0.09% |  | -0.02% |  | -0.08% |  | 99% |  | 93% |
| Mixed content | 0.04% |  | 0.67% |  | -0.21% |  | 101% |  | 102% |
| Camera-Captured content | -0.03% |  | 0.05% |  | 0.04% |  | 100% |  | 99% |
| Overall | 0.08% |  | 0.12% |  | 0.16% |  | 101% |  | 99% |

Table 2: Results of IBC = 1 and DualITree = 1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0_PLT1_IBC1_DT1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.00% | -0.02% | -0.01% | 100% | 101% |
| TGM 720p | 0.02% | 0.00% | 0.08% | 100% | 101% |
| Animation | 0.01% | 0.00% | -0.02% | 99% | 97% |
| Mixed content | 0.04% | 0.09% | 0.10% | 101% | 105% |
| Camera-Captured content | -0.03% | 0.06% | 0.01% | 100% | 100% |
| Overall | 0.01% | 0.02% | 0.03% | 100% | 101% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-6.0_PLT1_IBC1_DT1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.03% | 0.06% | 0.00% | 101% | 101% |
| TGM 720p | 0.01% | -0.02% | 0.06% | 100% | 99% |
| Animation | 0.15% | 0.13% | 0.11% | 99% | 98% |
| Mixed content | 0.03% | -0.34% | -0.37% | 99% | 100% |
| Camera-Captured content | -0.03% | 0.10% | -0.01% | 99% | 98% |
| Overall | 0.02% | -0.02% | -0.03% | 100% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0_PLT1_IBC1_DT1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.14% | 0.18% | 0.22% | 101% | 101% |
| TGM 720p | 0.26% | 0.04% | 0.30% | 98% | 98% |
| Animation | 0.08% | 0.18% | 0.02% | 98% | 98% |
| Mixed content | -0.18% | 0.20% | -0.17% | 98% | 99% |
| Camera-Captured content | -0.02% | 0.04% | -0.19% | 97% | 93% |
| Overall | 0.08% | 0.13% | 0.08% | 99% | 98% |

3.2. YUV4:4:4 IBC OFF

The proposed method is evaluated according to the CE8 description [1] for the intra-only (AI), random-access (RA) and low delay B slice (LB) configurations. When IBC is turned off, the IBC is turned off while HashME, BDPCM, and PLT are turned on for all Classes. The results with DualITree OFF and DualITree ON are shown in Table 1 and Table 2.

Table 3: Results of IBC = 0 and DualITree = 0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0_PLT1_IBC0_DT0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.00% | 0.00% | 0.00% | 100% | 99% |
| TGM 720p | 0.00% | 0.00% | 0.00% | 101% | 103% |
| Animation | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Mixed content | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 99% | 97% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-6.0_PLT1_IBC0_DT0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.00% | 0.02% | 0.03% | 100% | 100% |
| TGM 720p | -0.03% | 0.00% | 0.01% | 100% | 101% |
| Animation | 0.07% | 0.06% | 0.08% | 100% | 101% |
| Mixed content | -0.01% | 0.06% | 0.06% | 100% | 100% |
| Camera-Captured content | -0.11% | 0.05% | -0.05% | 100% | 101% |
| Overall | -0.01% | 0.04% | 0.03% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0_PLT1_IBC0_DT0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.08% | 0.07% | 0.06% | 101% | 101% |
| TGM 720p | -0.13% | -0.03% | -0.08% | 101% | 98% |
| Animation | 0.02% | -0.01% | 0.09% | 100% | 99% |
| Mixed content | -0.02% | -0.21% | -0.11% | 102% | 100% |
| Camera-Captured content | -0.08% | 0.03% | -0.06% | 102% | 101% |
| Overall | -0.06% | -0.03% | -0.02% | 101% | 100% |

Table 4: Results of IBC = 0 and DualITree = 1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0_PLT1_IBC0_DT1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.00% | 0.00% | 0.00% | 100% | 103% |
| TGM 720p | 0.00% | 0.00% | 0.00% | 100% | 103% |
| Animation | 0.00% | 0.00% | 0.00% | 101% | 106% |
| Mixed content | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 102% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-6.0_PLT1_IBC0_DT1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.01% | 0.02% | 0.03% | 100% | 100% |
| TGM 720p | 0.00% | -0.02% | 0.00% | 99% | 99% |
| Animation | -0.03% | 0.06% | -0.01% | 101% | 101% |
| Mixed content | -0.08% | -0.03% | -0.15% | 101% | 100% |
| Camera-Captured content | -0.11% | -0.02% | -0.02% | 101% | 101% |
| Overall | -0.03% | 0.00% | -0.02% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0_PLT1_IBC0_DT1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.12% | 0.21% | 0.23% | 101% | 100% |
| TGM 720p | -0.02% | -0.10% | -0.07% | 100% | 98% |
| Animation | -0.10% | 0.13% | 0.05% | 102% | 103% |
| Mixed content | -0.03% | -0.04% | 0.02% | 100% | 101% |
| Camera-Captured content | -0.03% | -0.03% | 0.03% | 100% | 97% |
| Overall | 0.00% | 0.04% | 0.06% | 100% | 100% |

3.3. YUV4:2:0.

The proposed method is evaluated according to the CTC description [2] for the intra-only (AI), random-access (RA) and low delay B slice (LB) configurations. The results are shown in Table 5.

Table 5: Results of YUV 4:2:0

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0, CTC |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Y | U |  |  | V |  |  | EncT |  |  | DecT |  |  |
| Class A1 | 0.00% | 0.00% |  |  | 0.00% |  |  | 98% |  |  | 98% |  |  |
| Class A2 | 0.00% | 0.00% |  |  | 0.00% |  |  | 100% |  |  | 102% |  |  |
| Class B | 0.00% | 0.00% |  |  | 0.00% |  |  | 98% |  |  | 97% |  |  |
| Class C | 0.00% | 0.00% |  |  | 0.00% |  |  | 99% |  |  | 94% |  |  |
| Class E | 0.00% | 0.00% |  |  | 0.00% |  |  | 99% |  |  | 91% |  |  |
| Overall | 0.00% | 0.00% |  |  | 0.00% |  |  | 99% |  |  | 96% |  |  |
| Class D | 0.00% | 0.00% |  |  | 0.00% |  |  | 98% |  |  | 97% |  |  |
| Class F | 0.01% | 0.06% |  |  | -0.08% |  |  | 98% |  |  | 98% |  |  |
| Class TGM | 0.02% | 0.02% |  |  | 0.03% |  |  | 99% |  |  | 97% |  |  |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Over VTM-6.0, CTC |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Y |  | U |  |  | V |  |  | EncT |  |  | DecT |  |
| Class A1 | -0.01% |  | -0.05% |  |  | -0.04% |  |  | 100% |  |  | 101% |  |
| Class A2 | 0.03% |  | 0.09% |  |  | 0.05% |  |  | 98% |  |  | 97% |  |
| Class B | -0.02% |  | 0.10% |  |  | 0.06% |  |  | 99% |  |  | 99% |  |
| Class C | 0.01% |  | 0.10% |  |  | -0.08% |  |  | 100% |  |  | 98% |  |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.00% |  | 0.07% |  |  | 0.00% |  |  | 99% |  |  | 99% |  |
| Class D | -0.01% |  | -0.05% |  |  | 0.02% |  |  | 96% |  |  | 93% |  |
| Class F | 0.03% |  | -0.05% |  |  | 0.00% |  |  | 99% |  |  | 99% |  |
| Class TGM | -0.01% |  | -0.02% |  |  | -0.03% |  |  | 98% |  |  | 99% |  |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Over VTM-6.0, CTC |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Y |  |  | U |  |  | V |  |  | EncT |  |  | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.00% |  |  | 0.09% |  |  | 0.09% |  |  | 98% |  |  | 92% |
| Class C | -0.01% |  |  | 0.18% |  |  | 0.03% |  |  | 97% |  |  | 86% |
| Class E | -0.02% |  |  | 0.35% |  |  | -0.09% |  |  | 100% |  |  | 99% |
| Overall | -0.01% |  |  | 0.19% |  |  | 0.02% |  |  | 98% |  |  | 92% |
| Class D | -0.03% |  |  | 0.01% |  |  | 0.25% |  |  | 93% |  |  | 88% |
| Class F | 0.10% |  |  | -0.29% |  |  | 0.66% |  |  | 99% |  |  | 95% |
| Class TGM | -0.21% |  |  | -0.08% |  |  | -0.12% |  |  | 94% |  |  | 87% |

# Reference

- X. Xu, Y.-H. Chao, Y.-C. Sun, J. Xu, “Description of Core Experiment 8 (CE8): 4:4:4 Screen Content Coding Tools”, JVET-O2028, 15th meeting, Gothenburg, SE, July 2019.

- F. Bossen, J. Boyce, K. Sühring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Doc. JVET-N1010, 14th Meeting: Geneva, CH, 19–27 Mar. 2019.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).