| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0477-v4 |
| --- | --- |

| Title: | AHG18: On Last Position Signaling for Lossless |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Alican Nalci, Hilmi E. Egilmez, Muhammed Coban, and Marta Karczewicz 5775 Morehouse Drive San Diego, CA, U.S.A. 92121 | Tel: Email: | analci@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In AHG18 VTM-6.0-lossless branch, trans quant bypass (QB) mode provides a lossless alternative in VVC. This can be achieved by setting a CU-level cu_transquant_bypass_flag=1. In the lossy version of VVC, last position of primary transform coefficients is explicitly signaled and used in various inference schemes for LFNST and signaling of significant flags/groups. In the lossless setting, last position signaling may carry signaling overhead. This is because transform coefficients are in the frequency domain and lossless residuals are in the spatial domain and lossless residuals are not necessarily clustered in the top-left corner of a transform block.

# Proposal

In AHG18 VTM-6.0-lossless branch, trans quant bypass (QB) mode provides a lossless alternative in VVC  by setting cu_transquant_bypass_flag=1 [1, 2]. In this proposal we propose to disable last position signaling when a CU is lossless coded. Specifically, last position is assumed to be at the right bottom location of a transform block and hence is not signaled. As lossy mode of VVC uses an inference scheme for significant sub blocks and significance flag we, always signal significant sub-blocks and significant flags in the proposed method.

The following variants are proposed:

- Proposal 1: Disable Last Position Signaling for Lossless

- Proposal 2: Disable Last Position on top of JVET-O0082 Proposal #1

  - In JVET-O0082 Proposal #1, BDPCM is applied to both in TS residual coding and transform residual (TR) coding.

- Proposal 3: Disable Last Position on top of JVET-O0082 Proposal #4

  - JVET-O0082 Proposal #4, BDPCM is applied to both in TS residual coding and transform residual (TR) coding and chroma BDPCM is applied to only TS residual coding.

# Results

Proposal 1: Disable Last Position Signaling for Lossless

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | AHG18 LL | Proposal #1 |  | AHG18 LL | Proposal #1 |  | AHG18 LL | Proposal #1 |  |
| Class A1 | 2.2 | 2.2 | -0.41% | 2.3 | 2.3 | -0.40% |  |  |  |
| Class A2 | 1.7 | 1.8 | -0.44% | 1.9 | 1.9 | -0.36% |  |  |  |
| Class B | 2.1 | 2.1 | -0.50% | 2.3 | 2.3 | -0.51% | 2.3 | 2.3 | -25.50% |
| Class C | 2.1 | 2.1 | -0.62% | 2.6 | 2.6 | -0.55% | 2.6 | 2.6 | -0.55% |
| Class D | 1.9 | 2.0 | -0.59% | 2.8 | 2.8 | -0.56% | 2.7 | 2.8 | -0.56% |
| Class E | 2.9 | 3.0 | -1.01% |  |  |  | 3.3 | 3.3 | -0.57% |
| Class F | 5.5 | 5.5 | -0.13% | 39.4 | 39.2 | 0.01% | 60.7 | 60.2 | 0.09% |
| TGM | 12.3 | 12.1 | 1.01% | 112.6 | 111.6 | 0.93% | 142.4 | 141.0 | 1.04% |
| Overall | 2.2 | 2.2 | -0.59% | 2.3 | 2.3 | -0.47% | 2.6 | 2.5 | -8.82% |
| Enc Time[%] | 103% |  |  | 103% |  |  | #NUM! |  |  |
| Dec Time[%] | 95% |  |  | 98% |  |  | #NUM! |  |  |

Table 1. Results after disabling last position signaling for lossless.

(AI and RA results are complete, pending LDB results)

Proposal 2: Disable Last Position Signaling for Lossless on top of JVET-P0082 Proposal #1

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | AHG18 LL | Proposal #2 |  | AHG18 LL | Proposal #2 |  | AHG18 LL | Proposal #2 |  |
| Class A1 | 2.2 | 2.3 | -1.27% | 2.3 | 2.3 | -0.76% |  |  |  |
| Class A2 | 1.7 | 1.8 | -1.56% | 1.9 | 1.9 | -0.62% |  |  |  |
| Class B | 2.1 | 2.1 | -1.80% | 2.3 | 2.3 | -0.76% | 2.3 | 2.3 | -67.47% |
| Class C | 2.1 | 2.1 | -2.00% | 2.6 | 2.6 | -0.88% | 2.6 | 2.6 | -0.92% |
| Class D | 1.9 | 2.0 | -3.12% | 2.8 | 2.8 | -1.06% | 2.7 | 2.8 | -1.02% |
| Class E | 2.9 | 3.0 | -2.49% |  |  |  | 3.3 | 3.4 | -1.51% |
| Class F | 5.5 | 5.7 | -3.96% | 39.4 | 41.2 | -2.86% | 60.7 | 64.2 | -3.26% |
| TGM | 12.3 | 12.6 | -2.83% | 112.6 | 115.2 | -2.14% | 142.4 | 145.6 | -2.04% |
| Overall | 2.2 | 2.2 | -1.83% | 2.3 | 2.3 | -0.76% | 2.6 | 2.3 | -17.55% |
| Enc Time[%] | 117% |  |  | 114% |  |  | #NUM! |  |  |
| Dec Time[%] | 89% |  |  | 98% |  |  | #NUM! |  |  |

Table 2. Results after disabling last position signaling for lossless on top of JVET-P0082 Proposal #1. BDPCM is enabled for both TS residual coding and transform residual coding.

(AI and RA results are complete, pending LDB results)

Proposal 3: Disable Last Position Signaling for Lossless on top of JVET-P0082 Proposal #4

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | AHG18 LL | Proposal #3 |  | AHG18 LL | Proposal #3 |  | AHG18 LL | Proposal #3 |  |
| Class A1 | 2.2 | 2.3 | -2.07% | 2.3 | 2.3 | -1.10% |  |  |  |
| Class A2 | 1.7 | 1.8 | -2.00% | 1.9 | 1.9 | -0.66% |  |  |  |
| Class B | 2.1 | 2.1 | -2.42% | 2.3 | 2.3 | -0.89% | 2.3 | 2.3 | -67.63% |
| Class C | 2.1 | 2.1 | -2.76% | 2.6 | 2.6 | -1.04% | 2.6 | 2.6 | -1.04% |
| Class D | 1.9 | 2.0 | -4.25% | 2.8 | 2.8 | -1.28% | 2.7 | 2.8 | -1.18% |
| Class E | 2.9 | 3.1 | -4.28% |  |  |  | 3.3 | 3.4 | -1.71% |
| Class F | 5.5 | 5.9 | -5.59% | 39.4 | 41.7 | -3.66% | 60.7 | 64.7 | -3.82% |
| TGM | 12.3 | 13.1 | -5.98% | 112.6 | 116.4 | -3.08% | 142.4 | 146.4 | -2.65% |
| Overall | 2.2 | 2.3 | -2.68% | 2.3 | 2.3 | -0.92% | 2.6 | 2.3 | -17.68% |
| Enc Time[%] | 121% |  |  | 115% |  |  | #NUM! |  |  |
| Dec Time[%] | 85% |  |  | 98% |  |  | #NUM! |  |  |

Table 3. Results after disabling last position signaling for lossless on top of JVET-P0082 Proposal (#4). BDPCM is enabled for both TS residual coding and transform residual coding and chroma BDPCM is enabled for transform skip residual coding.

(AI and RA results are complete, pending LDB results)

# Conclusion

In this contribution we proposed to disable last position signaling for lossless coding. We presented our results on top of JVET-O0082 Proposals #1 and #4. Our results indicate that disabling last position provides overall bit-rate savings.

It is recommended to disable last position signalling for lossless coding in VVC.

# Spec Changes

#### 7.3.8.11 Residual coding syntax

| residual_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |  |
| --- | --- | --- |
| … |  |  |
| if( log2TbWidth > 0 && !cu_transquant_bypass_flag ) |  |  |
| last_sig_coeff_x_prefix | ae(v) |
| if( log2TbHeight > 0 && !cu_transquant_bypass_flag ) |  |
| last_sig_coeff_y_prefix | ae(v) |
| if( last_sig_coeff_x_prefix > 3 && !cu_transquant_bypass_flag ) |  |
| last_sig_coeff_x_suffix | ae(v) |
| if( last_sig_coeff_y_prefix > 3 && !cu_transquant_bypass_flag ) |  |
| last_sig_coeff_y_suffix | ae(v) |
| … |  |
| do { |  |
| if( lastScanPos = = 0 ) { |  |
| lastScanPos = numSbCoeff |  |
| lastSubBlock− − |  |
| } |  |
| lastScanPos− − |  |
| xS = DiagScanOrder[ log2TbWidth − log2SbW ][ log2TbHeight − log2SbH ] [ lastSubBlock ][ 0 ] |  |
| yS = DiagScanOrder[ log2TbWidth − log2SbW ][ log2TbHeight − log2SbH ] [ lastSubBlock ][ 1 ] |  |
| xC = ( xS << log2SbW ) + DiagScanOrder[ log2SbW ][ log2SbH ][ lastScanPos ][ 0 ] |  |
| yC = ( yS << log2SbH ) + DiagScanOrder[ log2SbW ][ log2SbH ][ lastScanPos ][ 1 ] |  |
| } while( ( xC != LastSignificantCoeffX ¦ ¦ cu_transquant_bypass_flag ) ¦ ¦ ( yC != LastSignificantCoeffY ¦ ¦ cu_transquant_bypass_flag ) ) |  |
| … |  |
| firstPosMode1 = −1 |  |
| for( n = firstPosMode0; n >= 0 && remBinsPass1 >= 4; n− − ) { |  |
| xC = ( xS << log2SbW ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 0 ] |  |
| yC = ( yS << log2SbH ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 1 ] |  |
| if( coded_sub_block_flag[ xS ][ yS ] && ( n > 0 ¦ ¦ !inferSbDcSigCoeffFlag ) && ( ( xC != LastSignificantCoeffX ¦ ¦ cu_transquant_bypass_flag ) ¦ ¦ ( yC != Last SignificantCoeffY ¦ ¦ cu_transquant_bypass_flag ) ) ) { |  |
| sig_coeff_flag[ xC ][ yC ] | ae(v) |
| remBinsPass1− − |  |
| if( sig_coeff_flag[ xC ][ yC ] ) |  |
| inferSbDcSigCoeffFlag = 0 |  |
| } |  |
| … |  |

# References

- AHG18 VTM-6.0-lossless, https://vcgit.hhi.fraunhofer.de/jvet-o-ahg-18/VVCSoftware_VTM

- B. Bross, J. Chen and S. Liu, “Versatile Video Coding (Draft 6),” JVET-O2001, 15th Meeting, Gothenburg, SE, June 2019.

- JVET-P0082 “AHG18:  BDPCM for Lossless”, A. Nalci, H. Wang, H. E. Egilmez, M. Coban, and M. Karczewicz (Qualcomm)

# Patent rights declaration

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).