| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0478-v2 |
| --- | --- |

| Title: | AHG17: On (sub)profiles signalling in the DPS |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Robert Skupin Yago Sanchez Karsten Sühring Thomas Schierl | Email: | {firstname.lastname}@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution proposes to indicate more than one profile_tier_level() syntax in the DPS as spliced bitstreams may adhere to different profiles. Especially, in view of sub-profiling, non-onion-shelled profiles are conceivable.

Revision 1 of this document contains changes according to comments received during BoG review.

# Introduction

The current VVC Specification specifies a new parameter set, namely the DPS, that describes the properties of a bitstream, i.e. by providing information beyond the CVS. This information mainly includes the profile_tier_level() to which the whole bitstream conforms.

It is not clear whether it is possible to express conformance of a bitstream with a single profile_tier_level(), particularly when more than one CVS is spliced together into a single bitstream. This is particularly worrisome in view of sub-profiles that are not defined in an onion shell manner.

# Proposal

The proposal is to extend the DPS to allow signalling more than one profile_tier_level() that need to be supported and express that the bitstream conforms to the union of all (sub-)profiles indicated into the DPS.

| decoding_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| dps_decoding_parameter_set_id | u(4) |
| dps_max_sub_layers_minus1 | u(3) |
| dps_reserved_zero_5bits | u(5) |
| dps_num_profile_tier_level_minus1 | u(4) |
| for( i = 0; i <= dps_num_profile_tier_level_minus1; i++ ) { |  |
| profile_tier_level( dps_max_sub_layers_minus1 ) |  |
| } |  |
| dps_extension_flag | u(1) |
| if( dps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| dps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

dps_num_profile_tier_level_minus1 plus 1 specifies the number of profile level tier definitions contained in the DPS. The value shall be in the range of 0 to 16.

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).