| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0479 |
| --- | --- |

| Title: | CE8-related: Restriction on context coded bins in palette |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chao-Hsiung Hung, Yung-Hsuan Chao, Marta Karczewicz | Tel: Email: | +1-858-6581293 chaohsiu@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This proposal proposes to restrict the context coded bin number in palette. The context coded bin number upper bound limit is set to 1.75 × CU height × CU width.

The results on YUV4:4:4 compared to VTM6.0 are as follows.

Dual tree OFF: xxx% AI, xxx% RA, xxx% LB

Dual tree ON: xxx% AI, xxx% RA, xxx% LB

# Introduction

Context coded bins increase hardware complexity. Some proposals [1][2][3] try to limit context coded bin number on transform skip. This proposal extends this idea to palette.

# Proposal

Palette need to code palette color predictor, color index map, run type map, run length map, and quantized escape value. Coding copy_above_indices_for_final_run_flag, run type map, and run length map uses context coded bins. This proposal restricts total number of context coded bins to 1.75 × CU height × CU width. After reaching the limit, remaining context coded bins will be bypass coded.

# Simulation Results

The proposed method is evaluated according to the CE8 description [4] for the intra-only (AI), random-access (RA) and low delay B slice (LB) configurations. The IBC, HashME, BDPCM, PLT are turned on for all Classes. The results with DualITree OFF and DualITree ON are shown in Table 1 and Table 2.

Table 1: Results of DualITree = 0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0_PLT1_IBC1_DT0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.07% | 0.05% | 0.07% | 101% | 100% |
| TGM 720p | 0.03% | 0.10% | 0.05% | 99% | 93% |
| Animation | 0.02% | 0.06% | 0.06% | 100% | 100% |
| Mixed content | 0.04% | 0.03% | 0.05% | 103% | 104% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Overall | 0.04% | 0.06% | 0.05% | 101% | 99% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-6.0_PLT1_IBC1_DT0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.11% | 0.05% | 0.17% | 101% | 101% |
| TGM 720p | 0.01% | 0.05% | 0.06% | 98% | 97% |
| Animation | 0.03% | -0.01% | 0.01% | 101% | 100% |
| Mixed content | -0.09% | 0.12% | 0.07% | 100% | 100% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Overall | 0.02% | 0.05% | 0.07% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0_PLT1_IBC1_DT0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.02% | 0.02% | 0.01% | 100% | 100% |
| TGM 720p | 0.09% | -0.03% | 0.64% | 99% | 95% |
| Animation | -0.02% | 0.02% | 0.00% | 100% | 96% |
| Mixed content | 0.16% | -0.07% | 0.01% | 100% | 97% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 103% | 103% |
| Overall | 0.04% | -0.01% | 0.16% | 100% | 98% |

Table 2: Results of DualITree = 1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0_PLT1_IBC1_DT1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.11% | 0.17% | 0.15% | 102% | 102% |
| TGM 720p | 0.05% | 0.09% | 0.14% | 99% | 95% |
| Animation | -0.01% | 0.01% | 0.01% | 99% | 98% |
| Mixed content | 0.04% | 0.08% | 0.10% | 101% | 102% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Overall | 0.05% | 0.08% | 0.09% | 100% | 99% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-6.0_PLT1_IBC1_DT1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.03% | 0.05% | 0.04% | 101% | 101% |
| TGM 720p | 0.13% | 0.10% | 0.06% | 99% | 97% |
| Animation | 0.00% | 0.00% | -0.01% | 100% | 99% |
| Mixed content | 0.07% | 0.21% | 0.12% | 100% | 100% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 102% | 102% |
| Overall | 0.05% | 0.08% | 0.05% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0_PLT1_IBC1_DT1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.08% | 0.01% | 0.05% | 96% | 94% |
| TGM 720p | 0.14% | -0.14% | 0.46% | 94% | 90% |
| Animation | 0.03% | 0.09% | 0.05% | 99% | 98% |
| Mixed content | -0.29% | -0.19% | -0.09% | 99% | 100% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 101% | 99% |
| Overall | 0.01% | -0.05% | 0.12% | 97% | 96% |

# Reference

- S. Yoo, J. Choi, J. Heo, J. Choi, L. Li, J. Lim, S. Kim, “Non-CE7 : Sign coding for transform skip”, JVET-M0279, 13th meeting, Marrakech, MC, January 2019.

- B. Bross, T. Nguyen, P. Keydel, H. Schwarz, D. Marpe, T. Wiegand, “Non-CE8: Unified Transform Type Signalling and Residual Coding for Transform Skip”, JVET-M0464, 13th meeting, Marrakech, MC, January 2019.

- B. Bross, T. Nguyen, H. Schwarz, D. Marpe, T. Wiegand, “CE8: Residual Coding for Transform Skip Mode (CE8-4.3a, CE8-4.3b, CE8-4.4a, and CE8-4.4b)”, JVET-N0280, 14th meeting, Geneva, CH, March 2019.

- Y.-H. Chao, C.-H. Hung, W.-J. Chien, T. Hsieh, and M. Karczewicz, “CE8-2.4: Line-based CG Palette Mode”, JVET-O0120, 15th meeting, Gothenburg, SE, July 2019.

- X. Xu, Y.-H. Chao, Y.-C. Sun, J. Xu, “Description of Core Experiment 8 (CE8): 4:4:4 Screen Content Coding Tools”, JVET-O2028, 15th meeting, Gothenburg, SE, July 2019.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).